| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0157 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0046 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On DPB - to share or not to share, that is the question |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang Ying Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

A sub-DPB in an SHVC/MV-HEVC decoder is shared by all layers that have the same representation format indicated in the VPS. This document asserts that sharing of sub-DPBs may result in decoded picture buffer overflow when bitstream down-switching or bitstream thinning (discarding of individual or a set of discardable pictures) occurs. Several possible solutions, with none asserted to be seemingly satisfying, are presented. Another discussion of the question on whether to share or not to share seems needed, and a final decision on this is suggested to be made at this meeting.

# Problem description

In the last meeting, a design choice was made such that the DPB in SHVC/MV-HEVC is partitioned into sub-DPBs, where picture storage buffers in a sub-DPB contain the decoded pictures of the same resolution, colour format, and bit depth (collectively called the representation format) as signalled in the Video Parameter Set (VPS). In other words, decoded pictures of the same representation format, assuming they are the same as signalled in the VPS, share the same sub-DPB.

When decoded pictures of different layers share the same sub-DPB, if some coded pictures are removed from the bitstream, e.g. as a result of when bitstream down-switching or bitstream thinning, decoding of the resultant bitstream could lead to (decoded picture) buffer overflow. Even though such bitstreams may be non-conforming, it would be in the interest of widespread use of the standard and it has been a design principle followed by the JCT-VC and the JCT-3V that the design supports such use cases.

The following use-case scenarios describe a non-exhaustive list of cases where such overflow could occur. The basic problem that is underlying all these examples is the following: a set of decoded pictures D of a particular layer L1 would have been removed from the sub-DPB if a particular coded picture X were not removed from the bitstream (e.g. due to down-switching, discarding, or loss). When a bitstream wherein the optimal sub-DPB memory size is indicated is decoded by a decoder that uses the minimum required sub-DPB memory size, another layer L2 sharing the same sub-DPB might have expected these decoded pictures in D to have been removed from the sub-DPB and would thus try to utilize the corresponding picture storage buffers. In this case, when the coded picture X is absent from the bitstream, decoded pictures in D are not removed and resultant decoding could result in buffer overflow (when there is a decoded picture to be stored into the sub-DPB but the sub-DPB is full).

## Down-switching scenario

Consider the bitstream given in Figure 1. The base layer (BL) and the enhancement layer (EL) share the same sub-DPB, and when the coded pictures are all present in the bitstream, a sub-DPB size of 6 is sufficient to decode the two layers. Details of the prediction structures are given in the figure. Note that arrows indicate prediction relationship, with the head pointing to the reference picture.

Figure 1 – Example of a bitstream before down-switching

Suppose that the bitstream obtained by the decoder is a down-switched bitstream from the bitstream illustrated in Figure 1, as shown in Figure 2. The first EL IDR picture B whose decoding would have removed the three picture buffers is not present in the bitstream, consequently the decoded pictures E, F and G linger in the sub-DPB. At the start of decoding picture A, the sub-DPB contains six decoded pictures and is therefore full, therefore buffer overflow occurs as there is no place to store the decoded picture A.

Figure 2 – Example of buffer overflow resulting from removing pictures in EL in down-switching scenario

## Single-loop like decoding scenario

Consider the case where we have a two-layer bitstream where the two layers share the same sub-DPB and the EL is coded in single-loop-like fashion. Only the IRAP pictures in the BL are used for inter-layer prediction. The IRAP pictures (more specifically, CRA pictures) are not aligned in the bitstream, as shown in Figure 3.

Figure 3 – Example of original bitstream before removal of non-IRAP picture in the BL

Suppose that the non-IRAP BL pictures (which are not used for inter-layer prediction) are removed from the bitstream after a particular access unit in order to reduce the bitrate. The resultant bitstream is shown in Figure 4. Once again, because the picture buffers that would have been emptied by picture B are not emptied, a buffer overflow would occur while decoding picture A.

Figure 4 – Example of buffer overflow problem resulting from removing non-IRAP picture in BL

## Removal of discardable pictures scenario

Removal of discardable pictures can also result in buffer overflow in some cases. Consider the example in Figure 5, where the sub-DPB size is 5. Picture 6 in the enhancement layer is marked as a discardable picture, as it is not used for reference by any other picture. Picture 6 also contains an empty temporal short-term reference picture set (RPS), and thus when the decoder decodes picture 6, pictures 2 and 4 are marked as unused for reference.

Figure 5 – Example of original bitstream before individual discardable picture is removed

When a middle box or other entities removes the discardable picture 6, and the bitstream (as shown in Figure 6) is sent to the decoder, a buffer overflow occurs while decoding picture 7. Since picture 6 is not present, pictures 2 and 4 are not marked as unused for reference when picture 7 is being decoded. Hence six picture storage buffers are required, thus resulting in an overflow.

Figure 6 -- Example of buffer overflow problem resulting from removing discardable picture

# Possible solutions

## Signalling of additional RPS

The buffer overflows in the examples given in Section Error: Reference source not found are the result of lack of timely reference picture set update. In Figure 2, if the RPS update occurred in the order in which picture B would have been decoded, the buffer overflow would not have occurred.

Additional signalling of the RPS of a picture in a particular layer could solve this problem. This information can be signalled in SEI message (described in 4.1.2) or in slice header extension of pictures that present in the access unit containing the picture (described in 4.1.3). The syntax and the semantics of the RPS information are given below.

### RPS information syntax

| rps_information( ) { | Descriptor |
| --- | --- |
| layer_rps_info_id | u(6) |
| active_sps_id | ue(v) |
| st_rps_from_sps_flag | u(1) |
| if( st_rps_from_sps_flag ) |  |
| st_rps_from_sps_idx | ue(v) |
| else |  |
| st_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| num_rps_info_long_term_pics | ue(v) |
| for( i = 0; i < num_rps_info_long_term_pics; i++ ) { |  |
| poc_rps_info_lsb_lt[ i ] | u(v) |
| used_by_curr_rps_info_pic_lt_flag[ i ] | u(1) |
| delta_poc_msb_ present_rps_info_flag[ i ] | u(1) |
| if( delta_poc_msb_present_rps_info_flag[ i ] ) |  |
| delta_poc_msb_cycle_rps_info_lt[ i ] | ue(v) |
| } |  |
| } |  |

### Variant 1: carriage of RPS information in SEI message

| rps_info( payloadSize ) { | Descriptor |
| --- | --- |
| rps_information( ) |  |
| } |  |

RPS information semantics

The RPS information syntax structure provides the information that can be used to derive the RPS of the picture that is contained in the same access unit as the SEI message and that has nuh_layer_id equal to layer_rps_info_id. When the access unit containing the SEI message does not contain a picture with nuh_layer_id equal to layer_rps_info_id, the decoder can use the information contained in the SEI message to update the RPS and thus the decoded picture buffer associated with the layer with nuh_layer_id equal to layer_rps_info_id.

The reference pictures indicated by the RPS information may or may not present in the DPB and no action is required when the indicated reference pictures are not present in the DPB. Reference pictures with nuh_layer_id equal to layer_rps_info_id and are not indicated by the RPS information shall be marked as "unused for reference".

layer_rps_info_id specifies the nuh_layer_id of the picture that is in the access unit containing the SEI message and for which the SEI message provides information about the RPS.

active_sps_id identifies the active SPS of the layer with nuh_layer_id equal to layer_rps_info_id which may be used to derive the RPS information specified by st_rps_from_sps_idx.

The syntax structure of st_ref_pic_set( ) is the same as short_term_ref_pic_set( stRpsIdx ) with stRpsIdx being equal to num_short_term_ref_pic_sets in the active SPS.

The semantics of the syntax elements st_rps_from_sps_flag , st_rps_from_sps_idx, num_rps_info_long_term_pics[ i ], poc_rps_info_lsb_lt[ i ], used_by_curr_rps_info_pic_lt_flag[ i ], delta_poc_msb_present_rps_info_flag[ i ] and delta_poc_msb_cycle_rps_info_lt[ i ] are similar to the semantics of short_term_ref_pic_set_sps_flag , short_term_ref_pic_set_idx, num_long_term_pics, poc_lsb_lt[ i ], used_by_curr_pic_lt_flag[ i ], delta_poc_msb_present_flag[ i ], and delta_poc_msb_cycle_lt[ i ], and they are used to indicate the RPS information of the picture in the current access unit with nuh_layer_id equal to layer_rps_info_id.

### Variant 2: carriage of RPS information in slice segment header extension

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| … |  |
| num_rps_information | ue(v) |
| for( i = 0; i < num_rps_information; i++ ) |  |
| rps_information( ) |  |
| while( more_data_in_slice_segment_header_extension( ) ) |  |
| slice_segment_header_extension_data_bit | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

RPS information semantics

The RPS information provides the information required to derive the RPS of the picture that is contained in the same access unit as the current picture and that has nuh_layer_id equal to layer_rps_info_id. When the access unit containing the current picture does not contain a picture with nuh_layer_id equal to layer_rps_info_id, the decoder can use the RPS information to update the RPS and thus the decoded picture buffer associated with the layer with nuh_layer_id equal to layer_rps_info_id.

The reference pictures indicated by the RPS information may or may not present in the DPB and decoder is not required to take action when the indicated reference pictures are not present in the DPB. Reference pictures with nuh_layer_id equal to layer_rps_info_id and are not indicated by the RPS information shall be marked as "unused for reference"

num_rps_information specifies the number of RPS information entries that are signalled in the current slice.

layer_rps_info_id specifies the nuh_layer_id of the picture that is in the access unit containing the current picture.

The syntax structure of st_ref_pic_set( ) is the same as short_term_ref_pic_set( stRpsIdx ) with stRpsIdx being equal to num_short_term_ref_pic_sets in the active SPS.

## Buffer overflow indication

In many typical bitstreams that have a regular coding structure, the buffer overflow would most likely not occur. It would be useful for a middle box/network entity to know whether operations such as those described in Section Error: Reference source not found would cause buffer overflow. Such indication can be described as follow:

partial_bitstream_buffer_overflow equal to 0 specifies that any subset of a bitstream that is obtained by removing from a conforming bitstream those pictures that would not be used to predict other pictures in the resultant subset bitstream would not result in a buffer overflow when decoded. partial_bitstream_buffer_overflow equal to 1 specifies that any subset of a bitstream that is obtained by removing from a conforming bitstream those pictures that would not be used to predict other pictures in the resultant subset bitstream may or may not result in a buffer overflow when decoded. When not present, the value of partial_bitstream_buffer_overflow is inferred to be equal to 1.

The above syntax element would be signalled in the VPS VUI.

## Indication to allow removal of the oldest reference picture

In many typical bitstreams, except for long-term reference pictures, reference pictures that are selected for inter prediction are those that are closer to the picture being decoded. To overcome the above identified problems, the solution presented in 2.2 can have further semantic such whether the earliest short-term reference pictures shall be removed from the sub-DPB when buffer overflow happens.

partial_bitstream_buffer_overflow equal to 0 specifies that any subset of a bitstream that is obtained from a conforming bitstream shall not result in a buffer overflow when decoded. partial_bitstream_buffer_overflow equal to 1 specifies that any subset of a bitstream that is obtained from a conforming bitstream may or may not result in a buffer overflow when decoded. When partial_bitstream_buffer_overflow is equal to 1 and the number of pictures in the sub-DPB is greater than or equal to the size of the sub-DPB the short-term reference pictures that are in the sub-DPB assigned for the layer of the current picture with the smallest POC and the highest layerId value shall be marked as "unused for reference". When not present, the value of partial_bitstream_buffer_overflow is inferred to be equal to 1.

The changes to subclause 8.3.2 are highlighted below in yellow.

The derivation process for the RPS and picture marking are performed according to the following ordered steps:

…

- All reference pictures in the DPB that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetStFoll and have nuh_layer_id equal to currPicLayerId are marked as "unused for reference".

- When the value of partial_bitstream_buffer_overflow is equal to 1 and the number of reference pictures in the DPB is greater than or equal to MaxDecPicBufferingMinus1 + 1, the reference picture in the DPB marked as "needed for short-term reference" and has smallest POC and has the highest layerId is marked as "unused for reference".

## Reverting the shared sub-DPB decision back to layer-specific sub-DPB

If there is no good mechanism that can be agreed to solve the buffer overflow problem in shared sub-DPB mechanism, the worst-case scenario would be to revert the decision of allowing sharing of sub-DPB for layers that have the same representation format back to layer-specific sub-DPB.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).