| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0159r1 |
| --- | --- |

| Title: | AHG 13: Sub-region extraction – position calculation and comparison of different approaches |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto Tomohiro Ikai Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution firstly reports an analysis comparing different approaches to realize sub-region extraction with SHVC. Based on the analysis, it is proposed to support sub-region extraction with extracting both enhance- and base-layer data by 1) adding new syntax in SPS extension and 2) modifying scale and reference pixel location derivation. The proposed changes are asserted to enhance the application scenarios using sub-region extraction.

In revision 1, experimental results of applying the proposed method for 520p to 720p scalability is added. The experimental results assertedly indicate that the proposed method can achieve the performance gain equivalent to those reported in JCTVC-Q0104.

# Introduction

To date, several contributions have discussed about the applications supporting region of interest (ROI) in HEVC or SHVC. Extracting sub region corresponding to tiles or tile sets is a key functionality that accomplishes such ROI applications. In the previous meeting, a method is proposed to signal additional information, to keep the exact pixel correspondence between layers in the bitstream generated by the sub-region extraction [1].

During the discussion, it was generally understood that the proposed approach should be correct in terms of inter-layer filtering processing. However, concerns were raised regarding the necessity of supporting the exact pixel correspondence. A particular concern was that the other approaches may also realize sub-region extraction scenario without base-layer extraction.

To address the concern, chapter 2 of this contribution provides comparison among several approaches to realize sub-region extraction scenario with SHVC. Chapter 3 states the proposed syntax for the pixel correspondence and pixel calculation. (The proposed changes are the same as the method in JCTVC-P0049).

# Comparison of SHVC sub-region extraction approaches

The SHVC sub-region extraction is to rewrite or transcode original SHVC bitstream for full region to generate new SHVC bitstream for sub-region. Fig. 1 illustrates four different approaches for SHVC sub-region extraction.

(A) Drop EL/BL tiles: Both EL (Enhancement Layer) and BL (Base Layer) are encoded with multiple tiles. In the sub-region extraction (rewriting), the tiles outside of ROI are disposed for both EL and BL. Note that this approach can be further categorized by whether the exact phase and scale alignment are supported.

(B) Drop EL tiles, Empty BL tiles: Both EL and BL are encoded with multiple tiles. In rewriting, the tiles outside of ROI are disposed for EL. The tiles outside of ROI in BL are emptied by replacing existing slice data with the slice data containing minimal data.

(C) Drop EL tiles: Only EL is encoded with multiple tiles. In rewriting, the tiles outside of ROI are disposed for EL. No change in BL.

(D) Re-encoding: No tiles need to be used for both EL and BL. In rewriting, the whole EL and BL are fully decoded and then ROI region is cropped and re-encoded.

Fig. 1: Approaches for SHVC sub-region extraction

Fig. 2 summarizes the comparison among different approaches in terms of bitrate, rewriting complexity, and decoding complexity.

Bitrate (full-region): Encoding with multiple tiles reduces coding efficiency. To keep the same quality, approach A, B, and C might require higher bitrate.

Rewriting complexity: Approach D is very complex in terms of computing and requires full decoding and re-encoding implementation. Approach A and C are relatively easy for both computation and implementation. They just require discarding some slice data and modifying header/parameter set. Approach B is a bit complex compared to A and C since it requires encoding empty slice data, which requires cabac imitation implementation.

Bitrate (sub-region): Approach D could achieve the smallest bitstream. Approach A and B causes increased bitrate due to the tile division. Approach C results in the highest bitrate because the whole BL data remains.

Decoding complexity: Approach D has the minimal complexity for the decoder that only decode sub region since it only requires decoding of extracted regions. Approach A achieves the second minimal complexity next to approach D, which needs a couple of additional add in the resampling process if exact phase/scale position derivation is utilized. Approach B is relatively complex compared to approach D since it requires full decoding of BL although some CTU are empty CTUs. Approach C is much complex since it requires decoding of whole BL irrespective of the extracted ROI.

Fig. 2: Comparison of different SHVC sub-region extraction approaches

Bitrate numbers in Fig. 2 are calculated with several assumptions as shown in right-bottom in Fig. 2.

- Ratio of EL bitrate relative to BL bitrate is set to 200% based on the results of SHM-4.0 anchor results.

- Bitrate increase by tiling is set to 3% for EL and 9% for BL based on the experimental results which encode HD sequences with 4x3 independent tiles for both EL and BL with 2x random access configuration.

- Sub-region area is set to 1/3 of the full-region. It roughly corresponds to 720p in 4K frame.

With the above assumptions, the bitrate value in Fig 2. were calculated by the following equations.

- Variables

  - rBL,full : bitrate of BL full region

  - c : ratio of EL bitrate relative to bitrate of corresponding BL region

  - tEL, tBL : ratio of tiled EL/BL bitrate relative to non-tiled EL/BL

  - bEL, bBL : ratio of extracted region relative to full region (ratio in area)

- rAL,full : Bitrate (full-region, EL+BL)

  - rAL,full = rBL,full (c・tEL + tBL )

- rAL,sub : Bitrate (sub-region, EL+BL)

  - rAL,sub = rBL,full (c・tEL・bEL + tBL・bBL )

From the comparison, it can be safe to say the winner depends on the application. Therefore, it is preferable to support all approaches and to give the user an opportunity to select the one best fit for their application. In our perspective, approach A (Drop EL/BL tiles) is attractive since rewriting and decoding complexity are low in terms of both computation and implementation while the bitrate of sub-region bitstream is also small compared with other appoachs.

# Proposal

(Note: This chapter contains the same contents as in JCTVC-P0049 discussed in San Jose meeting.)

In the SHVC draft spec, the scale factor (ScaleFactorX and ScaleFactorY) is derived based on the ratio of the reference-layer picture size and the scaled reference-layer picture size. If either of the sizes is changed in the extraction, the scale factor may not be kept exactly the same because the precisions of those sizes are restricted (i.e. the reference picture size is multiple of minimum luma CTB size and the scaled reference picture size is in units of two-luma pixels).

To achieve scale and phase match in the case of sub-region extraction, where both EL and BL tiles are extracted, we propose additional syntax and reference position derivation process. Conceptually, the reference and scaled reference picture sizes before extraction are signalled using the proposed syntax. Fig 1 depicts relations among extracted sub-region and additional parameters.

Fig. 1: Relations among extracted region and proposed parameters

Our proposal includes following modifications.

- new syntax to signal

A) offset between top-left pixels of reference picture before and after extraction (cf. RLOffsetTL in Fig. 1), and

B) offset between bottom-right pixels of reference picture before and after extraction (cf. RLOffsetBR in Fig. 1)

- modified calculation of scale factor and reference position

- change semantics of scaled reference layer offset parmeters – top-left pixel of scaled reference picture now corresponds to the position which is 'top-left pixel of reference picture' plus RLOffsetTL.

Proposed syntax for those parameters are added in sps_multilayer_extension() as shown in Table 1. The additional parameters can be signalled selectively for specific reference layers using the similar syntax as scaled reference layer offset information.

Table 1: Proposed additional syntax in SPS extension

| sps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_id[ i ] | u(6) |
| scaled_ref_layer_left_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_right_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_bottom_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| vert_phase_position_enable_flag[ scaled_ref_layer_id[ i ] ] | u(1) |
| } |  |
| num_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_ref_layer_offsets; i++) { |  |
| ref_layer_id[ i ] | u(6) |
| ref_layer_left_offset[ ref_layer_id[ i ] ] | se(v) |
| ref_layer_top_offset[ ref_layer_id[ i ] ] | se(v) |
| ref_layer_right_offset[ ref_layer_id[ i ] ] | se(v) |
| ref_layer_bottom_offset[ ref_layer_id[ i ] ] | se(v) |
| } |  |
| } |  |

Reference layer offset variables are derived as follows.

RefLayerLeftOffset  = ref_layer_left_offset[ rLId ] << 1;

RefLayerRightOffset = ref_layer_right_offset[ rLId ] << 1;

RefLayerTopOffset = ref_layer_top_offset[ rLId ] << 1;

RefLayerBottomOffset = ref_layer_bottom_offset[ rLId ] << 1;

Scale factors are calculated with the following equations.

AltRefLayerPicWidthInSamplesY =	 
	RefLayerPicWidthInSamplesY ‒ RefLayerLeftOffset ‒ RefLayerRightOffset

AltRefLayerPicHightInSamplesY =	
	RefLayerPicHightInSamplesY ‒ RefLayerTopOffset ‒ RefLayerBottomOffset

ScaleFactorX = ( ( AltRefLayerPicWidthInSamplesY << 16 )
	+ ( ScaledRefLayerPicWidthInSamplesY >> 1 ) ) / ScaledRefLayerPicWidthInSamplesY

ScaleFactorY = ( ( AltRefLayerPicHeightInSamplesY << 16 )
	+ ( ScaledRefLayerPicHeightInSamplesY >> 1 ) ) / ScaledRefLayerPicHeightInSamplesY

When not present, all reference layer offset values are inferred to be 0 and alternative reference layer picture size parameters are inferred to be equal to be the size of the reference picture indicated by ref_layer_id[i]. Therefore, when those offsets are not signalled, above equations result in the same scale factors as in the current draft.

Reference pixel location in 1/16 pixel accuracy (xRef16 and yRef16) are derived with the following equations.

The variables xRef16 and yRef16 are derived as follows:

Here variables offsetX and offsetY are derived as follows:

offsetXRef = ( RefLayerLeftOffset << 4 )/ ( ( cIdx = = 0)  ?  1 :  SubWidthC)	
offsetYRef = ( RefLayerTopOffset << 4 )/ ( ( cIdx = = 0)  ?  1 :  SubHeightC)

xRef16 = ( ( ( xP – offsetX ) * ScaleFactorX  + addX + ( 1 << 11 ) ) >> 12 ) – ( phaseX << 2 )
        + offsetXRef
yRef16 = ( ( ( yP – offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) – ( phaseY << 2 )
        + offsetYRef

Note that the complexity increase of proposed changes is minor because scale factors and reference layer offset variables are constant in a picture.

With the above scale and reference pixel location calculation, the same prediction values can be derived by setting the appropriate scaled reference layer offset values and reference layer offset values.

# Experimental results on 540p to 720p scalability

JCTVC-Q0104 raised the problem of inter-layer prediction for the case of 540p to 720p spatial scalability. The problem stems the fact that 540p base layer is encoded as 544p picture due to the limitation of picture size (ie. should be multiple of 8). In that sense, it can be seen as extracting 540p base layer from 544p base layer. Therefore, the problem can be handled with the proposed method in this contibution by setting a reference layer bottom offset to -4 (ie. set ref_layer_bottom_offset to -2).

The proposed method is implemented on SHM-5.0 and experiments have been conducted for 540p to 720p spatial scalability. The resutls are shown in Table 2 and the xls sheet is attached. The results indicates that the proposed method can achieve almost the same gain as the method proposed in Q0104.

Table 2: Result of the proposed method for 540p to 720p scalability

# Conclusion

This contribution reports an analysis comparing different approaches to realize sub-region extraction with SHVC. This contribution also proposes the new syntax in SPS extension and modification of scale and reference pixel location derivation. It is recommended to review comparison of different approaches and understand there are some supporting results for EL and BL extracting approach. It is also recommended to adopt the proposed syntax and reference pixel location derivation in SHVC.

# Reference

[1] T. Yamamoto, T. Tsukuba, T. Ikai, E. Alshina "JCTVC-P0049: AHG 13: Scale and reference position derivation for sub-region extraction," San José, US, 9–17 Jan. 2014

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).