| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0161 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0049 |
| --- | --- |

| Title: | Layer association of auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

Currently primary and auxiliary picture association is specified by the equality of ScalabilityId. Under this definition, one auxiliary picture could be associated with several primary pictures if there are several layers which have the same ScalabilityId. This characteristics are thought to be useful for auxiliary picture sharing. However this definition also can bring about non-intentional association in multiple dependent tree case. To address this problem, Proposal 1 is to define new ScalabilityId, named UserId, to distinguish the primary picture which has the same ScalabilityId.

The other problem is that, when ScalabilityId is different, the auxiliary picture sharing cannot be achieved, e.g. normal resolution base layer and high resolution enhancement layer cannot share one common auxiliary picture. To address this problem, Proposal 2 is to add a new flag to control whether a specific ScalabilityId is used or not for primary picture association.

# Introduction

Distinguish auxiliary picture association among layers with the same ScalabilityId

A problem arises in the case of multiple dependent trees as shown Fig 1. In which, one or more primary pictures and auxiliary pictures have the same ScalabilityId, but the intentional association cannot be distinguished by the existing ScalableId.  The problem could be more critical and confusing if we allow multiple primary picture associations for one auxiliary picture as shown Fig 2.

Fig 1. Misassociation in multiple dependent tree case

Fig 2. Confusing association

Auxiliary picture sharing

Currently WD specifies a “one” primary picture is associated with the auxiliary picture. However we should relax this restriction to allow several primary pictures to have a common auxiliary picture considering the following use cases.

- In SNR or spatial scalability case, the base layer and the enhanced layer is supposed to be associated with the same alpha plane.

- In stereo case, the base view (left) and the enhance view (right) may have the common depth view (center).

Since the ScalableId is different in above use case, it is also inevitable to allow auxiliary picture association between different ScalabilityId.

Fig 3. Not supported auxiliary picture sharing (in SHVC use case)

# Proposal

The proposal consists of two parts:

Proposal 1

- Add new scalability dimension, named “UserId”, to distinguish a layer among layers which have the same ScalabilityId[ i ][ j ].

UserId helps disinguishing the associated primary picture. Without UserId, it could be difficult to select associated primary pictures because they can be plural.

Note: The name UserId is one suggestion. OrderId or something is also ok.

Proposal 2

- Add a  flag, called aux_mask_flag [j], for ScalabilityId[ ][ j ] to enable the use case that layers which have different ScalabilityId[ i ][ j ] share one auxiliary layer.

With proposal 1 and 2, the problems shown in Fig.1 and Fig.2 are solved as Fig 4.

With proposal 2, the problem shown in Fig. 3 is solved as Fig 5.

Fig. 4 Streamlined layer association in multiple dependent tree case with proposal 1

Fig. 5 Achieved intended association with proposal 1

Fig. 6 Supported auxiliary picture sharing with proposal 2

# Text of Proposal 1

The text change is highlighted by Yellow.

F.2.4.3.1.1 Video parameter set extension semantics

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table Error: Reference source not found-1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table Error: Reference source not found-1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Multiview | View Order Index |
| 2 | Spatial/quality scalability | DependencyId |
| 3 | Auxiliary | AuxId |
| 4-14 | Reserved |  |
| 15 | User | UserId |

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

…

AuxId[ lId ] shall be in the range of 0 to 2, inclusive, or 128 to 143, inclusive, for bitstreams conforming to this version of this Specification. Although the value of AuxId[ lId ] shall be in the range of 0 to 2, inclusive, or 128 to 143, inclusive, in this version of this Specification, decoders shall allow values of AuxId[ lId ] in the range of 0 to 255, inclusive.

For an auxiliary picture with nuh_layer_id equal to nuhLayerIdA, one or morean associated primary pictures, if any, isare the pictures in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ nuhLayerIdA ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

# Text of Proposal 2

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| if (scalability_mask_flag[ 3]) |  |
| for( j = 0; j < NumScalabilityTypes ; j++ ) |  |
| if (scalability_mask_flag [j] && j != 3) |  |
| aux_mask_flag[ j ] | u(1) |

aux_mask_flag[ j ] equal to 1 specify that the primary picture association with nuh_layer_id is specified by the equality of ScalabilityId[  ][ j ]. aux_mask_flag[ j ] equal to 0 specify that auxiliary picture association is not specified by the equality of ScalabilityId[  ][ j ]. When not present, aux_mask_flag[ j ] is inferred to be 0.

For an auxiliary picture with nuh_layer_id equal to nuhLayerIdA, one or morean associated primary pictures, if any, isare the picture in the same access unit having AuxId[ nuhLayerIdB ] equal to 0 such that ScalabilityId[ LayerIdxInVps[ nuhLayerIdA ] ][ j ] is equal to ScalabilityId[ LayerIdxInVps[ nuhLayerIdB ] ][ j ] and aux_mask_flag[j] is equal to 1 for all values of j in the range of 0 to 2, inclusive, and 4 to 15, inclusive.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).