| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0164 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0052 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On Sub-bitstream property SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to add new sub bitstream mode in Sub-bitstream property SEI to support signalling bitrate information of only output layers in output layer set. The new sub bitstream mode enable signalling bitrate of enhacement-layer only layer bunch.

# Introduction

It has been asserted in JCTVC-P0045 that enhancement layer (EL) only layer set is important taken into account of realistic use case, where several dependent trees, e.g {0, 1, 2} and {3,4,5}, exist in bitstream, and decoder just decodes and plays one layer tree not-having base layer (e.g. {3,4,5}). There argued that a way should be provided to enable HEVC version1 decoder should understand bitstream property by SPS and decide to decode or skip the extracted bistream. The authors have understood that is no easy way and that

EL only layer set was not allowed, or layer set would always include layer 0 (e,g {0,3,4,5}).

In this case, bitrate information in VPS also includes the bitrate of layer 0. Considering this situation, we should provide, a capability of sending EL only bitrate information and think  that Sub-bitstream property SEI takes on this capability by adding a sub-bitstream extraction mode.

# Proposal

It is proposed to add an additional sub-bitstream extraction mode to signal EL bitrate information:

The mode is to

- Remove all VCL-NAL units with output_flag [ nuh_layer_id] equal to 0 and with nuh_layer_id values not among the nuh_layer_id values of the direct or indirect reference layers of the target output layers which have nul_layer_id values included in TargetOptLayerIdList.

With this extraction mode, in the case that output layer set is {0,3,4,5} with output_flag {0,1,1,1}, layer 0 is removed So the SEI can signal bitrare for actual use case in which practical decoder (not HRD decoder) only decodes necessary layers, e.g. {3, 4, 5}.

- F.14.1.6 Sub-bitstream property SEI message syntax

| sub_bitstream_property( payloadSize ) { | Descriptor |
| --- | --- |
| active_vps_id | u(4) |
| num_additional_sub_streams_minus1 | ue(v) |
| for( i = 0; i <= num_additional_sub_streams_minus1; i++) { |  |
| sub_bitstream_mode[ i ] | u(3) |
| output_layer_set_idx_to_vps[ i ] | ue(v) |
| highest_sublayer_id[ i ] | u(3) |
| avg_bit_rate[ i ] | u(16) |
| max_bit_rate[ i ] | u(16) |
| } |  |
| } |  |

- F.14.2.6 Sub-bitstream property SEI message semantics

The sub-bitstream property SEI message, when present, provides the bit rate information for a sub-bitstream created by discarding those pictures in the layers that do not belong to the output layers of the output layer sets specified by the active VPS and that do not affect the decoding of the output layers.

When present, the sub-bitstream property SEI message shall be associated with an initial IRAP access unit, and the information provided by the SEI messages applies to the bitstream corresponding to the CVS containing the associated initial IRAP access unit.

active_vps_id identifies the active VPS. The value of active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS referred to by the VCL NAL units of the associated access unit.

num_additional_sub_streams_minus1 plus 1 specifies the number of the sub-bitstreams for which the bit rate information may be provided by this SEI message. The value of num_additional_sub_streams_minus1 shall be in the range of 0 to 210− 1, inclusive.

sub_bitstream_mode[ i ] specifies how the i-th sub-bitstream is generated. The value of sub_bitstream_mode[ i ] shall be in the range of 0 to 2, inclusive. The values 3 to 7 are reserved for future use by ITU-T and ISO/IEC. When sub_bitstream_mode[ i ] is the greater than 1, decoders shall ignore the syntax elements output_layer_set_idx_to_vps[ i ], highest_sublayer_id[ i ], avg_bit_rate[ i ], and max_bit_rate[ i ].

When sub_bitstream_mode[ i ] is equal to 0, the i-th sub-bitstream is generated as specified by the following steps:

–	The sub-bitstream extraction process as specified in clause 10 is invoked with the bitstream corresponding to the CVS containing the sub-bitstream property SEI message, highest_sublayer_id[ i ], and LayerSetLayerIdList[ LayerSetIdxForOutputLayerSet[ output_layer_set_idx_to_vps[ i ] ] ] as inputs.

–	Remove all NAL units for which the nuh_layer_id is not included in TargetOptLayerIdList and either of the following conditions is true:

–	The value of nal_unit_type is not in the range of BLA_W_LP to RSV_IRAP_VCL23, inclusive, and max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ][LayerIdxInVps[ layerId ] ] is equal to 0 for layerId values included in TargetOptLayerIdList.

–	TemporalId is greater than the maximum value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ][LayerIdxInVps[ layerId ] ] − 1 for all layerId values included in TargetOptLayerIdList.

When sub_bitstream_mode[ i ] is equal to 1, the i-th sub-bitstream is generated as specified by the above steps followed by:

–	Remove all NAL units with nuh_layer_id not among the values included in TargetOptLayerIdList and with discardable_flag equal to 1.

When sub_bitstream_mode[ i ] is equal to 2, the i-th sub-bitstream is generated as specified by the following steps:

–	The sub-bitstream extraction process as specified in clause 10 is invoked with the bitstream corresponding to the CVS containing the sub-bitstream property SEI message, highest_sublayer_id[ i ], and LayerSetLayerIdList[ LayerSetIdxForOutputLayerSet[ output_layer_set_idx_to_vps[ i ] ] ] as inputs.

–	Remove all VCL-NAL units with output_flag [ nuh_layer_id] equal to 0 and with nuh_layer_id values not among the nuh_layer_id values of the direct or indirect reference layers of the target output layers which have nul_layer_id values included in TargetOptLayerIdList.

output_layer_set_idx_to_vps[ i ] specifies the index of the output layer set corresponding to the i-th sub-bitstream.

highest_sublayer_id[ i ] specifies the highest TemporalId of access units in the i-th sub-bitstream.

avg_bit_rate[ i ] indicates the average bit rate of the i-th sub-bitstream, in bits per second. The value is given by BitRateBPS( avg_bit_rate[ i ] ) with the function BitRateBPS( ) being specified as follows:

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x  >>  14 ) )		(Error: Reference source not found-1)

The average bit rate is derived according to the access unit removal time specified in clause Error: Reference source not found. In the following, bTotal is the number of bits in all NAL units of the i-th sub-bitstream, t1 is the removal time (in seconds) of the first access unit to which the VPS applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies. With x specifying the value of avg_bit_rate[ i ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) ) )	(Error: Reference source not found-2)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0		(Error: Reference source not found-3)

max_bit_rate[ i ] indicates an upper bound for the bit rate of the i-th sub-bitstream in any one-second time window of access unit removal time as specified in clause Error: Reference source not found. The upper bound for the bit rate in bits per second is given by BitRateBPS( max_bit_rate[ i ] ). The bit rate values are derived according to the access unit removal time specified in clause Error: Reference source not found. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_bit_rate[ i ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) )	(Error: Reference source not found-4)

# Patent rights declaration

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] Y. Chen, Y.-K. Wang, A. K. Ramasubramonian, Hendry, MV-HEVC/SHVC HLS: Sub-bitstream property SEI message, JCTVC-P0204, JCT-VC 16th Meeting: San José, US, 9–17 Jan. 2014

[2] T. Ikai, T. Tsukuba, T. Yamamoto, “MV-HEVC/SHVC HLS: On layer set definition”, JCTVC-P0045, JCT-VC 16th Meeting: San José, US, 9–17 Jan. 2014