| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0168 |
| --- | --- |

| Title: | On phase offset for resampling process in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Krishna Rapaka Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 651 8028 cjianle@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an alternative signalling method for phase offset used in resampling process.

# Proposed phase offset signalling

In the current SHVC draft, a VPS flag (cross_layer_phase_alignment_flag) is used to indicate whether the sample grids are top-left-aligned or center-aligned among different layers in downsampling such that the phase derivation process used in upsampling can be matched with that used in downsampling. In addition to this flag, the vertical phase of upsampling process can be further adjusted with additional flags signalled in the bitstream for efficient inter-layer prediction of interlaced-to-progressive scalability. Altogether, those flags are used to derive the variables phaseX and phaseY as follows, which are used for reference sample location in upsampling process.

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag

phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : ( ( cIdx  = =  0 ) ?
	( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)

In this contribution, instead of deriving the value of phaseX and phaseY by above equation at decoder side，it’s proposed to signal the value of phase offset in the bitstream directly. Four syntax elements, hor_phase_luma, hor_phase_chroma, ver_phase_luma and ver_phase_chroma, which represent horizontal luma phase offset, horizontal chroma phase offset, vertical luma phase offset and vertical chroma phase offset, are signalled.

With the proposed method, the encoder can choose the appropriate phase offset according to the downsampling mechanism and scalability type. The phase offset syntaxes are signaled in VPS for each direct dependent layer and at most two sets of phase offset are signaled for one reference layer. When two sets of of phase offset are signalled for a reference layer, a flag is signaled in slice segment header to indicate which one is used to resample the reference layer picture.

# Spec text

The newly added parts are highlighted in green and the deleted parts are marked as strikethrough.

F.7.3.2.1.1	Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| default_direct_dependency_flag | u(1) |
| if( default_direct_dependency_flag ) |  |
| default_direct_dependency_type | u(v) |
| else { |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |
| phase_offset_present_flag | u(1) |
| if( phase_offset_present_flag ) { |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ){ |  |
| phase_offset_num[ i ][ j ] | u(2) |
| for( k = 0; k < phase_offset_num[ i ][ j ]; k++ ) { |  |
| hor_phase_luma[ k ][ i ][ j ] | u(3) |
| ver_phase_luma[ k ][ i ][ j ] | u(3) |
| hor_phase_chroma[ k ][ i ][ j ] | u(3) |
| ver_phase_chroma[ k ][ i ][ j ] | u(3) |
| } |  |
| } |  |
| } |  |
| … |  |

F.7.4.3.1.1	Video parameter set extension semantics

phase_offset_num[ i ][ j ] specifies the number of phase offset sets signaled for the layer with nuh_layer_id equal to layer_id_in_nuh[ i ] and the reference layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. When not present, the value of phase_offset_num[ i ][ j ] is inferred to be equal to 0. The value of phase_offset_num[ i ][ j ] shall be in the range of 0 to 2, inclusively.

hor_phase_luma[ k ][ i ][ j ] specifies the phase offset with set index k used for reference layer luma sample location derivation in horizontal direction when reference layer nuh_layer_id equal to layer_id_in_nuh[ j ] and the current layer nuh_layer_id equal layer_id_in_nuh[ i ]. When not present, the value of hor_phase_luma[ k ][ i ][ j ] is inferred to be equal to 0.

ver_phase_luma[ k ][ i ][ j ] specifies the phase offset with set index k used for reference layer luma sample location derivation in vertical direction when reference layer nuh_layer_id equal to layer_id_in_nuh[ j ] and the current layer nuh_layer_id equal layer_id_in_nuh[ i ]. When not present, the value of ver_phase_luma [ k ][ i ][ j ] is inferred to be equal to 0.

hor_phase_chroma[ k ][ i ][ j ] specifies specifies the phase offset with set index k used for reference layer chroma sample location derivation in horizontal direction when reference layer nuh_layer_id equal to layer_id_in_nuh[ j ] and the current layer nuh_layer_id equal layer_id_in_nuh[ i ]. When not present, the value of hor_phase_chroma[ k ][ i ][ j ] is inferred to be equal to 0.

ver_phase_chroma[ k ][ i ][ j ] specifies specifies the phase offset with set index k used for reference layer chroma sample location derivation in vertical direction when reference layer nuh_layer_id equal to layer_id_in_nuh[ j ] and the current layer nuh_layer_id equal layer_id_in_nuh[ i ]. When not present, the value of ver_phase_chroma[ k ][ i ][ j ] is inferred to be equal to 1.

F.7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| if ( vert_phase_position_enable_flag[ RefPicLayerId[ i ] ] ) |  |
| vert_phase_position_flag[ RefPicLayerId[ i ]] | u(1) |
| if( phase_offset_num[ nuh_layer_id ][ RefPicLayerId[ i ] ] > 1 ) |  |
| phase_offset_set_flag[ RefPicLayerId[ i ]] | u(1) |
| … |  |

F.7.4.7.1	General slice segment header semantics

phase_offset_set_flag[ RefPicLayerId[ i ]] specifies the phase offset set index used in reference layer sample location derivation for the reference layer picture with nuh_layer_id equal to RefPicLayerId[ i ]. When not present, the value of phase_offset_set_flag[ RefPicLayerId[ i ]]is inferred to be equal to 0.

H.6.2	Derivation process for reference layer sample location used in resampling

…

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag		(H-1)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-2)

phaseX = ( cIdx  = =  0 ) ? HorPhaseOffsetY : HorPhaseOffsetC	(H-3)
phaseY = ( cIdx  = =  0 ) ? VerPhaseOffsetY : VerPhaseOffsetC	(H-4)

…

H.8.1.4	Resampling process for inter-layer reference pictures

…

VertPhasePositionAdjustFlag = vert_phase_position_enable_flag[ rLId ]	(H-5)
VertPhasePositionFlag = vert_phase_position_flag[ rLId ] 	(H-6)

HorPhaseOffsetY = hor_phase_luma[ phase_offset_set_flag[ rLId ] ][ currLayerId ][ rLId ]	(H-xx)
VerPhaseOffsetY = ver_phase_luma[ phase_offset_set_flag[ rLId ] ][ currLayerId ][ rLId ]	(H-xx)
HorPhaseOffsetC = hor_phase_chroma[ phase_offset_set_flag[ rLId ] ][ currLayerId ][ rLId ]	(H-xx)
VerPhaseOffsetC = ver_phase_chroma[ phase_offset_set_flag[ rLId ] ][ currLayerId ][ rLId ]	(H-xx)

…

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).