| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0170 |
| --- | --- |

| Title: | SHVC HLS: Resampling need for a scalable layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jonatan Samuelsson, Rickard Sjöberg, Jacob Ström Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

Currently a decoder may according to information given on a high level e.g. VPS, draw wrong conclusions about the need for resampling process or not for decoding of a layer of a scalable video bitstream. The need for resampling process could be useful for a decoder to know since it varies the amount of actions for the decoder to perform.

We propose to add a layer wise syntax element to indicate if resampling process is needed for decoding of a layer of a HEVC extension bitstream. The indication can also distinguish between resampling for fractional and integer sample positions or if the need for resampling is unknown.

# Problem

The scalable extension of HEVC supports both spatial scalability and quality scalability. Spatial scalability can be achieved by invoking a sample based- or motion based resampling process for a picture/motion field of a lower layer and use it as a reference picture for coding of another layer. Quality scalability can be achieved without any resampling process by copying a reconstructed picture/motion field of a lower quality and use it as a reference picture for the coding of another layer. Spatial scalability can also be achieved with resampling process with the use of the reference scaling offsets but without need for resampling for fractional sample positions. Reference scaling offsets can also be used to deploy spatial scalability only for a portion of a lower layer by invoking the resampling process although the resolutions of the two layers are the same.

Currently a decoder may according to information given on a high level e.g. VPS, draw wrong conclusions about the need for resampling process or not for decoding of a layer of a scalable video bitstream. The need for resampling process or not could be useful for a decoder to know since there is a difference in the amount of computations for resampling for fractional sample positions, resampling for integer sample positions or not doing any resampling at all.

On one hand the decoder may think that a layer of a scalable bitstream don’t require any resampling process since it seems to contain information covering the same spatial region when the resolutions of a reference and enhancement layer are defined to be the equal in the VPS. However the resolution of the reference and enhancement layer can also be defined in the sequence parameter set (SPS) and change the situation such that the reference layer is half of the one specified in the VPS, thus require resampling process. Another change of the situation can be made by the reference scaling offsets in the SPS extension since they can be set such that the enhancement layer corresponds to a portion of the reference layer, e.g. zoom in, which also requires resampling process although the resolutions of the two layers are the same.

Another example is when an enhancement and the reference layer have different resolutions according to the VPS. In this case the decoder may think that the enhancement layer requires resampling process for fractional sample positions although also in this case the reference scaling offsets in the SPS extension can be provided and be such that it enables the enhancement layer to use the reference layer with resampling for integer sample positions.

# Solution

It is proposed to add syntax to the VPS VUI extension [1] that clarifies the need for resampling process for decoding of a specific layer.

F.7.3.2.1.1.4 VPS VUI syntax

| vps_vui( ){ | Descriptor |
| --- | --- |
| .. |  |
| resampling_flag | u(1) |
| if(resampling_flag ){ |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| resampling [ i ] | u(2) |
| } |  |
| ... |  |
| } |  |

F.7.4.3.1.4 VPS VUI semantics

resampling_flag equal to 1 indicates that the syntax element resampling[i] is present. resampling_flag equal to 0 indicates that the syntax element resampling[i] is not present.

resampling[ i ] equal to 1 indicates that the resampling process is needed for decoding of the i-th layer since the layer has fractional sample positions compared to the sample positions of at least one of it’s direct reference layers. resampling[i] equal to 2 indicates that the resampling process is needed for decoding of the i-th layer but the layer has no fractional sample positions compared to the sample positions of any of its direct reference layers. If the resampling[i] is equal to 3 the decoding of the i-th layer requires no resampling process. If the resampling[i] is equal to 0 or if resampling[i] is not present it is unknown if the resampling process is required for decoding of the i:th layer.

# Conclusion

It is proposed to add syntax elements to the VPS VUI extension of the SHVC draft such that a decoder on a high level can understand the resampling need for decoding of a specific layer.

# References

- “High efficiency video coding (HEVC) scalable extension draft 5”,  JCTVC-P1008_v4, San Jose, January, 2014

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).