| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0174 |
| --- | --- |

| Title: | AHG10: Modified copy above mode for palette based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianqing Zhu, Jiangli Ye, Kimihiko Kazui 29F,Kerry Parkside,1155 fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5810 zhujianqing@cn.fujitsu.com threerabbit@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

Simplification of the process for copy mode on palette based coding is presented. Use pixel value directly rather than index for encoding and decoding process for copy mode. Test results show gain of 0.1%~0.2% AI-lossless on SCs.

# Introduction

CE on Palette base coding tools[1][2] were studied in the 16th JCT-VC meetings in San Jose and proved to be efficient, especially for screen contents. AHG10 was established for further study and a base software [3] for AHG10 investigation on palette coding tool were suggested. This contribution is an improvement on the base software.

In base software, the pixels in the CU are encoded in a raster scan order. There are 3 modes for each pixels: “copy-above”, “index-run” and “pixel-escape”. For “copy-above” mode, index derivation is needed for both encoder and decoder. For “copy run” derivation in encoder side, index is derived firstly for each pixel. Then, compare index of current position and that of its above position to judge whether use copy or not. In decoder side, if “copy mode” is used, the index value of current position is copied from its above position. Then, pixel value is derived by looking up palette table. And for the pixel in escape mode, copy mode will not be applicable.

For example, for location (0,1) (first pixel of second line) in Fig1, “copy run ” is 7.

Figure 1 Coding unit to be coded in palette base coding

This contribution presents a simplification of the process for copy mode on palette based coding.

# Proposed method

## Simplified process for copy above mode

Use pixel value directly for “copy-above” (in horizontal scan order) or “copy-left” (in vertical scan order) mode in encoder and decoder. For “copy run” derivation in encoder side, there is no index derivation. Just compare pixel value of current position and its above/left to judge whether it can be copy or not. In decoder side, for pixel in “copy mode”, the reconstructed pixel value of current position is the reconstructed pixel value of its above/left position.

In this way, copy mode will not care about the pixel value is in escape or not. The “copy run” might be larger so that coding efficient is higher. For the CU in Figure1 case, for location (0,1) (first pixel of second line), “copy run ” is 15 as shown in Figure2.

Figure 2

Meanwhile, calculation complexity is reduced since no index derivation process is needed for “copy mode”.

# Experimental results

The anchor is the base software in P0303 a bug fixed on calculate run. Test condition is CTC of rang extension in P1006 for AHG8 with all intra configuration.

## Result of simplified process for copy above mode

Table 1. Simulation results in lossless coding mode

# References

- X. Guo, Y. Lu, and S. Li, “RCE4: Test1. Major-color-based screen content coding,” JCTVC-P0108, San José, US, Jan. 2014.

- L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, and F. Zou, “RCE4: Results of test 2 on palette mode for screen content coding,” JCTVC-P0198, San José, US, Jan. 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, and J. Xu, “Suggested Software for the AHG on Investigation of Palette Mode Coding Tools,” JCTVC-P0303, Geneva, Switzerland, Oct. 2013.

# Patent rights declaration

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).