| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0175 |
| --- | --- |

| Title: | Intra block copy with encoder search using chroma component |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an encoder search method for Intra block copy. In the proposed method, both luma and chroma components are used in the encoder search for Intra block copy in a final step. Reported average BD-rates are -1.3%, -0.8% and -0.9% for YUV 4:4:4 SC sequences in AI-MT, RA-MT and LB-MT, respectively.

# Introduction

In the current HEVC Range Extensions, Intra block copy (Intra BC) is adopted to predict repetitive patterns in the frame and leads to large bit-rate savings. In order to get the blocks vectors (BVs) used for Intra BC, block matching algorithm (BMA) is performed at encoder, and the BV with the minimum luma R-D cost is selected as the final vector.

# Proposed method

The proposed encoder search method for Intra BC includes the SAD for chroma component in the second step:

- Step 1: The best N (i.e. 4) BVs are selected according to their R-D cost, which is calculated as

RD_Cost = SADLuma + Lambda * BVbits

Note that here only the SAD of luma component is used.

- Step 2: Both luma and chroma components are used in the calculation of SAD for the selected best N BVs from step 1. The BV with the minimum R-D cost out of the N BVs is selected as the final BV, and the R-D cost in this step is calculated as

RD_Cost = SADLuma + SADChroma + Lambda * BVbits

# Results

In this section, experimental results are presented for the proposed encoder search method for Intra BC. The proposed method is implemented on top of HM13.0_RExt6.0, and AHG8 test conditions [1] are used. The performance is compared to the anchor for lossy (Table 1) and lossless (Table 2) configuration in terms of BD-rates and rate savings, respectively.

Table 1 Lossy results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.4% | -0.4% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.6% | -1.8% | -1.9% | -1.3% | -1.5% | -1.5% | -1.0% | -1.2% | -1.1% |
| RGB 4:4:4 Animation | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC | -1.3% | -2.1% | -2.1% | -1.4% | -2.0% | -1.9% | -1.3% | -1.6% | -1.7% |
| YCbCr 4:4:4 Animation | 0.0% | -0.3% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -6.1% | -6.2% | -6.3% | -5.7% | -6.1% | -6.4% | -4.0% | -3.4% | -3.6% |
| YCbCr 4:4:4 SC (Optional) | -1.6% | -2.0% | -1.9% | -1.2% | -1.5% | -1.4% | -0.9% | -1.1% | -1.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 98% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -1.3% | -1.5% | -1.5% | -1.0% | -1.2% | -1.2% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 SC | -0.8% | -1.4% | -1.5% | -0.9% | -1.3% | -1.4% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | -0.3% | -0.4% | 0.0% | -0.2% | -0.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -5.2% | -5.4% | -5.4% | -4.6% | -5.2% | -5.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.4% | -1.6% | -1.8% | -1.3% | -1.5% | -1.6% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.4% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.9% | -1.1% | -1.1% | -0.8% | -0.9% | -0.9% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
| YCbCr 4:4:4 SC | -0.9% | -1.4% | -1.3% | -0.9% | -1.3% | -1.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -5.0% | -5.1% | -5.2% | -4.9% | -4.8% | -5.0% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.7% | -1.8% | -1.9% | -1.1% | -1.2% | -1.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 2 Lossless results

| Average bit-rate increase | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.2% | -0.1% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.4% | -0.3% | -0.2% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 2.1% | 1.8% | 1.8% |
| YCbCr 4:4:4 SC (Optional) | 0.2% | 0.3% | 0.3% |
| Enc Time[%] | 102% | 103% | 103% |
| Dec Time[%] | 105% | 101% | 110% |

# References

[1] C. Rosewarne, K. Sharman, and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, San José, US, January 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).