| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0176-r1 |
| --- | --- |

| Title: | AHG8: String match in coding of screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Zhan Ma, Meng Xu, Xian Wang, Haoping Yu 2330 Central Expy, Santa Clara, CA 95050, United States | Tel: Email: | +1 408 330 5142 wei.wang.sc@huawei.com, zhan.ma@huawei.com, haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

In this contribution, the string match method presented in JCTVC-Q0034 is extended to include 2-D rectangular blocks. Based on this method, a hybrid of 1-D and 2-D string match method is developed for coding of the color index map proposed in JCTVC-Q0034. In addition, several improvements, over the screen content coding solution in Q0034, are also described for better coding performance and reduced complexity. These improvements include simplified color table processing, fast color table merge decision, fast string search, etc. These proposed methods have been implemented in the HEVC RExt Draft 5 associated reference software HM12.1+RExt-5.1 [3] (RExt5.1). The simulation results have shown an average bit-rate reduction over the CfP anchors for lossless coding by 16.2%, 10.5%, and 9.3% for All Intra (AI), Low-Delay with B-picture (LB), and Random Access (RA), respectively. For the lossy coding mode, an average BD rate reduction of 14.4% for AI, 10.9% for RA, and 7.4% for LB have been observed. The complexity is measured using the encoding and decoding times. However, due to the heterogeneous computing nodes utilized, the running times presented in the test results may not accurately reflect the relative complexity.

# Introduction

JCTVC-Q0034 proposed an advanced screen content coding solution with a “color table and index map coding mode” designed specifically for coding of screen content. The basic algorithms in this mode include pixel representation using color table, color table compression, color index map compression, 1-D string search, etc., which is summaried below:

- Color table processing

  - Color table merge: only left merge is enabled in JCTVC Q0034

  - Inter-table color sharing

  - Intra-table color DPCM

  - Lossy color grouping

- Color index map processing

  - 1-D string match with upper and left buffer

  - Adaptive horizontal and vertical index map scan: (only horizontal scan is enabled in JCTVC Q0034)

- LMChroma (same technology from HM8.1)

In this contribution, the main framework of Q0034 is kept without modification. However, we have extended the string match algorithm to include 2-D rectangular sub-blocks. In addition, several improvements have been made in the areas of color table processing, color table merge decision, string search, etc. The main benefits of these improvements are better coding performance and reduced complexity. major few improvements have been deployed to further the coding efficiency and speed-up both encoder and decoder. These changes are highlighted as follows, and are discussed in details in the following section.

- Color index map processing

  - Hybrid 1-D and 2-D Color Index Map Processing

    - Hashing function is utilized to boost the encoder speed

- Color table processing

  - Color table merge: both left and upper merge are enabled

    - A 2-stage fast merge decision is implemented

# Algorithm description

## Hybrid 1-D and 2-D Color Index Map Processing

After obtaining the color table, each and everyone of the basic processing units (three components packed together) is translated to the corresponding index in the table. As the packed components are used for each pixel to derive the proper index, there is only one index map to be encoded for each CU. The index assignment is based on the minimum error criteria.

Index map of each CU could be seen as the block with reduced dynamic range, but it still contains the dominant local feature such as the directions.  Thus, adaptive horizontal and vertical scanning is introduced to process the index map considering the local direction feature.

Note: same to JCTVC-Q0034, the vertical scan was skipped while generating the test results shown in this contribution.

For each horizontal or vertical scanning pattern, instead of using only 1-D block search to derive the matched blocks, 2-D block search is also added to derive the matched blocks.  Previously processed indices will be used as the reference for the successive index search.  Multiple CTUs, e.g. up to 3 CTUs, to the left of current CTU will also be used as the reference area for 2-D block search.

- Both 1-D and 2-D search are performed for the current index. For the 1-D search result, search engine records matched length and corresponding distance or location.  For the 2-D search result, search engine records matched block width and height, corresponding reference LCU index and the location of reference index in the LCU.

- If the current index couldn’t find its match from previously processed indices, it is encoded as the unmatched index by encoding a flag indicating unmatched pixel and the index value itself.

- If the current index chooses a 1-D matched string as its search result, the encoding is done by sending a flag indicating matched pixel,  the match type flag (1-D), the distance (offset between current position and matched position) and the length of successive matched indices.

  - If there are multiple 1-D matches for current index, the pair providing the best R-D performance should be chosen. (To save the complexity, current implementation uses a heuristic manner to select the pair with the longest matched length).

- If the current index chooses a 2-D match as its search result, the encoding is done by sending a flag indicating the matched pixel, the match type flag (2-D), the reference LCU index, the distance (the address of matched position in reference LCU) and the length (width/height) of successive matched block.

  - If there are multiple 2-D matches for current index, the pair providing the best R-D performance should be chosen. (To save the complexity, current implementation uses a heuristic manner to select the pair with the biggest matched area).

- A Coded Map is introduced in the coding process.  Whenever a match is found, the area covered by 1-D length or 2-D block is marked in the Coded Map. After the coding of the current index is finished, the coding process scans through the coded map to find the first un-coded location as the next index to process.

- Also in the current 1-D/2-D string search, if any of the location in the current string has already been coded in previous 2-D search, the match result for this location is always treated as a match even though the index value between the current location and reference location may be different.

- Note the flag mentioned above for unmatched and matched signaling may be just a one-bit flag used in all cases.

For the current proposal, 2-D search is performed on CTU level only.

In 1-D search description, curIdx is the starting address of to-be-coded string; refIdx is all possible starting address of reference string.  For every given curIdx and refIdx, program performs a modified memory compare operation to find the first unmatched value.  It then compares all the matched-length and records the longest matched-length (len, refIdx) as the winner of 1-D search for location curIdx.

It is possible that a certain location in the current 1-D search string has already been encoded by previous 2-D search; in this case, this location is treated as a match regardless of its value.  A mismatch happens only when this location is not encoded AND its ColorMap value doesn’t equal to the ColorMap value at corresponding location in reference string.

In 2-D search description, curIdx is the top-left corner of to-be-coded block; refIdx is all possible top-left corner of reference block.

Program calculates the maximum search window in the Current Color Map and saves them in variables cw0 and ch0.  It also calculates the maximum search window in the Reference Color Map and saves them in variable rw0 and rh0.

The final search window is defined by minw = min(rw0, cw0) and minh = min(rh0, ch0).

For every given curIdx and refIdx, the search process performs a modified memory comparison operation to find the first unmatched value in every line of the search window (minw, minh). It then compares all the matched-size for any height and records the longest matched-size (width, height, refIdx and index of reference LCU) as the winner of the 2-D search for location curIdx.

It is possible that a certain region in the current 2-D search window has already been coded by a previous 2-D match, in this case, this location is treated as a match regardless of its value.  It is also possible that a certain region in reference 2-D search window has not been coded yet; in this case, this location is treated as a mismatch regardless of its value.

A mismatch happens only when this location is not coded AND when this location in the reference search window is not coded, or its value in the Current Color Map isn’t equal to the color map value at the corresponding location in the reference string.)

After the search for 1-D string match and 2-D area match is completed, the 1-D result is used if the 1-D maxLen is bigger than the 2-D max2DSize or the 2-D area is smaller than a threshold (currently 4x4).  Otherwise, the 2-D result is used for coding of curIdx.  CodedMap is updated and the curIdx is advanced to next location.

Due to the nature of hybrid 1-D and 2-D search, the next curIdx location may have already been coded by previous 2-D search, a raster scan is performed in CodedMap to find the first location that has not been coded as the new curIdx.

To encode the length of 1-D string match, the bit stream syntax uses an adjusted length instead of actual maxLen to encode 1-D result.  Given that some of the locations in maxLen have already been encoded during previous 2-D match, these locations are removed from maxLen and the adjusted length is written to bit stream.  If the adjusted length equals to 1, then this match is treated as an unmatched data.

To encode the width/height of 2-D block, the bit stream syntax uses length instead of width/height to encode 2-D result.  The length here is defined as the raster location of the bottom-right corner of the matched window in the search window defined by (minw, minh). It is further modified to reflect the fact that the width/height of both search window and matched window are bigger than the threshold window using below pseudo code:

2DLength = (2DWidth-MIN_2D_WIDTH)+(2DHeight-MIN_2D_HEIGHT)*(1+minw-MIN_2D_WIDTH)

Decoding process can retrieve 2D width and height using below pseudo code:

2DWidth = (2DLength%(1+minw-MIN_2D_WIDTH))+MIN_2D_WIDTH;

2DHeight = (2DLength/(1+minw-MIN_2D_WIDTH))+MIN_2D_HEIGHT;

Decoder should maintain the same CodedMap operation for correct processing sequence.

- After it receives a 1-D length or 2-D width/height, the decoder adjusts the 1-D length if there are any already-coded-locations during the CodedMap.

- In the process of reconstructing current 1-D string or 2-D block from reference location, decoder would copy Color Map index from reference location only when current location is not coded (as indicated in CodedMap).

- It updates the CodedMap, and scans through the CodedMap to find the next un-coded location as the new curIdx.

## Color Table Derivation

Instead of applying the neighboring color grouping in JCTVC-Q0034, a simple yet efficient hard decision is used to group the colors for color table derivation, where if the difference between two colors is less than a pre-defined threshold (i.e., 9), these two colors are grouped together. The grouping is now conducted within a single scan of the histogram of current CU.

## Color Table Merge

Both left and upper merge are applied in this contribution.  However, instead of checking left merge, upper merge and no merge one-by-one using R-D behaviour, a simple 2-stage merge decision is applied, where colorTableIdx mode is examined with both merge modes disabled during the first stage. Only when current CU selects the colorTableIdx mode (i.e., normal HEVC mode shows worse R-D cost), left and upper merge will be checked in the second stage.

# Syntax and semantics description

Here is the syntax for the proposed color table and index map coding method.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| … |  |
| … |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| hybrid_color_index_search_eabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| …. |  |
| … |  |
| } |  |

| color_index_map(x0, y0, nCbS, nCbS ) { | Descriptor |
| --- | --- |
| color_idx_map_scan_dir | ae(v) |
| for( i = 0; i < nCbS * nCbS) { |  |
| idx_match _flag | ae(v) |
| if( idx_match_flag) { |  |
| unmatched_idx | ae(v) |
| } else { |  |
| if( sps_hybrid_search_enabled_flag ) |  |
| 2d_block_search_flag | ae(v) |
| if( 2d_block_search_flag ) { |  |
| 2d_lcu_index | ae(v) |
| 2d_distance | ae(v) |
| 2d_length | ae(v) |
| } |  |
| } else { |  |
| sig_dist_flag | ae(v) |
| if( sig_dist_flag ) { |  |
| dist_equal_one_flag | ae(v) |
| } else { |  |
| distance_minus2 | ae(v) |
| } |  |
| sig_len_ flag | ae(v) |
| if ( sig_len_ flag ) { |  |
| length_minus1 | ae(v) |
| i += length_minus1 + 1; |  |
| } else { |  |
| length_minus_cuWidth | ae(v) |
| i += length_minus_cuWidth + cuWidth; |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

hybrid_color_index_search_eabled_flag: equal to 1 specifies that current coding unit is using hybrid 1D and 2D search mode. hybrid_color_index_search_eabled_flag equal to 0 specifies that current coding unit uses only 1D search method.

2d_block_search_flag[x0][y0]: equal to 1 specifies that the type of current matched pair is a 2D block match.  2d_block_search_flag[x0][y0] equal to 0 specifies that the type of current matched pair is a 1D string match.

2d_lcu_index[x0][y0][cIdx]: specifies the index of referenced CTU where 2D match was found. It is the nth CTU to the left side of current CTU. n=0 means that 2D match was found within current CTU.

2d_distance[x0][y0][cIdx]: specifies the distance of current matched pair.  It is the raster scan index of top-left corner of the matched reference block within the reference CTU.

2d_length[x0][y0][cIdx]: specifies the length of current matched 2D pair.

The variable 2D width and height are derived as follows:

2DWidth[x0][y0][cIdx] = (2d_length[x0][y0][cIdx]%(1+minw-MIN_2D_WIDTH))+MIN_2D_WIDTH;

2DHeight[x0][y0][cIdx] = (2d_length[x0][y0][cIdx]/(1+minw-MIN_2D_WIDTH))+MIN_2D_HEIGHT;

# Compression performance discussion

Proposed technical solution is implemented on top of the HEVC RExt 5.1 for comparison. Both lossless and lossy cases are performed to demonstrate the coding efficiency.  For lossless case, bit rate reduction is measured.  For lossy case, bit rate matching is first conducted and then the BD-Rate is calculated using the QPs derived from the bit rate matching.

The complexity is measured using the encoding and decoding time, however, due to the heterogeneous computing nodes utilized, the running time is not reliable.

## Lossy test condition

The summaries of lossy BD-Rate reduction for both with PSNR clipping and without PSNR clipping in comparison to the RExt 5.1 are shown below. More detailed simulation data is enclosed as excel attachment.

(a)

(b)

(c)

Figure 1: summary of BD-Rate reduction for (a) AI, (b) RA, and (c) LB. (without hash speedup)

(a)

(b)

(c)

Figure 1: summary of BD-Rate reduction for (a) AI, (b) RA, and (c) LB. (with hash speedup)

## Mathematically lossless test condition

The summary of lossless bit rate reduction in comparison to the RExt 5.1 is shown below. More detailed simulation data is enclosed as excel attachment.

- AI

- RA

- LB

Figure 3: lossless bit rate reduction summary (without hash speedup)

- AI

- RA

- LB

Figure 4: lossless bit rate reduction summary (with hash speedup)

Because of the heterogeneous computing nodes utilized for simulation, running time shown above is not reliable.

# Complexity analysis

Additional memory requirement:

Hybrid 1-D and 2-D search use approximately additional 80KB memory (20 CTUs).

An 8-bit CRC hashing is used to increase encoding speed; other type of hash function can also be adopted.

For 1-D search, a 2-point hashing is calculated for every Color Map Index, a hash dictionary array is formulated using calculated hash value.

For 2-D search, a 4x4 block hashing is calculated for every Color Map Index, a hash array and a hash dictionary array are formulated using calculated hash value.

# Closing remarks

This contribution presents the improvements on top of the JCT-VC Q0034 with enhanced performance and reduced complexity. Improvements include extended string match algorithm, simplified color table processing, fast merge decision, 2-D search. The proposed solution is implemented on top of the HEVC RExt Draft 5 associated reference software HM12.1+RExt-5.1 [3] (RExt5.1), demonstrating an average bitrate reduction over the anchors by 16.2%, 10.5%, 9.3% bit rate reduction for respective lossless All Intra (AI), Random Access (RA) and Low-Delay with B-picture (LB) coding structures. The simulations results for the lossy coding made also show an average BD rate reduction by 14.4%, 10.9%, 7.4% for AI, RA, and LB coding scenarios, respectively. The complexity is measured using the encoding and decoding time, however, due to the heterogeneous computing nodes utilized, the running time is not reliable.

We suggest the committee adopts these improvements into the CE or TE for further study.

Reference

[1] Joint Call for Proposal for coding of screen content, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q6, Doc. MPEG2014/N14175, San Jose, USA, Jan. 2014.

[2] D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, HEVC Range Extensions Draft 5, Joint Collaborative Team on Video Coding (JCT-VC) of ITU SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JCTVC-O1005, Geneva, CH, Oct. 2013.

[3] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.1+RExt-5.1

[4] G. Bjontegaard, Calculation of average PSNR differences between RD-curves, ITU-T SG 16 Q.6, Doc. VCEG-M33, Austin, USA, April 2001.

# Patent rights declaration(s)

Huawei Technologies, Inc., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).