| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0179 |
| --- | --- |

| Title: | AHG8: Crosscheck of Coding the prediction differences of the intra BC vectors (Q0095) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-Q0095, “AHG8: Coding the prediction differences of the intra BC vectors”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform, not modifying the level and profile set by the CTC configuration files. As a result, the BD rate results performed during this crosscheck match those provided in the contribution Q0095.

# Analysis of the proposal

The contribution JCTVC-Q0095 [1] proposes a method for coding the block vector difference associated with the Intra BC coding mode. The proposed method represents the absolute value | d |, of each component of the block difference vector by its most significant bit (MSB) index followed by its refinement bits. The MSB index for syntax value 0 is assigned to -1. The MSB index plus one is binarized by the unary code. The remaining refinement bits are represented in the fixed-length binary code with the codeword length determined by the MSB index.

It is also important to note that this new binarization scheme leads to exactly the same bin string as that of the binarization scheme employed in the current HEVC for coding motion vector difference.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the AhG8 Common Testing Conditions [2], except the level and tier are unmodified from the value found in the configuration files. Table 1 and Table 2 show the results of the proposed scheme compared with HM13.0+RExt6.0 anchors for lossy configurations. The encoding runtime are not modified compared to the anchors.

## Lossy results

## Lossless results

# Conclusion

This contribution reports cross-verification results on contribution JCTVC-Q0095, “AHG8: Coding the prediction differences of the intra BC vectors”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. The BD rate results provided by this cross-check match the ones provided by the proponent.

# References

- S.-T. Hsiang and S. Lei, “AHG8: Coding the prediction differences of the intra BC vectors”, JCTVC-Q0095, 17th meeting, Valencia, March/April 2014.

- C. Rosewarne, K. Sharman and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, 16th meeting, San-Jose, January 2014.