| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0181 |
| --- | --- |

| Title: | Crosscheck of quantization rounding for RDPCM (JCTVC-Q0148) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-Q0148, “Quantization rounding for RDPCM”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform, not modifying the level and profile set by the CTC configuration files. BD rate results matched, while decoding runtimes were inaccurate during those experiments and should not be considered.

# Analysis of the proposal

Contribution JCTVC-Q0181 asserts that the traditional uniform quantization with deadzone, used for residual transform coefficients, is detrimental to the coding of spatial residual during transform skip, in particular when RPDCM is active. It therefore proposes to modify, in the case of the RDPCM mode, the rounding used in computing the quantized value: for a quantization step size of Q, the rounding offset used is always Q/2.

The implementation constitutes a minor modification and matches the description. It is implemented by modifying the rounding offset in the last shift before the clipping of the quantized coefficient, located in TComTrQuant::transformSkipQuantOneSample.

The change does not constitute a normative change, as is further evidenced by the code not changing the dequantization formulae.

# Simulation results

## RExt6.0 anchor

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the CTC, except the level and tier are unmodified from the value found in the configuration files. Table 1 and Table 2 show the results of the proposed scheme compared with HM13.0+RExt6.0 anchors for lossy configurations. The encoding runtime are not modified compared to the anchors. The decoding runtimes are less trustworthy as a higher instability has been observed on our system.

## Other informal tests

The proposed changes were introduced in the AhG10 palette software. While the BD-rate results show lessened gains, the behaviour is maintained, in particular in RA and LDB configurations, where RPDCM may be applied to inter CUs.

# Conclusion

This contribution reports cross-verification results on contribution JCTVC-Q0181, “Quantization rounding for RDPCM”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. BD rate results matched, while runtimes were inaccurate during those experiments and should not be considered.