| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0184 |
| --- | --- |

| Title: | Cross-check of JCTVC-Q0135 AMP for Intra BC prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | PoLin Lai, Xiaozhong Xu, Shan Liu MediaTek USA 2860 Junction Ave. San Jose, CA 95134 | Email: | {polin.lai, xiaozhong.xu. shan.liu}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document presents cross-check report for JCTVC-Q0135 AMP for IntraBC prediction. Software study indicates that by adding AMP to IntraBC, the proposed IntraBC partition mode singalling is aligned towards Inter partition mode signalling, with the exception of the existence of N×N partition mode in IntraBC. When the proposed addition of AMP is further combined with the removal of N×N partition mode in the other method in the proposal, the IntraBC partition mode singalling is fully aligned with the Inter partition mode signalling. The cross-check reports exact match of the rate-distortion measurements to data provided by in the proposal.

# Software study

The proposed AMP for IntraBC is implemented as additional partition mode options:

At encoder RD optimization, it is added within TEncCu::xCompressCU when evaluating IntraBC partition modes, and is tested only for 16×16 CU.

The cross-check studied the signalling of IntraBC partition mode in the proposal (TEncSbac.cpp / TDncSbac.cpp):

- Method 1: Add AMP to IntraBC: By adding AMP to IntraBC, the proposed IntraBC partition mode singalling is aligned towards Inter partition mode signalling, with the exception of the existence of N×N partition mode in IntraBC.

- Method 2: Add AMP + remove N×N partition mode for IntraBC: The IntraBC partition mode singalling is fully aligned with the Inter partition mode signalling.

# Simulation results

The cross-check tested the following cases as proposed in the contribution, lossy AI condition only:

|  | New Fast search | NxN | AMP | In Doc Q0135 |
| --- | --- | --- | --- | --- |
| Case 1 | No | Yes | Yes | Table 1 |
| Case 2 | Yes | Yes | Yes | Table 2 |
| Case 3 | No | No | Yes | Table 3 |

The rate-distortion measurements match exactly to the proponent’s data. The cross-check runtime measurements are not reliable as the test and anchor were conducted in different environments.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).