| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0185 |
| --- | --- |

| Title: | Cross-check of 'AHG18: Worst-case Escape Code Length Mitigation' (JCTVC-Q0073) by Sony |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of JCTVC-Q0073 by Sony.  In this contribution, the worst case length of the coeff_abs_level_remaining syntax element is reduced from 46 bypass bins to 32 bypass bins.  The cross check result confirms the simulation result reported by Sony.

# Cross check procedure

The source code was provided by Sony. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under the AHG18 lossy and AHG18 lossless conditions, as defined in [1]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code and working draft text was studied and the following observations were made:

- The maximum prefix length is set to 32 – MAX_TR_DYNAMIC_RANGE – 3.

  - For a given bit-depth (when extended precision processing is enabled), MAX_TR_DYNAMIC_RANGE is set to bit_depth + 6 bits.

- When the maximum prefix length is reached, the suffix length is set to MAX_TR_DYNAMIC_RANGE – Rice parameter value.

- Binarisation of coeff_abs_level_remaining is modified accordingly.

- The RDOQ process is modified to account for the cost of coding coeff_abs_level_remaining.

These observations are consistent with the description in [2].

# Coding performance measurement

Table 1 and Table 2 provide the cross-check results for JCTVC-P0061 under AHG18 lossy and AHG18 lossless conditions, respectively.

Note that run-times are not comparable, as the cluster used to perform the simulation was loaded with multiple simulations at the time of running the cross-check.

Table 1. JCTVC-Q0073 AHG18 lossy simulation result.

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| HDR |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 4K from HDR |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  |
| 4:0:0 Medical |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  | 0.0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Encoding Time % |  |  | 103% |  |  | 103% |  |  | #NUM! |  |  | #NUM! |  |  |
| Decoding Time % |  |  | 101% |  |  | 102% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 2. JCTVC-Q0073 AHG18 lossless simulation result.

|  |  |  | Average Bit Rate Change (%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) | Intra 12-bit (internal bit depth) | Lowdelay 16-bit (internal bit depth) | Lowdelay 12-bit (internal bit depth) |
| SVT |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| RExt RGB |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| HDR |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| 4K from HDR |  |  | 0.0% | 0.0% |  |  |
| HDR 4:0:0 Medical |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| 4:0:0 Medical |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Encoding Time % |  |  | 105% | 104% | #NUM! | #NUM! |
| Decoding Time % |  |  | 103% | 102% | #NUM! | #NUM! |

# Conclusion

This cross-check includes a simulation result for contribution JCTVC-Q0073 by Sony. The provided software matches the description in the contribution. The cross-check result confirms the simulation result reported by the proponent.

# References

- C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, 16th JCT-VC meeting, San Jose, US, Jan. 2014.

- K. Sharman, N. Saunders, J. Gamei, “AHG18: Worst-case Escape Code Length Mitigation,” JCTVC-Q0073, 17th JCT-VC meeting, Valencia, ES, Mar. 2014.