| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0186 |
| --- | --- |

| Title: | AHG5: Super-high Tier Specification Targeted at the Intra 16-bit 4:4:4 Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | T. Suzuki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
|  | A. Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | +1 408 352 4715 ali.tabatabai@am.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

At the 16th JCT-VC meeting in San José, the use of a new super tier was adopted, which could be used in combination with the Intra 16-bit 4:4:4 profile. Following directives from the parent body where the previously adopted super-tier was dropped from the currently defined profiles, this contribution proposes that a “High Intra 16-bit 4:4:4” profile is adopted, enabling the compression ratio defined by MinCR to be approached for all levels as a sequence average when using intra-only coding.

# Introduction

At the 16th JCT-VC meeting in San José the use of a new super tier was adopted, which could be used in combination with the Intra 16-bit 4:4:4 profile. This super tier had data bit rates 4 times higher than high tier.

When coding intra frames, it is common for the compression ratio of the sequence as a whole to be similar to that of each frame. Considering MinCR is defined to be in the range of 1.0 to 2.0 for levels 4 to 6.2, a maximum sequence average bit rate should be defined that would allow these compression ratios to be approached. Note that tests run using the AHG18 lossy test conditions already produce compression ratios within the 1.0 to 2.0 range, and therefore this high operating point has been extensively tested by the JCT-VC. For such a stream to be compliant in super tier, a level intended for a resolution higher than the actual image resolution would have to be used. In the case of 8k, this is impossible as no higher-resolution level is defined.

At the previous meeting when the super tier was first mentioned it transpired that this super tier could be integrated without affecting profiles other than those targeting high operating points. Therefore this proposal aims to close the final gap between MinCR and the minimum compression ratio defined by the maximum bit rate and maximum sample rate.

However, following directives from the parent standard bodies, the super tier was removed from all profiles defined in Range Extensions. It is therefore proposed that a new intra-only profile be created to enable the minimum compression ratio to approach MinCR-like operating points and to specify tools that make decoding such streams in real-time practical. This new profile would have the following features:

- It is based upon Main 16-bit Intra 4:4:4 profile, using just two tiers.

- It is suggested to be called “High 4:4:4 16 Intra”.

- The bit rate of the system would be increased. This can be achieved either by:

  - CpbBrVclFactor is set to 48000*HbrFactor for this profile, (and CpbBrNalFactor 10% higher).

  - Alternative, scaling HbrFactor by a factor of 16 when applied to this profile (assuming the base CpbBrVclFactor is 3000).

- The CPB size would be defined such that it would always be able to hold at least one frame compressed at the MinCr ratio.

  - In P1005_v2, the CPB size is scaled by CpbBrVclFactor, so that it can always buffer 1s of video. At these operating points, such a buffer size is unnecessarily large, and it is proposed that it is reduced by a factor of 16.

- Decoders supporting this profile would not automatically be required to support any other profiles.

- The value of cabac_bypass_alignment_enabled_flag would be specified to be 1.

# MinCR and the Sequence Average Minimum Compression Ratio

Table 1 shows the value of MinCR for each tier and level alongside the sequence average compression ratio defined by:

| Level | MaxBR (in CpbBRVclFactor or CpbBrNalFactor bits per second) |  |  | Minimum sequence average compression ratio (4:4:4 16-bit) |  |  | MinCR (HbrFactor = 2, MinCRScaleFactor = 0.5) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main | High | Super (High×4) | Main | High | Super |  |
| 1 | 128 |  |  | 34.6 |  |  | 0.5 |
| 2 | 1 500 |  |  | 19.7 |  |  | 0.5 |
| 2.1 | 3 000 |  |  | 19.7 |  |  | 0.5 |
| 3 | 6 000 |  |  | 22.1 |  |  | 0.5 |
| 3.1 | 10 000 |  |  | 26.5 |  |  | 0.5 |
| 4 | 12 000 | 30 000 | 120 000 | 44.6 | 17.8 | 4.5 | 1.0 |
| 4.1 | 20 000 | 50 000 | 200 000 | 53.5 | 21.4 | 5.3 | 1.0 |
| 5 | 25 000 | 100 000 | 400 000 | 85.6 | 21.4 | 5.3 | 1.5 |
| 5.1 | 40 000 | 160 000 | 640 000 | 107.0 | 26.7 | 6.7 | 2.0 |
| 5.2 | 60 000 | 240 000 | 960 000 | 142.6 | 35.7 | 8.9 | 2.0 |
| 6 | 60 000 | 240 000 | 960 000 | 142.6 | 35.7 | 8.9 | 2.0 |
| 6.1 | 120 000 | 480 000 | 1 920 000 | 142.6 | 35.7 | 8.9 | 2.0 |
| 6.2 | 240 000 | 800 000 | 3 200 000 | 142.6 | 42.8 | 10.7 | 1.5 |

Table 1 - MinCR (where HbrFactor = 2 and MinCRScaleFactor = 0.5) and minimum sequence average compression ratio for all current tiers and levels using the three tier system described in JCTVC-P1005_v2

As can be seen from the above table, considering the minimum sequence average compression ratio, it would be impossible to approach MinCR for intra-only coding. Therefore, increasing the bit rates by a further factor of 4 over those defined previously for the super tier is proposed. This can be achieved using the scheme defined in Section 1.

# Working Text Changes

Working text changes to follow due to the large changes required to the working text in this area.

-

-evel limits

- -

-

-

- Conclusion

An issue has been raised where, though MinCR is defined such that PSNRs equivalent to 16-bit precision can be attained, it would be impossible for an intra-only coder to compress at MinCR while still meeting the minimum compression ratio constraints defined by the maximum bit rate and maximum luma sample rate.

To allow MinCR to be approached, it is proposed that a new intra-only high profile be added to the standard (using a new profile-idc) that allows for higher maximum bit rates, such as those being considered for use in future communication standards.

- Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).