| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0187 |
| --- | --- |

| Title: | Resampling of reference layer frames in interlaced-to-progressive scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kemal Ugur Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that when a reference layer frame represents interlaced source content, the present SHVC resampling process is unable to generate an inter-layer reference picture on the basis of a selected field of the reference layer frame for interlaced-to-progressive scalability. This contribution proposes a vertical decimation factor syntax element for the slice segment header to indicate whether every sample row or every other sample row of the reference layer frame takes part in the resampling process. Additionally, this contribution proposes a vertical offset syntax element for the slice segment to indicate whether the top field or the bottom field of the reference layer frame is resampled, when the vertical decimation is indicated to be in use. The inter-layer resampling process is proposed to be appended according to the proposed syntax elements.

# Introduction

Interlaced source content can be coded as frames or fields. In the targeted use case, interlaced source content is coded as frames in the base layer, whereas the enhancement layer represents a progressive scan enhancement of the base layer.

When a BL frame is used as a reference for inter-layer prediction, every other sample row of the BL frame should take part in the resampling process. It is asserted that SHVC currently uses every sample row in the resampling process and hence is not well suited for interlaced-to-progressive scalability when the base layer contains coded frames. This contribution proposes a vertical decimation factor syntax element to indicate whether every sample row or every other sample row of the BL frame takes part in the resampling process.

In interlaced-to-progressive scalability, a base-layer frame should be as reference for inter-layer prediction for two enhancement-layer pictures (one for each field parity). This can be realized by either of the following two ways:

- Diagonal prediction as proposed in JCTVC-Q0105/JCT3V-H0035 and illustrated in Figure 1. The first EL picture in each pair of EL pictures is predicted using conventional inter-layer prediction, and the second EL picture is predicted using diagonal inter-layer prediction.

Figure 1. Interlaced-to-progressive scalability with coded frames in the base layer

- Coding of "skip pictures" in the base layer which use the previous picture as reference, use zero motion vectors, no prediction error coding and no loop filtering as illustrated in Figure 2.

Figure 2. Interlaced-to-progressive scalability with coded frames and skip pictures in the base layer.

It is remarked that scaled reference layer offsets could be used for selecting whether the top or bottom field is resampled from coded frames representing interlaced source content. However, scaled reference layer offsets are indicated in the SPS and hence stay unchanged for the entire coded video sequence. Therefore, this contribution proposes a vertical offset syntax element to indicate whether an additional offset to be used in the derivation of the vertical sample location of the reference layer pictures. The vertical offset syntax element is present only when the vertical decimation is in use.

# Overview of the proposal

Two flags are proposed for the slice segment header as follows:

| … |  |
| --- | --- |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| if ( vert_phase_position_enable_flag[ RefPicLayerId[ i ] ] ) { |  |
| vert_phase_position_flag[ RefPicLayerId[ i ]] | u(1) |
| vert_decimation_flag[ RefPicLayerId[ i ] ] | u(1) |
| if( vert_decimation_flag[ RefPicLayerId[ i ] ] ) |  |
| vert_offset_flag[ RefPicLayerId[ i ] ] | u(1) |
| } |  |

The plain English semantics of the proposed syntax elements are as follows (please refer to the accompanying specitication text to see the exact specification text language):

vert_decimation_flag equal to 1 causes the inter-layer resampling process to use every other sample row of the reference-layer picture.

vert_offset_flag equal to 0 and 1 cause the inter-layer resampling process to resample on the basis of even sample lines and odd sample lines, respectively, of the reference-layer picture (when sample lines are indexed starting from 0).

The following figure explains the impact of vert_decimation_flag equal to 1 visually, where the top field is resampled with vert_offset_flag equal to 0 and the bottom field is resampled with vert_offset_flag equal to 1.

Figure 3. Proposed inter-layer resampling with vert_decimation_flag equal to 1.

In addition to the syntax and semantics, the proposed specification text proposes:

- Modification in the derivation of the reference layer sample location (subclause H.6.2), which chooses the initial sample row on the basis of the target sample row in the resampled picture (the variable yP) and the value of the vert_offset_flag.

- In the luma and chroma sample interpolation processes (subclauses H.8.1.4.1.3 and H.8.1.4.1.4) every other sample row is skipped when vert_decimation_flag is equal to 1.

The detailed proposal is indicated with the change marks in the accompanying specification text document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).