| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0189 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0138 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On slice_temporal_mvp_enabled_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that there are several problems related to slice_temporal_mvp_enabled_flag. The proposal contains two parts reportedly fixing the asserted problems:

- The proposed slice segment header changes allow the presence of slice_temporal_mvp_enabled_flag in slice segment headers of IDR pictures of predicted layers and clarify that the bitstream constraints related to slice_temporal_mvp_enabled_flag apply only to independent layers.

- Similar bitstream constraints as those indicated by slice_temporal_mvp_enabled_flag equal to 0 for independent layers are proposed to be indicated for predicted layers by the proposed temporal motion vector constraints SEI message. The SEI message is also proposed to indicate whether the pictures associated with the SEI message may be used as collocated pictures for temporal motion vector prediction for any subsequent picture, in decoding order, and hence whether their motion fields need to be stored.

# Introduction

## Relevant parts of HEVC, MV-HEVC and SHVC

The syntax and semantics related to the slice_temporal_mvp_enabled_flag syntax element is specified as follows in the slice segment header of HEVC, and these parts have been kept unchanged in the MV-HEVC and SHVC specifications.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| … |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| … |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| … |  |

slice_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction. If slice_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no temporal motion vector predictor is used in decoding of the current picture. Otherwise (slice_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of slice_temporal_mvp_enabled_flag is inferred to be equal to 0.

When both slice_temporal_mvp_enabled_flag and TemporalId are equal to 0, the syntax elements for all coded pictures that follow the current picture in decoding order shall be constrained such that no temporal motion vector from any picture that precedes the current picture in decoding order is used in decoding of any coded picture that follows the current picture in decoding order.

NOTE 1 – When slice_temporal_mvp_enabled_flag is equal to 0 in an I slice, it has no impact on the normative decoding process of the picture but merely expresses a bitstream constraint.

NOTE 2 – When slice_temporal_mvp_enabled_flag is equal to 0 in a slice with TemporalId are equal to 0, decoders may empty "motion vector storage" for all reference pictures in the decoded picture buffer.

## Problem statement

It is asserted that there are the following problems related to slice_temporal_mvp_enabled_flag:

- In MV-HEVC/SHVC, IDR pictures in predicted layers may use B slice type and could therefore make use of TMVP too. For example, in the IPP inter-view prediction hierarchy, an IDR picture in the second P view could use a picture of the first P view as a collocated picture for TMVP. However, currently the syntax excludes the slice_temporal_mvp_enabled flag, when nal_unit_type indicates an IDR picture.

- slice_temporal_mvp_enabled_flag is used for two purposes, to indicate the use of TMVP in the prediction of the current picture and to specify a bitstream constraint. However, the bitstream constraint and the related informative note 3 are specific to a single-layer bitstream. It is unclear how the bitstream constraint and the note would be applied to a scalable bitstream.

- In enhancement layers it seems incorrect to couple the indication whether to use the TMVP mechanism within the current picture and the bitstream constraint whether the current and earlier pictures may be used as collocated pictures for TMVP in subsequent pictures, in decoding order. Particularly, an enhancement-layer picture may use TMVP (e.g. with an inter-layer reference picture as a collocated picture) while the subsequent pictures, in decoding order, do not use any earlier pictures, in decoding order, as collocated pictures for TMVP. In this case, slice_temporal_mvp_enabled_flag has to be equal to 1 to enable TMVP for the enhancement-layer picture, but no mechanism exists to indicate that the subsequent pictures, in decoding order, do not use any earlier pictures, in decoding order, as collocated pictures for TMVP.

# Proposal

The proposal contains two parts: The proposed slice segment header changes (Section 2.1) allow the presence of slice_temporal_mvp_enabled_flag in slice segment headers of IDR pictures of predicted layers and clarify that the bitstream constraints related to slice_temporal_mvo_enabled_flag apply only to independent layers. Similar bitstream constraints for predicted layers are proposed to be indicated by a new SEI message (Section 2.2), which also enables to indicate whether the motion fields of the current picture and the subsequent pictures need to be stored.

## Proposed slice segment header changes

It is proposed to allow the presence of slice_temporal_mvp_enabled_flag in slice segment headers of IDR pictures of predicted layers.

It is proposed to clarify the bitstream constraint and the respective informative notes related to slice_temporal_mvp_enabled_flag apply to each independent layer only.

The following changes in the specification text are proposed:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| … |  |
| } |  |
| if( ( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| … |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| … |  |

slice_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction. If slice_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no temporal motion vector predictor is used in decoding of the current picture. Otherwise (slice_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of slice_temporal_mvp_enabled_flag is inferred to be equal to 0.

Let currLayerId be equal to nuh_layer_id of the current NAL unit. When NumDirectRefLayers[ currLayerId ] is equal to 0, slice_temporal_mvp_enabled_flag is equal to 0 and TemporalId is equal to 0, the syntax elements for all coded pictures with nuh_layer_id equal to currLayerId that follow the current picture in decoding order shall be constrained such that no temporal motion vector from any picture that precedes the current picture in decoding order is used in decoding of any coded picture with nuh_layer_id equal to currLayerId that follows the current picture in decoding order.

NOTE 3 – When NumDirectRefLayers[ currLayerId ] is equal to 0 and slice_temporal_mvp_enabled_flag is equal to 0 in an I slice, it has no impact on the normative decoding process of the picture but merely expresses a bitstream constraint.

NOTE 4 – When NumDirectRefLayers[ currLayerId ] is equal to 0 and slice_temporal_mvp_enabled_flag is equal to 0 in a slice with TemporalId are equal to 0, decoders may empty "motion vector storage" for all reference pictures with nuh_layer_id equal to currLayerId in the decoded picture buffer.

## Proposed temporal motion vector constraints SEI message

It is proposed to specify a new SEI message to indicate whether the earlier pictures of a layer may be used as collocated pictures for TMVP of any subsequent picture, in decoding order, of the same layer. This functionality of the SEI message is the same as the bitstream constraint of indicated by slice_temporal_mvp_enabled_flag equal to 0 for independent layers. The indication can be used to empty the motion vector storage for the layer.

It is additionally proposed that the SEI message can indicate the motion fields of the current and subsequent pictures have to be stored, i.e. if there may be subsequent pictures using the current picture or the subsequent pictures as collocated pictures for TMVP.

The proposed syntax and semantics of the SEI message are as follows.

| temporal_motion_vector_prediction_constraints( payloadSize ) { | Descriptor |
| --- | --- |
| prev_pics_not_used_flag | u(1) |
| no_intra_layer_col_pic_flag | u(1) |
| } |  |

The temporal motion vector prediction constraints SEI message indicates constraints on collocated pictures for temporal motion vector prediction. The SEI message may be used to determine if the motion vectors of earlier pictures in decoding order need no longer be stored or if the motion vectors of this picture and subsequent pictures need to be stored.

When the message is not included in a scalable nesting SEI message, it applies to pictures with nuh_layer_id equal to that of the SEI NAL unit containing the message. When the message is included in a scalable nesting SEI message, it applies to the pictures with nuh_layer_id equal to all values of nestingLayerIdList[ i ] derived through the semantics of the containing scalable nesting SEI message. Let associatedLayerIdList be the list of nuh_layer_id values to which the temporal motion vector prediction constraints SEI message applies.

The temporal motion vector prediction constraints SEI message is a prefix SEI message. The temporal motion vector prediction constraints SEI message may be present for a picture with TemporalId equal to 0 and shall not be present for a picture with TemporalId greater than 0.

Let a set of access units associatedAuSet be the access units from the access unit containing the SEI message until the next access unit, in decoding order, that contains a temporal motion vector prediction constraints SEI message with an identical associatedLayerIdList to that of the present message, exclusive, or until the end of the CVS, exclusive, whichever is earlier in decoding order. The temporal motion vector prediction constraints SEI message applies to associatedAuSet.

Let currLayerId be equal to any single value of associatedLayerIdList.

prev_pics_not_used_flag equal to 1 indicates that the syntax elements for all coded pictures that follow the access unit containing the current picture in decoding order are constrained such that no temporal motion vector from any picture that has nuh_layer_id equal to any value in associatedLayerIdList and precedes the access unit containing the current picture in decoding order is used in decoding of any coded picture that follows the access unit containing the current picture in decoding order. prev_pics_not_used_flag equal to 0 indicates that the bitstream may or may not fulfill the constraints indicated by prev_pics_not_used_flag equal to.

NOTE – When prev_pics_not_used_flag is equal to 1, decoders may empty the "motion vector storage" for all reference pictures with nuh_layer_id equal to any value in associatedLayerIdList in the decoded picture buffer.

no_intra_layer_col_pic_flag equal to 1 indicates the following:

–	If NumDirectRefLayers[ currLayerId ] is equal to 0, slice_temporal_mvp_enabled_flag is not present or is equal to 0 in each picture with nuh_layer_id equal to currLayerId in associatedAuSet.

–	Otherwise, all the pictures with nuh_layer_id equal to currLayerId in associatedAuSet do not use temporal motion vector prediction or use collocated pictures with nuh_layer_id different from currLayerId.

When no_intra_layer_col_pic_flag is equal to 0, no constraint for the collocated picture of the pictures with nuh_layer_id equal to curreLayerId in associatedAuSet is indicated.

Let NoIntraLayerColPicFlag[ currLayerId ] be equal to no_intra_layer_col_pic_flag.

NOTE – The motion vectors of the current picture with nuh_layer_id equal to layerId have to be stored when they may be used for temporal motion vector prediction of other pictures in the same layer or when they may be used for inter-layer motion prediction. In other words, the motion vectors of the current picture have to be stored when at least one of the following is true:

- sps_temporal_mvp_enabled_flag in the active SPS for the current picture is equal to 1 and NoIntraLayerColPicFlag[ layerId ] is equal 0.

- NoIntraLayerColPicFlag[ layerId ] is equal to 1 and there is a nuh_layer_id value nuhLayerIdA such that VpsInterLayerMotionPredictionEnabled[ LayerIdxInVps[ nuhLayerIdA ] ][ LayerIdxInVps[ layerId ] ] is equal to 1.

NOTE – The motion vectors of a picture with nuh_layer_id equal to layerId need no longer be stored when the picture is marked as "unused for reference", or the picture is not used for temporal motion vector prediction of other pictures in the same layer and all pictures in the same access unit that may use the picture as a reference for inter-layer motion prediction have been decoded, or the access unit containing the picture precedes the current access unit in decoding order, where this SEI message is present with associatedLayerIdList including the nuh_layer_id of the picture and prev_pics_not_used_flag equal to 1. In other words, the motion vectors of a picture need no longer be stored when at least one of the following is true:

- The picture is marked as "unused for reference".

- NoIntraLayerColPicFlag[ layerId ] is equal to 1 for the picture and the access unit containing the picture has been decoded.

- The access unit containing the picture precedes the current access unit, in decoding order, and the current access unit contains this SEI message with associatedLayerIdList including the nuh_layer_id value of the picture and with prev_pics_not_used_flag equal to 1.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).