| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0192 |
| --- | --- |

| Title: | Insights and open questions on HDR/WCG video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | David Singer Alexandros Tourapis | Email: | singer@apple.com atourapis@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

There has recently been considerable interest within the MPEG and JCT-VC standardization communities [1]-[10] in supporting the encoding of High Dynamic Range (HDR) and Wide Color Gamut (WCG) content in codecs such as HEVC and its future extensions. This was driven, among others, by new developments in content capture and production, new displays, new standards, as well as the desire of providing a more realistic as well as dramatic visual experience to the user. Even though it is well understood that, it would likely be best to create a new video coding solution that best fits and is optimized based on such new content characteristics, it seems to be also highly desirable that simple, preferably only high level syntax extensions, are performed onto the existing HEVC specification. This may permit immediate deployment of HDR/WCG systems, which would be of substantial interest for the entire industry.

This document poses several questions that appear relevant to this investigation, i.e. in understanding whether it is possible to support the encoding of HDR/WCG content within the context of the already available coding tools and specified profiles in HEVC (and, indeed, other codecs such as AVC). The intention of this document is not to dismiss the rather comprehensive and well thought of work that is currently been undertaken in [10] within the context of the MPEG HDR/WCG Adhoc activity, but instead seeks to possibly accelerate some of these efforts. Some of the questions here, for example, may already be addressed in [10] or elsewhere, in one-way or another. Our contribution builds also on some of the efforts already done in [1][2][3][4][5] but without particularly insisting on a specific color space or electro-optical transfer function.

In general, we are asking “minimum perturbation” questions: What standards are already in place, are missing but are already in development, and what is completely missing and needs to be started for there to be a multi-vendor interoperability on this topic? How much can we fit into existing structures and decompose the problem into orthogonal, smaller, problems?

# Open Questions on HDR/WCG content distribution

Our investigation into HDR and WCG content and how such would likely fit within the existing core coding tools of HEVC has basically raised the following questions:

- We have classically coded in a de-correlated YUV or YCbCr space. Can this continue to work, or do we need a new representation such as XYZ or its de-correlated version under definition by SMPTE YDxDz? If so, what properties should these representations have, especially given that for consumer applications bitrates are far more constrained? Should such representations be based on the human visual system characteristics, for example? Is there evidence or are there simple experiments that can be conducted to provide such an answer?

- Are there aspects of our current codecs that take advantage of the fact that they are working in YCbCr and which would be less true if YDzDx or another space were used?  Can we continue to use the ‘guts’ of our existing codecs, at a suitable bit-depth, and just tag the output appropriately for display?

- How important are 4:2:2 and 4:4:4 chroma subsampling formats for HDR/WCG distribution? And even if subsampling to 4:2:0 is utilized, can more intelligent downsampling/upsampling be used to improve performance?

- Codecs have classically been tagged with a transfer function, color primaries, and matrix coefficients. Is there a set (or more) of these that is suitable to address this problem, or do we need to change to a different model?  If there is a set, is it already defined in the HEVC specification or do we need to extend the specification in supporting these? Examples include the ITU-R Recommendation BT.2020 [11], Sony’s S-Gamut [14], or P3 D60/DCI color primaries, or, in terms of transform functions, Sony’s S-Log electro-optical transfer function (EOTF), Dolby’s PQ EOTF [12] or the more recent BBC EOTF [13] that provides backward compatibility with the existing gamma function. What are the benefits of these, and is there any harm in specifying any of these now?

- If we adopt a new color space, or dynamic range, can we define it to be ‘parameterized’ for a given region/picture/sequence?  For example, instead of restricting the dynamic range to a fixed range, can and should we specify parameters that specify the peak brightness and black level that are encountered for a frame or sequence, and appropriately scale the transfer function only within that range, so we may make maximum use all the values provided by our bit-depth (e.g. 256 values for 8-bit)?

- What are the desired characteristics of a consumer HDR system? We probably need to separate discussion of dynamic range (contrast ratio), and the number of discernable differences, from discussion of brightness. We need to consider the dynamic range and the peak/minimum brightness we can achieve, as well as maybe the “sustained” brightness that a user would likely tolerate comfortably. The rate of change may also impact tolerance. Some people are susceptible to high-level stimuli especially when cycled (e.g. bright repeated flashes). Has anyone ever performed any such investigation including whether there could be health risks involved with such systems?

- Are there any differences that we should be considering for small factor mobile applications/displays versus large factor non-mobile displays? Do we wish to provide the same dynamic range across all devices and are there any limiting factors that one needs to consider? Are there differences in technology that are more appropriate for mobile displays or vice versa that relate to weight/size of the display and power consumption?

- Does the move to HDR and/or WCG effectively force us to go to a greater bit-depth to provide sufficient values across the desired dynamic range to avoid banding? What is the desired dynamic range for each application, and how is that determined? And if a greater bit-depth is required, what is that bit-depth value? Do we need to consider something more radical, such as floating point representations, making the existing codecs obsolete?

- Do we gain anything by using a scalable solution system, e.g. a system with a standard-dynamic range base layer and an HDR/WCG enhancement layer, compared to a single layer solution?  Would, in fact, our older systems, or systems with standard-range displays, benefit from having this base layer?  Or would it be easy enough for new systems to decode the HDR/WCG content and then map it as best they can to the display in hand?

- How is the post-processing chain of a display impacted by the consideration of HDR content? Do the existing algorithms/blocks that we have work as is, or do they need modifications? If so, how much modification do they need? Can our current approach to display adaptation – mapping from the decoded content to particular displays with different characteristics – continue to work?

- Do we need to consider compositing the video with other information such as overlays and captions? It is not just about how, but also about where.

- A natural question is what happens with content creation. Is it easy to provide those tools to the user (as well as to professional content creators)?

- How about still images? Should a universal strategy exist between both stills and video?

# Some Insights

Given some of the work that has already been done in this area, and especially the work presented in [1][2][3][4][5], it appears that maybe some of these questions could be answered to some extent. In particular, it has already been shown that HDR delivery could be achieved using the existing HEVC specification, by utilizing some of the HDR specific EOTFs, and minimal high-level syntax modifications. It might be sufficient for the deployment and adoption of consumer HDR systems in the near-term future. Even though we wouldn’t advocate the support of the XYZ color space for consumer applications, we would strongly recommend to the JCT-VC to consider adopting simple modifications in the HEVC specification, such as extending some of the available parameters in the VUI semantics, so as to better enable the consideration and distribution of HDR/WCG using this codec.

In particular, we would recommend

- extending the colour_primaries element to also indicate the P3 color space (with different white points such as D60 and D65 and not be limited to the DCI white point) and the XYZ color space, if it is so desired,

- supporting the various HDR targeted EOTFs in the transfer_characteristics element,

- and, if needed, extending the matrix_coeffs element accordingly.

In essence, it appears to us that codecs using these values, and specifically HEVC, may, with slight modifications and suitable choice of other parameters (e.g. bit-depth), already be capable of supporting HDR/WCG encoding in their current form. It is therefore recommended, even though further investigation on the optimal encoding of such content is advised, that appropriate actions are taken by the JCT-VC to enable this very important application.

# Conclusions

In this contribution, we have presented several open questions that relate to the encoding, distribution, and consumption of HDR/WCG content in the context of MPEG video-coding standards. It was also recommended that certain actions are considered by the JCT-VC so as to appropriately enable support of such content using the existing HEVC specification.

# References

- C. Fogg and J. Helman, “Indication of SMPTE 2084 and 2085 and carriage of 2086 metadata in HEVC”, JCTVC-P0084, 16th Meeting: San José, US, 9–17 Jan. 2014

- B. Mandel, C. Fogg, and J. Helman, “HDR coding results for FruitStall and Oblivion”, JCTVC-P0080, 16th Meeting: San José, US, 9–17 Jan. 2014

- B. Mandel, C. Fogg, and J. Helman, “High dynamic range video coding results”, JCTVC-O0101, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013

- C. Fogg, J. Helman, and B. Mandel, “Profiles for Extended Precision”, JCTVC-O0105, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013

- C. Fogg, “VUI color description set for XYZ”, JCTVC-O0106, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013

- H. Basse, W. Aylsworth, S. Stephens, M. DeValue, R. Kisor, B. Mandel, J. Helman, A. Luthra, and W. Wan, “Proposed Standardization of XYZ Image”, M30167, 105th Meeting, Vienna, AT, 29 Jul. – 2 Aug. 2013

- A. Luthra, A. Chalmers, A. Artusi, K. Debattista, and M. Agostine, “Requirements for Higher Dynamic Range (HDR) and Wider Color Gamut video coding”, M31825, 107th Meeting: San Jose, US, 13-17 Jan. 2014

- E.François, S.Lasserre, F.LeLeannec, and Y.Chen, “Use cases and requirements for HDR and WCG video coding”, M31956, 107th Meeting: San Jose, US, 13-17 Jan. 2014

- H. Thoma, “On chroma subsampling for HDR video”, M32222, 107th Meeting: San Jose, US, 13-17 Jan. 2014

- A. Luthra, E. François, W. Husak, “Draft requirements for Higher Dynamic Range (HDR) and Wide Color Gamut (WCG) video coding for Broadcasting, OTT, and Storage Media”, M32716, 107th Meeting: Valencia, ES, 31 Mar.-4 Apr. 2014

- "BT.2020 : Parameter values for ultra-high definition television systems for production and international programme exchange". International Telecommunication Union. 2012-08-23.

- S. Miller, M. Nezamabadi, S. Daly, “Perceptual Signal Coding for More Efficient Usage of Bit Codes”, SMPTE Journal, Vol. 122, No. 4, May-June 2013.

- British Broadcasting Corporation (BBC), “Proposed OETF/EOTF transfer characteristics for high dynamic range signals compatible with the conventional gamma curve”, Document 6C/276E, ITU-R Radiocommunication Study Groups, March 5, 2014

- H. Gaggioni, P. Dhanendra, J. Yamashita, N. Kawada, K. Endo, and C. Clark, “S-Log: A new LUT for digital production mastering and interchange applications”, http://pro.sony.com/bbsccms/assets/files/mkt/cinema/solutions/slog_manual.pdf

# Patent rights declaration(s)

Apple Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The XYZ color space seems to be quite sensitive to errors induced by the quantization process.

This appears to only be necessary if the YdZdX color space is considered for any application.