| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0193 |
| --- | --- |

| Title: | Combination of screen content coding proposals JCTVC-Q0034/JCTVC-Q0176 and JCTVC-Q0036 |  |
| --- | --- | --- |
| Status: | Input Document to JCT-VC |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Robert Cohen, Xingyu Zhang, Anthony Vetro, Kazuo Sugimoto Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139, U.S.A Akira Minezawa, Kazuyuki Miyazawa, Shun-ichi Sekiguchi, Tokumichi Murakami Information Technology R&D Center 5-1-1, Ofuna, Kamakura, JAPAN Zhan Ma, Wei Wang, Meng Xu, Xian Wang, Haoping Yu 2330 Central Expy, Santa Clara, CA 95050, United States | +1-617-621-7500 cohen@merl.com, xzhang@merl.com, avetro@merl.com, sugimoto@merl.com +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp, Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp, Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp, Murakami.Tokumichi@eb.MitsubishiElectric.co.jp +1 408 330 5142 zhan.ma@huawei.com, haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) and Mitsubishi Electric Corporation |  |

_____________________________

# Abstract

This document proposes a combination of screen content coding proposals JCTVC-Q0034/JCTVC-Q0176 and and JCTVC-Q0176. Using HEVC Range Extensions Draft 6 and the HM-13.0_RExt-6.0 software as a foundation, this combined proposal adds a color table and index map coding mode, block-level inter-component prediction, a histogram correction mode for Sample Adaptive Offset, and an independent uniform prediction mode. Using the same test conditions as specified in the Joint Call for Proposals for Coding of Screen Content, with HM 12.1_RExt 5.1 coded bitstreams as anchors, average improvements in BD-Rate for various content types under the lossy All Intra (AI) configuration are up to 39% for RGB sequences and up to 34% for the Y component of YCbCr. For Random Access (RA) conditions, the corresponding averages were up to 26% for RGB and 21% for YCbCr, and for Low Delay (LB) conditions, the averages were up to 18% for RGB and 13% for YCbCr. For lossless conditions , averaged bit rate reduction is up to 45%, 46% for AI RGB and AI YCbCr, 39% and 36% for RA RGB and RA YCbCr, and 38% and 32% for LB RGB and LB YCbCr, respectively.

# Combination of proposals JCTVC-Q0034/JCTVC-Q0176 and JCTVC-Q0036

Documents JCTVC-Q0034 [1] and JCTVC-Q0036 [2] were both submitted as responses to the Joint Call for Proposals for Coding of Screen Content (SCC CfP) [4]. Both proposals contain new tools that improve the performance of HEVC Range Extensions 5 [5] and 6 [6] when used to code screen content video. In addition, the improvements to JCTVC-Q0034 proposed in JCTVC-Q0176 [3] are also integrated into this proposal. A list of tools for the separate and combined proposals is shown in Table 1. The new tools shown in bold text under the individual proposals were used in the combined proposal.

Table 1. New tools in separate and combined proposals

| JCTVC-Q0034/JCTVC-Q0176 | JCTVC-Q0036 | Combined proposal |
| --- | --- | --- |
| Color table and index map mode | Palette mode | Color table and index map mode |
| Inter-component prediction | Inter-component prediction | Inter-component prediction |
|  | Histogram Correction for SAO | Histogram Correction for SAO |
|  | Independent Uniform Prediction | Independent Uniform Prediction |
| Also includes RExt 5.1 tools | Also includes RExt 6.0 tools | Also includes RExt 6.0 tools |

# Summary of proposed tools

This section gives a basic idea on what each new tool does. Full details on can be found in JCTVC-Q0034/JCTVC-Q0176 and JCTVC-Q0036.

Color table and index map mode: This mode mainly contains two parts: color table processing and the corresponding color index map processing. For each CU, a color table is derived from itself or from a neighboring CU. The three components packed together are then translated into a color index map, which is scanned and coded using a string search method.

Inter-component prediction: For each PU, this mode performs a prediction among the collocated input components. The second component is predicted from the first, and the third component is predicted either from the first or second components.

Histogram Correction for SAO: This mode targets subjective improvements by adding a Histogram Correction mode to Sample Adaptive Offset process. Histograms are computed for each block, and pixel values inside a window near strong peaks are replaced with the peak pixel value. The size of the window and number of peaks is optimized in the encoder.

Independent Uniform Prediction: As an additional mode to the existing inter and intra prediction modes, this Independent Uniform Prediction mode computes a prediction block in which all pixels have the same color within each component. For this proposal, the encoder signals two color triplets at the slice level, and then a flag selecting one of those triplets is signaled at the CU level.

Additional RExt 6.0 tools [7]: The anchors for the SCC CfP were based on the HM-12.1_RExt-5.1 software. This combined proposal was built upon HM-13.0_RExt-6.0, without modification to the settings in the configuration files included in HM-13.0_RExt-6.0 release. This combined proposal therefore includes the tools in RExt-6.0 that provide additional improvements when coding screen content material. Some examples of these tools include Transform Skip improvements, cross-component prediction, the rotation of Transform Skip blocks, and RDPCM.

# Compression performance

The results reported in here use the templates and measurement methodologies specified by the SCC CfP [1]. After the CfP was issued, proponents were asked to clip each picture’s PSNR to 58.9226 dB, given that with screen content material some pictures under lossy conditions may still be coded losslessly. In this document and the accompanying spreadsheets, both the anchor and proposed per-picture PSNR values were clipped to 58.9226 dB prior to averaging.

## Lossy test condition

Table 2 shows the simulation results of this combined proposal under lossy test condition defined in the SCC CfP.

Table 2. The simulation results under lossy test conditions

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -38.4% | -39.6% | -39.0% |
| RGB, text & graphics with motion,720p | -28.2% | -27.7% | -28.2% |
| RGB, mixed content, 1440p | -13.8% | -13.8% | -13.9% |
| RGB, mixed content, 1080p | -15.5% | -16.3% | -16.1% |
| RGB, Animation, 720p | -8.4% | -10.7% | -8.3% |
| YUV, text & graphics with motion, 1080p | -34.4% | -36.8% | -37.0% |
| YUV, text & graphics with motion,720p | -18.8% | -25.9% | -28.7% |
| YUV, mixed content, 1440p | -11.7% | -17.5% | -17.3% |
| YUV, mixed content, 1080p | -13.6% | -18.2% | -17.7% |
| YUV, Animation, 720p | -1.9% | -6.4% | -5.8% |
| Enc Time[%] | 307% |  |  |
| Dec Time[%] | 88% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -24.0% | -25.4% | -24.9% |
| RGB, text & graphics with motion,720p | -26.4% | -25.5% | -26.3% |
| RGB, mixed content, 1440p | -10.8% | -10.7% | -10.8% |
| RGB, mixed content, 1080p | -9.6% | -10.0% | -9.8% |
| RGB, Animation, 720p | -4.1% | -5.7% | -4.6% |
| YUV, text & graphics with motion, 1080p | -21.1% | -26.5% | -26.4% |
| YUV, text & graphics with motion,720p | -17.4% | -24.5% | -28.8% |
| YUV, mixed content, 1440p | -9.0% | -18.6% | -17.8% |
| YUV, mixed content, 1080p | -8.6% | -14.4% | -13.7% |
| YUV, Animation, 720p | -0.7% | -4.9% | -4.7% |
| Enc Time[%] | 156% |  |  |
| Dec Time[%] | 132% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -17.3% | -18.5% | -17.9% |
| RGB, text & graphics with motion,720p | -20.3% | -19.3% | -20.1% |
| RGB, mixed content, 1440p | -7.7% | -7.5% | -7.6% |
| RGB, mixed content, 1080p | -6.3% | -6.2% | -6.1% |
| RGB, Animation, 720p | -1.6% | -2.3% | -1.6% |
| YUV, text & graphics with motion, 1080p | -13.1% | -17.3% | -17.5% |
| YUV, text & graphics with motion,720p | -10.7% | -17.4% | -21.5% |
| YUV, mixed content, 1440p | -6.1% | -15.8% | -15.2% |
| YUV, mixed content, 1080p | -4.8% | -9.1% | -8.3% |
| YUV, Animation, 720p | 0.0% | -2.6% | -2.4% |
| Enc Time[%] | 136% |  |  |
| Dec Time[%] | 134% |  |  |

### Mathematically lossless test condition

Table 3 shows the simulation results of this combined proposal under lossless test conditions defined in the SCC CfP.

Table 3. The simulation results under lossless test conditions

|  | AI |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 42.3% | 44.9% | 31.4% | 59.7% |
| RGB, text & graphics with motion,720p | 23.6% | 24.4% | 4.3% | 44.4% |
| RGB, mixed content, 1440p | 9.0% | 9.3% | 7.8% | 10.8% |
| RGB, mixed content, 1080p | 13.9% | 12.8% | 10.2% | 15.4% |
| RGB, Animation, 720p | 4.5% | 4.5% | 4.5% | 4.5% |
| YUV, text & graphics with motion, 1080p | 43.4% | 46.4% | 28.2% | 62.7% |
| YUV, text & graphics with motion,720p | 22.4% | 24.3% | 3.1% | 50.4% |
| YUV, mixed content, 1440p | 5.5% | 6.1% | 2.6% | 9.6% |
| YUV, mixed content, 1080p | 10.8% | 10.0% | 8.1% | 11.9% |
| YUV, Animation, 720p | 2.7% | 2.7% | 2.7% | 2.7% |
| Enc Time[%] | 329% |  |  |  |
| Dec Time[%] | 142% |  |  |  |
|  | RA |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 27.9% | 39.2% | 26.5% | 56.5% |
| RGB, text & graphics with motion,720p | 10.0% | 19.8% | 7.0% | 44.0% |
| RGB, mixed content, 1440p | 8.8% | 8.8% | 8.0% | 9.6% |
| RGB, mixed content, 1080p | 12.3% | 10.8% | 9.1% | 12.6% |
| RGB, Animation, 720p | 7.3% | 7.3% | 7.3% | 7.3% |
| YUV, text & graphics with motion, 1080p | 21.7% | 35.7% | 19.6% | 52.6% |
| YUV, text & graphics with motion,720p | 6.9% | 17.7% | 2.1% | 49.6% |
| YUV, mixed content, 1440p | 2.1% | 2.1% | 1.7% | 2.5% |
| YUV, mixed content, 1080p | 7.3% | 5.0% | 2.3% | 7.6% |
| YUV, Animation, 720p | 2.0% | 2.0% | 2.0% | 2.0% |
| Enc Time[%] | 149% |  |  |  |
| Dec Time[%] | 137% |  |  |  |
|  | LB |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 27.6% | 37.9% | 26.8% | 54.2% |
| RGB, text & graphics with motion,720p | 9.9% | 19.5% | 7.3% | 41.9% |
| RGB, mixed content, 1440p | 9.4% | 9.4% | 8.5% | 10.3% |
| RGB, mixed content, 1080p | 12.1% | 10.9% | 9.6% | 12.3% |
| RGB, Animation, 720p | 7.7% | 7.7% | 7.7% | 7.7% |
| YUV, text & graphics with motion, 1080p | 19.9% | 31.7% | 18.5% | 44.0% |
| YUV, text & graphics with motion,720p | 5.8% | 16.0% | 1.9% | 46.8% |
| YUV, mixed content, 1440p | 2.0% | 2.0% | 1.4% | 2.5% |
| YUV, mixed content, 1080p | 7.2% | 4.5% | 1.6% | 7.5% |
| YUV, Animation, 720p | 1.9% | 1.9% | 1.9% | 1.9% |
| Enc Time[%] | 141% |  |  |  |
| Dec Time[%] | 140% |  |  |  |

# Summary and conclusions

This document proposes a combination of screen content coding proposals JCTVC-Q0034 and JCTVC-Q0036, including additional improvements from JCTVC-Q0176. The combined proposal adds a color table and index map coding mode, block-level inter-component prediction, a histogram correction mode for Sample Adaptive Offset, and an independent uniform prediction mode. Using the test conditions specified in the Joint Call for Proposals for Coding of Screen Content, this combined proposal yields average improvements in BD-Rate for various content types under the lossy All Intra configuration of up to 39% for RGB sequences and up to 34% for the Y component of YCbCr. For Random Access conditions, the corresponding averages were up to 26% for RGB and 21% for YCbCr, and for Low Delay (LB) conditions, the averages were up to 18% for RGB and 13% for YCbCr. For lossless conditions, averaged bit rate reductions are up to 45% and 46% for AI RGB and YCbCr, 39% and 36% for RA RGB and YCbCr, and 38% and 32% for LB RGB and YCbCr, respectively. This combined proposal provides additional improvements in compression efficiency above that already achieved by the separate proposals.

# Patent rights declaration(s)

Huawei Technologies (USA), may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Z. Ma et al., “Description of screen content coding technology proposal by Huawei Technologies (USA),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0034, 17th Meeting: Valencia, ES, March-April 2014.

- R. Cohen et al., “Description of screen content coding technology proposal by Mitsubishi Electric Corporation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0036, 17th Meeting: Valencia, ES, March-April 2014.

- W. Wang et al., “AHG8: String match in coding of screen content,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0176, 17th Meeting: Valencia, ES, March-April 2014.

- “Joint Call for Proposals for Coding of Screen Content,” ISO/IEC JTC1/SC29/WG11, MPEG2014/N14175, San José, US, January, 2014.

- D. Flynn, J. Sole, G. Sullivan and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 5,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-O1005, 15th Meeting: Geneva, CH, October/November, 2013.

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. Sullivan and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 6,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P1005, 16th Meeting: San José, US, January, 2014.

- HM RExt software, available at https://hevc.kw.bbc.co.uk/svn/jctvc-hm/branches/HM-range-extensions