| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0196 |
| --- | --- |

| Title: | SCE1: Crosscheck result of Test 1.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document reports the crosscheck result of SCE1: Test 1.3.

# Introduction / Problem Statement / …

Proposals on color gamut and bit-depth scalability have been studied under SCE1. Three methods are proposed by JCTVC-P0124 as follow:

- Region-based

- Matrix-based

- Polynominal-based

and the following configurations are being tested under SCE1:

Configuration 1: N=8, linear matrix mapping (YUV colorspace is divided into 8x8x8 regions)
Configuration 2: N=1, linear matrix mapping (Region based WP is disabled and only difference to HEVC WP is linear matrix based equations)
Configuration 3: N=8, no matrix based WP (Matrix based WP is disabled and only difference to HEVC WP is signaling different WP parameters for different regions)
Configuration 4: N=1, polynomial matrix mapping enabled (Region based WP is disabled and only difference to HEVC WP is polynomial matrix based equations)

This document crosscheck result of Configuration 3 as described above.

# Simulation

## Simulation Condition

SHM5.0 including the bit-depth scalability feature and the option to set the weighted prediction restricted to inter-layer prediction, as adopted in JCTVC-O0194, is used as anchor. It is equipped with functionality of SPS/PPS insertion at regular period insertion, but when creating anchor data this functionality is turned-off. The anchors will be based on SHM5.0 with WP restricted to inter layer prediction (ILP).

The proposed method has been implemented by the proponent and provided to the author. The implementation has been verified, but the following problems are found:

- The values of some parameters are fixed as 1024. It means that the src accepts only 10-bit input for the enhancement layer. At least 8-bit input should be able to be accepted toward the finalization of SHVC, and if future SHVC based on RExt are standardized more than or equal to 12-bit input should also be accepted.

- Reconstructed images with 2x scalability test (including bit-depth scalability) do not match at the encoder and the decoder. This problem originally existed in the anchor software, but src to fix the problem has been provided by the SCE coordinator on February 25. This fix has not been reflected to the src of the proposal.

{AI,RA}_{1x,2x} conditions as specified by JCTVC-P1101 are tested.

## Simulation Result

Summary of the result is shown in Table 2.2.1. Complete result is shown in JCTVC-Q0196.doc. The result matched the one provided by the proponent.

Of the proposals in JCTVC-P0124, only Configuration 3 does not require cross-color component processing. It is already adopted in HEVC RExt, but it can be processed by block-basis. Methods proposed in JCTVC-P0124 requires cross-color component processing by pixel-basis, which requires more computational complexity.

Table 2.2.1

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -5.2% | -6.1% | -12.0% | -5.1% | -4.4% | -9.8% |
| Overall (Test vs Ref) | -5.2% | -6.1% | -12.0% | -5.1% | -4.4% | -9.8% |
| Overall (Test vs single layer) | 25.3% | 23.7% | -1.3% | 14.9% | 19.3% | 7.9% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -21.4% | -21.7% | -27.2% | -10.7% | -9.6% | -15.0% |
| Overall (Test EL+BL vs single EL+BL) | -33.4% | -34.7% | -48.5% | -24.6% | -21.8% | -30.2% |
| Enc Time[%] | 94.8% |  |  | 102.5% |  |  |
| Dec Time[%] | 105.9% |  |  | 128.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -6.9% | -8.3% | -13.8% | -4.3% | -4.8% | -9.4% |
| Overall (Test vs Ref) | -6.9% | -8.3% | -13.8% | -4.3% | -4.8% | -9.4% |
| Overall (Test vs single layer) | 34.9% | 36.2% | 9.0% | 23.8% | 27.1% | 15.5% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.3% | 27.6% |
| EL only (Test vs Ref) | -21.3% | -22.0% | -26.8% | -8.3% | -8.4% | -13.0% |
| Overall (Test EL+BL vs single EL+BL) | -26.9% | -26.3% | -40.8% | -17.6% | -13.9% | -22.8% |
| Enc Time[%] | 67.9% |  |  | 66.2% |  |  |
| Dec Time[%] | 116.6% |  |  | 141.6% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

This document reports the crosscheck result of SCE1: Test 1.3. Result matched the one provided by the proponent, but a few problems with srcs are reported.

Of the proposals in JCTVC-P0124, only Test 1.3 does not require cross-color component processing by pixel-basis, which requires computational complexity.

It is recommended that the proposed method be discussed among other proposals, both in the aspects of coding efficiency and complexity.

# References

[1] “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-P1101, Jan 2014

[2] “Non-SCE1: Color gamut scalability using modified weighted prediction”, JCTVC-P0124, Jan 2014

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.