| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0197 |
| --- | --- |

| Title: | Cross-check for increased precision for deriving the reference layer sample location |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | E.Alshina | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd |  |  |

_____________________________

# Abstract

This is report theoretically analyses potential benefits increased precision for deriving the reference layer sample location.

# Proposed increased precision for deriving the reference layer sample location

ARRIS’s contribution [1] proposes to modify the reference layer sample location computations in order to provide accurate phase offset calculations as follows.

- H.8.1.4	Resampling process for inter-layer reference pictures

The variables shiftX and shiftY are defined as:

shiftX  = 31 – Ceil( Log2 (RefLayerPicWidthInSamplesY));

shiftY  = 31 – Ceil( Log2 (RefLayerPicHeightInSamplesY));

The variables ScaleFactorX and ScaleFactorY are derived as follows:

ScaleFactorX = ( ( RefLayerPicWidthInSamplesY << 16 shiftX ) + ( ScaledRefLayerPicWidthInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicWidthInSamplesY		(H-1)
ScaleFactorY = ( ( RefLayerPicHeightInSamplesY << 16 shiftY ) + ( ScaledRefLayerPicHeightInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicHeightInSamplesY		(H-2)

H.6.1	Derivation process for reference layer sample location

Input to this process is a luma location ( xP, yP ) relative to the top-left luma sample of the current picture.

Output of this process is a luma location ( xRef, yRef ) relative to the top-left luma sample of the reference layer picture.

The variables xRef and yRef are derived as follows:

xRef = ( ( xP − ScaledRefLayerLeftOffset ) * ScaleFactorX + ( 1 << 15 (shiftX – 1) ) ) >> 16 shiftX 	(H-1)
yRef = ( ( yP − ScaledRefLayerTopOffset ) * ScaleFactorY + ( 1 << 15 (shiftY – 1) ) ) >> 16 shiftY 	(H-2)

H.6.2	Derivation process for reference layer sample location used in resampling

The variables phaseX, phaseY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-3)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-4)

addX = ( ScaleFactorX * phaseX + 2 ) >> 2 		(H-5) 
addY = ( ScaleFactorY * phaseY + 2 ) >> 2 		(H-6)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX + addX + ( 1 << 11 (shiftX – 5)  ) ) >> 12 (shiftX – 4) ) − ( phaseX << 2 )	(H-7)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY + addY + ( 1 << 11 (shiftY– 5) ) ) >> 12 (shiftY – 4) ) − ( phaseY << 2 )	(H-8)

# Analysis

The precise reference sample location in (H-1) – (H-2) would be

xRef_double =(double) xP *(double) ScaledRefLayerPicWidthInSamplesY / (double) ScaledRefLayerPicWidthInSamplesY                                                                                                                       (1)

yRef_double =(double) yP *(double) ScaledRefLayerPicHeighInSamplesY / (double) ScaledRefLayerPicHeighInSamplesY                                                                                                                       (2)

if division is done w/o accuracy loss.

We have analyzed the accuracy of reference sample location calculation from current SHVC draft  and proposed in [JCTVC-Q0122]  for different spatial ratio and picture sizes.

Results are summarized in Table 1.

Table 1. Summary of test results.

| # ELsize | # BLsize | # MaxError (max(abs(·)/N) |  | # Comments |
| --- | --- | --- | --- | --- |
|  |  | # SHVC | # Q0122 |  |
| # 2560 | # 1280 | # 0.0000 | # 0.0000 | # classA x2 |
| # 1600 | # 800 | # 0.0000 | # 0.0000 |  |
| # 1920 | # 1280 | # 0.0208 | # 0.0208 | # classB x1.5 |
| # 1080 | # 720 | # 0.0208 | # 0.0208 | # classB x1.5 |
| # 1920 | # 960 | # 0.0000 | # 0.0000 | # classB x2 |
| # 1080 | # 540 | # 0.0000 | # 0.0000 |  |
| # 3840 | # 1920 | # 0.0000 | # 0.0000 | # SCE1 x2 |
| # 2160 | # 1080 | # 0.0000 | # 0.0000 |  |
| # 2560 | # 1456 | # 0.0438 | # 0.0313 | # Arbitrary ratio SCE |
| # 1920 | # 1096 | # 0.0333 | # 0.0292 |  |
| # 3840 | # 2560 | # 0.0417 | # 0.0208 | # 4K x1.5 |
| # 7680 | # 3840 | # 0.0000 | # 0.0000 | # 8K x2 |
| # 7680 | # 5120 | # 0.0625 | # 0.0208 | # 8K x1.5 |
| # 24576 | # 36864 | # 0.1458 | # 0.1458 | # >>8K x1.5 |

This is clear form Table 1 that for both common test condition and SCE1 tests. Even for 8K size and ratio x2 results of SHVC draft and JCTVC-Q0122 are identical. There is a difference in reference sample position calculation for 8K x1.5 ratio and resolutions used in SCE on arbitrary scalability ratio. Even for the cases JCTVC-Q0122 reduces an error in reference sample position calculation compare to current SHVC draft, but

- Not illuminate entirely

- Sometimes there is no error reduction

- Reduction is not significant

Figure 1 shows typical example of error in reference sample position calculation black dots represent SHVC draft, red represent JCTVC-Q0122

Fig. 1. Error in reference sample position calculation for SHVC draft and JCTVC-Q0122 (25601456).

# Conclusion

JCTVC-Q0122 reduces error in reference sample position calculation, but not illuminate entirely. In many application cases (including very high resolution video) both JCTVC-Q0122 and current SHVC draft produce identical results. In particular modification proposed in JCTVC-Q0122 doesn’t affect common test conditions or SCE1 results.

# Reference

- K. Minoo, D. Baylon(ARRIS), “Increased precision for deriving the reference layer sample locations,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0122, Mar. 2014

# Patent rights declaration(s)

Samsung Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).