| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0203 |
| --- | --- |

| Title: | AhG13: Methodology for evaluation an accuracy of re-sampling process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | proposal |  |  |
| Author(s) or Contact(s): | E.Alshina | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd |  |  |

_____________________________

# Abstract

Improvement for accuracy in re-sampling was reported in different contributions listed in AhG13 report. This document describes the methodology for re-sampling process accuracy evaluation.

# Introduction

Re-sampling process in SHVC draft uses integer implementation which approximates division operation. An error of this integer re-sampling process depends on scaling factor, which is affected by for example scaling reference off-set parameters and cropping window size of reference level. The precision of reference position calculation in SHVC draft is 1/16 of inter-samples value.

Verification of performance impact of various contributions proposing re-sampling process verification is hard task, since available test content is limited.

Instead running performance tests it is suggested to measure the difference between results of reference position calculation in integer implementation and true reference position value calculated with division.

Excel template attached to this contribution was circulated between BoG [1] on phase adjustment participants. This tool illustrates the accuracy of re-sampling process. User may set size of reference layer and current layer picture size in cells “N3” and “N1” correspondently. Other parameters which affect re-sampling process also can be specified in column “N”. Colum “A” contains integer positions in current layer which should be mapped to reference layer samples. There is result of integer re-sampling process specified in JCTVC-Q0200 [2] in column “D”. Column “E” contains the result of “true” reference position calculation performed with division, w/o accuracy loss. Blue line in a graph represents the difference between integer and “true” reference sample position derivation for JCTVC-Q0200 [2]. This error is zero for given spatial scalability case 540p720p conversion. For arbitrary ratio SHVC re-sampling process error is not zero, by modifying values in “N3” and “N1” cells user might investigate the error of SHVC re-sampling process.

As an example the combination JCTVC-Q0159 [3] which suggests the ROI extraction is investigated in attached excel template. In cells “Q13” and “Q14” start and end position for ROI is specified. Column “H” is result of integer mapping between ROI and current layer picture and column “I” is results of this mapping w/o accuracy loss (with division). Start position of ROI should be even number, end position is odd number. For ROI extraction process scaling ratio is generally speaking arbitrary and so there is non -zero error in reference position calculation. Red line in a graph illustrates this error. As use can see this error is within 1/16=0,0625 (precision allowed by SHVC specification), so the performance of SHVC if ROI extraction feature described in [3] is expected to be on a level reported in core experiment on arbitrary scalability ratio support [4].

Fig. 1. Accuracy of reference sample calcuation.

Another excel template attached to [2] compares the accuracy of 2 re-sampling processes which use decoded and cropping picture sizes for scaling factor calculation. In that example one re-sampling process has zero error but another has an error which reaches 0,5 in magnitude. Performance comparison done in [5] reports that this 0,5 error in re-sampling process results in ~9% performance gain.

Proposed technology for evaluation of accuracy in reference position calculation allows judgement about precision comparison w/o running actual simulation. If reference sample position calculation doesn’t increase 1/16 allowed in SHVC draft then no surprising affect in performance test is expected. But error in reference position calculation essentially higher than 1/16 indicates the problem in re-sampling process. If re-sampling process problem is illustrated in this simple way there is a need for BD-rate performance test to verify performance impact.

# Example of gerenalized phase adjustment

BoG [1] was mandated to work-out generic solution which allows phase signalling which adjusts re-sampling process for various application.

Instead of deriving phases from system of flags explicit phase signalling is possible.

Resampling process suggested in [2] is described below:

xRef16 = ( ( ( xP – offsetX) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) – ( phaseX << 2 ) +

+(SubWidthC *conf_win_left_offset* (( cIdx = = 0)  ?    SubWidthC:1)<<4)

yRef16 = ( ( ( yP − offsetY) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) −  ( phaseY << 2 ) ) +

+(SubHeightC *conf_win_top_offset* (( cIdx = = 0)  ?    SubWidthC:1)<<4)

addX=(( ScaleFactorX * phaseX + 2 ) >> 2)

addY=(( ScaleFactorY * phaseY + 2 ) >> 2)

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)

These formulas can be re-written

xRef16 =

(((xP − offsetX) * ScaleFactorX + (( ScaleFactorX * phaseX                      + 2 ) >> 2) +

( 1 << 11 ) ) >> 12 ) − ( phaseX << 2 ) – ((conf_win_left_offset[ rLId ] * (( cIdx = = 0)  ? SubWidthC:1))<<4)

yRef16 = (((yP − offsetY) * ScaleFactorY + (( ScaleFactorY * phaseY                      + 2 ) >> 2) +

( 1 << 11 ) ) >> 12 ) −( phaseY << 2 ) – ((conf_win_top_offset [ rLId ]* ( cIdx = = 0)  ? SubHeightC:1))<<4)

xRef16 = ( ( ( xP – offsetX) * ScaleFactorX + (( (ScaleFactorX   –  ( 1 << 16 )) * phaseX + 2 ) >> 2)   +( 1 << 11 ) ) >> 12 )  – ((conf_win_left_offset[ rLId ] * (( cIdx = = 0)  ? SubWidthC:1))<<4)

yRef16 = ( ( ( yP − offsetY) * ScaleFactorY + ((( ScaleFactorY   − ( 1 << 16 )) * phaseY + 2 ) >> 2)  + ( 1 << 11 ) ) >> 12 )  – ((conf_win_top_offset [ rLId ]* ( cIdx = = 0)  ? SubHeightC:1))<<4)

And modified re-sampling process looks as follows:

xRef16 = ( ( ( xP – offsetX) * ScaleFactorX +AddX   +( 1 << 11 ) ) >> 12 )   –

((conf_win_left_offset[ rLId ] * (( cIdx = = 0)  ? SubWidthC:1))<<4)

yRef16 = ( ( ( yP − offsetY) * ScaleFactorY +AddY + ( 1 << 11 ) ) >> 12 )  –

((conf_win_top_offset [ rLId ]* ( cIdx = = 0)  ? SubHeightC:1))<<4)

with new variables which only depending of phases:

AddX  =  (((ScaleFactorX   –  ( 1 << 16 )) * phaseX + 2 ) >> 2)

AddY  =  ((( ScaleFactorY   − ( 1 << 16 )) * phaseY + 2 ) >> 2)

From this derivation it seems that only 4 values phaseX(Luma), phaseY (Luma), values phaseX(Chroma), phaseY (Chroma) are sufficient to specify re-sampling process. Precision of phaseX(Luma), phaseY (Luma), values phaseX(Chroma), phaseY (Chroma) is subject for further study. Table 1 maps phaseX(Luma), phaseY (Luma), values phaseX(Chroma), phaseY (Chroma) values and combinations of syntax elements specifying re-sampling process in current SHVC draft spec

Table 1 Mappimg between generic phases to be signalled and combinations of existing syntax elements specifying re-sampling process.

| cross_layer_phase_alignment_flag | VertPhasePositionAdjustFlag | VertPhasePositionFlag | phaseX (L) | phaseY (L) | phaseX (Ch) | phaseY (Ch) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 2 | 1 | 1 | 2 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 2 | 0 | 2 |
| 1 | 1 | 0 | 2 | 0 | 1 | 0 |
| 1 | 1 | 1 | 2 | 2 | 1 | 2 |

.

It is suggested to study described in this section generic solution using methodology described in section 1.

# Suggestions

Use proposed technology for evaluation of accuracy in reference position calculation in future AhG on re-sampling process study.

# References

[1] JCTVC-Q0234 BoG report on SHVC upsampling process and phase offset [J. Boyce]

[2] JCTVC-Q0200 Re-sampling process modification considering cropping information [E.Alshina (Samsung), J.Chen (Qualcomm)]

[3] JCTVC-Q0159 AHG 13: Sub-region extraction – position calculation and comparison of different approaches [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

[4] JCTVC-O0031 SCE1: Summary report of SHVC Core Experiment on support for arbitrary scalability ratio [E. François, E. Alshina, J. Chen]

[5] JCTVC-Q0104 Fix for the computation of scaling factors used in inter-layer prediction [Kenneth Andersson, Jonatan Samuelsson (Ericsson)]

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).