| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0206 |
| --- | --- |

| Title: | Proposal to support 12 bit video in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki and Kazushi Sato 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
|  | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
| Source: | Sony |  |  |

_____________________________

# Abstract

This document proposes to investigate 4:2:0 12 bit support in SHVC. Currently bit depth scalability in SHVC is limited up to 10 bit. However, in UHDTV, for example ITU-R BT.2020, 4K video is defined for both 10 bit and 12 bit. In addition, 4:2:0 12 bit profile is defined in RExt and the profile is used for consumer application in near future. Therefore, in SHVC bit depth scalability profile in SHVC, 12 bit video should be allowed for both BL and EL. This contribution proposes to allow to use 4:2:0 12 bit video in bit depth scalability profiles in SHVC.

# Proposal

In this contribution, the followings are proposed.

- To add 4:2:0 12 bit video capability in SHVC (with the tools limited to the tools in 4:2:0 12 bit profile)

- To check the performance of bit depth scalability with 12 bit video, e.g. by SCE

- To consider to include 12 bit video capability in profiles supporting bit depth scalability

4:2:0 12 bit profile in RExt will be used for consumer applications, and 4:2:0 12 bit video should be allowed for both BL and EL. Since the tools in 4:2:0 12 bit profile are basically the same as 4:2:0 8/10 bit profiles, only the tools in 4:2:0 12 bit profiles should be included in both BL and EL. No other tools in RExt should be added for this capability.

# Initial test results

Preliminary test results were conducted to confirm the performance of bit depth scalability in SHVC. The current SHM software can support up to 12 bit.

To confirm the compatibility of SHM with bit depth scalability and 12 bits input, a test were conducted with RExt-6.0, HM-13.0, and bug fixed version of SHM-5.0-IRAP from JCTVC-Q0130 [2] using the QPs for 1x scalability from SCE1 [1].

One test sequence was used in this study. The test sequence is FruitStall in 4096x2160 at 24fps in YUV 4:2:0 format in 10 bits and 12 bits. The 10 bit version is used for base layer input. The 12 bit version is used for enhancement layer input and single layer input.

As shown in Table 1, the bug fixed version of SHM-5.0-IRAP [2] is compatible with 12 bits EL input and 10 bits BL input in YUV 4:2:0. Its coding performance is comparable to single layer 12 bits encoding with RExt-6.0 and HM-13.0.

Table 1: Confirmation of SHM with AI and RA configurations for supporting 12 bit enhancement layer input and 10 bit base layer input and 1x scalability.

|  | AI HEVC 1x 10-bit base |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RExt vs SHM BL+EL | -3.8% | -5.3% | -5.9% |
| RExt vs HM | -0.3% | 0.1% | 0.5% |
| SHM BL+EL vs HM | 3.8% | 6.1% | 7.5% |
|  | RA HEVC 1x 10-bit base |  |  |
|  | Y | U | V |
| RExt vs SHM BL+EL | -6.5% | -11.9% | -13.5% |
| RExt vs HM | 0.2% | 0.5% | 0.8% |
| SHM BL+EL vs HM | 7.2% | 15.1% | 18.1% |

# Conclusions

This contribution proposes the followings.

- To add 4:2:0 12 bit video capability in SHVC (with the tools limited to the tools in 4:2:0 12 bit profile)

- To check the performance of bit depth scalability with 12 bit video, e.g. by SCE

- To consider to include 12 bit video capability in profiles supporting bit depth scalability

Preliminary results are shown in this contribution. This capability should be investigated by SCE by July and appropriate profiles for such capability should be consider for SHVC profiles.

# References

- A. Duenas, P. Andrivon, Y. Ye, E. Alshina, X. Li, K. Ugur, K. Sato, S.H. Kim, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability,” JCTVC-P1101, San José, US, 9–17 Jan. 2014.

- C. Auyeung, “AHG12: Mismatch of SHVC draft 5 and SHM-5.0 software in bit-depth scalability with 1x scalability,” JCTVC-Q0130, Valencia, ES, 27 March – 4 April 2014

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).