| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0211 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0188 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On HighestTid and MaxSubLayersInLayerSetMinus1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Takeshi Tsukuba Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

The contribution proposes a modification related to maximum sub-layer value, called MaxSubLayersInLayerSetMinus1[ i ], in the output layer set which is used to signals sub-layer base DPB parameters and restrict HighestTid in the range of 0 and MaxSubLayersInLayerSetMinus1[ i ]. The modification uses maximum sub-layer value of output layers instead of maximum sub-layer value of decoding layers. It is asserted that the modification are useful to reduce DPB size and remove unnecessary DPB parameters.

# Introduction

There is a use-case that the frame rate of enhancement is lower than base layer, e.g. 2160p@30fps enhancement layer and 1080p@60fps base layer. There the sub-layer value of enhancement layer, sublayers_vps_max_minus1[1] is less than sublayers_vps_max_minus1[0]. In that case, the current spec has the following problems.

- Unnecessary decoding and DPB size increase (shown in Figure 1)

- Unnecessary DPB parameters signaling (shown in Figure 2)

As shown Figure 1, if output layer is enhancement layer only (OutputLayerFlag[][0] = 0 and OutputLayerFlag[][1] = 1), it is considered that we don’t need to decode sub-layers (TID=2) that are higher than highest sub-layer of output layers. However, in the current spec, there is no such restriction on HighestTid related to output layers. Since the decoder is required to decode unnecessary sub-layer for supporting such un-useful HighestTid case, DPB size increases.

Sub-layer base DPB parameters are signaled in the range of 0 to maximum sub-layer of decoding layers. The range is not restricted in terms of maximum sub-layer of output layers. Although, the current spec allows omitting the direct signaling by setting sub_layer_dpb_info_present_flag equal to 0 and inferring it from the lower sub-layer, the inference doesn’t work. It is because that the DPB size of higher sub-layer , which is with unnecessary decoding, is difference from the DPB size of lower sub-layer, which is sufficient for output pictures (See Figure 2).

Lastly it is inevitable to consider alternative output layer case. In alternative output layer case, the lower layer can be outputted in the case that the output layer is not prepared to be output. In the current spec, decoding process and output frame rate depends on HighestTid, which can be higher than the maximum sub-layer of output layers. This flexibility is undesirable for implementation which is required to support alternative sub-layer output/decoding as well as alternative layer output/decoding. It can also complicate testing.

Figure 1: Example of unnecessary decoding

Figure 2 Unnecessary DPB parameters signalling

| dpb_size( ) { |  |
| --- | --- |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| sub_layer_flag_info_present_flag[ i ] | u(1) |
| for( j = 0; j <= MaxSubLayersInLayerSetMinus1[ i ]; j++ ) { |  |
| if( j > 0 && sub_layer_flag_info_present_flag[ i ] ) |  |
| sub_layer_dpb_info_present_flag[ i ][ j ] | u(1) |
| if( sub_layer_dpb_info_present_flag[ i ][ j ] ) { |  |
| for( k = 0; k < NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ]; k++ ) |  |
| max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] | ue(v) |
| max_vps_num_reorder_pics[ i ][ j ] | ue(v) |
| if( NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ] != NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ] ) |  |
| for( k = 0; k < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; k++ ) |  |
| max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] | ue(v) |
| max_vps_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- The variable MaxSubLayersInLayerSetMinus1[ i ] is derived as follows:

for( i = 1; i < NumOutputLayerSets; i++ ) {
	maxSLMinus1 = 0
	optLsIdx = LayerSetIdxForOutputLayerSet[ i ]
	for( k = 0; k < NumLayersInIdList[ optLsIdx ]; k++ ) {
		lId = LayerSetLayerIdList[ optLsIdx ][ k ]	(F-1)
		maxSLMinus1 =Max( maxSLMinus1, sub_layers_vps_max_minus1[ LayerIdxInVps[ lId ] ] )
	}
	MaxSubLayersInLayerSetMinus1[ i ] = maxSLMinus1
}

# Proposal

The proposals are

- to restrict HighestTid in the range of 0 and MaxSubLayersInLayerSetMinus1, which is highlighted in yellow

- to modify the value of MaxSubLayersInLayerSetMinus1 from maximum sublayer value of decoding layers to maximum sub-layer value of output layers, which is highlighted in cyan

8.1 General decoding process

…

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, HighestTid is set as specified in subclause C.1.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

If TargetOutputLayerSetidx is not equal to 0, HighestTid is set equal to Min (MaxSubLayersInLayersSetMinus1[TargetOutputLayerSetIdx], HighestTid)

…

Annex C Hypothetical reference decoder

C.1 General

…

Multiple tests may be needed for checking the conformance of a bitstream, which is referred to as the bitstream under test. For each test, the following steps apply in the order listed:

- An operation point under test, denoted as TargetOp, is selected by selecting a target output layer set identified by TargetOutputLayerSetIdx and selecting a target highest TemporalId value HighestTid. The value of TargetOutputLayerSetIdx shall be in the range of 0 to NumOutputLayerSets − 1, inclusive. The value of HighestTid shall be in the range of 0 to MaxSubLayersInLayerSetMinus1[ TargetOutputLayerSetIdx ]vps_max_sub_layers_minus1, inclusive. The variables TargetDecLayerSetIdx, TargetOptLayerIdList, and TargetDecLayerIdList are then derived as specified by Equation Error: Reference source not found. The operation point under test has OptLayerIdList equal to TargetOptLayerIdList, OpLayerIdList equal to TargetDecLayerIdList, and OpTid equal to HighestTid.

- The sub-bitstream extraction process as specified in clause 10 is invoked with the bitstream under test, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

…

F.4.7.3.1.1 Video parameter set extension semantics

…

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the i-th output layer set is an output layer. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the i-th output layer set is not an output layer.

…

- The variable MaxSubLayersInLayerSetMinus1[ i ] is derived as follows:

for( i = 1; i < NumOutputLayerSets; i++ ) {
	maxSLMinus1 = 0
	optLsIdx = LayerSetIdxForOutputLayerSet[ i ]
	for( k = 0; k < NumLayersInIdList[ optLsIdx ]; k++ ) {
		lId = LayerSetLayerIdList[ optLsIdx ][ k ]	(F-2)

if (OutputLayerFlag[i][k])
			maxSLMinus1 =Max( maxSLMinus1, sub_layers_vps_max_minus1[ LayerIdxInVps[ lId ] ] )
	}
	MaxSubLayersInLayerSetMinus1[ i ] = maxSLMinus1
}

# Patent rights declaration

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).