| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0213_r1 |
| --- | --- |

| Title: | Fix for adaptive color space coding in JCTVC-Q0035 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document presents the fix for adaptive color space coding in JCTVC-Q0035 and shows experimental results. The bug was caused by not normalizing the inverse color conversion matrix when coding was performed in YCoCg color space. After fixing the bug, the coding efficiency for RGB sequences can be improved. For example, the coding gain compared with the CfP anchor increases from 6.7% to 20.5% for animation RGB in AI fixed QP coding. Compared with the CfP anchor, now on average the coding gain for AI is 32.3% (the average number includes YUV sequences, which are not changed). On average, about 2.2%, 1.9%, and 1.4% more bit saving is achieved for AI, RA, and LB respectively. (These average number includes YUV sequences, which are unchanged. If only RGB sequences are taken into consideration, the improvement will be doubled.)

# Introduction

In JCTVC-Q0035, if the input is of GBR format, an intra CU can choose YCoCg color space to perform prediction and coding. If YCoCg color space is used for a CU, after coding, the reconstructed CU is converted back to GBR and stored in the decoded buffer. The following conversion matrix is used –

(1)

Since the norm of the above conversion matrix is not 1, the QP applied in YCoCg color space should be different from that in GBR color space. We noticed that JCTVC-Q0031/JCTVC-Q0112 has mentioned this issue.

However, in JCTVC-Q0035, we did not take normalization into consideration so that the performance was compromised. In this document, we fix the normalization issue during inverse color space conversion for YCoCg coding and show the improved results.

# Solution

The fix is just to decrease the QP value when YCoCg color space is used for coding to make sure the quantization in YCoCg color space is comparable to that in GBR color space. If QP is used in RGB color space and the current CU uses YCoCg color space, QP-5 is used for Y component, QP-3 is used for for Co component, and QP-5 is used for Cg component. The QP offset is derived as follows.

As discussed in [1], the norm of inverse conversion are

NY=3NCo=2NCg=3

The quantization step size is Qs=2(QP−4)6. And the quantization error relates to Qs2. To guarantee that the RGB signal converted from the YCoCg signal have similar distortion level, we should have

2(QP−4)3=3∗2(QPY−4)3

We can derive that

QPY = QP – 3*log23 ≈ QP–4.75 ≈ QP–5

Similar, we can have

QPCo = QP – 3*log22 = QP–3

QPCg = QP – 3*log23 ≈ QP–4.75 ≈ QP–5

# Experimental results

Fix QP (same input QP as Call for Proposal anchor) encoding is used. The anchor is the joint CfP anchor, i.e., HM-12.1+RExt-5.1. The fix only has impacts on RGB lossy coding. The PSNR values are not clipped. BD-Rate results of JCTVC-Q0035 and the proposed fix are shown in Table 1 and 2, respectively.

Table 1 BD-Rate of JCTVC-Q0035

Table 2 BD-Rate of JCTVC-Q0035 with the proposed fix

# References

- D. Schonberg, S. Sun, G. J. Sullivan, S. Regunathan, Z. Zhou, and S. Srinivasan, “Techniques for enhancing JPEG XR / HD Photo rate-distortion performance for particular fidelity metrics,” Proc. of SPIE Vol. 7073, 707314, (2008).

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).