| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0216 |
| --- | --- |

| Title: | Cross-check for modified phase offset calculation in JCTVC-Q0120 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jianle Chen | Tel: Email: | +1 858 999 4243 cjianle@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the crosscheck result of the modified phase offset calculation calculation used for reference sample location derivation proposed in JCTVC-Q0120. The software was checked to be consistent to the proposed description and the coding results match those results provided by the proponent.

# Proposed phase offset calculation used reference sample location derivation

In JCTVC-Q0120, it was proposed to to modify the existing phase offset calculation by using the existing flags for the support of spatial scalability and interlaced-to-progressive scalability.

The modifications are as follows:

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-1)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-2)

When cIdx is 0, variables phaseX, deltaX, phaseY, and deltaY are derived using Table 1 as follows:

phaseX = phaseY = CrossLayerPhaseAlignmentFlag <<1

deltaX = CrossLayerPhaseAlignmentFlag <<3

deltaY = ((CrossLayerPhaseAlignmentFlag <<3)>>VertPhasePositionAdjustFlag) + (VertPhasePositionFlag<<3)

When cIdx is 1, variables phaseX, deltaX, phaseY, and deltaY are derived using Table 2 as follows:

phaseX = CrossLayerPhaseAlignmentFlag

deltaX = CrossLayerPhaseAlignmentFlag << 2

phaseY = CrossLayerPhaseAlignmentFlag + 1

deltaY=((( CrossLayerPhaseAlignmentFlag +1)<<2)>>VertPhasePositionAdjustFlag)+(VertPhasePositionFlag<<3)

The variables phaseX, phaseY, addX and addY, are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-3)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-4)

addX = ( ScaleFactorX * phaseX + 2 ) >> 2		(H-5) 
addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(H-6)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) − ( phaseX << 2 ) deltaX	(H-7)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) − ( phaseY << 2 ) deltaY	(H-8)

# Observation and experimental results

In the current SHVC, CrossLayerPhaseAlignmentFlag is used to calculate the phase offset for spatial scalability. VertPhasePositionAdjustFlag and VertPhasePositionFlag are used to calculate the phase offset for interlaced-to-progressive scalability. In the proposed method, the three lags are jointly considered for the case of combination of spatial and interlaced-to-progressive scalabilities by assuming that the lower layer input sequence is generated by firstly downsampling the upper layer sequence, and then an interlacing process is performed. The proposed phase offset derivation for this user case is correct according to mental crosscheck.

The experiment on common test condition is performed to verify that the proposed method provides identical results to the current SHVC.

# Reference

- K. Minoo, D. Baylon, “Re-sampling using existing phase offset flag signaling” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0120, Mar. 2014