| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0223_v6 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0207 |

| Title: | BoG report on High Level Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce Ye-Kui Wang | Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met 27 March 2014.

The BoG met again on 28 March 2014.

The BoG met again on 29 March 2014.

The BoG met again on 30 March 2014.

The BoG met again on 1 April 2014.

# Recommendations

The BoG recommends adopting the following:

- JCTVC-Q0108 Proposal #1: Remove restriction on alignment of TSA and STSA with dependent layers

- JCTVC-Q0108 Proposal #2: Allow STSA pics of non-base layers to have TemporalId equal to 0

- JCTVC-Q0158 Proposal #2: Change range of values for HighestTid in Annex C to reduce number of encoder conformance tests

- JCTVC-Q0146 Proposal 1.1: Change inference rule in semantics of poc_msb_val_present_flag

- JCTVC-Q0146 Proposal 1.4, Add an inference value of 0 when poc_reset_info_present_flag is not present

- JCTVC-Q0060 Proposal 2: Add a condition to the derivation of refLayerPicIdc of (TemporalId == 0)

- JCTVC-Q0165 proposal 1.a), add a constraint to disallow an empty output layer set

- JCTVC-Q0165 proposal 2. change condition for presence of num_add_output_layer_sets to avoid sending it when there is only one layer set.

- JCTVC-Q0195 proposal 1: restructure syntax table for sending of rep_format_idx_present_flag and vps_num_ref_formats_minus1

- JCTVC-Q0195 proposal 2: add a semantic constraint on the value of update_ref_format_flag

- JCTVC-Q0177 proposal 1: modify inference rule for sps_temporal_id_nesting_flag when it is not present

- JCTVC-Q0177 proposal 2: clarification of description of end of sequence NAL unit

- JCTVC-Q0183, numerous small clean-ups on SEI messages

- JCTVC-Q0078, Items 3 b,c and 4, clarifying which pictures in an output layer sets are applied the values of general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag.

The BoG recommends the following activities take place:

- Revisit JCTVC-Q0105 in track, “On temporal enhancement layers and diagonal inter-layer prediction”

- Revisit JCTVC-Q0166 in track, “On scaled reference layer offset”

- Revisit JVTVC-Q0170 in track, “Resampling need for a scalable layer”

- Revisit JCTVC-Q0115 in track, “On indication of decoding process”

- Revisit aspects of JCTVC-Q0078 in track, “On additional layer sets, rewriting of simulcast layers, and profile-tier-level indication for auxiliary picture layers”

- Revisit JCTVC-Q0090 in track, “Redundant picture SEI message”

The BoG refers the following to the editors:

- Several editorial issues regarding HRD operation from JCTVC-Q0055.  Review text modifications with contribution author.

- Editorial improvements are needed for the association of NAL units within an access unit and access unit boundary detection.  Language in JCTVC-0091 may be useful to study.

- JCTVC-Q0146, proposal 1.2, consider adding a NOTE and check if usage of HandleCraAsBlaFlag and EOS behavior is clearly described, and proposal 1.3 specification bug fix

- JCTVC-Q0054 proposal #2, bug fix in the semantics of vps_profile_present_flag[ ]

- JCTVC-Q0116, consider adding a note regarding how random accessibility is affected by the recovery point SEI message

- .

The recommendations listed above have been reviewed by JCT-VC.

The BoG recommends adopting the following:

- JCTVC-Q0102 proposal #3, change the max loop index for signaling bit rate and pic rate info to MaxSubLayersInLayerSetMinus1

- Add a profile constraint to the Scalable Main, Scalable Main 10, and Stereo Main profiles against allowing layers with duplicate values of DependencyId (or ViewOrderIdx) when AuxId equal to 0.

- JCTVC-Q042v2: Add semantic constraints to poc_lsb_not_present_flag.

- JCTVC-Q0160v2: Add constraint to stereo main profile that it must contain exactly two texture views, and add a note to state that the constraint implies a restriction that alt_output_flag equal to 0.

- JCTVC-Q0189 proposal 2.2, clarification of semantics of slice temporal mvp enabled flag

- JCTVC-Q0189 proposal 2.3,  SEI message for indicating constraints on TMVP

The BoG refers the following to the editors:

- Clarify that an auxiliary picture can be associated with more than one primary picture.  Consider if the language associating an alpha auxiliary picture with a primary picture in the semantics of dimension_id[ ] [ ] near the AuxId derivation could be moved to the alpha SEI message.

- JCTVC-Q014, consider adding a note to clarify that if IDR alignment is indicated, the constraint on poc_lsb_not_present[ ] does not apply.

The BoG recommends the following activities take place:

- Discuss in track that no sub-bitstream extraction capability exists in the reference software, even though it is normatively described in the text.  Seek volunteer to provide software.

- Revisit JCTVC-Q0078 in track

- Discuss in track the association of auxiliary pictures with primary pictures. Key questions to consider are:

  - Should an association be defined in the specification, and if so, how?

  - Can an auxiliary picture be associated with more than one primary picture?

  - Can more than one auxiliary picture of a particular type be associated with a primary picture?  If so, how does the decoder know which one to use?

  - Should a default association behavior be described, which uses the existing Scalability Id mechanism?

  - Should the association be defined in an SEI message?  Could it override the default association?

  - Does the alpha SEI message need an indication of which is the associate primary picture layer?

JCT-VC decisions related to joint SHVC/MV-HEVC HLS topics are also summarized below.

- JCTVC-Q0045/JCTVC-Q0096: Overlay info SEI message

- Hybrid scalability text in the AHG15 report approved

- Remove sub-DPB sharing and processes that mark inter-layer reference pictures as "unused for reference".

- Sub-bitstream extraction capability will be added to reference software.

- Normative rewriting process to be defined, so that independent non-base layers can be rewritten to be decodable by non-layered profiles (i.e.,. Main, Main 10)

- Auxiliary picture concepts:

  - Profile indication in the SPS of an aux pic is the profile that would apply after rewriting.

  - When an output layer set that doesn't include the base layer, the bitstream is rewritten and the same output layer set is required to be present for the rewritten bitstream, which contains a base layer

  - When an aux picture is part of an output layer set but is not part of a target output layer, the buffer flow includes the aux picture but the decoding process requirements do not.

  - An auxiliary picture can be associated with more than one primary picture.

  - More than one auxiliary picture of a particular type may be associated with the same primary picture

- Document review details

## 7.5.1 Generic HLS issues

JCTVC-Q0055 MV-HEVC/SHVC HLS: Comments on general decoding process and selection of CPB operation in the HRD operation [Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung)]

This contribution proposes modifications and improvements of texts in the subclause 8.1 General decoding procedure and C.1 General of Annex C with the following items:

First proposal: Re-definition of ‘output layer’

Refer to editors

Second Proposal: to substitute ‘TargetOptLayerSetIdx’ with ‘TargetOutputLayerSetIdx’

Seems to be an editorial bug. Refer to editors.

Third Proposal; Modification of TargetOutputLayerSetIdx setting procedure

Refer to editors to improve C.1 description of setting the value of TargetOutputLayerSetIdx

Fourth Proposal: Modification of HighestTid setting procedure

Refer to editors.  Suggest to introduce a variable which is set to vps_max_sub_layers_minus1 when nuh_layer_id > 0.  (Base layer uses SPS value, other layer use VPS value)

Fifth Proposal: Modification of NoRaslOutputFlag setting procedure

Proposal seems to change technical behavior.  No action.

Sixth proposal: Modification of selecting CPB operation for a bitstream conformance test

Refer to the editors to clarify this section regarding “both” bitstream specific and bitstream partition specific conformance

If possible, desirable to have editorial changes made during this meeting, and reviewed by the proponent.

JCTVC-Q0091 MV-HEVC/SHVC HLS: On access unit definition and allowing different decoding orders in different layer trees [M. M. Hannuksela (Nokia)]

It is asserted that it may be desirable to allow different decoding orders of pictures in different layer trees to provide flexibility for encoders to use different prediction structures in different layer trees and to enable a leaner sub-DPB capacity requirement for some layer trees, such as auxiliary picture layers.

The following aspects are proposed regardless of whether the access unit is specified as proposed in this contribution:

- Definition of a layer tree (which, when phrased in a simplified manner, consists of the layers that have dependency relations with each other).

- Allocation of layers to sub-DPBs in a manner that different layer trees are allocated to different sub-DPBs. Consequently, the extraction of a layer tree does not affect the sub-DPB parameters or operation of another layer tree even if the layer trees would have the same resolution, bit-depth and chroma format.

Revisit after other related topics are reviewed.

Additionally, the contribution proposes the following items related to relaxing the access unit definition:

- Specification of the order of NAL units and coded pictures and association to access units for MV-HEVC/SHVC.

Such specification has been missing so far in MV-HEVC/SHVC, which have instead referred to HEVC version 1 specification text, which is asserted to be wrong or incomplete in many aspects for MV-HEVC/SHVC.

- Relaxation of the access unit definition so that an access unit is required to include coded pictures associated with the same output time and belonging to the same layer tree. When the bitstream has multiple layer trees, an access unit may but is not required to include coded pictures associated with the same output time and belonging to different layer trees.

- A VPS VUI flag cross_layer_dec_order_aligned_flag. When the flag is equal to 1, access units are constrained to include all coded pictures associated with the same output time.

- A bitstream conformance constraint that pictures associated with the same output time shall be within the same POC resetting period and shall have the same value of PicOrderCntVal. This constraint is needed to make picture output order of the output order conforming decoder the same as that of the output timing conforming decoder.

Aspects a-b and 1-4 can be considered separately.

It was asserted that the proposal gives encoders more flexibility, and that it could allow more efficient sub-DPB optimization.

A participant questioned what interaction this proposal would have with systems specifications.  It was suggested that independent layer trees would need to be placed in different tracks.

Access unit boundary detection might be made more complex, but not clear.

A question was raised about interaction with SEI message persistence and scalable nesting SEI messages.

Refer to editors: It was noted that editorial improvements are needed for the association of NAL units with an access unit, and access unit boundary detection, independent from this proposal.  The language in this contribution may be useful to study.

JCTVC-Q0105 MV-HEVC/SHVC HLS: On temporal enhancement layers and diagonal inter-layer prediction [M. M. Hannuksela (Nokia)]

This contribution asserts that diagonal inter-layer prediction would be useful in the following use cases:

- An enhancement layer provides a temporal enhancement, possibly along with spatial or quality enhancement, relative to the base layer, where the picture rate ratio is non-dyadic, e.g. a 24-Hz base layer, a 30-Hz first enhancement layer, and a 120-Hz second enhancement layer.

- An SHVC-coded temporal enhancement layer is provided for an AVC or HEVC base layer.

- Interlaced-to-progressive scalability, when the base layer contains coded frames.

It is proposed to enable the use of diagonal inter-layer reference pictures as summarized in the following:

- A gating flag in SPS multilayer extension specifies if diagonal inter-layer prediction is enabled in the slice header level.

- A reference-layer short-term RPS syntax structure can be included in the slice header for a direct reference layer. The reference-layer short-term RPS syntax structure specifies the pictures from the direct reference layer that are included in the initial reference picture list(s) of the current picture, but causes no change on the marking of the pictures.

- The decoding process for reference picture lists construction is modified to include reference pictures from the reference-layer short-term RPS syntax structure for the current picture.

New syntax structure is proposed.  It was noted that the earlier proposal in JCTVC-P0141 was simpler and involved less change to the specification.

Revisit in track, with updated contribution to also provide simpler option.

JCTVC-Q0108 MV-HEVC/SHVC HLS: On TSA and STSA pictures [M. M. Hannuksela (Nokia)]

The contribution includes the following proposals:

- It is proposed to remove the constraints on aligning TSA and STSA pictures within an access unit due to the following asserted reasons:

  - The alignment constraints do not guarantee that sub-layer up-switching would be possible across all layers of the target output layer set, when a base-layer TSA or STSA picture at the desired switch-to sub-layer appears. In other words, layer-wise consideration of sub-layer up-switching is needed in any case.

  - Without the alignment constraints, it would be possible to adapt the highest transmitted TemporalId of the highest layer separately from other layers and hence achieve finer-grain bitrate adjustment.

  - Without the alignment constraints, reference pictures for inter prediction can be selected differently across layers, allowing some layers to be coded with more frequent sub-layer up-switch points and some layers with a larger number of reference pictures for inter prediction (aiming at better rate-distortion performance).

- It is proposed to allow STSA pictures with nuh_layer_id greater than 0 to have TemporalId equal to 0. Such STSA pictures enable layer up-switching where a subset of sub-layers of the highest layer can be decoded starting from the up-switch point. It is asserted that the proposed change does not affect the decoding process.

Agreed to remove restriction from #1, to no longer require TSA or STSA alignment of dependent layers with the current layer.

Agreed to remove restriction from proposal #2, to allow STSA pics of non-base layers to have TemporalId equal to 0.

JCTVC-Q0109 MV-HEVC/SHVC HLS: On TemporalId constraints [M. M. Hannuksela (Nokia)]

This contribution proposes that pictures with different TemporalId values are allowed in the same access unit. It is asserted that the following use cases or benefits are achieved with the proposal:

- It provides encoders flexibility in determining prediction hierarchies differently across layers, allowing some layers to be coded with a greater number of TemporalId values and frequent sub-layer up-switch points and some layers with a prediction hierarchy aiming at a better rate-distortion performance.

- It provides flexibility to encode layer trees of the same bitstream independently from each other.

A question was raised of interaction with SEI messages.

No action.

JCTVC-Q0158 MV-HEVC/SHVC HLS: On Highest TemporalId [S. Deshpande (Sharp)]

In this document:

- In proposal 1 it is proposed to derive HighestTemporalIdList - a list of values of highest TemporalId for each layer in the TargetDecLayerIdList. It is proposed to invoke the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in sub-clause F.8.1.4.1 based on the derived HighestTemporalIdList during the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than zero as specified in F.8.1.4.

It is asserted that without this proposed modification currently in a mixed frame rate and mixed number of temporal sub-layers scenario the marking process for sub-layer non-reference pictures not needed for inter-layer prediction in F.8.1.4.1 will not get invoked for layers which have lower frame rate and lower number of temporal sub-layers as the highest TemporalId value in those layers is less than HighestTid value.

- In proposal 2 a modification is proposed to the range of values for HighestTid in Annex C.

Proposal 1 asserts efficiency improvement for sub-DPB optimization.  One pic in the DPB could potentially be saved under particular use cases – an example was given of 3 layers, with 30 fps, 30fps, and 60 fps of the 3 layers. It was suggested that there would only be a benefit for shared sub-DPB.

Some non-proponent support for proposal 1 if shared sub-DPB is kept.  Revisit once shared sub-DPB decision is made.  Offline discussion about text for handling an additional use case.

Proposal 2 is asserted to reduce the number of tests than an encoder has to make for conformance.  Agreed to Proposal 2.

JCTVC-Q0163 MV-HEVC/SHVC HLS: On decoding non-output/non-reference layers [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

This contribution proposes to modify derivation process of the layer identifier list TargetDecLayerIdList to exclude decoding of non-output / non-reference layers.

It is asserted that the proposal is beneficial to provide a capability to ignore unnecessary layers for target output layers in decoding process. Considering base layer existence is set to be inevitable at any layer sets regardless of base layer usage, the proposed modification is assertively critical to signal the actual HDR parameter that depends on decoding process.

In the revision 2, Option 1 and Option 2 are updated.

It was suggested that JCTVC-Q0078 is related.

Need to consider the larger question about should a conformance point be defined which doesn’t contain a backwards compatible base layer, and if there should be a normatively defined re-writing process for an independent enhancement layer to be rewritten as a base layer.

Option 2 changes the sub-bitstream extraction process, but Option 1 does not.

Contribution specification text appears to be incomplete, because other areas would be affected.

Revisit after review of related contributions and discussion of larger questions.

### 7.5.2 IRAP alignment and POC derivation (2X)

JCTVC-Q0057 MV-HEVC/SHVC HLS: Comments on IRAP alignments and POC value derivation [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, S. Lee, C. Kim (Samsung)]

Several restrictions on the IRAP picture alignment and the POC value derivation are proposed. With the proposed restrictions for IRAP alignments, the behaviour of POC value derivation can be simplified.

In the first proposal, a flag layerwise_start_up_disable_flag disabling a layerwise start-up is proposed.  It was questioned whether this was proposing a change in the decoding process or a constraint on the bitstream.  The proposal text doesn’t describe a change in decoding process. No action.

In the second proposal, when all_layers_idr_aligned_flag is equal to 1, vps_poc_lsb_aligned_flag shall be equal to 1 and poc_reset_info_present_flag shall be equal to 0.  Spec text was not provided.  No action.

In the third proposal, replacing the restriction that “the value of poc_reset_idc of all pictures in an access unit shall be the same” with “the value of poc_reset_idc of all slices in a layer shall be the same” is proposed.  It was questioned if the proposal would impact error resilience.   No action.

In MV-HEVC profile (stereo profile), cross_layer_pic_type_aligned_flag and cross_layer_irap_aligned_flag are equal to 1 by default.  No action.

JCTVC-Q0146 MV-HEVC/SHVC HLS: On picture order count and related [A. K. Ramasubramonian, Hendry, Y.-K. Wang]

This document proposes several changes to the semantics of syntax elements related to POC, and also proposes a constraint to disallow an enhancement layer RASL picture of a CRA picture to refer to pictures that precede, in decoding order, a POC resetting picture that precedes the associated CRA picture in decoding order and that is in the same layer as the CRA picture.

Proposal 1.1 changes semantics of poc_msb_val_present_flag, inference rule when not present.  Agreed.

Proposal 1.2 proposes a semantic constraint.  It was suggested that if a splicer removed the RASL pictures associated with the IRAP pic at the upswitch point, this constraint would not be necessary.  Refer to editors to consider adding a NOTE and check if usage of HandleCraAsBlaFlag and EOS behavior is clearly described.  JCTVC-Q0177 may be related.

Proposal 1.3 spec bug fix, refer to editors.

Proposal 1.4 proposes an inference value of 0 when poc_reset_info_present_flag is not present.  Agreed.

### 7.5.3 RPS signalling and derivation (3)

JCTVC-Q0060 MV-HEVC/SHVC HLS : On inter-layer RPS signalling and derivation [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

This contribution proposes two changes for MV-HEVC and SHVC related to inter-layer RPS signalling in the slice segment header and derivation process. The first change targets to improve the inter-layer RPS signalling in the slice segment header by taking into account the number of direct reference layers presented in the same access unit as the current picture, which is indicated by the ‘numRefLayerPics’, instead of the number of direct reference layers indicated by the ‘NumDirectRefLayers’. The second change targets to improve the derivation process of ‘numRefLayerPics’ and ‘refLayerPicIdc’ to support the lightweight option scenario in that syntax element ‘max_tid_il_ref_pics_plus’ is set equal to 0.

In r1 revision, the simplified version is added for Item 2 and comments on JCTVC-Q0079, which includes the conceptually same proposals as this, are added.

Proposal 1 could provide some minor bit savings in the slice segment header under certain conditions.  JCTVC-Q0079 is related.

Proposal 2 adds a condition to the derivation of the value of refLayerPicIdc.  Agreed to add a condition with (TemporalId == 0). Also proposed as option 1 in JCTVC-Q0079.

JCTVC-Q0079 MV-HEVC/SHVC HLS: max_tid_il_ref_pics_plus1 in inter-layer RPS syntax and semantics [M. M. Hannuksela (Nokia)]

The contribution proposes changes for the following two asserted problems or shortcomings:

- When max_tid_il_ref_pics_plus1[ ][ ] is equal to 0 (i.e., when only the IRAP pictures of the reference layer may be used as reference for inter-layer prediction), the index of the reference layer is not included in the list of layer index values of the potential inter-layer reference pictures, refLayerPicIdc[ ], even though it should be included for a correct operation.

- max_tid_il_ref_pics_plus1[ ][ ] is not used in the inter-layer RPS syntax and semantics to prune the potential reference layers for inter-layer prediction.

Two options are proposed, where option 1 is asserted to fix the asserted bug only and option 2 is asserted to fix both issues. The proponent prefers option 2.

See notes for JCTVC-Q0060.

Could potentially gain a few bits per slice under certain conditions, but no estimate of gains.

JCTVC-Q0100 MV-HEVC/SHVC HLS: Constraints for Reference Picture Set Parameters [S. Deshpande (Sharp)]

In this document bitstream constraints are proposed for short-term reference picture set parameters and long-term reference picture set parameters for SHVC and MV-HEVC. These bitstream constraints are proposed for syntax elements num_negative_pics, num_positive_pics, num_long_term_pics, num_long_term_sps considering the DPB parameters signalled in VPS which are used for the DPB operation.

It seems that the existing constraints are not clear for the layered extensions, and should be addressed.  Was further discussed after offline discussion and refinement of proposed text.

JCTVC-Q0154 includes proposal for inferring the VPS max dpb size for layer 0 to be equal to the related value in the SPS.  Revisit after review of JCTVC-Q0154.

### 7.5.5 Parameter sets (5)

JCTVC-Q0054 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung)]

Modifications of semantics of dimension_id_len_minus1[ j ] and vps_profile_present_flag[ i ] are proposed.

In the first proposal to modify Semantics of dimension_id_len_minus1, no action.

In the second proposal, refer to editors a bug fix in the semantics of vps_profile_present_flag[ i ]

JCTVC-Q0061 MV-HEVC/SHVC HLS: On max_tid_il_ref_pics_plus1 in the VSP extension [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

This contribution proposes the extra syntax elements to indicate the default value for syntax element “max_tid_il_ref_pics_plus1” to remove signalling redundancy.

This change could save some bits in the VPS for the case when max tid is used, but the same value was used for all sub-layers.  Two options proposed.

It was suggested that we should be concentrating on design stability and bug fixes at this stage rather than optimizations.

JCTVC-Q0110 Support for out-of-band signaling in VPS to enable future layer additions [S. Narasimhan, A. Luthra (Arris)]

Presenter not available.

JCTVC-Q0165 MV-HEVC/SHVC HLS: Clean up for output layer set [T. Tsukuba, T. Yamamoto, T. Ikai, S. Deshpande (Sharp)]

This contribution presents several proposals regarding output layer set as follows:

- Prohibits 1) the case no output layer exists in an output layer set, and 2) duplicated output layer sets

- Clean up signaling condition related to num_add_output_layer_sets and default_target_output_layer_set.

- Add a flag, ptl_dpb_info_present_flag, indicating whether or not to explicitly signal profile_tier_level_idx and DPB parameters for additional output layer sets

In revision 1, the proposed text for proposal 1 is updated. In addition, bit-estimation analysis on proposal 3 is added. According to the analysis, it is asserted that proposal 3 can reduce bits with negligible overhead.

In revision 2, the proposed texts for proposal 3 is updated.

In revision 3, the proposed text for proposal 3 is further updated.

Proposal 1.a) Add a constraint to disallow an empty output layer set. Agreed.

Proposal 1.b) Proposes to add a constraint to disallow duplicated output layer sets.   No action.

Proposal 2.  Change condition for presence of num_add_output_layer_sets to avoid sending it when there is only one layer set.  Seen as a more logically consistent design.  Agreed.

Proposal 3.  Could provide bitrate savings.  Use case is not clear.  No action.

JCTVC-Q0211 MV-HEVC/SHVC HLS: On HighestTid and MaxSubLayersInLayerSetMinus1 [T. Ikai (Sharp)] [late]

The contribution proposes a modification related to maximum sub-layer value, called MaxSubLayersInLayerSetMinus1[ i ], in the output layer set which is used to signals sub-layer base DPB parameters and restrict HighestTid in the range of 0 and MaxSubLayersInLayerSetMinus1[ i ]. The modification uses maximum sub-layer value of output layers instead of maximum sub-layer value of decoding layers. It is asserted that the modification are useful to reduce DPB size and remove unnecessary DPB parameters.

It was questioned if there is an interaction with the alternate output layer process.  It was noted that this was a late contribution, so little opportunity for study in advance of meeting.

The proposal could save some bits in the VPS in certain conditions.

JCTVC-Q0195 MV-HEVC/SHVC HLS: On representation format signaling [S. Hattori, O. Nakagami, T. Suzuki (Sony)] [late]

This contribution discusses representation format (rep_format) signaling in VPS extension.

First, the default mapping between the rep_format and layer is discussed. Currently, this happens when rep_format_idx_present_flag is equal to 0. In this case, the number of rep_formats is constrained to be equal to the number of layers (MaxLayersMinus1). In the contribution, more flexible method is studied and proposed to release this constraint.

Second, a case in which vps_num_rep_formats_minus1 is equal to 0 is discussed. The contribution proposes a clean-up signaling vps_num_rep_formats_minus1. The contribution indicates that the update in SPS extension by update_rep_format_flag is redundant in this case.

The contribution provides comparison of signalled parameter in the bitstream with examples which are based on the use-cases from CTC in MV-HEVC and SHVC.

For proposal 1, it is asserted that the new design is cleaner and easier to understand.  Agreed to restructure rep_format_idx_present_flag and vps_num_ref_formats_minus1.

For proposal 2, a semantic constraint on the value of update_ref_format_flag.  Agreed.

### 7.5.7 Miscellaneous HLS topics (8)

JCTVC-Q0142 MV-HEVC/SHVC HLS: On extraction of independent non-base layer [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

This contribution asserts the presence of two problems related to the semantics of poc_lsb_not_present_flag[ i ] and the use of long-term reference pictures. The asserted problems can be summarized as follow:

- When poc_lsb_not_present_flag[ i ] is equal to 1 and the IDR picture in layer i is in the same access unit with a base layer picture that has slice_pic_order_cnt_lsb not equal to 0, the bitstream may become non-conformant as the derived picture order count (POC) for that IDR may be different from other pictures in the same access unit.

- When an independent non-base layer is extracted for input for HEVC decoder, if pictures in layer uses long-term reference picture (LTRP) and the original bitstream has non-cross layer aligned IDR, a problem may occur such that HEVC decoder cannot locate the correct LTRP.

The contribution also asserts that the proposed constraints to the semantics of poc_lsb_not_present_flag[ i ] and base_layer_parameter_set_compatibility_flag[ i ] would solve the problems above.

Regarding proposal 1, it was suggested to revise the language to make it more clear that if IDR alignment is indicated, the constraint does not apply.  Further BoG discussion after offline discussion. Refer to editors to consider adding the suggested note.

Regarding proposal 2, it was suggested that the proposed language was overly restrictive.  It was also questioned if the functionality of the existing VUI indictor flag should be expanded to include a general indication of rewritability, rather than just the parameter set aspect.

Further discussion in BoG.  A suggestion was made to not change the base_layer_parameter_set_compatability_flag as originally proposed, but to add semantic constraints to poc_lsb_not_present_flag.  Agreed to this suggestion, will be reflected in the v2 version of the document.

JCTVC-Q0177 MV-HEVC/SHVC HLS: Miscellaneous HLS topics [Hendry, A. K. Ramasubramonian, Y.-K. Wang, V. Seregin (Qualcomm)] [late]

This document proposes several changes to the current MV-HEVC/SHVC design. The proposed changes include the following:

In proposal 1, Modifying of the inference rule for sps_temporal_id_nesting_flag when it is not present.  This is seen as an editorial bug.  Agreed.

In proposal 2, Clarification of the description related to the end of sequence NAL unit and the end of bitstream NAL unit.  It was suggested the behavior for how a decoder handles an EOS with layer id > 0 should be disallowed in this version of the spec.  Agreed, subject to text revision to address this suggestion.

In proposal 3, Signalling of the presence of discardable pictures in the bitstream.  Proposes to add a flag in the slice segment header, and a gating flag in the VPS.  It is asserted to save bits in some cases, but no numbers were provided.  No action.

JCTVC-Q0102 MV-HEVC/SHVC HLS: Comments on HEVC Extensions [S. Deshpande (Sharp)]

Only proposals 3 and 4 belong to this category.

Proposal 3: It is proposed to signal the bit rate and picture rate information only up to the maximum temporal sub-layers in the corresponding layer set.

It was suggested that an existing variable could be used, MaxSubLayersInLayerSetMinus1. Some non-proponent support.  Further discussed after revised text with that change was available.  Agreed to change the max loop index for signaling bit rate and pic rate info, as included in the spec text provided in the r2 version.

In r1 revision following proposal 4 is added:

Proposal 4: A modification to the derivation of numRefLayerPics, and refLayerPicIdc[ i ] to handle discardable pictures is proposed.

It is asserted that the bits could be saved in some cases, when there are discardable pics in reference layers.  No bit rate savings numbers are provided.  No action.

JCTVC-Q0160 SHVC/MV-HEVC HLS: On alternative output layer flag [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes to introduce a gating flag for alt_output_layer_flag. The proposed change is asserted to reduce parsing complexity as well as to assure the extensibility of the usage of the alternative output layer. In addition, it is proposed to restrict usage of the alternative output layer in Stereo Main profile.

It was suggested the extensibility could alternatively be provided in an extension to the VPS extension.

Proposed to restrict stereo main profile from using the alt output layer.  Agreed in spirit, but using alternative text, which will be provided by M. Hannuksela.  Further discussed in BoG to review revised text.

An output layer set with index olsIdx conforming to the Stereo Main profile shall obey the following constraints:

–       Let numTextureViews be equal to the number of those layers that are not auxiliary picture layers and for which output_layer_flag[ olsIdx ][ j ] is equal to 1 for all values of j in the range of 0 to NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ olsIdx ] ] – 1, inclusive. numTextureViews shall be equal to 2.

Agreed, based on a v2 version of the document to be uploaded.  A note should also be added to state that this constraint implies a restriction of alt output flag equal to 0.

JCTVC-Q0166 MV-HEVC/SHVC HLS: On scaled reference layer offset [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

This contribution proposes change on scaled reference offset information signalling. It is proposed to signal the offset information in both VPS extension and slice header extension.

Contribution did not provide decoding process modification text.  Some support. Revisit in track when text of semantics and decoding process is available.

JCTVC-Q0170 SHVC HLS: Resampling need for a scalable layer [K. Andersson, J. Samuelsson, R. Sjöberg, J. Ström]

Currently a decoder may according to information given on a high level e.g. VPS, draw wrong conclusions about the need for resampling process or not for decoding of a layer of a scalable video bitstream. The need for resampling process could be useful for a decoder to know since it varies the amount of actions for the decoder to perform.

We propose to add a layer wise syntax element to indicate if resampling process is needed for decoding of a layer of a HEVC extension bitstream. The indication can also distinguish between resampling for fractional and integer sample positions or if the need for resampling is unknown.

Proposes new syntax elements in the VUI.

Revisit in track to consider if the indication functionality is considered useful.

JCTVC-Q0189 MV-HEVC/SHVC HLS: On slice_temporal_mvp_enabled_flag [M. M. Hannuksela (Nokia)] [late]

Chaired by Y.-K. Wang.

It is asserted that there are several problems related to slice_temporal_mvp_enabled_flag. The proposal contains two parts reportedly fixing the asserted problems:

- The proposed slice segment header changes allow the presence of slice_temporal_mvp_enabled_flag in slice segment headers of IDR pictures of predicted layers and clarify that the bitstream constraints related to slice_temporal_mvp_enabled_flag apply only to independent layers.

- Similar bitstream constraints as those indicated by slice_temporal_mvp_enabled_flag equal to 0 for independent layers are proposed to be indicated for predicted layers by the proposed temporal motion vector constraints SEI message. The SEI message is also proposed to indicate whether the pictures associated with the SEI message may be used as collocated pictures for temporal motion vector prediction for any subsequent picture, in decoding order, and hence whether their motion fields need to be stored.

Discussion:

It was remarked that this basically does not have a coding efficiency impact on SHVC, essentially it only affects coding efficiency of MV-HEVC bitstreams that have more than two layers.

It was concluded that the SH syntax only affects pictures in a layer that depends another layer, thus potential extraction of an independent layer is not affected.

There are no simulation results. However, it was commented and agreed by some participants that this is basically a clarification or bug fix for TMVP in multi-layer context.

There was some non-proponent support.

There is a need of a constraint on the value of the first SEI flag, and that constraint seems needing to have some dependency across a CVS.

The design adds one bit to slice headers of IDR pictures of enhancement layers that depend on other layer.

Further discussed in BoG after offline study and text is available for resolving the constraint issue and the "one-bit" issue mentioned above.

It was remarked that this issue is not relevant for SHVC and for MV-HEVC with 2 views, and is only relevant for 3 or more views.  Given that we only have a stereo main profile with 2 views now, do we need to take action now?

Could a reserved flag be added now in the sps_multi_layer extension without making slice header syntax table changes now?  That might impact ability for SPS sharing.

Reserved bits are available in the VPS now.  So no action on this aspect for slice header syntax modifications is required now.

The proposed SEI message contains two flags.  The second flag might also be relevant to single layer.

Agreed to recommend adoption for sections 2.2 and 2.3 in revision 1.  Section 2.2 clarification of semantics of slice temporal mvp enabled flag.  Section 2.3 is SEI message for indicating constraints on TMVP.

JCTVC-Q0078 MV-HEVC/SHVC HLS: On additional layer sets, rewriting of simulcast layers, and profile-tier-level indication for auxiliary picture layers [M. M. Hannuksela (Nokia)]

Chaired by Y.-K. Wang.

The proposal includes the following aspects:

- Additional layer sets can be specified in VPS extension in two value ranges of layer set indices:

- A first range of indices for additional layer sets when an external base layer is in use. The sub-bitstream extraction process is appended so that it can be applied to additional layer sets where an external base layer is in use.

- A second range of indices for additional independent layer sets (which may be converted to a new bitstream). These additional layer sets are not required to generate conforming bitstreams with the sub-bitstream extraction process of clause 10.

A loop of hrd_parameters( ) syntax structures may be provided for the additional layer sets where an external base layer is in use.

- The syntax of SPS and PPS with nuh_layer_id greater than 0 is modified so that the SPS and PPS syntax of HEVC version 1 can be used also when nuh_layer_id is that of the independent non-base layers. This is asserted to make it possible to rewrite independent non-base layers to standalone bitstreams by changing the nuh_layer_id of SPSs and PPSs for the independent non-base layers.

- A non-base layer subtree extraction process is specified for converting one or more non-base layer subtrees to a standalone conforming bitstream.

- Profile-tier-level combinations specified for additional independent layer sets apply to the output bitstream of the non-base layer subtree extraction process:

- Profile-tier-level combinations indicated in the VPS for the output layer sets of additional layer sets apply to the output bitstream of the non-base layer subtree extraction process.

- The profile-tier-level combination indicated in the SPS for a non-base independent layer applies to the base layer of an output bitstream of the non-base layer subtree extraction process.

- It is allowed to include auxiliary picture layers for output layer sets that are indicated to be conforming to the MV-HEVC and SHVC profile specifications (currently Stereo Main, Scalable Main, Scalable Main 10), but auxiliary picture layers are not taken into account in the profile and level constraints.

Additionally, complementary aspects numbered 1, 2, 3a to 3d, and 4 are proposed in Section 2.7 of the contribution.

Discussion:

On the 1st problem: it was remarked that the current AHG15 output text allows signalling of profile, tier, and level for bitstreams where the base layer is provided externally.

On the 2nd and 3rd problems: It was asked why not just add an auxiliary picture layer into an output layer set and then indicate the profile, tier, and level for the output layer set. The answer was that in some cases it is desirable not to require decoding of the auxiliary picture layer(s).

On overview:

Item 1: The two ranges of layer set indices may not be mutual exclusive.

Item 2: The benefit of this seems to make it easier to know which SPSs/PPSs are used by a particular independent enhancement layer.

On 2.3:

The proposed rewriting process allows changing of relative layer index value for two layers, consequently changing of the layer order or NAL unit decoding order within an AU, in the rewritten bitstream. This should be disallowed.

On 2.4:

The trick of using the value 7 for sps_max_sub_layer_minus1 needs a bit more clarification or checking.

On 2.5:

- Prepare VPSs for rewritten bitstreams and put them in SEI messages.

- A new type of nesting SEI message is defined for the three HRD-related SEI messages, because nesting of nesting is not allowed.

On 2.6:

For an output layer set that contains auxiliary pictures, bits of auxiliary pictures are counted in HRD operations but not in the level restrictions. This seems to be problematic as the level restrictions are based on HRD operations.

On 2.7:

Items 3 and 4 here are independent of the major topics.

Item 3: Agreed on sub-items b and c. Revisit sub-items a and d after reviewing of the picture timing SEI message issue for frame-field information in Q0183.

Item 4: Agreed.

General discussion:

Two relatively big topics are covered here, handling of auxiliary pictures from decoding capability requirement point of view and bitstream rewriting. It seems that the root reason of doing these is that currently planned SHVC and MV-HEVC profile definitions allow including of auxiliary pictures in output layer sets but the capability for decoding of auxiliary pictures is not counted in the profile, tier, level definition. It was questioned why cannot we just allow output layer sets to include auxiliary pictures and also allow those that don't include. And for those that include auxiliary pictures, indicate the decoding capability through profile, tier, and level.

Revisit in track.

Discussed again on 1 April in BoG, chaired by J. Boyce

In MVC, a rewriting process was included in an informative subclause.

A larger question is if we should normatively define conformance for an independent set of layers? Several participants supported this.

It was noted there is not a sub-bitstream extraction capability in the SHM reference software.  Revisit in track the fact that this software is missing.

If a rewriting process were normatively defined, some reference software should be made available.

It was questioned if the addition of a rewriting process could be done in a later version of the specification, using an extension to the VPS extension. It could be done, but with some bitrate overhead.

It was questioned if the new syntax elements in the VPS extension could be put in an SEI message. Moving to an SEI message would make that information not available in the VPS for session negotiation purposes.

Revisit in track.

## 7.4.2. Other (4)

JCTVC-Q0116 RExt HLS: Picture referencing across CRA pictures [R. Sjöberg, J. Samuelsson, Y. Wang (Ericsson)]

Chaired by Y.-K. Wang.

This contribution is a follow-up of contribution JCTVC-P0118 and claims that the HEVC/RExt restrictions that prohibit prediction between trailing and leading pictures hurt compression efficiency, especially for field coding referencing structures. The contribution reports that the proposed changes provide a 1.7% average bit-rate reduction for all eight test sequences used within the MPEG AHG on study of interlaced coding in HEVC using QPs [22, 27, 32, 37]. Using QPs [17, 22, 27, 32] the reported bit-rate reduction is 1.5%. Source code patches for generating this result are attached with the contribution.

The contribution proposes to use a reserved NUT to enable prediction across IRAP pictures while still enable usage of the existing picture types such as CRA, RADL, RASL.

The contribution claims that even though it is possible to indicate random access capability using recovery point SEI messages with TRAIL pictures, the contribution states that it is strongly preferable to be able to use the IRAP NAL unit types to in order to perform random-access-point detection at NAL unit header level and make use of the features associated with IRAP pictures such as alternative HRD parameters etc.

The following elements are proposed:

- Use a reserved VCL NAL unit type (type 11 is proposed) to be used for a new picture type, here called a CTR picture. The CTR picture is a trailing picture that, in contrast to the existing trailing picture types, can be referenced by leading pictures.

- Add a constraint that a CTR picture must immediately follow the associated IRAP picture in decoding order.

- Add a constraint that there can be at most one CTR picture for each IRAP picture (IDR, BLA, CRA).

The proposal notes that an alternative solution that was discussed at the last meeting was to add a flag in the RExt SPS extension to allow one trailing picture that directly follows an IRAP picture in decoding order to be used for reference by leading pictures associated with the same IRAP picture.

Version 2 of this contribution contains results for 8 sequences instead of 6, as well as results for QPs [17, 22, 27, 32].

Discussion:

It was suggested that a prefix NAL unit could be defined to indicate random accessibility – this way the bitstream can still be backward compatible to HEVC version 1, while when using a new VCL NAL unit type legacy decoders would ingore pictures containing VCL NAL units using the new VCL NAL unit type.

It was also suggested that systems specifications usually have their own ways of indciating random accessiblity, and such indications could be defined by systems specifications – one such example using access unit delimiter was mentioned.

Agreed to consider adding a note to the spec, mentioning how the random accessibility can be indicated by the recovery point SEI messsage, and possibly mention also that systems specs usually have their own ways of indicating random accessiblity.

JCTVC-Q0115 REXT/MV-HEVC/SHVC/3D-HEVC HLS: On indication of decoding process [M. M. Hannuksela (Nokia)]

The proposal can be summarized as follows:

- An indication of the applied decoding process is included in the VPS for each auxiliary picture layer and for each layer that is not included in any output layer set (excluding the additional independent output layer sets as proposed in JCTVC-Q0078/JCT3V-H0026).

  - independent_layer_dec_process_idc is provided for independent layers and can specify a decoding process of HEVC version 1 or range extensions.

  - predicted_layer_dec_process_idc is provided for predicted layers and can specify a decoding process of MV-HEVC or SHVC.

  - Both independent_layer_dec_process_idc and predicted_layer_dec_process_idc are ue(v)-coded and decoders shall ignore layers that have a value of independent_layer_dec_process_idc or predicted_layer_dec_process_idc that falls outside the currently allowed range.

  - independent_layer_dec_process_idc and predicted_layer_dec_process_idc also specify constraints on sps_extension_type_flag[ i ] values for the associated layer.

- The decoding process to be used for the current picture is selected as follows: If the current picture is within an auxiliary picture layer, the decoding process determined by the independent_layer_dec_process_idc or predicted_layer_dec_process_idc for the auxiliary picture layer is used. Otherwise, the decoding process determined by the profile of the target output layer set is used.

The contribution proposes indication of decoding process when a profile was not available to describe the aux pic or a layer that is not included in an output layer set.  It was questioned why not to just define a profile for the capability?

It was suggested that the new syntax elements could be in the VPS VUI rather than the VPS, but the syntax elements are used in the decoding process.

Revisit in track.

## 7.6.1 General

JCTVC-Q0183 MV-HEVC/SHVC HLS: SEI message cleanups [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm)] [late]

This document proposes some clean-ups on semantics of SEI messages specified in HEVC for their use in SHVC/MV-HEVC. These changes include 1) definitions of output operation point and picture unit, 2) some restrictions on order of SEI messages, 3) some clarifications on the applicable operation points or layers of SEI messages, 4) some restrictions on nesting of SEI messages as well as some updaates to the semantics of the scalable nesting SEI message, 5) some restrictions on the values of nuh_layer_id and TemporalId of SEI NAL units, 6) some udpates to the restrictions on the presence of SEI messages between two VCL NAL units, 7) some udpates to the restrictions on repetition of SEI messages, 8) some updates to the semantics of a list of specific SEI messages, such as the display orientation SEI message, the recovery point SEI message, and the region refresh information SEI message. Some of the changes are asserted also apply to HEVC version 1 and thus could be considered as corrigendum items.

Proposal 2.1-1 Agreed in spirit, need to check text

Proposal 2.1-2 Agreed in spirit, need to check text

Proposal 2.1-3 Agreed in spirit, need to check text

Proposal 2.1-4 Agreed in spirit, need to check text

Proposal 2.2-1 Agreed in spirit, revisit to confirm text

Proposal 2.2-2 No action, for future flexibility

Proposal 2.2-3 Agreed in spirit, need to check text, value should be 6

Proposal 2.2-4 Tentatively agreed in spirit, need to check text, revisit

Proposal 2.2-5 No action, for future flexibility

Proposal 2.3-1 Agreed in spirit, need to check text

Proposal 2.3-2 Agreed in spirit, need to check text

Proposal 2.3-3 Agreed in spirit, need to check text

Proposal 2.3-4 Agreed in spirit, need to check text

Proposal 2.3-5 Agreed in spirit, need to check text

Proposal 2.3-6 Agreed in spirit, need to check text

Proposal 2.3-7 revisit

Proposal 2.4-1 Agreed in spirit, need to check text

Proposal 2.4-2 Agreed in spirit for non-hybrid scalability, need to check text, revisit for hybrid scalability

Proposal 2.4-3 Agreed in spirit, need to check text

Proposal 2.5-1 revisit

Proposal 2.5-2 revisit

Proposal 2.5-3 No action, for future flexibility

Proposal 2.5-4 Tentatively agreed in spirit, need to check text, revisit

Proposal 2.5-5 Tentatively agreed in spirit, need to check text, revisit

### 7.6.4 Other SEI (3)

JCTVC-Q0090 HEVC/MV-HEVC/SHVC HLS: Redundant picture SEI message [M. Sychev, V. Stepin, S. Ikonin (Huawei)]

Chaired by Y.-K. Wang.

This contribution presents the support for redundant pictures in HEVC/SHVC/MV-HEVC via SEI message. The proposed solution describes the syntax and semantics of how to support redundant pictures. In opposite to H.263/H.264 redundant pictures the proposed approach allows to separate in time the protected primary picture with associated redundant picture. This increases probability of delivering one of them.

Discussion:

It was remarked that those restrictions related to POC and picture timing etc. for redundant pictures would not be needed, as normative decoding process for redundant pictures should not be specified.

It was asked whether the design is backward compatible to HEVC version 1, and it seems yes.

It was asked why not using auxiliray picture types for redundant pictures? One possible reason is that it is desriable to use it with HEVC version 1 in backward compatible.

One participant, who claimed to be the one who originally introduced the redundant picture feature into the world, commented that the redundant pictures feature is not useful anymore nowadays. Another participant commented that the design seems to work for what is intended, but his company has no plan to use the feature.

It was asked whether the AVC redundant pictures feature has been used. It seemed no.

It was asked any other participants besides the proponent think that the group should standardize the support of redunant pictures? There was no in the BoG.

It was requested to ask the same question to a larger audience.

Revisit in track.

JCTVC-Q0164 MV-HEVC/SHVC HLS: On sub-bitstream property SEI [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)]

This contribution proposes to add new sub bitstream mode in Sub-bitstream property SEI to support signalling bitrate information of only output layers in output layer set. The new sub bitstream mode enable signalling bitrate of enhacement-layer only layer bunch.

If a rewriting process is not specified for independent layer trees, this signalling may be useful.

Revisit after review of Q0078 is completed.

JCTVC-Q0167 MV-HEVC/SHVC HLS: On layers not present SEI [T. Ikai (Sharp)]

This contribution proposes to remove semantics restriction of Layers not present SEI message for clean up. Additionally, it is proposed to introduce a new syntax to support maximum sub-layers information signaling for a specific period.

It appears that there may be a bug in the SEI message semantics that may have been introduced when the message was changed from layers present to layers not present.  Revisit after corrected text is provided by Y.-K. Wang.

It was proposed to add a maximum sub-layers indication.  The usefulness of the information was questions.

## 7.4.1 Auxiliary pictures

JCTVC-Q0161 Layer association of auxiliary pictures [T. Ikai (Sharp)]

Currently primary and auxiliary picture association is specified by the equality of ScalabilityId. Under this definition, one auxiliary picture could be associated with several primary pictures if there are several layers which have the same ScalabilityId. This characteristics are thought to be useful for auxiliary picture sharing. However this definition also can bring about non-intentional association in multiple dependent tree case. To address this problem, Proposal 1 is to define new ScalabilityId, named UserId, to distinguish the primary picture which has the same ScalabilityId.

The other problem is that, when ScalabilityId is different, the auxiliary picture sharing cannot be achieved, e.g. normal resolution base layer and high resolution enhancement layer cannot share one common auxiliary picture. To address this problem, Proposal 2 is to add a new flag to control whether a specific ScalabilityId is used or not for primary picture association.

Revision1 fixed the syntax table and add the ScalablilityId usage relaxation for proposed UserId.

It was questioned if an inference rule could be defined that would automatically associate an aux pic with scalable dependent layers, if no aux pic associated with that layer.

It was suggested that if additional information signaling was needed, an SEI message may be the more appropriate location.

In proposal 2, it was suggested to modify the association rule to allow multiple primary picture association.

General discussion:

Agreed in spirit that an aux pic can be associated with more than one primary picture. Refer to editors the task to ensure that no language in the spec precludes this. In JCTVC-Q0081 discussion notes, a decision was made to remove the constraint that there must be a primary pic associated with each aux pic.  Refer to editors, that other language in the semantics near AuxId derivation could be moved to the alpha SEI message.

Is it possible to have more than one layer with the same value of DependencyId?  Similar for ViewOrderIdx.  It was suggested that the appropriate place for such a restriction would be in the profile definition.  Agreed that in the scalable main, scalable main 10 and stereo main profiles, should have a restriction against duplicate values of DependencyId (or ViewOrderIdx) when AuxId equal to 0.

It was proposed to allocate a value of scalability_mask[ i ] to unspecified.  This usage would be different than the intended use of this syntax element. No action.

JCTVC-Q0162 SHVC/MV-HEVC HLS: Scaled offset information of auxiliary pictures [T. Ikai (Sharp)]

This contribution proposes to constrain bitstream not to include scaled reference layer offset information for auxiliary pictures in SPS refered from primary pictures. The proposed restriction is asserted to help avoiding confusion and reducing implementation complexity caused by supporting two different ways of signalling spatial relation between primary and auxiliary pictures.

It was suggested that an existing constraint on the value of scaled_ref_layer_id[ i ]  is sufficient to avoid confusion. No action.

## 7.6.1 SEI and VUI General

JCTVC-Q0084 VUI entries for YZX and digital cinema EOTF [C. Fogg, J. Helman (Movielabs)]

## 7.6.3 Colour-related SEI

JCTVC-Q0074 SEI message for Colour Mapping Information [P. Andrivon, P. Bordes, E. François (Technicolor)]

JCTVC-Q0086 Mastering Display Color Volume SEI [C. Fogg, J. Helman (Movielabs)]

## General discussion about auxiliary picture association with primary picture

How to determine the primary picture that an aux pic is associated with?

The existing Scalability Id mechanism can be used for a default association process. (See examples #1 and #3 below.)  An SEI message could override the default association.

Should the default association behavior be described in the specification?

Does the alpha SEI message need an indication of which is the associated primary picture layer?

Revisit in track these issues.

An alpha picture could be scalably coded, if there was a Scalable Main profile indication for the alpha aux pic layer

Example #1:

layer_id 0, DepId 0, AuxId 0, base primary

layer_id 1, DepId 1, AuxId 0, enhan primary, direct depend on layer id 0

layer_id 2, DepId 0, AuxId 1: base layer alpha

layer_id 3, DepId 1 Aux Id 1 :enhan layers alpha, direct depend on layer_id 2

Example #2:

layer_id 0, DepId 0, base primary

layer_id 1, DepId 1, enhan primary, direct depend on layer id 0

layer_id 2, DepId 1 Aux Id 1 : high res alpha.

Could use some type of inference to make the aux pic associated with more than one primary pic

Also, existing syntax allows multiple aux pics to be associated with the same primary picture, as long as they have different values of nuh_layer_id

Example #3:

layer id 0, DepId 0, base primary

layer id 1, DepId 0, Aux Id 1, alpha A

layer id 2, DepId 0, Aux Id 1, alpha B

This example may need some external means for the decoder to determine whether to use alpha A or B