| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0229r1 |
| --- | --- |

| Title: | HEVC still picture coding performance evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Didier Nicholson Antonin Descampe Thomas Richter Chaker Larabi | Tel: Email: | +33 1 46 73 06 06 didier.nicholson@vitec.com a.descampe@intopix.com richter@rus.uni-stuttgart.de chaker.larabi@univ-poitiers.fr |
| Source: |  |  |  |

_____________________________

# Abstract

This document presents coding performance evaluation of HEVC for coding still pictures. JPEG reference test image set, available in 4:2:0, in 4:4:4 as well as in RGB 4:4:4 has been used for the evaluation. All Intra configuration was used and for 4:4:4 and RGB 4:4:4 content, HEVC Rext was used (HM13.0 + RExt 6.0), and results compared to JPEG 2000 (using OpenJPEG v2.1 release candidate).

# Introduction

The objective of this document is to present coding performance of HEVC for coding still pictures, done in cooperation with JPEG experts with the objective of verifying the performance of HEVC for coding still pictures.

For the purpose of this evaluation, conditions detailed in WG11 N14086 document, agreed by WG1 JPEG experts have been followed. The set of test pictures has been prepared from JPEG test set to be suitable to be encoded by HM HEVC reference software, by using a RGB to YCC software conversion tool (difftest_ng) provided by JPEG experts. This tool has also being used for objective measurement comparisons.

# Image test set

The JPEG test images test set has been used for this evaluation. In a first step some of the pictures have been slightly cropped to have images sizes multiple of minimum CU size as it seemed to be a restriction of the HEVC reference software. Cropping operation concerned only P04, P14, Zoo and Oahu images, and pixels cropped are bottom right pixels. Later it has been seen that HM can deal with images of any size with the appropriate option.

Table 1 Still test-set and respective characteristics

| Image | Resolution | Characteristic |
| --- | --- | --- |
| Bike | 2048x2560 | Part of the original JPEG 2000 data set (ISO12640) |
| Café | 2048x2560 | Part of the original JPEG 2000 data set (ISO12640) |
| Woman | 2048x2560 | Part of the original JPEG 2000 data set (ISO12640) |
| P01 | 4064x2704 | Image provided by Microsoft |
| P04 | 2264x1512 | Image provided by Microsoft |
| P06 | 4064x2704 | Image provided by Microsoft |
| P08 | 2592x1944 | Image provided by Microsoft |
| P10 | 2128x2832 | Image provided by Microsoft |
| P14 | 2704x3496 | Image provided by Microsoft |
| P26 | 4288x2848 | Image provided by Microsoft |
| P30 | 4064x2704 | Image provided by Microsoft |
| Zoo | 3904x2600 | Image provided by Olivier Pfeiffer |
| Oahu | 2600x3904 | Image provided by Olivier Pfeiffer |

Bike			Cafe	Woman	P01

P04	P06	P08

P10	P14	P26

P30	Zoo	Oahu

The test images have been converted from PPM file format as available on the JPEG ftp site (ftp:// etro6.vub.ac.be, under pub/testing/original directory), to .yuv and .rgb raw file formats compatible with HM software. This conversion has been made by using difftest_ng utility, provided by JPEG experts as included in wg1n6562 document (difftest_ng is a software under GPL licence).

PPM RGB have been converted to .rgb, 4:4:4 YCC and 4:2:0 YCC. The RGB to YCC colour transform used in difftest_ng conversion software is the same as the one embedded into JPEG 2000, named ICT (Irreversible Color Transform), and complies with sYCC IEC standard.

4:4:4 and 4:2:0 YCC data are therefore representative of YCC input to JPEG  2000 (or JPEG) codec. YCC data are full range data and care has to be taken when configuring HEVC encoders.

Command lines used for the conversion:

PPM RGB to raw RGB:

difftest_ng.exe --convert bike_2048x2560_444_8bit_444_rgb.raw@2048x2560x3:[8=0]:[8=1]:[8=2] bike.ppm

PPM RGB to 4:4:4 YCC:

difftest_ng.exe --toycbcr --convert bike_2048x2560_8bit_444.raw@2048x2560x3:[8=0]:[8=1]:[8=2] bike.ppm

PPM RGB to 4:2:0 YCC:

difftest_ng.exe --toycbcr --csub 2 2 --convert bike_2048x2560_8bit_420.raw@2048x2560x3:[8=0]:[8=1]/2x2:[8=2]/2x2 bike.ppm

# Encoders configurations

HEVC HM software.

HM13.0 + RExt 6.0 has been used, with All Intra (AI) configuration, as provided JCTVC-P1006 document.

RGB 4:4:4

For RGB 4:4:4, configuration files used are cfg/encoder_intra_main_rext.cfg and a per sequence configuration file, made from cfg/per-sequence configuration files made for AHG5 core experiments

YCC 4:4:4

For YCC 4:4:4, configuration files used are cfg/encoder_intra_main_rext.cfg and a per sequence configuration file, made from cfg/per-sequence configuration files made for AHG5 core experiments

YCC 4:2:0

For YCC 4:2:0, configuration files used are cfg/encoder_intra_main.cfg and a per sequence configuration file

For the two YCC configurations, as input data are full range data, VideoSignalTypePresent and        VideoFullRange have been set to 1.

As per N14086, QP values used were 12, 17, 2, 27, 32 and 37.

JPEG 2000 Openjpeg software.

OpenJPEG v2.1 release candidate software has been used for encoding the test pictures with JPEG 2000.

JPEG 2000 compression parameters used are conforming to WG11 N14086 document.

The following command lines have been used:

RGB 4:4:4

opj_compress -i image.ppm -o image_j2k_qp.j2k -OutFor J2K -n 6 -b 64,64 -I -r compression_rate

YCC 4:4:4

opj_compress -i cafe_2048x2560_8bit_444.raw –o image_widthxheight_8bit_444_ycc_j2k_qp.j2k -OutFor J2K -n 6 -b 64,64 -I -mct 0 -F width,height,number_component,bitdepth,u -r compression_rate

YCC 4:2:0

opj_compress -i cafe_2048x2560_8bit_420.raw -o image_widthxheight_8bit_420_ycc_j2k_qp.j2k -OutFor J2K -n 6 -b 64,64 -I -mct 0 -F width,height,number_component,bitdepth,u@1x1:2x2:2x2  -r compression_rate

For each configuration, the compression rate parameter has been calculated from the HEVC compressed image, to match exactly the same compression ratio between JPEG 2000 and HEVC. This is possible because JPEG 2000 includes a Post Compression Rate Control mechanism that allows precise compression ratio control. As a result each JPEG 2000 compressed image can be directly compared to the corresponding HEVC compressed imaged it referred to.

| Parameter | OpenJPEG v2.1 RC | Comment |
| --- | --- | --- |
| code‐block size | 64x64 | Default entropy coding mode |
| Number of quality layer | 1 | progression order LRCP |
| precincts | No |  |
| Wavelet transform | 9x7 |  |
| Number of decomposition levels | 5 |  |
| Rate control | 4.0 to 225 compression rate (2bpp to 0.035bpp) |  |
| Visual weighting | Off |  |
| Number of bits per component | 8 | 4:4:4 and 4:2:0 chroma subsampling. For YCC configurations ICT color transform was disabled |

For each of the two YCC configurations, RGB to YCC color transform (ICT) has been disabled. This color transform is used in JPEG 2000 to convert an RGB image when the data to be encoded have to be in YCC format, but direct compression of RGB components can be also performed. Using ICT is the common use with JPEG 2000, and has been used for RGB 4:4:4 encoding.

# Results

All detailed results are included in the attached Excel sheet.

Results are presented as for JCTVC-P1006, grouped in three categories (Main, High and Super-high tiers). Main tier correspond to the use of QP values of 22, 27, 32 and 37, High tier corresponds to the use of QP values of 17, 22, 27 and 32, and Super-high tier corresponds to QP values of 12, 17, 22 and 27.

The reference is corresponding to HEVC and the tested codec to JPEG 2000. The following table shows the BD-rate increase when encoding by using JPEG 2000. Negative values enlightened in green shows improvement of BD-rate when using JPEG 2000 instead of HEVC.

RGB 4:4:4

YCC 4:4:4

YCC 4:2:0

Detailed results for High tier configuration

# Recommendations

The above results validated the performances of HEVC for coding still pictures, both for 4:2:0 and 4:4:4 8bit images. The following actions are recommended:

- To propose a 4:4:4 Still image profile based upon one of the 4:4:4 RExt profiles

- To set-up subjective tests to complete these verification tests

- To continue verification tests with more than 8bit material

- To continue verification tests with video material (including more than 8bits, using HEVC verification test material as well as RExt video sequences) instead of still pictures.

- To create HEVC still picture compliance (V1 and RExt) test set, potentially by using the JPEG Still picture test set, as for HEVC Still Picture profile, as included in HEVC V1, compliance material is not available.

# Patent rights declaration(s)

VITEC does not have any current or pending patent rights relating to the technology described in this contribution.