| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0231 |
| --- | --- |

| Title: | Crosscheck of JCTVC-Q0095 on Coding the prediction differences of the intra BC vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chao Pang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The crosscheck results of JCTVC-Q0095 are reported in this document. In the contribution, a new method of coding Intra BC block vector difference is proposed. The coding results match those results provided by the proponent.

# Description

This document reports the crosscheck results of JCTVC-Q0095. In JCTVC-Q0095, the block vector difference for Intra BC is coded in two parts: most significant bits and refinement bits. Additional CABAC contexts are employed. The software provided by the proponent is compiled and tested on Linux cluster. The simulation is performed under the AHG8 common test condition.

# Results

In this section, the crosscheck results are presented, both lossy (Error: Reference source not found) and lossless (Error: Reference source not found), and they match the results provided by the proponent.

Table 1 Lossy results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.9% | -0.7% | -0.7% | -0.7% | -0.6% | -0.5% | -0.5% | -0.4% | -0.4% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.6% | -1.5% | -1.6% | -1.2% | -1.2% | -1.2% | -1.0% | -1.0% | -0.9% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -1.9% | -1.8% | -1.9% | -1.5% | -1.4% | -1.5% | -1.2% | -1.1% | -1.1% |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.7% | -1.7% | -1.7% | -1.6% | -1.5% | -1.6% | -1.5% | -1.3% | -1.4% |
| YCbCr 4:4:4 SC (Optional) | -2.3% | -2.3% | -2.2% | -2.0% | -2.0% | -1.9% | -1.8% | -1.7% | -1.6% |
| Enc Time[%] | 96% |  |  | 95% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 100% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.6% | -0.4% | -0.3% | -0.5% | -0.4% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -1.4% | -1.3% | -1.2% | -1.2% | -1.0% | -1.0% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.4% | -1.3% | -1.3% | -1.2% | -1.1% | -1.1% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.7% | -1.5% | -1.6% | -1.5% | -1.4% | -1.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -2.1% | -1.7% | -1.8% | -2.0% | -1.8% | -1.8% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.2% | -0.6% | -0.3% | -0.3% | -0.4% |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |  |  |  |
| RGB 4:4:4 SC | -1.2% | -1.2% | -1.2% | -0.9% | -0.9% | -1.0% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.6% | -1.4% | -1.5% | -1.3% | -1.2% | -1.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -2.4% | -2.4% | -2.4% | -2.1% | -2.0% | -1.9% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -2.3% | -2.4% | -2.4% | -2.2% | -2.3% | -2.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

Table 2 Lossless results

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.2% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.5% | -0.4% | -0.4% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.5% | -0.8% | -0.4% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.0% | -0.9% | -0.8% |
| YCbCr 4:4:4 SC (Optional) | -1.1% | -1.2% | -1.2% |
| Enc Time[%] | 101% | 103% | 106% |
| Dec Time[%] | 109% | 104% | 106% |