| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0232 |
| --- | --- |

| Title: | HDR/WCG StEM and Telescope clips encoded as Y’DzDx and BT.2020 Y’CbCr |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bill Mandel 100 Universal City Plaza Universal City, CA 91608 USA Chad Fogg, Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | bill.mandel@nbcuni.com chadfogg@gmail.com jhelman@movielabs.com |
| Source: | Universal Pictures, MovieLabs. |  |  |

_____________________________

# Abstract

For the investigations into both high bit-depth (JCT AdHoc 18) and the HDR/WGC adhoc (MPEG XYZ), eight new high dynamic range / wide color gamut test sequences (StEM: Confetti, MagicHour, WarmNight, CoolNight, Rain,; Telescope: SSInterior, HyperDebris, Nebula) contributed to the Valencia March 2014 JCT / MPEG meeting were coded as HEVC Main12 Random Access bitstreams according to the workflow process described in JCTVC-Q0085 / m32804. The accompanying Excel spreadsheet charts the PSNR and bitrates extracted from the HEVC HM reference encoder logs.

# Introduction

The tests used bitstreams from five source clips taken from the HDR version of the Digital Cinema Initiatives Standard Evaluation Material (DCI StEM) provided in [2] and three source clips taken from Telescope, a short that may be available soon for licensing for test use from Disney. The sources were professionally color graded for high dynamic range and mastered as wide color gamut content using the P3 colorspace and exported in 16-bit TIFFs encoded using the PQ EOTF.

The eight sequences that were used for this test are as follows:

| No. | Brief description | Master frames |
| --- | --- | --- |
| 1. | StEM Confetti through Juggler | 1784 to 2144 |
| 2. | StEM Magic Hour: Fountain to Table | 3527 to 3887 |
| 3. | StEM Warm Night: Torch to Table (before slomo) | 6280 to 6640 |
| 4. | StEM Cool Night: Fog | 8208 to 8568 |
| 5. | StEM Rain | 9872 to 10232 |
| 6. | Telescope SSInterior | 4212 to 4472 |
| 7. | Telescope Hyperspace Debris | 5900 to 6260 |
| 8. | Telescope Nebula | 8200 to 8560 |

Steps described in [1] were applied to generate both Y’DzDx and Y’CbCr BT.2020 bitstreams. Both sets of bitstreams were generated using an input scale factor of 1/169, and the Y’DzDx bitstreams used the formula optimized for the D65 white point at 500 cd/m2 as defined in tiff2ydzdx.cpp in [1].

The sequences were encoded using the HM-13.0+RExt-6.0. The software was complied with high bitdepth support (RExt__HIGH_BIT_DEPTH_SUPPORT = 1). The Main 12 profile in Random Access case was used, enabled by taking encoder_randomaccess_main10.cfg but changing profile to main-Rext and setting inputBitDepth and internalBitDepth to 12. A ChromaQPOffset value of -12 was applied to both Cb and Cr.

# References

[1] B. Mandel, C. Fogg, J. Helman, “High Dynamic Range, Wide Color Gamut workflow”, JCTVC-Q0085

[2] J. Helman, W. Husak, “HDR/WCG StEM Content Description”, JCTVC-Q0228.

# Patent rights declaration(s)

Universal Pictures and MovieLabs do not have any current or pending patent rights relating to the technology described in this contribution.