| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0234 |
| --- | --- |

| Title: | BoG report on SHVC upsampling process and phase offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met 29 March 2014.

The BoG recommends that the following activities take place:

- Discussion within the track of whether the phase adjustment should use explicit signaling of phases vs. the current method which derives phases based upon flags for interlace indication and cross-layer phase position?

- Side activity among proponents to consider a general solution for explicit signaling of related parameters to address multiple issues, including 540p->720p, ROI scalability, field-to-frame, and other phase adjustment issues, and revisit this meeting.

The BoG plans to meet again to revisit:

- Any new contributions based upon side activity discussion

- Revisit phase adjustment

- Revisit improving coding inefficiency for 540p -> 720p scalability

# Document review details

## Phase adjustment

JCTVC-Q0107 SHVC HLS: On picture level resampling phase filter selection [Y. Ye, Y. He, Y. He (Interdigital)]

At the 16th JCTVC meeting, a few flags for the purpose of vertical resampling phase adjustment were adopted into the SHVC draft 6. In combination, these flags allow the resampling phase filters to be adjusted at the picture level. One important use case to which vertical resampling phase adjustment applies is the case of interlaced to progressive scalability. In this contribution, alternative signaling methods for resampling phase filter selection are proposed. It is asserted that the proposed signaling methods harmonize all syntax elements related to resampling phase filter selection, and that they provide more flexibility than SHVC draft 6.

Two options proposed.  In first option, PPS signaling of one set of phases.  In second option, signal multiple sets of phases in PPS, and select in the slice header.

Remove three flags, one in SPS and one in VPS and one in slice header.

JCTVC-Q0120 Re-sampling using existing phase offset flag signaling [K. Minoo, D. Baylon (Arris)] [late]

This contribution proposes to modify the semantics of the existing phase offset flag signaling for spatial scalability to accommodate additional phase offset cases which are not currently included.  In particular, the proposed approach includes cases where field to frame scalability is combined with additional spatial scalability, e.g. 1080i to 4k. The suggested correction in phase calculation improves the coding efficiency compared to no vertical phase correction by 5.1% and 0.5% for All Intra and Random access in a conducted test.

No syntax changes in proposal.  Adds calculation of deltaX, at the per picture layer.

An asserted advantage is to handle the 1080p->4k case, or other unusual ratios.  However, SHVC vs simulcast is only about a 16% gain, generally, for those resolutions.  Should have no difference for 2:1 vertical ratios.

If explicit signaling used, as in Q0107, would also need to signal DeltaX, DeltaY, to get the same per sample behavior as proposed here.

JCTVC-Q0168 On phase offset for resampling process in SHVC [J. Chen, K. Rapaka, M. Karczewicz (Qualcomm)]

This contribution proposes an alternative signalling method for phase offset used in resampling process.

Similar to Q0107 option 2, but signals in the VPS.

Could also remove flag in VUI.

General discussion:

Calculation of delta based on current indicators can be helpful for non 2:1 upsampling ratios, might be useful for 480i -> 720p.

Delta values could be explicitly signaled, if desired, rather than derived from existing syntax elements.

Explicit signaling could support different chroma sample locations.

Options:

- No change from current design

- Add delta calculation from Q0120

- Have explicit phase signaling (details of which param sets TBD)

- Have explicit phase and delta signaling (details TBD)

No significant computational complexity differences in #1-4

Overhead syntax efficiency differences

- 1 VPS flag, 1 SPS flag, 1 slice header flag

when feature not used, slice header flag not needed

- same as 1

- several options

  - 1 gating flag in a PS, 4 3-bit syntax elements in a PS per set plus num sets, set selector idc (1 bit if 2 sets)

- several options

  - 2 gating flag in a PS, 8 3-bit syntax elements in a PS per set plus num sets, set selector idc (1 bit if 2 sets)

Questions for track

- Use explicit phase signaling vs current method which derives from flags target at interlace indication and current/reference layer phase shift?

Ask track to decide this question, and BoG to meet again later to work out details based on this track decision.

JCTVC-Q0119 Increased resolution for scaled reference layer offset [K. Minoo, D. Baylon, A. Luthra (Arris)] [late]

This contribution presents an approach to signaling phase offsets to improve inter-layer prediction precision and hence the compression performance of SHVC. The proposed method increases the resolution of the scaled reference layer offset to provide phase alignment between layers.

Proposes moving scale ref offset signaling to PPS, to allow per picture changes. (JCTVC-Q0166 is related.)   Explicit signaling of phase and delta, for luma and chroma.

No cross-check.

Per sample calculation similar to before. Per picture change for the Phase and Delta.

JCTVC-Q0216 Cross-check for modified phase offset calculation in JCTVC-Q0120 [J. Chen (Qualcomm)] [late]

Confirmed the simulation results.

## Scaling Factor

JCTVC-Q0122 Increased precision for deriving the reference layer sample locations [K. Minoo, D. Baylon (Arris)] [late]

It was observed in [1] that the current precision in computing the reference layer sample location can lead to errors in filter phase selection, especially as the picture size increases.  It is proposed that the precision be increased without changing the maximum bit depth.

Per sample calculation changes the shift from a fixed value to a variable value.  Could also increase the ScaleFactorX (and Y) bit depth to 32 from 16.

Results not uploaded.  4k with 1.5x scalability has gain 0.07% for AI and 0.05% for RA.

Cross-check in JCTVC-Q0197.

Further study.  Interesting to see results for 8k.

JCTVC-Q0197 Cross-check for increased precision for deriving the reference layer sample location [E. Alshina (Samsung)] [late]

Crosscheck of Q0122; includes a counter-proposal. However, the counter-proposal aspect was withdrawn in a revised version of the contribution.

JCTVC-Q0159 AHG 13: Sub-region extraction – position calculation and comparison of different approaches [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

This contribution firstly reports an analysis comparing different approaches to realize sub-region extraction with SHVC. Based on the analysis, it is proposed to support sub-region extraction with extracting both enhance- and base-layer data by 1) adding new syntax in SPS extension and 2) modifying scale and reference pixel location derivation. The proposed changes are asserted to enhance the application scenarios using sub-region extraction.

In revision 1, experimental results of applying the proposed method for 520p to 720p scalability is added. The experimental results assertedly indicate that the proposed method can achieve the performance gain equivalent to those reported in JCTVC-Q0104.

If the equations were editorially modified, the per sample calculation complexity is not changed.

Some additional per picture calculations.

JCTVC-Q0104 Fix for the computation of scaling factors used in inter-layer prediction [K. Andersson, J. Samuelsson (Ericsson)]

Reference scaling offsets fails to align the reference and the enhancement layer for inter-layer prediction for the case of 540p to 720p spatial scalability. It is proposed to fix this by the use of fractional reference scaling offsets in the computation of the scale factors. In comparison to SHM-5.0 this improves the coding efficiency by 12.1% for intra, 9.6% for random access and 8.8% for low delay B. No change in performance for common conditions.

Affects calculation of ScaleFactor, which is on a per sequence or per picture basis.  Doesn’t affect per sample operations.

In the proposal, a single syntax element is used for both horizontal and vertical adjustment.  It was suggested that perhaps two separate syntax elements for horizontal and vertical might be preferable.

JCTVC-Q0215 Cross-check for refined scaling factor calculation in JCTVC-Q0104 [J. Chen (Qualcomm)] [late]

Confirmed the experimental results.

General discussion:

There was consensus that there is a problem with 540p to 720p scalability which should be addressed in some way.

Both Q0104 and Q0159 can address the 540p to 720p scalability issue.  Another option is to use the base layer cropping parameters.  The signaling in Q0159 could be used to re-signal the base layer cropping parameters.

Q0159 address the ROI scalability problems.  Q0104 may help reduce the error, but won’t necessarily eliminate it for all possible resolution combinations.

Inter-layer SEI message may address the ROI scalability issue.

Explicit signaling of phases is not sufficient to solve the 540p to 720p scalability issue because it doesn’t adjust the ScalingFactor calculation.

Tentative decision is to use the base layer cropping factors for computation of ScalingFactor.

Encourage side activity by proponents to solve a general explicit signaling of related parameters to address multiple issues, including 540p->720p, ROI scalability, field-to-frame, and other phase adjustment issues, and revisit this meeting.

## Other

JCTVC-Q0187 Resampling of reference layer frames in interlaced-to-progressive scalability [M. M. Hannuksela, K. Ugur (Nokia)] [late]

It is asserted that when a reference layer frame represents interlaced source content, the present SHVC resampling process is unable to generate an inter-layer reference picture on the basis of a selected field of the reference layer frame for interlaced-to-progressive scalability. This contribution proposes a vertical decimation factor syntax element for the slice segment header to indicate whether every sample row or every other sample row of the reference layer frame takes part in the resampling process. Additionally, this contribution proposes a vertical offset syntax element for the slice segment to indicate whether the top field or the bottom field of the reference layer frame is resampled, when the vertical decimation is indicated to be in use. The inter-layer resampling process is proposed to be appended according to the proposed syntax elements.

No experimental results are provided.

Computational changes are required more than per picture.

New syntax elements signaled at slice segment header, but are the same for the entire picture.

In the use case, the base layer has two interlaced fields coded as a frame.  The fit within the scope of the current project was questioned.