| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 Mar. – 4 Apr. 2014 | Document: JCTVC-Q0236_r1 |
| --- | --- |

| Title: | BoG report for Non-CfP SCC and related documents |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Cohen | Email: | cohen@merl.com |
| Source: | BoG |  |  |

_____________________________

- Summary

A BoG was held on Saturday March 29 to go over the Non-CfP screen content coding and related technical contributions. These documents include those in Sections 6.2 and 7.3 of the JCT-VC meeting report JCTVC-Q_Notes_d2.doc, except for the ones in Section 7.3 that were already covered during previous sessions.

A few changes to the document categorization from the JCT-VC meeting notes were made: JCTVC-Q0138 and JCTVC-Q0128 were move into Section 7.3 and covered in this BoG, and JCTVC-Q0140 was moved out to Section 7.1.4 on Intra block copy.

The documents included both new proposals and proposals related to one or more tools also included in the formal responses to the SCC CfP. Improvements to and combinations of existing tools were also proposed. Most, but not all, of the proposals were related to tools proposed in response to the SCC CfP, or to SCC-related tools proposed at earlier meetings. Many of these tools are not in the current HEVC Range Extensions Draft Text, and because they were related to the SCC Joint CfP responses, no formal recommendations were made by the BoG to adopt any specific proposal into RExt.

Many documents proposed identical or very similar tools, or combinations of tools.The outcomes of most discussions on these documents were that the proposals should be considered when defining common software platforms or common conditions for subsequent CEs, TEs, or AHG experiments. This happened for several cases, including Palette mode color table generation, palette mode color index coding, SAO improvements, single-color CU coding, string-matching/dictionary coding, and related encoder improvements.

In addition to discussing specific proposals, methodologies for evaluating them were also discussed. There was discussion of and consensus on using a higher, resolution-dependent PSNR limit than was used in the SCC CfP for clipping per-frame PSNR values, because the current clipping value of 58.92 dB caused problems with both computing BD-Rate and plotting PSNR vs. Rate curves. There was also some discussion on how to evaluate SAO tools in a way that isn’t affected by the encoder’s RD-optimized mode decision process. That would be an issue more for RA and LB coding constraints as compared to AI. Issues related to reporting average PSNR values across RGB and YUV results were also discussed.

The general recommendation from the BoG for most of these proposals is that they be considered when defining common software platforms or common conditions for related CEs, TEs, or AHG experiments. Further details can be found in the document and discussion summaries included below in this report. Some convergence prior to the end of the meeting is desirable, as opposed to waiting for the post-meeting editing period for CEs/TEs.When the joint discussions on the CfP responses begin next week, how to consider these CfP-related documents may need to be addressed as well.

- Detailed meeting notes

Below are the detailed BoG notes. The section numbers match those in the JCT-VC meeting report JCTVC-Q_Notes_d2.doc. Documents for which there were notes from the previous two days are not included in this report, as they were not discussed in this BoG.

## Related contributions

JCTVC-Q0052 Low complexity encoders for JCTVC-Q0035 [B. Li, J. Xu (Microsoft)]

This document presents several low complexity encoder implementations and their coding results based on JCTVC-Q0035. The tested methods were coded using the same QPs as the RExt5.1 anchor, using the SCC CfP test conditions. Several methods for encoder speed-ups are proposed, e.g. early termination of RDO checking loops.

Lossy:

| Test Option | AI |  | RA |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. T | BD-Rate | Enc. T | BD-Rate | Enc. T |
| RExt-6.0 | −8.8% | 140% | −5.7% | 109% | −3.6% | 109% |
| Option 1 | −30.1% | 292% | −27.2% | 88% | −21.0% | 90% |
| Option 2 | −29.6% | 226% | −26.8% | 81% | −20.7% | 84% |
| Option 3 | −28.9% | 186% | −26.4% | 80% | −20.5% | 83% |
| Option 4 | −28.2% | 169% | −25.4% | 72% | −19.4% | 75% |
| Option 5 | −19.6% | 110% | −20.1% | 65% | −15.9% | 68% |
| Option 6 | −16.4% | 76% | −17.2% | 49% | −13.6% | 53% |
| Option 7 | −16.4% | 74% | −11.5% | 35% | −4.5% | 37% |

Lossless:

| Test Option | AI |  | RA |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bitrate | Enc. T | bitrate | Enc. T | bitrate | Enc. T |
| RExt-6.0 | 9.8% | 142% | 10.2% | 110% | 10.6% | 110% |
| Option 1 | 29.9% | 369% | 24.0% | 95% | 23.1% | 93% |
| Option 2 | 29.7% | 311% | 23.9% | 91% | 23.0% | 88% |
| Option 3 | 29.5% | 281% | 23.8% | 89% | 22.9% | 86% |
| Option 4 | 29.4% | 259% | 23.5% | 81% | 22.6% | 79% |
| Option 5 | 27.1% | 165% | 22.1% | 66% | 21.3% | 66% |
| Option 6 | 25.5% | 131% | 21.1% | 51% | 20.3% | 53% |
| Option 7 | 25.5% | 128% | 19.5% | 37% | 18.6% | 39% |

Some individual-sequences may show an increase in encode time, i.e. these techniques are optimized for screen-content material.

A participant suggested that it would be useful to have some encoder speed-up techniques in the common software used for the next set of screen-content related CE/TE/AHG tests.

JCTVC-Q0053 On BD-Rate results [B. Li, J. Xu (Microsoft)]

Informational: This document reports several problems when using BD-Rate to interpret the results submitted for the joint Call for Proposals of Screen Content Coding. It is noted that the SCC CfP template fails to calculate a valid number in some cases or may report a number that is far from expected. Thus, the authors recommend caution when using the BD-Rate results reported in the SCC CfP template. It is suggested to use PSNR difference as objective numbers instead. This document also reports that visual differences can be observed even when the PSNR is around 64 dB, which may not be expected for camera-captured content. Thus, it is suggested to use a higher threshold of 114.31 dB to clip PSNR values for screen content coding, if PSNR clipping is applied.

The clipping for the SCC CfP was approximately 58.92 dB, which is associated with a 1-pel error in an 8-bit value. It is asserted that this value is too low, because visual artifacts are observed, for example, on small regions or objects within a mostly flat or uniform picture. Also, the BD-Rate formula in the current template fails when two points have the same PSNR, which happens when more than one of the four test points is clipped. It was noted that using a higher threshold won’t necessary solve this problem, although it would reduce how often it happens with the existing lower clipping value.

If 1 pixel had an error of 1 level over a 1080p picture, the computed maximum achievable PSNR would be approximately 111.30 dB = 10log10(255*255*1920*1080). If 2 pels had a 1-level error, the max PSNR would be 108.29 dB, implying the cost to eliminate a 1-pel level difference is 3 dB. Defining a 0-pel differene as producing a maximum PSNR of PSNR0 = 2*PSR1 – PSNR2 = 114.31 dB. (so a lossless picture in lossy coding conditions would be reported as having a PSNR of 114.31 dB, which is slightly higher than what the PSNR would there would be a 1-pel error.

Participants asked whether differences between very high PSNRs, e.g. 75 – 110 dB, are visible in general, and whether special considerations should be made when evaluating differences in BD-Rate measures among different proposals when their PSNR values are in that high range.

Some participants commented that looing at R-D plots are more informative than only using BD-Rate metrics.

Some participants felt this would be useful, some felt that the results may be misleading.

It was noted that RExt currently computes an average MSE over all frames, which may be helpful as compared to per-picture PSNR, when some pictures are coded losslessly.

It was noted that the next set of CE/TE/AHG experiments on SCC may need to consider solutions to theh problems exhibited in the earlier tests, e.g. due to lossless frames in lossy coding conditions, and their PSNR/BD-Rate measures. A participanted noted that the 114.31 dB is better than the 999 dB value currently reported by the HM-Rext software for lossless frames under lossy conditions.

It was noted that this PSNR limit is dependent upon picture size.

There appeared to be consensus to use this method for computing a PSNR limit, where each resolution would have a corresponding limit.

JCTVC-Q0193 Combination of screen content coding proposals JCTVC-Q0034/JCTVC-Q0176 and JCTVC-Q0036 [R. Cohen, X. Zhang, A. Vetro, K. Sugimoto (MERL), A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi Electric), Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu (Huawei Technologies (USA)] [late]

The discussion of this proposal was chaired by Gary Sullivan. Please refer to tomorrow’s JCT-VC meeting report for BoG notes on this document.

JCTVC-Q0194 Cross-check of JCTVC-Q0034 [X. Zhang, R. Cohen (MERL), K. Miyazawa (Mitsubishi Electric)] [late] [miss]

JCTVC-Q0213 Fix for adaptive color space coding in JCTVC-Q0035 [B. Li, J. Xu (Microsoft)] [late]

This document presents the fix for adaptive color space coding in JCTVC-Q0035, including experimental results. The bug was caused by not normalizing the inverse color conversion matrix when coding was performed in YCoCg color space. After fixing the bug, the coding efficiency for RGB sequences can be improved. For example, the coding gain compared with the CfP anchor increases from 6.7% to 20.5% for animation RGB in AI fixed-QP coding. Compared with the CfP anchor, now on average the coding gain for AI is 32.3% (the average number includes YUV sequences, which are not changed).

Results are provided for both fixed-QP (same as anchor), and for SCC CfP test conditions, where the QPs were selected to reach a target file size.

Improvements from this fix were larger for AI as compared to RA or LB, as this was an intra-block only fix.

## Screen content coding

### Palette mode

JCTVC-Q0047 AHG10: Simplification of Palette Based Coding [W. Pu, R. Joshi, M. Karczewicz, F. Zou, J. Sole (Qualcomm)]

This document presents encoder improvements and codec simplifications in palette based coding. First, an improvement of palette generation is proposed, which achieves better coding efficiency without any syntax or semantic change. Second, bypassing index coding when the palette has only one color is proposed. Third, when the palette is not full, the redundant escape pixel flag signalling is removed. Fourth, the CABAC context used in escape pixel coding is removed by change fixed length code to truncated binary code. Index is also coded using truncated binary codes without any CABAC context to align with escape pixel coding.

Bug fixes related to JCTVC-P0303 (AHG10) RGB SC -0.6/-0.9/-1.2%; YCbCr SC -0.1/-0.4/-0.8

Removing macro from P0108: -.9/-1.4/-1.8; -.3/-.6/-1.1%

Also adding encoder-only improvements: -3.3/-3.8/-4.2; -1.8/-2.1/-2.8%

Simplifiation of index and escape pixel coding: Trunc. Binary code for both idx and escape; All bins coded in bypass mode;

Reduced index codeword by 1 in some cases (separate results not available for this case).

All above methods yields: -5.9/-6.2/-6.3%; -3.6/-4.2/-5.0%

Cross-checker stated that they had similar contributions with similar suggestions.

Proponent and cross-checker agree that the truncated binary coding of index and esc. values should replace the fixed-length coding.

Question: Should these and similar changes be integrated into SW for AHG10, and should this be used as basis for others to use for TE’s or RCE’s? A participant pointed out that P0108 could be used as basis as well. Discuss this and related common-platform improvements after palette-related contributions are reviewed, or as part of AHG or TE formation discussions.

JCTVC-Q0178 AHG10: Crosscheck of Simplification of Palette Based Coding (JCTVC-Q0047) [C. Gisquet (Canon)] [late] [miss]

JCTVC-Q0063 AhG10: Palette predictor stuffing [C. Gisquet, G. Laroche, P. Onno (Canon)]

Software of AhG10 on palette mode coding uses the last palette to predict the current palette, indicating through flags which elements of a palette are reused. It is proposed in the present contribution to combine the last coded palette and the last predictor palette by stuffing unused elements of the last predictor at the end of the last coded palette, using the resulting palette as predictor for the next palette-coded CU, and adding flags to indicate when there are no predicted elements left. It is reported that the use of such a prediction method provides gains for AI-MT, AI-HT and AI-SHT configurations, compared to AhG10 software of more than 2.3%/3.0%/3.5% for SC classes, and up to 4.3%/5.3%/5.9% on optional classes.

For SC sequences, gains were -3.1/-3.6/-3.8% RGB; -2.3/-3.0/-3.5% YCbCr (reference was AHG10 software that was integrated into RExt 6.0 (JCTVC-Q0094)

The palette size is still 32 elements, but the predictor size is doubled to 64 elements.

It was clarified that the predictor will not contain duplicate colors, so a pruning process is not needed.

Proponent suggests this be included in any new AHG or TE-related software.

Cross-checker has a new proposal that combines this method (JCTVC-Q0225)

Recommendation: Discuss this when considering what should be in future palette software for AHG/TE.

JCTVC-Q0150 Crosscheck AhG10: Palette predictor stuffing (JCTVC-Q0063) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0064 AhG10: Palette index coding [C. Gisquet, G. Laroche, P. Onno (Canon)]

Palette mode in AhG10 uses FLC for coding the index. This proposition instead uses two codelengths depending on the index, as well as removing impossible values from the set. It is reported that the use of one such prediction mode provides gains for respectively AI-MT, AI-HT and AI-SHT configurations, on top of AhG10 RExt6.0-based software, of more than 1.6%/1.7%/1.6% for SC classes, and up to 6.1%/6.2%/6.4% on optional classes.

    - Truncated binary coding (same as Q0047)

    - “impossible” values – can be used to remove one value (from either copy-left or copy-above).

    - Escape value can only be signaled if palette has maximal size

For SC sequences: -1.8/-1.7/-1.6% RGB; -1.6/-1.7/-1.7% over RExt6.0+P0303 (Q0094)

First two methods also proposed in JCTVC-Q0047; Second method also proposed in CfP response JCTVC-Q0037

JCTVC-Q0151 Crosscheck AhG10: Palette index coding (JCTVC-Q0064) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0065 AhG10: Transition copy mode for Palette mode [C. Gisquet, G. Laroche, P. Onno (Canon)]

During the previous meeting, a new index prediction method was proposed, using the position of so-called transitions to determine a position from which the prediction copies indexes. This contribution presents a small modification to the update mechanism of the proposition, and also describes a new prediction method that encodes a run of values being the expected transition. It is reported that the use of one such prediction mode provides gains for respectively AI-MT, AI-HT and AI-SHT configurations, on top of AhG10 RExt6.0-based software, of more than 2.4%/2.8%/2.9% for SC classes, and up to 5.8%/6.0%/6.1% on optional classes. It is also reported that a simpler mode only predicting the index value provides gains for respectively AI-MT, AI-HT and AI-SHT configurations, on top of AhG10 software, of more than 1.9%/2.3%/2.1% for SC classes, and up to 4.9%/5.0%/5.1% on optional classes.

This method was also presented at previous meeting as JCTVC-P0115. This method was also used in JCTVC-Q0037 and JCTVC-Q0035 CfP responses, and similar approach was done in JCTVC-Q0033 CfP response.

For SC sequences: -2.9/-2.8/-2.7% RGB; -2.4/-2.8/-2.9% YCbCr

Simplified mode: -2.3/-2.3/-2.1 RGB; -1.9/-2.3/-2.4% YCbCr

Proponent suggests including these in palette tools used in any related CE/TE/AHG.

Crosschecker commented that it’s not clear yet how all these proposed methods (this and other palette-related ones) interact, so studying them together in AHG or CE may be of interest.

JCTVC-Q0152 Crosscheck AhG10: Transition copy mode for Palette mode (JCTVC-Q0065) [W. Pu, R. Joshi (Qualcomm)] [late] [miss]

JCTVC-Q0066 AhG10: Run coding for palette mode [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

This contribution proposes a modification of the palette mode of the AhG10 software. This modification relates to the run coding and was proposed in JCTVC-P0113. The principle is to limit the number of runs coded after a palette index of the run mode. A syntax element is transmitted for each palette CU to decode or not decode the run. The proposed modification of the AhG10 software gives 0.4% gain on top of AhG10 software for screen content sequences for Intra configurations.

For SC sequences: -.6/-.6/-.6% RGB; -.4/-.5/-.6% YCbCr

Q0071 proposed similar method

Q0047 included alternative encoder modification, asserted to have higher gain.

Proponent will look into whether this method is still applicable given Q0047 changes.

JCTVC-Q0153 Crosscheck AhG10: Run coding for palette mode (JCTVC-Q0066) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0069 AhG10: Combination of Palette coding tools [C. Gisquet, G. Laroche, P. Onno (Canon)]

Several palette coding tools were proposed in the scope of AhG10. This proposal studies their combination, together with several bugfixes and more limited tools. It is reported that the combination provides gains for respectively AI-MT, AI-HT and AI-SHT configurations, on top of AhG10 RExt6.0-based software, of more than 5.3%/7.0%/8.0% for SC classes, and up to 11.6%/12.6%/13.8% on optional classes.

Includes JCTVC-Q0064, JCTVC-Q0065, JCTVC-Q0066, JCTVC-Q0063, along with some bug fixes.

There are several similar tools proposed in the current and previous meetings.

SC gains: -8.0/-8.4/-8.3% RGB; -5.2/-7.0/-8.0% YCbCr

They also compared to SCC CfP anchor (RExt 5.1), getting -30.8% RGB; -27.0% YCbCr for AI coding constraint, with 149% encoding time ratio. It was noted that this comparison also includes gains from RExt6.0.

There were some questions about encoding time ratios. Palette on top of RExt 6.0 gives about -9.0/-10.6/-12.5% RGB -6.6/-9.3/-11.9% YCbCr with 105% encoder time ratio according to JCTVC-Q0094, which gives an indication of the approximate encoder run-time impact from Palette alone.

Proponents ask that this continued to be studied, in particular with respect to SCC CfP.

JCTVC-Q0224 Crosscheck AhG10: Combination of Palette coding tools (JCTVC-Q0069) [W. Pu, R. Joshi (Qualcomm)] [late] [miss]

JCTVC-Q0071 AHG10: Improvements on palette coding [D. Bugdayci, J. Lainema, K. Ugur, M. M. Hannuksela (Nokia)]

This contribution proposes three tools to improve coding efficiency of palette based coding of screen content. The tools; (i) long-term palette prediction, (ii) restricted run-length coding and (iii) adaptive scanning of palette coded CUs; are implemented on top of the RExt 5.1 based JCTVC-P0303 software. The luma gain in main tier with respect to that software model is reportedly 2.2%, 1.4% and 0.1% for the lossy all intra RGB 4:4:4 SC, YUV 4:4:4 SC and Class F categories, respectively.

Long-term palette prediction: Slice-level long-term palette; max size 16 entries per color component, as triplet representation. Uses most frequently-used explicitly signaled palette colors of the previous frame as the long-term palette. Entries for long-term palette come from I-slices only.For All-Intra conditions, the first long-term palette has size zero, and subsequent long-term palette is available for the next intra frames; i.e. an Intra frame uses this previously-computed long-term palette based on earlier pictures.

Run mode restriction: Identical to one of Canon’s earlier proposals

Alternative scan for palette CUs: Flag to signal whether to scan the palette-coded CU horizontally or vertically. An RD-optimized check is done in the encoder to decide which direction to use.

SC: -2.2/-2.3/-2.3% RGB; -1.4;-1.6;-1.7% (Encode/decode times not reliable)

Individual gains also reported, e.g. -1.6% RGB/-0.9% YCbCr for long-term palette; Run mode restriction similar to Canon’s proposal (-0.4%); alternative scan (-0.3%).

Proponent suggests studing this further in CE or TE and in any related tests.

No crosscheck done (although two of these methods were proposed elsewhere, with crosschecks)

JCTVC-Q0083 AHG10: A triplet palette mode combining JCTVC-P0108 and JCTVC-P0198 [Y.-C. Sun, T.-D. Chuang, P. Lai, S.-T. Hsiang, Y.-W. Chen, X. Zhang, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

In JCTVC-P0108 and JCTVC-P0198, palette modes were proposed and combined into JCTVC-P0303 with several improvements. This contribution proposes a triplet palette mode similar to modes described in the aforementioned and other previous proposals, with some modifications. The proposed modifications are implemented on top of HM-13.0+RExt-6.0. Experimental results show that, compared with HM-13.0+RExt-6.0, 14.2%, 13.1%, and 10.2% BD-rate savings are achieved for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively. For SC RGB 444, savings are -17.4, -16.5, -14.5%.

Includes palette copy mode, palette modification mode, and palette predictor updating

It was asked how the palette modification mode differs from the AHG10 method. It was confirmed that element-wise it is the same.

Coding of color index map: Three index line modes (vertical copy/horizontal copy/normal)

Transition-copy prediction (similar to JCTVC-Q0065)

Index map rotation (horizontal/vertical scan): Similar to JCTVC-Q0071and JCTVC-Q0169

For AI tiers, gains were: -17.4/-17.2/-17.3% RGB; -14.2/-16.2/-17.0% YCbCr

Maximum palette size for this proposal is 64 (whereas P0303-based proposals used 32). Stuffing is used, and the predictor also has 64 elements.

Proponent would like this to be in a related TE or CE

Cross-checker confirms results and agrees with having a study on this.

JCTVC-Q0205 AHG10: Cross-check report for JCTVC-Q0083 X. Guo (Microsoft) [late]

JCTVC-Q0094 Suggested software for the AHG10 Palette Based Coding based RExt6.0 [W. Pu, X. Guo, P. Onno, P. Lai, J. Xu] [late]

During San Jose meeting, AHG10 was established to study palette based coding. Suggested software based on RExt5.1 was released in JCTVC-P0303. This contribution presents the results and source code of porting JCTVC-P0303 to RExt6.0.

It was noted that the “Source” for this document is the AHG10 coordinators.

For SC content, AI gains were -9.0/-10.6/-12.6% RGB; -6.6%/-9.3/-11.9% YCbCr

This was a straight porting of the RExt5.1-based JCTVC-P0303 software into RExt6.0, with no bugfixes or modifications.

JCTVC-Q0169 AHG10: Adaptive Scan Order on Palette Based Coding [J. Zhu, J. Ye, K. Kazui (Fujitsu)

An adaptive scan order on palette based coding is presented to improve coding gain. Horizontal and vertical scan orders are considered. No copy mode is used for the 1st line of the CU for horizontal scan. Copy mode is used for the 1st column of the CU for vertical scan. No line buffer is needed for copy mode. Test results show gain of 0.3% AI-lossless and AI-lossy on SCs, 1.4% AI-lossless and 1.8% AI-lossy on optional SCs (both RGB and YCbCr).

Reference for BD-Rates was JCTVC-P0303 plus a bug fixed on the calculation of the run.

Similar method used in Q0071 and Q0083, except in this case copy mode is used for 1st column of CU for vertical scan order.

No cross-check done.

JCTVC-Q0138 AHG10: Line-matching above copy method [Yao-Jen Chang, Chun-Lung Lin, Chao-Hsiung Hung (ITRI), Chun-Chi Chen, Wen-Hsiao Peng, Hsueh-Ming Hang (NCTU)]

In JCTVC-P0108, there are three modes including “VERTICAL (VER)” mode, “HORIZONTAL (HOR)” mode and “NORMAL” mode to code the color indices for palette-based coding. In this contribution, three modes are modified as “VER” mode, “HOR” mode and “MATCHING (MATCH)” mode. The MATCH mode is designed based on a line-matching above copy method to code color indices. The experimental results show that the coding gains of “Text, Graphics w/ Motion” class under AI-lossy coding are –8.7% for RGB(1080p), –4.0% for RGB(720p), –9.2% for YUV(1080p) and –3.1% for YUV(720p), respectively, while anchor is HM13.0+RExt6.0.

A 2-bit preamble is used for an initial search of the above lines, and then lines with matching preambles are checked for a full match.

Anchor does not have palette. JCTVC-P0108 was added to RExt6.0 and then the proosed matching mode was used to replace the normal mode of the JCTVC-P0108 method for coding the color indices.

The gains of the proposed matching mode as compared to RExt6.0+palette were not in the document, but were verbally reported as being -0.4% to -0.5%.

The cross-checker confirmed that the results matched, and they commented that the results were interesting.

JCTVC-Q0218 Crosscheck of JCTVC-Q0138 on line-matching above copy method [Y.-C. Sun, Y.-W. Huang (MediaTek)] [late]

JCTVC-Q0174 AHG10: Modified copy above mode for palette based coding [J. Zhu, J. Ye, K. Kazui (Fujitsu)] [late]

A simplification of the process for copy mode on palette based coding is presented. It uses the pixel value in the current position directly rather than an index, for the encoding and decoding process for copy mode. Test results show gains of 0.2% (RGB) 0.1% (YCbCr) AI-lossless on SC material.

Uses pixel value directly for “copy above” mode

Only lossless results provided for this meeting.

No cross-check for this proposal.

JCTVC-Q0225 Improvements to palette based coding [W. Pu, F. Zou, R. Joshi, M. Karczewicz, J. Sole (Qualcomm)] [late]

This document proposes combination of JCTVC-Q0047 and the palette stuffing method proposed in JCTVC-Q0063. Additionally a method for coding the palette prediction vector using binary tree is proposed. It is reported that under AHG8 test conditions, for the All-Intra configurations, BD-rate savings of more than 7.4% (MT), 8.1% (HT) and 8.3% (SHT) for SC RGB classes under lossy coding conditions, compared with the software based on AHG10 recommendations (RExt6.0 version of JCTVC-P0303).

YCbCr savings were -4.8/-58/-6.7%

Palette size is 64; predictor size is 128

Example comparison of gains: RGB SC: -5.9% for JCTVC-Q0047; -3.1% for JCTVC-Q0063; and for this proposal the corresponding improvement is -7.4%.

Gains somewhat similar to those in JCTVC-Q0069, which has some similarities to this combination.

RA and LB results will be provided in a revision.

No crosscheck available at the moment.

### Lossless coding

No documents were in this section.

### Other

JCTVC-Q0068 AhG8: On Sample Adaptive Band Value [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

This contribution proposes a new SAO type for screen content coding. This type is similar to the sample adaptive band offset. The difference is that all filtered pixels of a band are systematically equal to a decoded value instead of being equal to the addition of the decoded pixels and the decoded offset. The proposed method gives an average BDR gain of 0.9% for respectively AI, RA and LDB configurations compared to the RExt6.0, using the screen content call for proposal test conditions.

SC RGB gains -1.6% AI, -1.7% RA, -2.0% LB; YCbCr -2.6% AI%, -2.3 RA%, -1.9%

JCTVC-Q0036 uses related method (histogram correction), by signaling the pixel value to use for that band.

This is an additional SAO mode.

There was discussion on how SAO proposals can be compared, e.g. with and without a given SAO tool, without having the differences due to RDO mode decisions in the encoder. This may be resolved by modifying the encoder to use the same Intra result with both SAO modes, but it was unclear how this would be done for RA and LB conditions.

JCTVC-Q0208 Crosscheck of JCTVC-Q0068 on AHG8: On Sample Adaptive Band Value [J. Xu, A. Tabatabai (Sony)] [late]

JCTVC-Q0093 Single color intra mode for screen content coding [Y.-W. Chen, Y.-C. Sun, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, an intra coding mode termed as single color mode is proposed to code the smooth areas within a picture for screen content coding (SCC). A CU-level flag is signaled to indicate whether a CU is coded as single color mode. When a CU is coded as single color mode, it is reconstructed by filling this CU with one single value. Additionally, an index is transmitted to select one out of several available color sample candidates derived from the spatial neighboring pixels. Compared to the anchor HM 13.0_RExt 6.0, the proposed method reportedly shows 0.8% BD-rate savings for the average over different classes of SCC call for proposal (CfP) sequences under All-Intra conditions. The BD-rate savings are reported to be 1.3%, 1.7%, 1.0% and 1.6% under All-Intra conditions for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively.

CU is coded using one color for all pixels. No residue or transform is computed.

This mode was included in the JCTVC-Q0033 SCC CfP response.

It was asked what the gains of this were over palette. The presenter asserted that the gains would be less, but still over 1%.

A related technique using single colors for all pels in a CU was proposed in the JCTVC-Q0036.

Yesterday, it was commented that these methods might be suitable in a TE or for a sub-TE in the potential upcoming palette TE.

JCTVC-Q0171 Cross-check of JCTVC-Q0093 [J. Xu (Microsoft)] [late] [miss]

JCTVC-Q0112 In-loop color-space transform [L. Zhang, J. Chen, J. Sole, M. Karczewicz]

This contribution proposes an in-loop adaptive color-space transform for HEVC Range Extensions. The color-space transform converts prediction error original signals in 4:4:4 chroma format into those in a sub-optimal color space. As a result, redundancy among three color components is further reduced. For CU with intra BC and inter mode, the color-space transform is applied to prediction error. For CU with intra modes, the color-space transform is applied to both neighboring reference samples and the current block, and then the prediction residue is generated. One flag is signaled for each CU to indicate the usage of color-space transform. Experimental results show that the proposed method can achieve 22.0%, 26.9%, 23.1% BD-rate savings respectively for RGB, text & graphics with motion, 720p / RGB, mixed content, 1440p/ RGB animation, respectively in Low-delay B configurations under the test conditions of the Call for Proposals for Screen Content Coding. The proposed method can also reportedly achieve 18.1%, 20.8% and 9.1% BD-rate reduction respectively for RGB SC, RGB animation, and optional RGB SC in Low-delay B Main-Tier configurations under the AhG8 common test conditions.In-loop color transform (ICT): Lossy YCoCg with range extensions; Lossless YCog-R (Reversible)

ICT applied to Intra-BC and inter residuals, and for Intra modes, ICT applied to input signals and surrounding piels.

CU-level flag for enabling

QP is increased to QP+8 in the YCgCo space

To reduce encode time, RD selection process first checked in transformed (YCgCo) space, then a subset used in RGB; vice versa for YCbCr

Implemented on and compared to RExt-6.0

BD-Rate improvement for RGB text and graphics with motion 1080p and 720p: 13.4% and 19.3% for AI; 12.2% and 21.1% for RA; 10.8% and 22.0% for LB (lossy)

This transform sometimes yields improvement for YCbCr sequences: YCbCr: 1.0% and 0.4%; 1.0% and 1.3%, 0.7% and 1.3% (using ordering shown above)

It was noted that CCP is enabled for both the anchor and tested.

This transform was used in the same way in the corresponding CfP response JCTVC-Q0031

Question: How does this in-loop colorspace transform compare to out-of-loop conversion? Answer: 6.7% and 5.7% AI; 5.9% and 9.2% RA; 4.1% and 5.8% (1080p and 720p for RGB input) improvement from in-loop transform (from Table 5 of this proposal)

Comment made on complexity/latentency for this method on CUs, esp. in Intra case.

JCTVC-Q0124 Improvements on 1D dictionary coding [J. Ye, S. Liu, S. Lei (MediaTek), X. Chen, L. Zhao, T. Lin (Tongji)]

This document presents some improvements on a 1D dictionary coding technique. Firstly the 1D dictionary is implemented in CU level. Secondly the binarization and coding of the matching length is modified for enhancing coding efficiency. The proposed implementation example using HM-13.0+RExt-6.0 as anchor reports -13.1%, -8.0%, and -4.8% BD-rate savings for SC RGB sequences under AI, RA, and LB, respectively.

Modification of JCTVC-L0303 string-matching technique: CU instead of CTU-based; coding of the matching length modified.

CU modification yields AI RGB text & graphics with motion BD-rate change: -11.5% RGB, -7.9% YCbCr, for AI.

Combined with modified coding of matching length: -13.1% RGB, -10.1% YCbCr, for AI

The proponents mentioned that their encode/decode times may not be reliable. Generally the increase in encoder and decoder run time ratios are about 10%, perhaps less for the decoder.

Some other CfP responses also included string-matching methods.

JCTVC-Q0125 Crosscheck for JCTVC-Q0124 Improvements on 1D dictionary coding mode [R. Cohen (MERL)] [late]

JCTVC-Q0137 AHG 8: Adaptive Scaling of SAO offset [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)] [late]

The scale factor for the offset in sample adaptive offset (SAO) is signaled in the picture parameter set (PPS). In this contribution an adaptive offset scaling scheme is proposed, in which the signaling level is changed to the slice header, and the range of the scaling factor is modified. The reported coding gain is 0.3/0.6% for luma/chroma, respectively in low delay B main-tier.This proposal tests whether this adaptive offset scaling method will give gain for bit-depths of 10 and 8, and proposes:

    - changing the limit on the scale factor range to [0, BitDepth-5], which allows for this to be applied to bit-depths of 10 bits or less;

    - signaling in the slice header instead of PPS;

    - enabling/disabling flag in the SPS (no signaling in slice header if this flag is 0 or if SAO is disabled).

Gains for luma averages are about 0.0%-0.3% over all configurations.

Cross-checker confirmed results; cross-checker had similar method described in JCTVC-N0246, which was reported to have gains of about 1% for 4:4:4 LB and roughly 0.5% for AI and RA, with more changes than in this proposal.

Several participants questioned whether it’s worth making this change given the gains.

Note that this proposal was not part of a CfP response, it’s more of an RExt proposal.

JCTVC-Q0217 Crosscheck of JCTVC-Q0137 on adaptive scaling of SAO offsets [X. Zhang, Y.-W. Huang (MediaTek)] [late]

JCTVC-Q0140 AHG8: Performance of encoder and parameter only changes for Screen Content Coding [J. Sole, C. Pang, L. Zhang, K. Rapaka, M. Karczewicz (Qualcomm)] [late]

This document was moved to Section 7.1.4 Intra block copy, in the JCT-VC meeting report

JCTVC-Q0149 1D Dictionary Coding [F. Zou (Qualcomm)]

In this proposal, a pixel based 1D dictionary method is proposed to improve video coding efficiency. The matching in 1D dictionary is applied in units of pixels. The proposed 1D dictionary is implemented on LCU level, and associated syntaxes are modified to further improve the coding efficiency of the 1D dictionary. Simulation results show that the proposed method achieves 14.7%, 15.2% and 12.1% BD-rate savings for SC RGB sequences for AI RA and LB main tiers respectively.

Proposed as a lossless coding method in the JCTVC-Q0031 CfP response, as gains for lossy conditions weren’t sufficient in that framework. This proposal (JCTVC-Q0149) reports both lossy and lossless performance. Approximately 5% of the lossless gain in JCTVC-Q0031 comes from this method.

Compared to RExt6.0, adding this method yields -17.0% AI, -10.1 RA, -6.2% LB for RGB text and graphics with motion; -13.6% AI, -8.8% RA, -4.3% YCbCr.

Instead of single-pixel matching as in JCTVC-L0303, this uses full-pixel matching (all three components). A horizontal scan was also used.

No cross-check performed.

JCTVC-Q0176 AHG8: String match in coding of screen content [W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu (Huawei)] [late]

In this contribution, the string match method presented in JCTVC-Q0034 is extended to include 2-D rectangular blocks. Based on this method, a hybrid of the 1-D and 2-D string match methods is developed for coding of the color index map proposed in JCTVC-Q0034. In addition, several improvements over the screen content coding solution in JCTVC-Q0034 are described for targeting coding performance and complexity. These improvements include simplified color table processing, fast color table merge decision, fast string search, etc. These proposed methods have been implemented in the HEVC RExt Draft 5 associated reference software HM12.1+RExt-5.1 (RExt5.1). The simulation results have shown an average bit-rate reduction over the CfP anchors for lossless coding by 16.2%, 10.5%, and 9.3% for All Intra (AI), Low-Delay with B-picture (LB), and Random Access (RA), respectively. For the lossy coding mode, an average BD rate reduction of 14.4% for AI, 10.9% for RA, and 7.4% for LB have been observed. The complexity is measured using the encoding and decoding times.The above averages were averaged over both RGB and YUV (as is the case in some other proposals)

Note that this method was implemented on top of RExt 5.1. For RGB Text & graphics with motion 1080p, using RExt 5.1 as an anchor, BD-Rate changes were -37.7% RGB, -33.85 YCbCr for AI; -23.6/-20.7% RA; -16.4%/-12.5%.

Their JCTVC-Q0034 used a 1D-only string search. This proposal uses a hybrid 1D and 2D string search.

Encode/decode times may not be reliable.