| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0238 |
| --- | --- |

| Title: | Encoding time reduction for Independent Uniform Prediction in SCC CfP response JCTVC-Q0036 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway, 8th Floor Cambridge, MA 02139 | Email: | xzhang@merl.com cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the coding performance and encoding time ratios resulting from modifications of the encoder decision process associated with the Independent Uniform Prediction (IUP) mode in screen content CfP response JCTVC-Q0036. One modification reduces the number of rate-distortion optimized mode decisions, and the other modification limits the blocks that are considered for IUP. For AI conditions, the first method reduces the encoding time ratio from 301% to 258%, with a BD-Rate loss of 0.0% - 0.1%. The second method reduces the encoding time from 301% to 237%, with a BD-Rate loss of 0.0% - 0.7%. Both methods combined reduce the encoding time from 301% to 228% with a BD-Rate loss of 0.0% - 0.7%. Partial results for LB conditions are included in the accompanying spreadsheets.

# Introduction

JCTVC-Q0036 [1], a response to the Joint Call for Proposals for Coding of Screen Content [2] includes an Independent Uniform Prediction (IUP) mode, which generates a single-color prediction block using a limited set of colors for a slice. In the encoder implementation used for JCTVC-Q0036, IUP mode was implemented in a straightforward fashion as an additional mode, using the encoder’s rate-distortion optimized mode decision process. No encoder speed-up techniques were used in association with IUP. The purpose of this document is to present the results of two techniques applied to IUP that reduce the encoding time with a negligible effect on coding performance.

# Method 1: Reducing number of RDO mode decision checks

In JCTVC-Q0036, the number of colors available to IUP was set to 2. The encoder performed a complete RDO mode check using each color. Method 1 eliminates one of the RDO checks by selecting the color before performing the RDO process. Results are shown in Table 1.

Table 1. Method 1 results

|  | AI, Before mod |  |  | AI, After mod |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB, text & graphics with motion, 1080p | -27.0% | -28.0% | -27.6% | -27.0% | -28.0% | -27.6% |
| RGB, text & graphics with motion,720p | -19.1% | -19.3% | -20.0% | -19.0% | -19.2% | -20.0% |
| RGB, mixed content, 1440p | -11.0% | -10.9% | -11.0% | -10.9% | -10.8% | -10.9% |
| RGB, mixed content, 1080p | -13.4% | -13.6% | -13.4% | -13.4% | -13.6% | -13.4% |
| RGB, Animation, 720p | -8.2% | -10.4% | -8.0% | -8.2% | -10.4% | -8.0% |
| YUV, text & graphics with motion, 1080p | -25.5% | -27.7% | -27.6% | -25.5% | -27.7% | -27.6% |
| YUV, text & graphics with motion,720p | -14.6% | -19.6% | -22.4% | -14.6% | -19.5% | -22.4% |
| YUV, mixed content, 1440p | -8.3% | -12.8% | -12.5% | -8.1% | -12.7% | -12.5% |
| YUV, mixed content, 1080p | -11.5% | -13.8% | -13.7% | -11.5% | -13.7% | -13.7% |
| YUV, Animation, 720p | -1.8% | -6.4% | -5.8% | -1.8% | -6.4% | -5.8% |
| Enc Time[%] | 301% |  |  | 258% |  |  |

# Method 2: Limiting blocks for which IUP is considered

This method skips the IUP mode in the RDO mode decision process, based on the pixel values in the block. Results are shown in Table 2.

Table 2. Method 2 results

|  | AI, Before mod |  |  | AI, After mod |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB, text & graphics with motion, 1080p | -27.0% | -28.0% | -27.6% | -26.8% | -27.8% | -27.4% |
| RGB, text & graphics with motion,720p | -19.1% | -19.3% | -20.0% | -18.8% | -18.9% | -19.7% |
| RGB, mixed content, 1440p | -11.0% | -10.9% | -11.0% | -10.6% | -10.6% | -10.6% |
| RGB, mixed content, 1080p | -13.4% | -13.6% | -13.4% | -13.3% | -13.5% | -13.3% |
| RGB, Animation, 720p | -8.2% | -10.4% | -8.0% | -8.2% | -10.4% | -8.0% |
| YUV, text & graphics with motion, 1080p | -25.5% | -27.7% | -27.6% | -25.2% | -27.3% | -27.3% |
| YUV, text & graphics with motion,720p | -14.6% | -19.6% | -22.4% | -14.1% | -18.9% | -21.8% |
| YUV, mixed content, 1440p | -8.3% | -12.8% | -12.5% | -7.8% | -12.3% | -12.2% |
| YUV, mixed content, 1080p | -11.5% | -13.8% | -13.7% | -11.4% | -13.5% | -13.4% |
| YUV, Animation, 720p | -1.8% | -6.4% | -5.8% | -1.8% | -6.4% | -5.8% |
| Enc Time[%] | 301% |  |  | 237% |  |  |

# Method 1 and Method 2 combined

Results for the combination of methods 1 and 2 are shown in Table 3.

Table 3. Results for methods 1 and 2 combined

|  | AI, Before mods |  |  | AI, After mods |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB, text & graphics with motion, 1080p | -27.0% | -28.0% | -27.6% | -26.8% | -27.8% | -27.4% |
| RGB, text & graphics with motion,720p | -19.1% | -19.3% | -20.0% | -18.8% | -18.9% | -19.7% |
| RGB, mixed content, 1440p | -11.0% | -10.9% | -11.0% | -10.6% | -10.5% | -10.6% |
| RGB, mixed content, 1080p | -13.4% | -13.6% | -13.4% | -13.3% | -13.5% | -13.3% |
| RGB, Animation, 720p | -8.2% | -10.4% | -8.0% | -8.2% | -10.4% | -8.0% |
| YUV, text & graphics with motion, 1080p | -25.5% | -27.7% | -27.6% | -25.2% | -27.3% | -27.3% |
| YUV, text & graphics with motion,720p | -14.6% | -19.6% | -22.4% | -14.1% | -18.9% | -21.8% |
| YUV, mixed content, 1440p | -8.3% | -12.8% | -12.5% | -7.8% | -12.3% | -12.2% |
| YUV, mixed content, 1080p | -11.5% | -13.8% | -13.7% | -11.4% | -13.5% | -13.4% |
| YUV, Animation, 720p | -1.8% | -6.4% | -5.8% | -1.8% | -6.4% | -5.8% |
| Enc Time[%] | 301% |  |  | 228% |  |  |

# Summary and conclusions

Two methods for improving the encoding time of the JCTVC-Q0036 encoder are demonstrated in this document. For AI conditions, the first method reduces the encoding time ratio from 301% to 258%, with a BD-Rate loss of 0.0% - 0.1%. The second method reduces the encoding time from 301% to 237%, with a BD-Rate loss of 0.0% - 0.7%. Both methods combined reduce the encoding time from 301% to 228% with a BD-Rate loss of 0.0% - 0.7%. Partial results for LB conditions are available in the accompanying spreadsheet.

# Patent rights declaration

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Cohen, X. Zhang, A. Vetro, K. Sugimoto, A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami, “Description of screen content coding technology proposal by Mitsubishi Electric Corporation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0036, 17th Meeting: Valencia, ES, 27 March – 4 April, 2014.

- “Joint Call for Proposals for Coding of Screen Content,” ISO/IEC JTC1/SC29/WG11, MPEG2014/N14175, San José, US, January, 2014.