| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 Mar. – 4 Apr. 2014 | Document: JCTVC-Q0239 |
| --- | --- |

| Title: | BoG report on summary of objective performance and tools for SCC CfP responses |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rajan Joshi Robert Cohen Haoping Yu | Email: | rajanj@qti.qualcomm.com cohen@merl.com haoping.yu@huawei.com |
| Source: | BoG |  |  |

_____________________________

- Abstract

- This document provides a report of the activities of the BoG on summary of objective performance and tools for SCC CfP responses. The following information is provided:

- A summary of B-D rates for lossy conditions excluding ‘desktop’, ‘console’ and ‘webBrowsing’

- A summary of compression ratio gains for lossless conditions for all sequences

- A summary of tools used in the CfP responses

- The rate-distortion plots for AI, RA, and LB test conditions for all sequences and rate points

- Summary

A BoG was held on Friday March 28 to provide a summary of objective performance of screen content coding CfP responses. The mandates of the BoG were:

- Discuss the problems that occurred in computing BD rates, and align the different ways it was done

- Prepare a table of encoding/decoding times

- Report back with all proposals in comparison, provide RD curves

- Provide a survey of the proposals (tools used in the proposals)

During the BoG meeting there was extensive discussion regarding the problems in the calculation of BD-rates. The main discussion points were:

- Whether clipping the PSNR to 58.92 dB was appropriate or some other clip value or unclipped values should be used for calculating the BD-rates. It was concluded that using unclipped PSNR values was unworkable when there was even a single lossless point.

- Some proponents had lossless coding at two or more rate points. In one case, all the rate-points were coded losslessly. This presented a challenge for the calculation of BD-rates. Some modifications such as adding small offsets to clipped PSNR values corresponding to lossless points were suggested. But, upon further testing, the BD-rate calculations were found to be unreliable in problem cases.

- Another suggestion was to use BD-PSNR. There was some concern that this measure had been very rarely used in JCT-VC. So it would be harder to interpret, and its behavior had not been well-tested under different conditions. A concern was also expressed that due to PSNR clipping, this may favor proposals with lower rates than the anchor rates.

- Another suggestion was to calculate PSNR difference assuming that the proposal and anchor bit-rates were the same and also list the percentage difference in rates. But there was objection that averaging the difference in PSNRs across rate points was meaningless.

- Another suggestion was to take the lossless results out of the lossy group, as lossless coding, in general, has never been evaluated in the RD space.

After an interim report was presented to the JCT-VC committee along with the rate-distortion plots, the committee advised the BoG to provide BD-rates for lossy conditions excluding the sequences ‘desktop’, ‘console’ and ‘webBrowsing’, which were the sequences that were most affected by the presence of lossless frames under lossy coding conditions. It was noted, however, that these screen-content sequences were the ones that exhibited the most improvement over the anchors when coded, so any BD-rate average tables that exclude these sequences should not be used alone when evaluating the proposals or tools. Performance on the three excluded sequences should be included in those considerations.

## Summary of the objective performance

The accompanying Excel spreadsheet ‘SCC-CfP-BD-rates-summary.xlsx’ provides the following information based on the initial uploads and presentations from the proponents:

- Lossy conditions: The BD-rates (excluding the 3 sequences) and encoding/decoding times for each proposal.

- Lossless conditions: The bit-rate improvements and encoding/decoding times for each proposal for each sequence.

The BD-rates for lossy conditions and bit-rate improvements for lossless conditions are summarized in Tables Table 1 and Table 2, respectively, for the luminance component (YCbCr) or the Green component (GBR). Results for all the components can be found in the spreadsheet. Tables Table 3 and Table 4  provide average encoding/decoding times for the different proposals under lossy and lossless conditions, respectively.

Table 1. Summary of Y and G component BD-rates of the CfP proposals under lossy conditions (excluding ‘desktop’, ‘console’ and ‘webBrowsing’)

Table 2. Summary of the bit-rate improvements of the CfP proposals under lossless conditions

Table 3. Summary of encoding/decoding times of the CfP proposals under lossy conditions

Table 4. Summary of encoding/decoding times of the CfP proposals under lossless conditions

## Summary of features in different proposals

A comparison of features in different proposals is provided in the spreadsheet “SCC-CfP-tool-summary.xlsx”. The table is organized by functionality, using the following categories for each proposal:

- Block-based IntraBC

  - Partition, range, search

  - Block-based IntraBC: Modes

  - Block-based IntraBC: Block Vector coding

- Line-based Intra Copy

- Palette

  - Coding of the palette/color table

  - Coding of color indices

- String matching

- Combined IntraBC, Palette, and/or string-matching

- Cross-component

- Color-space transform

- Loop-filtering

- Rice parameter initialization

- Other tools

- Encoder-only changes

## Rate-Distortion plots

Rate-distortion plots for every sequence, rate-point and configuration (AI, RA and LB) are provided in the accompanying zip file “SCC-CfP-RD-curves.zip”. In each plot, rate-distortion curves for each CfP proposal as well as the anchor are included.