| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0245 |
| --- | --- |

| Title: | Hash-based motion search |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document presents the hash based motion search strategy used in JCTVC-Q0035. The experimental results based on the first second in each sequence used in CfP shows that average 8.3% and 6.6% bit saving is obtained for RA and LB, respectively. The encoding time is also reduced by 40% and 32% on average for RA and LB, respectively.

# Introduction

This document presents the hash-based motion search strategy used in JCTVC-Q0035. The experimental results based on the first second in each sequence used in CfP shows that average 8.3% and 6.6% bit saving is obtained for RA and LB, respectively. The encoding time is also reduced by 40% and 32% on average for RA and LB, respectively.

# Solution

Hash-based search is applied to 2Nx2N block. 18-bit hash based on original pixels are used. The first 2 bits are determined by the block size, e.g. 00 for 8x8, 01 for 16x16, 10 for 32x32, and 11 for 64x64. The following 16 bits are determined by the original pixels.

For one block, we calculate two hash values using the similar way but different CRC truncated polynomial. The first hash value is used for retrieval and the second hash value is used to exclude some of the hash conflicts. The hash value is calculated as follows:

- For each row, calculate the 16-bit CRC value for all the pixels Hash[i].

- Group the row hash values together (Hash[0]Hash[1]…) and then calculate the 24-bit CRC value H.

- The lower 16 bits of H will be used as the lower 16 bits of hash value of the current block.

To avoid one hash value corresponds to too many entries, we avoid to add the blocks satisfying following conditions into the hash table

- There is only one pixel value in every row. Or

- There is only one pixel value in every column.

Because for this kind of blocks, the block with shifting one pixel may have the same content. So, we exclude these blocks from hash table. And this kind of blocks may be predicted perfectly by horizontal prediction or vertical prediction, so excluding them from hash table may not lead to significant performance drop.

After building the hash table, the motion search is performed as follows.

For 2Nx2N block,

- Perform hash based search first.

- If hash match is found, skip the normal integer pixel motion search and fraction pixel motion search. Otherwise, perform normal motion search.

- Save the integer MV found for 2Nx2N block.

For non-2Nx2N block, compare the cost of using MVP as ME start point and using the stored MV of 2Nx2N block using the same reference picture. And then select the one with smaller cost as the motion search start point.

We also developed early termination algorithm based on hash search, if all the following conditions are satisfied, the RDO process will be terminated, without checking other modes and CU splitting.

- Hash match is found.

- The quality of the reference block is no worse than the expected quality of the current block (the QP of the reference block is no greater than the QP of the current block).

- Current CU depth is 0.

# Experimental results

The software is implemented based on HM-range-extensions r3869. The unmodified HM-range-extensions r3869 is used as anchor and the proposed method is used as test. The (roughly) first second in every CfP sequence is used to test. (The first 32 pictures are tested for WebBrowsing and Robot. The first 48 pictures are tested for SlideShow (as the frame rate of slide show is low, about 1/10 length). And the first 64 pictures are tested for the other sequences.) Fix QP encoding is applied. The QP setting is the same as the QP specified for CfP. The summary results are shown in the following tables.

Table 1 Lossy coding result

Table 2 Lossless coding result

The following figures show an example of the memory increase in the software by enabling hash-based search. The sequence MissionControlClip2 RGB with 2560x1440 was used.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).