| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0251 |
| --- | --- |

| Title: | On HEVC 4:4:4 profiles with intra block copying |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the complexity and performance of intra block copying and, consequently, proposes to include this tool in the 4:4:4 profiles for HEVC Amd1.

# Intra Block Copying Performance

The following consumer applications would make use of the HEVC Amd1 profile:

- Wireless display

- Video conferencing with screen sharing

- Compression and transmission of computer generated sequences

- Mixed content scenarios

Intra block copying (IBC) provides large coding performance gains for sequences representative for these applications. Table 1 shows the BD-rate losses for AhG8 SCC test conditions if IBC is not part of the design. Gains of HEVC 4:4:4 (with IBC) with respect to AVC 4:4:4 high are and shown in Table 2.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 16.1% | 15.2% | 15.5% | 13.5% | 13.5% | 13.5% | 11.0% | 11.1% | 11.1% |
| RGB 4:4:4 SC | 51.5% | 52.0% | 52.4% | 51.0% | 51.7% | 51.6% | 49.5% | 49.3% | 49.5% |
| YCbCr 4:4:4 SC | 48.9% | 48.5% | 48.3% | 48.6% | 48.2% | 48.0% | 46.6% | 45.7% | 45.8% |
| RGB 4:4:4 SC (Optional) | 105.5% | 104.6% | 106.0% | 109.8% | 111.3% | 113.5% | 113.8% | 112.3% | 113.3% |
| YCbCr 4:4:4 SC (Optional) | 120.0% | 118.6% | 118.4% | 125.0% | 123.3% | 123.9% | 128.5% | 126.0% | 127.2% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 11.9% | 12.0% | 12.0% | 10.1% | 10.5% | 10.3% |  |  |  |
| RGB 4:4:4 SC | 39.6% | 39.6% | 40.1% | 39.2% | 39.2% | 39.4% |  |  |  |
| YCbCr 4:4:4 SC | 37.1% | 38.0% | 38.0% | 37.3% | 37.8% | 37.9% |  |  |  |
| RGB 4:4:4 SC (Optional) | 84.8% | 84.3% | 85.2% | 89.7% | 89.8% | 91.8% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 101.4% | 101.6% | 101.9% | 107.3% | 107.0% | 107.4% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 6.6% | 6.5% | 6.6% | 6.1% | 5.8% | 6.0% |  |  |  |
| RGB 4:4:4 SC | 25.3% | 24.0% | 24.5% | 26.4% | 25.1% | 25.4% |  |  |  |
| YCbCr 4:4:4 SC | 22.5% | 22.0% | 22.0% | 24.6% | 23.6% | 23.5% |  |  |  |
| RGB 4:4:4 SC (Optional) | 55.2% | 53.5% | 53.2% | 58.9% | 56.9% | 56.9% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 74.1% | 73.3% | 73.9% | 85.4% | 83.9% | 84.5% |  |  |  |

Table 1. Losses for screen content when IBC is disabled

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F 4:2:0 SC | -40.4% | -37.9% | -38.9% | -42.3% | -35.4% | -36.0% | -42.1% | -33.4% | -34.1% |
| RGB 4:4:4 SC | -71.6% | -70.4% | -70.5% | -74.4% | -73.0% | -73.2% | -73.2% | -71.8% | -71.8% |
| YCbCr 4:4:4 SC | -56.6% | -53.7% | -55.2% | -58.8% | -54.1% | -55.6% | -58.5% | -53.9% | -55.0% |
| RGB 4:4:4 SC (Optional) | -90.7% | -90.5% | -90.7% | -90.2% | -90.2% | -90.3% | -89.2% | -88.9% | -88.8% |
| YCbCr 4:4:4 SC (Optional) | -84.6% | -79.8% | -79.7% | -84.9% | -80.7% | -80.3% | -83.4% | -81.8% | -81.6% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F 4:2:0 SC | -39.8% | -40.2% | -41.6% | -38.7% | -37.2% | -37.8% |  |  |  |
| RGB 4:4:4 SC | -65.8% | -63.2% | -63.8% | -65.2% | -63.4% | -63.9% |  |  |  |
| YCbCr 4:4:4 SC | -52.1% | -51.1% | -53.2% | -52.6% | -50.7% | -52.5% |  |  |  |
| RGB 4:4:4 SC (Optional) | -86.8% | -86.6% | -86.9% | -88.0% | -87.9% | -87.9% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -77.8% | -76.6% | -76.0% | -80.1% | -77.8% | -77.2% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F 4:2:0 SC | -38.9% | -40.5% | -42.7% | -37.6% | -35.9% | -37.5% |  |  |  |
| RGB 4:4:4 SC | -59.2% | -57.0% | -57.8% | -59.2% | -57.8% | -58.4% |  |  |  |
| YCbCr 4:4:4 SC | -49.3% | -48.2% | -50.3% | -49.6% | -47.2% | -49.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -84.0% | -83.6% | -84.2% | -86.4% | -86.2% | -86.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -75.1% | -74.4% | -73.5% | -78.7% | -76.1% | -75.4% |  |  |  |

Table 2. HEVC 4:4:4 (with IBC) versus AVC 4:4:4

# Complexity

Complexity analysis of IBC is discussed in JCTVC-P0200. Main conclusions of that document are the following:

- IBC’s storage accounts for only low percentage of total area of decoder or codec.

- IBC’s cost is much lower than deblocking and SAO. Based on real HEVC HW implementation, the IBC’s area percentages are listed below:

  - About 4% of decoder area,

  - About 1% of combined encoder+decoder total area

  - Comparison IBC with deblocking/SAO implementation:

|  | Decoder | Enc+Dec |
| --- | --- | --- |
| Intra BC | 4% | 1% |
| DB/SAO | 16.5% | 4.4% |

From HW architecture point of view, if it is implemented in LCU pipeline style, then storage can be shared between stages of IBC and deblocking/SAO, which means no extra cost involved.

# Further Considerations

- Decoding time increase of IBC reported to be negligible

- IBC can provide more than 5% BD-rate gains for natural sequences (JCTVC-P0200)

- Stable design: IBC is a widely studied tool for more than a year  and no changes to the WD text done this meeting

- Tool included in all the CfP responses for the Coding of Screen Content

# Conclusions

IBC has small impact on the decoders while providing large gains for some classes of sequences. The design is well studied and stable.

IBC should be included in the 4:4:4 HEVC Amd1 profiles.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).