| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0254 |
| --- | --- |

| Title: | Report of side activity on reference position derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, Yan Ye, Jianle Chen, Tomoyuki Yamamoto, Kenneth Andersson | Tel: Email: | Koohyar.minoo@arrisi.com Yan.ye@interdigital.com cjianle@qti.qualcomm.com yamamoto.tomoyuki@sharp.co.jp kenneth.r.andersson@ericsson.com |
| Source: | Arris, InterDigital Communications, Qualcomm Incorporated, SHARP Corporation, Ericsson |  |  |

_____________________________

# Abstract

Side activity was conducted to prepare a solution for "General-purpose explicit signaling design, to solve all of functionalities present in the existing design, and possibly additional functionalities including flexible ROI".

This contribution reports on the identified solution(s) for the following spatial scalability topics:

- On the explicit phase offset signalling for the field to frame scalability: This contribution provides an explicit phase offset signalling which provides accurate phase offset for the existing spatial only scalability and field to frame only scalability. It also allows for accurate phase offset derivation for combination of field to frame and spatial scalability.

- On fixing the inaccurate scaling factor derivation: This document asserts that either of solutions provided in Q0104 or Q0159 can solve the problem.

- On inaccurate reference position derivation in cases similar to spatial scalability from 540 to 720: This contribution asserts that the problem can be fixed by either Q0104 or Q0159 contributions or it can be solved by using the cropping values from the base layer.

- On ROI scalability: The current SHVC draft already supports ROI scalability by the use of scaled reference layer offsets.

# Introduction / Problem Statement / …

Per recommendation of BoG on SHVC upsampling process and phase offset [1], the proponents of related contributions on reference location derivation process held a meeting on 3-30-2014 to provide one solution to address the needs of respective proposals.

After consideration of possible solutions, the group provided one solution for explicit signalling of phase offsets to address the needs of field to frame scalability.

The group also qualified both solutions provided in Q0159 and Q0104 as fixes for inaccurate scaling ratio derivation. It was also noted that the specific inaccurate scaling ratio derivation for cases such as 540 to 720 scalability, can also be fixed by using the cropping offsets from the base layer

# Improved accuracy of scaling factors

The SHVC draft 5 has already ROI capability such that a region in the EL can be a zoom in of an area in the BL or the BL could correspond to a cropped version of the EL.

The main problem that we have to fix is to improve the accuracy of the scaling factors used in inter-layer prediction, for example to be able to support 540p to 720p.

By enabling a possibility to use scaled reference offsets with higher precision accurate scaling factors for inter-layer prediction can be derived.

A single syntax element is introduced to determine the needed accuracy. The single syntax element can be set such that the behaviour is identical with SHVC draft 5 (e.g. use of scaled reference offsets with an accuracy of 2 luma samples).

| scaled_ref_layer_frac_factor_minus1 | ue(v) |
| --- | --- |

scaled_ref_layer_frac_factor_minus1[ scaled_ref_layer_id[ i ] ] specifies a factor that can enable fractional accuracy in the calculation of the scale factors of the associated inter-layer picture with nuh_layer_id equal to scaled_ref_layer_id[ i ] and the current picture. When not present, the value of scaled_ref_layer_frac_factor_minus1 [ scaled_ref_layer_id[ i ] ] is inferred to be equal to 0. When scaled_ref_layer_frac_factor _minus1[ scaled_ref_layer_id[ i ] ] is larger than 0 fractional accuracy is enabled with accuracy 1/( scaled_ref_layer_frac_factor _minus1[ scaled_ref_layer_id[ i ] +1).

H.8.1.4 Resampling process for inter-layer reference pictures

…

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftOffset = (scaled_ref_layer_left_offset[ rLId ]/ FractionalScalingFactor) << 1 	(H-1)
ScaledRefLayerTopOffset = (scaled_ref_layer_top_offset[ rLId ]/ FractionalScalingFactor) <<  1 	(H-2)
ScaledRefLayerRightOffset = (scaled_ref_layer_right_offset[ rLId  ]/FractionalScalingFactor) << 1	(H-3)
ScaledRefLayerBottomOffset = (scaled_ref_layer_bottom_offset[ rLId ] /FractionalScalingFactor) << 1 	(H-4)

FractionalScalingFactor = scaled_ref_layer_frac_factor_minus1[ rLId ] +1

The variables FractionalWidthOffset and FractionalHeightOffset are derived as follows:

FractionalWidthOffset = (scaled_ref_layer_left_offset[ rLId ] << 1) + (scaled_ref_layer_right_offset[ rLId  ]  << 1)

FractionalHeightOffset =  (scaled_ref_layer_top_offset[ rLId ]  << 1) + (scaled_ref_layer_bottom_offset[ rLId ]  << 1)

ScaledRefLayerPicWidthInSamplesY = FractionalScalingFactor *PicWidthInSamplesY − 
			FractionalWidthOffset	(H-5)
ScaledRefLayerPicHeightInSamplesY = FractionalScalingFactor *PicHeightInSamplesY − 
			FractionalHeightOffset	(H-6)

ScaleFactorX = ( ( (FractionalScalingFactor *RefLayerPicWidthInSamplesY) << 16 ) + ( ScaledRefLayerPicWidthInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicWidthInSamplesY		(H-7)
ScaleFactorY = ( ( (FractionalScalingFactor *RefLayerPicHeightInSamplesY) << 16 ) + ( ScaledRefLayerPicHeightInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicHeightInSamplesY		(H-8)

- H.8.1.4.2.1 Derivation process for inter layer motion parameters

..

scaleFactorMVX = Clip3( −4096, 4095, ( ( ScaledRefLayerPicWidthInSamplesY << 8 ) + ( (FractionalScalingFactor *RefLayerPicWidthInSamplesY) >> 1 ) ) / (FractionalScalingFactor *RefLayerPicWidthInSamplesY))	(H-9)

scaleFactorMVY = Clip3( −4096, 4095, ( ( ScaledRefLayerPicHeightInSamplesY<< 8 ) + ( (FractionalScalingFactor *RefLayerPicHeightInSamplesY) >> 1 ) ) / (FractionalScalingFactor *RefLayerPicHeightInSamplesY))	(H-10)

# Explicit phase derivation

In order to provide the exact phase derivation for any combination of spatial scalability and field to frame scalability, the following parameters are proposed to be sent.

| … |  |
| --- | --- |
| scaled_ref_layer_left_phase_offset | u(1) |
| scaled_ref_layer_top_phase_offset | u(1) |
| scaled_ref_layer_horizontal_chroma_position | u(2) |
| scaled_ref_layer_vertical_chroma_position | u(2) |
| ref_layer_left_phase_offset | u(2) |
| ref_layer_top_phase_offset | u(2) |
| ref_layer_horizontal_chroma_position | u(2) |
| ref_layer_vertical_chroma_position | u(2) |
| … |  |

scaled_ref_layer_left_phase_offset: specifies the horizontal luma alignment of top left sample of scaled-reference-picture in unit of ½ luma samples. When scaled_ref_layer_left_phase_offset is not present the value of this syntax element is assumed to be 0.

scaled_ref_layer_top_phase_offset: specifies the vertical luma alignment of top left sample of scaled-reference-picture in unit of ½ luma samples. When scaled_ref_layer_top_phase_offset is not present the value of this syntax element is assumed to be 0.

scaled_ref_layer_horizontal_chroma_position: specifies the horizontal chroma offset of the current picture relative to luma sample in units of ¼ luma samples. The value of scaled_ref_layer_horizontal_chroma_position shall be in the range of 0 and 4, inclusive. When not present, the value of scaled_ref_layer_horizontal_chroma_position is inferred to be equal to 0

scaled_ref_layer_vertical_chroma_position: specifies the vertical chroma offset of the current picture relative to luma sample in units of ¼ luma samples.The value of scaled_ref_layer_horizontal_chroma_position shall be in the range of 0 and 4, inclusive. When not present, the value of scaled_ref_layer_vertical_chroma_position is inferred to be equal to 2.

ref_layer_left_phase_offset: specifies the horizontal luma alignment of top left sample of reference-picture in unit of ¼ luma samples. When ref_layer_left_phase_offset is not present the value of this syntax element is assumed to be 0.

ref_layer_top_phase_offset: specifies the vertical luma alignment of top left sample of reference-picture in unit of ¼ luma samples. When ref_layer_top_phase_offset is not present the value of this syntax element is assumed to be 0.

ref_layer_horizontal_chroma_position: specifies the horizontal chroma offset of the reference picture relative to luma sample in units of ¼ luma samples. When not present, the value of ref_layer_horizontal_chroma_position is inferred to be equal to 0

ref_layer_vertical_chroma_position: specifies the vertical chroma offset of the reference picture relative to luma sample in units of ¼ luma samples. When not present, the value of ref_layer_vertical_chroma_position is inferred to be equal to 2.

- H.8.1.4	Resampling process for inter-layer reference pictures

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftPhaseOffset  = scaled_ref_layer_left_phase_offset

ScaledRefLayerTopPhaseOffset  = scaled_ref_layer_top_phase_offset

RefLayerLeftPhaseOffset  = ref_layer_ left_phase_offset

RefLayerTopPhaseOffset = ref_layer_top_phase_offset

ScaledRefLayerHorizontalChromaPosition  = scaled_ref_layer_horizontal_chroma_position

ScaledRefLayerVerticalChromaPosition  = scaled_ref_layer_vertical_chroma_position

RefLayerHorizontalChromaPosition  = ref_layer_horizontal_chroma_position

RefLayerVerticalChromaPosition  = ref_layer_vertical_chroma_position

H.6.2	Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-11)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-12)

The variables phaseX, phaseY, deltaX, and deltaY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-13)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-14)

phaseX = ( cIdx  = =  0 ) ? (ScaledRefLayerLeftPhaseOffset <<2)  : (ScaledRefLayerLeftPhaseOffset<<1 + ScaledRefLayerHorizontalChromaPosition)	(H-15)

phaseY = ( cIdx  = =  0 ) ? (ScaledRefLayerTopPhaseOffset <<2)  : (ScaledRefLayerTopPhaseOffset <<1 + ScaledRefLayerVerticalChromaPosition)	(H-16)

deltaX = ( cIdx  = =  0 ) ? (RefLayerLeftPhaseOffset <<2)  : (RefLayerLeftPhaseOffset<<1  + RefLayerHorizontalChromaPosition  <<1)	(H-17) 
deltaY = ( cIdx  = =  0 ) ? (RefLayerTopPhaseOffset <<2)  : (RefLayerTopPhaseOffset<<1 + RefLayerVerticalChromaPosition  <<1)	(H-18)

addX = ( ScaleFactorX * phaseX + 24 ) >> 23		(H-19) 
addY = ( ScaleFactorY * phaseY + 24 ) >> 23		(H-20)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP – offsetX) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) – deltaX ( phaseX << 2 )	(H-21)
yRef16 = ( ( ( yP − offsetY) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) − deltaY ( phaseY << 2 )	(H-22)

# Example of signalled values

SRLLPO  = scaled_ref_layer_left_phase_offset

SRLTPO  = scaled_ref_layer_top_phase_offset

RLLPO  = ref_layer_ left_phase_offset

RLTPO  = ref_layer_top_phase_offset

SRLHCP  = scaled_ref_layer_horizontal_chroma_position

SRLVCP  = scaled_ref_layer_vertical_chroma_position

RLHCP  = ref_layer_horizontal_chroma_position

RLVCP  = ref_layer_vertical_chroma_position

| VertPhasePositionAdjustFlag | CrossLayerPhaseAlignmentFlag | VertPhasePositionFlag | SRLLPO | RLLPO | SRLTPO | RLTPO |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 2 | 1 | 2 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 2 |
| 1 | 1 | 0 | 1 | 2 | 1 | 1 |
| 1 | 1 | 1 | 1 | 2 | 1 | 3 |

Table 1. Signaling of alignment values when cIdx = 0.

| VertPhasePositionAdjustFlag | CrossLayerPhaseAlignmentFlag | VertPhasePositionFlag | SRLHCP | RLHCP | SRLVCP | RLVCP |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 2 | 2 |
| 0 | 1 | 0 | 0 | 0 | 2 | 2 |
| 1 | 0 | 0 | 0 | 0 | 2 | 1 |
| 1 | 0 | 1 | 0 | 0 | 2 | 3 |
| 1 | 1 | 0 | 0 | 0 | 2 | 1 |
| 1 | 1 | 1 | 0 | 0 | 2 | 3 |

Table 2. Signaling of alignment values when cIdx = 1.

# Alternative solution to include "ROI extraction" functinality

Alternative solution is desribed which additionally support "ROI extraction" functionality on top of the solution presented in the previous section.

Changes are marked with green and blue mark.

Blue part achieves "ROI extraction" functionality which helps preserving correct scale and phase in ROI application even when non-significant BL independent tiles are discarded.

Note: Current SHVC can support both BL and EL ROI if no disposal of part of BL bitstream happens.

| … |  |
| --- | --- |
| scaled_ref_layer_left_phase_offset | u(1) |
| scaled_ref_layer_top_phase_offset | u(1) |
| scaled_ref_layer_horizontal_chroma_position | u(3) |
| scaled_ref_layer_vertical_chroma_position | u(3) |
| ref_layer_left_phase_offset | u(2) |
| ref_layer_top_phase_offset | u(2) |
| ref_layer_horizontal_chroma_position | u(2) |
| ref_layer_vertical_chroma_position | u(2) |
| ref_layer_left_offset | se(v) |
| ref_layer_top_offset | se(v) |
| ref_layer_right_offset | se(v) |
| ref_layer_bottom_offset | se(v) |
| … |  |

scaled_ref_layer_left_phase_offset: specifies the horizontal luma alignment of top left sample of scaled-reference-picture in unit of ½ luma samples. When scaled_ref_layer_left_phase_offset is not present the value of this syntax element is assumed to be 0.

scaled_ref_layer_top_phase_offset: specifies the vertical luma alignment of top left sample of scaled-reference-picture in unit of ½ luma samples. When scaled_ref_layer_top_phase_offset is not present the value of this syntax element is assumed to be 0.

scaled_ref_layer_horizontal_chroma_position: specifies the horizontal chroma offset of the current picture relative to luma sample in units of ¼ luma samples. The value of scaled_ref_layer_horizontal_chroma_position shall be in the range of 0 and 4, inclusive. When not present, the value of scaled_ref_layer_horizontal_chroma_position is inferred to be equal to 0

scaled_ref_layer_vertical_chroma_position: specifies the vertical chroma offset of the current picture relative to luma sample in units of ¼ luma samples.The value of scaled_ref_layer_horizontal_chroma_position shall be in the range of 0 and 4, inclusive. When not present, the value of scaled_ref_layer_vertical_chroma_position is inferred to be equal to 2.

ref_layer_left_phase_offset: specifies the horizontal luma alignment of top left sample of reference-picture in unit of ¼ luma samples. When ref_layer_left_phase_offset is not present the value of this syntax element is assumed to be 0.

ref_layer_top_phase_offset: specifies the vertical luma alignment of top left sample of reference-picture in unit of ¼ luma samples. When ref_layer_top_phase_offset is not present the value of this syntax element is assumed to be 0.

ref_layer_horizontal_chroma_position: specifies the horizontal chroma offset of the reference picture relative to luma sample in units of ¼ luma samples. When not present, the value of ref_layer_horizontal_chroma_position is inferred to be equal to 0

ref_layer_vertical_chroma_position: specifies the vertical chroma offset of the reference picture relative to luma sample in units of ¼ luma samples. When not present, the value of ref_layer_vertical_chroma_position is inferred to be equal to 2.

ref_layer_left_offset[ ref_layer_id[ i ] ] specifies the horizontal offset between the top-left luma sample of the alternative inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ i ] and the top-left luma sample of the inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ I ] in units of two luma samples. When not present, the value of ref_layer_left_offset[ ref_layer_id[ i ] ] is inferred to be equal to 0.

ref_layer_top_offset[ ref_layer_id[ i ] ] specifies the vertical offset between the top-left luma sample of the alternative inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ i ] and the top-left luma sample of the inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ I ] in units of two luma samples. When not present, the value of ref_layer_left_offset[ ref_layer_id[ i ] ] is inferred to be equal to 0.

ref_layer_bottom_offset[ ref_layer_id[ i ] ] specifies the horizontal offset between the bottom-right luma sample of the alternative inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ i ] and the bottom-right luma sample of the inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ I ] in units of two luma samples. When not present, the value of ref_layer_left_offset[ ref_layer_id[ i ] ] is inferred to be equal to 0.

ref_layer_right_offset[ ref_layer_id[ i ] ] specifies the vertical offset between the bottom-right luma sample of the alternative inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ i ] and the bottom-right luma sample of the inter-layer reference picture with nuh_layer_id equal to ref_layer_id[ I ] in units of two luma samples. When not present, the value of ref_layer_left_offset[ ref_layer_id[ i ] ] is inferred to be equal to 0.

- H.8.1.4	Resampling process for inter-layer reference pictures

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftPhaseOffset  = scaled_ref_layer_left_phase_offset

ScaledRefLayerTopPhaseOffset  = scaled_ref_layer_top_phase_offset

RefLayerLeftPhaseOffset  = ref_layer_ left_phase_offset

RefLayerTopPhaseOffset = ref_layer_top_phase_offset

ScaledRefLayerHorizontalChromaPosition  = scaled_ref_layer_horizontal_chroma_position

ScaledRefLayerVerticalChromaPosition  = scaled_ref_layer_vertical_chroma_position

RefLayerHorizontalChromaPosition  = ref_layer_horizontal_chroma_position

RefLayerVerticalChromaPosition  = ref_layer_vertical_chroma_position

The variables RefLayerLeftOffset, RefLayerRightOffset, RefLayerTopOffset, RefLayerBottomOffset, AltRefLayerPicWidthInSamplesY and AltRefLayerPicHeightInSamplesY are derived as follows:

RefLayerLeftOffset  = ref_layer_left_offset[ rLId ] << 1;

RefLayerRightOffset = ref_layer_right_offset[ rLId ] << 1;

RefLayerTopOffset = ref_layer_top_offset[ rLId ] << 1;

RefLayerBottomOffset = ref_layer_bottom_offset[ rLId ] << 1;
AltRefLayerPicWidthInSamplesY =	 
	RefLayerPicWidthInSamplesY + RefLayerLeftOffset + RefLayerRightOffset
AltRefLayerPicHightInSamplesY =	
	RefLayerPicHightInSamplesY + RefLayerTopOffset + RefLayerBottomOffset

The variables ScaleFactorX and ScaleFactorY are derived as follows:

ScaleFactorX = ( ( AltRefLayerPicWidthInSamplesY << 16 ) 
			+ ( ScaledRefLayerPicWidthInSamplesY >> 1 ) ) / ScaledRefLayerPicWidthInSamplesY		(H-23)
ScaleFactorY = ( ( AltRefLayerPicHeightInSamplesY << 16 )
		+ ( ScaledRefLayerPicHeightInSamplesY >> 1 ) ) / ScaledRefLayerPicHeightInSamplesY		(H-24)

H.6.2	Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-25)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-26)

The variables phaseX, phaseY, deltaX, deltaY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-27)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-28)

When cIdx is equal to 0, following applies.

phaseX = ScaledRefLayerLeftPhaseOffset <<2		 (H-29)
	phaseY = ScaledRefLayerTopPhaseOffset <<2		 (H-30)
	deltaX = RefLayerLeftPhaseOffset <<2 + RefLayerLeftOffset<<4	(H-31)
	deltaY = RefLayerTopPhaseOffset <<2 + RefLayerTopOffset<<4	(H-32)

Otherwise (cIdx is not equal to 0), following applies.

phaseX = ScaledRefLayerLeftPhaseOffset<<1 + ScaledRefLayerHorizontalChromaPosition	(H-33')
	phaseY = ScaledRefLayerTopPhaseOffset <<1 + ScaledRefLayerVerticalChromaPosition	(H-34')
	deltaX =  RefLayerLeftPhaseOffset<<1 + RefLayerHorizontalChromaPosition<<1
		+ (RefLayerLeftOffset<<4 / SubWidthC)	(H-35')
	deltaY = RefLayerTopPhaseOffset<<1 + RefLayerVerticalChromaPosition<<1
		+ (RefLayerTopOffset<<4 / SubHeightC)	(H-36')

addX = ( ScaleFactorX * phaseX + 24 ) >> 23		(H-37) 
addY = ( ScaleFactorY * phaseY + 24 ) >> 23		(H-38)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP – offsetX) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) – deltaX ( phaseX << 2 )	(H-39)
yRef16 = ( ( ( yP − offsetY) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) − deltaY ( phaseY << 2 )	(H-40)

H.6.1	Derivation process for reference layer sample location

Input to this process is a luma location ( xP, yP ) relative to the top-left luma sample of the current picture.

Output of this process is a luma location ( xRef, yRef ) relative to the top-left luma sample of the reference layer picture.

The variables xRef and yRef are derived as follows:

xRef = ( ( xP − ScaledRefLayerLeftOffset ) * ScaleFactorX + ( 1 << 15 ) ) >> 16 ‒ RefLayerLeftOffset	(H-1)
yRef = ( ( yP − ScaledRefLayerTopOffset ) * ScaleFactorY + ( 1 << 15 ) ) >> 16 ‒ RefLayerTopOffset	(H-2)

H.8.1.4.2.1	Derivation process for inter layer motion parameters

...

scaleFactorMVX = Clip3( −4096, 4095, ( ( ScaledRefLayerPicWidthInSamplesY << 8 ) + ( AltRefLayerPicWidthInSamplesY >> 1 ) ) / AltRefLayerPicWidthInSamplesY)	(H-3)

...

scaleFactorMVY = Clip3( −4096, 4095, ( ( ScaledRefLayerPicHeightInSamplesY << 8 ) + ( AltRefLayerPicHeightInSamplesY >> 1 ) ) / AltRefLayerPicHeightInSamplesY)	(H-4)

# References

[1]  J. Boyce, “BoG report on SHVC upsampling process and phase offset,” JCTVC-Q0234, March 2014.

# Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).