| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0255 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0209 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On sharing of sub-DPBs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry 5775 Morehouse Drive San Diego, CA 92121-1714 USA Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Qualcomm Incorporated, Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

It is asserted that an example of a bitstream which provides DPB picture storage buffer saving when layers that share the same representation format also share the same sub-DPB even when bitstream thinning operations are applied does exist. It is proposed that a flag be signalled at the VPS-level indicating whether sub-DPB sizes are explicitly signalled, and this is asserted to enable signalling of smaller buffer sizes than in those cases that have layer-specific sub-DPB sizes. When the sub-DPB sizes are not explicitly signalled, the shared sub-DPB sizes are inferred to be equal to the sum of the layer-specific number of picture storage buffers needed for the layers that are contained in that sub-DPB.

# Introduction

During the discussion of JCTVC-Q0154/JCT3V-H0046 in the 17th JCT-VC meeting, it was suggested to study examples of bitstreams where thinning operations, including down-switch, would not have effect of DPB-overflow when the thinned bitstream is decoded. This document provides one such example, and also proposes an alternate signalling mechanism to indicate the size of sub-DPBs that would not result in any overflow.

# Example

The document (slides) attached with this contribution gives an illustration of a bitstream with two layers. The prediction structures are also described, and it is shown that for bitstream down-switch operation, there is no risk of buffer overflow.

It is to be noted that the DPB-savings shown in the attached example also include the proposed changes in the document JCTVC-Q0158/JCT3V-H0047; the proposal in that document (marking process for sub-layer non-reference picture of the highest temporal ID within each layer) was waiting for a decision regarding shared sub-DPB.

# Proposal

It is proposed to signal a flag in the dpb_size( ) table that indicates whether the sub-DPB sizes are explicitly signalled for the bitstream. When indicated that the sub-DPB sizes are not signalled, they are inferred to be equal to the sum of the number of picture buffers that are indicated on a layer-specific basis. The changes to the Specification are highlighted below.

## DPB size syntax

| dpb_size( ) { |  |
| --- | --- |
| shared_sub_dpb_info_present_flag | u(1) |
| for( i = 1; i < NumOutputLayerSets; i++ ) { |  |
| sub_layer_flag_info_present_flag[ i ] | u(1) |
| for( j = 0; j <= MaxSubLayersInLayerSetMinus1[ i ]; j++ ) { |  |
| if( j > 0 && sub_layer_flag_info_present_flag[ i ] ) |  |
| sub_layer_dpb_info_present_flag[ i ][ j ] | u(1) |
| if( sub_layer_dpb_info_present_flag[ i ][ j ] ) { |  |
| if( shared_sub_dpb_info_present_flag ) |  |
| for( k = 0; k < NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ]; k++ ) |  |
| max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] | ue(v) |
| max_vps_num_reorder_pics[ i ][ j ] | ue(v) |
| if( NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ] != NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ] ¦ ¦ !shared_sub_dpb_info_present_flag ) |  |
| for( k = 0; k < NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ]; k++ ) |  |
| max_vps_layer_dec_pic_buff_minus1[ i ][ k ][ j ] | ue(v) |
| max_vps_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

## DPB size semantics

- shared_sub_dpb_info_present_flag equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] parameters are present. shared_sub_dpb_info_present_flag equal to 0 specifies that max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] parameters are not present. When shared_sub_dpb_info_present_flag is not present, its value is inferred to be equal to 0.

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] plus 1 specifies the maximum required size of the k-th sub-DPB for the CVS in the i-th output layer set in units of picture storage buffers when HighestTid is equal to j. When j is greater than 0, max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] shall be greater than or equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j - 1 ]. When max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is not present due to shared_sub_dpb_info_present_flag is being equal to 0, the value of max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] it is derived as follows.

for( m = 0, lsIdx = LayerSetIdxForOutputLayerSet[ i ], subDpbSize = 0; m < NumLayersInIdList[ lsIdx ]; m++ )
	if( SubDpbAssigned[ lsIdx ][ m ]  = =  k )
		subDpbSize += max_vps_layer_dec_pic_buffering_minus[ i ][ m ][ j ]
max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] = subDpbSize

When max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is not present for j in the range of 1 to MaxSubLayersInLayerSetMinus1[ i ], inclusive, and shared_sub_dpb_info_present_flag is equal to 1, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j - 1].

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).