| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 Mar. – 4 Apr. 2014 | Document: JCTVC-Q0256_r1 |
| --- | --- |

| Title: | Rectangular Region Frame Packing Arrangement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Giovanni Ballocca via Castagnole 59 10060, None, Italia | Tel: Email: | +390119904770 giovanni.ballocca@sisveltech.com |
| Source: | Sisvel Technology |  |  |

_____________________________

# Abstract

This contribution presents the draft text for rectangular region frame packing arrangement messages to be included in HEVC Amendment 1.

# Introduction

Resolution 13.2.1 from the 105th MPEG meeting, Vienna, July 2013 (N13648) states

WG11 intends to use a "versioned" approach to frame packing arrangement (FPA) SEI messages.  Accordingly, in each amendment, a single additional SEI message will include any new agreed FPA formats.  WG11 has approved the tiled FPA (m29229) for inclusion in the new FPA SEI message of ISO/IEC 23008-2:201x/amd1 (HEVC Amendment 1) at the time of AMD1 reaching FDAM, and further FPAs are currently under consideration.

Aim of this contribution is to propose a draft text for inclusion in the above mentioned HEVC Amendment 1.

# Proposed text

In clause 7.4.4 replace

general_non_packed_constraint_flag equal to 1 specifies that there are no frame packing arrangement SEI messages present in the CVS. general non_packed_constraint_flag equal to 0 indicates that there may or may not be one or more frame packing arrangement SEI messages present in the CVS.

NOTE 1 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages.

with

general_non_packed_constraint_flag equal to 1 specifies that there are no frame packing arrangement SEI messages and rectangular region frame packing arrangement SEI messages present in the CVS. general non_packed_constraint_flag equal to 0 indicates that one or more frame packing arrangement SEI messages or rectangular region frame packing arrangement SEI messages may (or may not) be present in the CVS.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages and rectangular region frame packing arrangement SEI messages.

In clause D.2 add the text highlighted in yellow

D.2	SEI payload syntax

D.2.1	General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 2 ) |  |
| pan_scan_rect( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| … |  |
| else if( payloadType = = XX [Ed. Insert next available number] ) |  |
| rect_region_frame_packing_arrangement ( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| … |  |
| post_filter_hint( payloadSize ) |  |
| else if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

[…]

D.2.17	Rectangular region frame packing arrangement SEI message syntax

| rect_region_frame_packing_arrangement ( payloadSize ) { | Descriptor |
| --- | --- |
| rect_region_frame_packing_arrangement_cancel_flag | u(1) |
| if( ! rect_region_frame_packing_arrangement_cancel_flag ) { |  |
| content_interpretation_type | u(2) |
| rect_region_frame_packing_arrangement_persistence_flag | u(1) |
| } |  |
| } |  |

[…]

In clause D.3.1 add the text highlighted in yellow

[…]

Table D-1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Buffering period | - The remainder of the bitstream |
| Picture timing | - The access unit containing the SEI message |
| Pan-scan rectangle | - Specified by the syntax of the SEI message |
| Filler payload | - The access unit containing the SEI message |
| User data registered by Rec. ITU-T T.35 | - Unspecified |
| User data unregistered | - Unspecified |
| Recovery point | - Specified by the syntax of the SEI message |
| Scene information | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a scene information SEI message |
| Picture snapshot | - The access unit containing the SEI message |
| Progressive refinement segment start | - Specified by the syntax of the SEI message |
| Progressive refinement segment end | - The access unit containing the SEI message |
| Film grain characteristics | - Specified by the syntax of the SEI message |
| Post-filter hint | - The access unit containing the SEI message |
| Tone mapping information | - Specified by the syntax of the SEI message |
| Frame packing arrangement | - Specified by the syntax of the SEI message |
| Rectangular region frame packing arrangement | - Specified by the syntax of the SEI message |
| Display orientation | - Specified by the syntax of the SEI message |
| Structure of pictures information | - The set of access units in the CVS that correspond to entries listed in the SEI message |
| Decoded picture hash | - The access unit containing the SEI message |
| Active parameter sets | - The CVS containing the SEI message |
| Decoding unit information | - The decoding unit containing the SEI message |
| Temporal sub-layer zero index | - The access unit containing the SEI message |
| Scalable nesting | - Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested |
| Region refresh information | - The set of VCL NAL units within the access unit starting from the VCL NAL unit following the SEI message up to but not including the VCL NAL unit following the next SEI NAL unit containing a region refresh information SEI message (if any) |

[…]

It is a requirement of bitstream conformance that when an SEI message with payloadType equal to 45 (frame packing arrangement) is present in an access unit, an SEI message with payloadType equal to XX (rectangular region frame packing arrangement) shall not be present in the same access unit access unit.

[…]

In the expression of constraints on non-nested SEI messages, add the number corresponding to the rectangular region frame packing arrangement SEI message to the list of messages in which the message with type code 45 is listed.

In clause D.3 add the text highlighted in yellow

[…]

D.3.17	Rectangular region frame packing arrangement SEI message semantics

This SEI message informs the decoder that the output cropped decoded picture contains samples of multiple distinct spatially packed constituent frames that are packed into one frame using a rectangular region frame packing arrangement. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent frames appropriately for display or other purposes (which are outside the scope of this Specification).

Each component plane of the decoded frames contains a rectangular region frame packing arrangement of corresponding planes of two constituent frames as illustrated in Figure XFigure X.

NOTE 1 – Figure X  provides an illustration of the rearrangement process for the rectangular region frame packing arrangemen7.

This SEI message may be associated with pictures that are either frames (when field_seq_flag is equal to 0) or fields (when field_seq_flag is equal to 1). The rectangular region frame packing arrangement of the samples is specified in terms of the sampling structure of a frame in order to define a frame packing arrangement structure that is invariant with respect to whether a picture is a single field of such a packed frame or is a complete packed frame.

When general_non_packed_constraint_flag is equal to 1 for a CVS, there shall be no rectangular region frame packing arrangement SEI messages in the CVS.

rect_region_frame_packing_arrangement_cancel_flag equal to 1 indicates that the rectangular region frame packing arrangement SEI message cancels the persistence of any previous rectangular region frame packing arrangement SEI message in output order. rect_region_frame_packing_arrangement_cancel_flag equal to 0 indicates that rectangular region frame packing arrangement information follows.

content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table XTable . Values of content_interpretation_type that do not appear in Table XTable  are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore rectangular region frame packing arrangement SEI messages that contain reserved values of content_interpretation_type.

For the specified frame packing arrangement scheme, there are two constituent frames that are referred to as frame 0 and frame 1.

Table -X – Definition of content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the frame packed constituent frames |
| 1 | Indicates that the two constituent frames form the left and right views of a stereo view scene, with frame 0 being associated with the left view and frame 1 being associated with the right view |
| 2 | Indicates that the two constituent frames form the right and left views of a stereo view scene, with frame 0 being associated with the right view and frame 1 being associated with the left view |

NOTE 2 – The value 2 for content_interpretation_type is not expected to be prevalent in industry use of this SEI message. However, the value was specified herein for purposes of completeness.

rect_region_packing_arrangement_persistence_flag specifies the persistence of the rectangular region frame packing arrangement SEI message.

rect_region_packing_arrangement_persistence_flag equal to 0 specifies that the rectangular region frame packing arrangement SEI message applies to the current decoded frame only.

rect_region_packing_arrangement_persistence_flag equal to 1 specifies that the rectangular region frame packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A frame in an access unit containing a rectangular region frame packing arrangement SEI message is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

NOTE 3 – The default display window parameters in the VUI parameters of the SPS should be used by an encoder to indicate to a decoder that does not interpret the rectangular region frame packing arrangement SEI message that the default display window is an area within only one of the two constituent frames.

Let croppedWidth and croppedHeight be the width and height, respectively, of the cropped frame output from the decoder in units of luma samples, derived as follows:

croppedWidth = pic_width_in_luma_samples −
			SubWidthC * ( conf_win_right_offset + conf_win_left_offset )	(D-X)

croppedHeight = pic_height_in_luma_samples −
			SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset )	(D-X)

It is a requirement of bitstream conformance for the rectangular region frame packing arrangement that

croppedWidth and croppedHeight shall be integer multiples of 3.

Let oneThirdWidth and oneThirdHeight be derived as follows:

oneThirdWidth = croppedWidth / 3		(D-X)

oneThirdHeight = croppedHeight / 3		(D-X)

The rectangular region frame packing arrangement is composed of five rectangular regions identified as R0, R1, R2, R3, and R4 as illustrated in Figure X-.

The width and height of the region R0 are specified in units of frame luma samples as follows:

r0W = 2 * oneThirdWidth		(D-X)

r0H = 2* oneThirdHeight		(D-X)

The width and height of the region R1 are specified in units of frame luma samples as follows:

r1W = oneThirdWidth		(D-X)

r1H = 2* oneThirdHeight		(D-X)

The width and height of the region R2 are specified in units of frame luma samples as follows:

r2W = oneThirdWidth		(D-X)

r2H = oneThirdHeight		(D-X)

The width and height of the region R3 are specified in units of frame luma samples as follows:

r3W = oneThirdWidth		(D-X)

r3H = oneThirdHeight		(D-X)

The width and height of the region R4 are specified in units of frame luma samples as follows:

r4W = oneThirdWidth		(D-X)

r4H = oneThirdHeight		(D-X)

Constituent frame 0 is obtained by cropping from the decoded frames the region R0, and constituent frame 1 is obtained by stacking vertically the regions R2 and R3 and placing the resulting rectangle to the right of the region R1. The region R4 is not part of either constituent frame and is discarded.

[…]

- Figure X- – Rearrangement and upconversion of rectangular region frame packing arrangement

[…]

# Patent rights declaration(s)

Sisvel Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).