| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q1004 |
| --- | --- |

| Title: | HEVC Conformance Draft 7 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document from JCT-VC |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
|  | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
|  | Wade Wan 5300 California Ave Irvine, CA 92617 USA | Tel: Email: | wwan@broadcom.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is a proposed draft 7 of conformance test of HEVC.

- Conformance specification for HEVC

# Scope

This Recommendation | International Standard specifies a set of tests and procedures designed to indicate whether encoders or decoders meet the normative requirements specified in ITU-T H.265 | ISO/IEC 23008-2.

# Normative references

## General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None.

## Paired Recommendations | International Standards equivalent in technical content

–	ITU-T H.265 (in force), High efficiency coding.

–	ISO/IEC 23008-2: in force, Information technology – High efficiency video coding and media delivery in heterogeneous environment – Part 2: High Efficiency Video Coding.

–	ITU-T H.265.2 (in force), High efficiency coding reference software.

–	ISO/IEC 23008-5: in force, Information technology – High efficiency video coding and media delivery in heterogeneous environment – Part 2: High Efficiency Video Coding Reference Software.

## Additional references

–	None.

# Definitions

For the purposes of this Recommendation | International Standard, the terms, definitions, abbreviations and symbols specified in ITU-T H.265 | ISO/IEC 23008-2 (particularly in clauses 3) apply. The following terms are further clarified for purposes herein as follows.

3.1	bitstream: An ITU-T H.265 | ISO/IEC 23008-2 video bitstream. A bitstream may contain I, P and B slices.

3.2	decoder: An ITU-T H.265 | ISO/IEC 23008-2 video decoder, i.e., an embodiment of the decoding process specified by ITU-T H.265 | ISO/IEC 23008-2. The decoder does not include the display process, which is outside the scope of this Recommendation | International Standard.

3.3	encoder: An embodiment of a process, not specified in this Recommendation | International Standard, that produces a bitstream.

3.4	reference software decoder: The software decoder contained in ITU-T H.265.2 | ISO/IEC 23008-5.

3.5	reference software encoder: The software decoder contained in ITU-T H.265.2 | ISO/IEC 23008-5.

# Abbreviations and acronyms

For the purposes of this Recommendation | International Standard, relevant abbreviations and acronyms are specified in clause 4 of ITU-T H.265 | ISO/IEC 23008-2.

# Conventions

For the purposes of this Recommendation | International Standard, relevant conventions are specified in clause 5 in ITU-T H.265 | ISO/IEC 23008-2.

# Conformance for ITU-T H.265 | ISO/IEC 23008-2

## Introduction

The following clauses specify the normative tests for verifying conformance of video bitstreams as well as decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this Recommendation | International Standard and the reference software decoder with source code included in electronic format.

## Bitstream conformance

The bitstream conformance of ITU-T H.265 | ISO/IEC 23008-2 is specified by clause C.4 of ITU-T H.265 | ISO/IEC 23008-2.

## Decoder conformance

The decoder conformance of ITU-T H.265 | ISO/IEC 23008-2 is specified by clause C.5 of ITU-T H.265 | ISO/IEC 23008-2.

## Procedure to test bitstreams

A bitstream that claims conformance with ITU-T H.265 | ISO/IEC 23008-2 shall pass the following normative test:

The bitstream shall be decoded by processing it with the reference software decoder. When processed by the reference software decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstreams that lack conformance to ITU-T H.265 | ISO/IEC 23008-2.

Successfully passing the reference software decoder test provides only a strong presumption that the bitstream under test is conforming to the video layer, i.e., that it does indeed meet all the requirements for the video layer (except Annexes C, D and E) specified in ITU-T H.265 | ISO/IEC 23008-2 that are tested by the reference software decoder.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in ITU-T H.265 | ISO/IEC 23008-2 including the hypothetical reference decoder (HRD) conformance (based on Annexes C, D and E). These complementary tests may be performed using other video bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

ITU-T H.265 | ISO/IEC 23008-2 contains several informative recommendations that are not an integral part of that Recommendation | International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntax elements and other values derived from those syntactic elements and used by the decoding process specified in ITU-T H.265 | ISO/IEC 23008-2.

A verifier may not necessarily perform all stages of the decoding process specified in ITU-T H.265 | ISO/IEC 23008-2 in order to verify bitstream correctness. Many tests can be performed on syntax elements in a state prior to their use in some processing stages.

## Procedure to test decoder conformance

### Conformance bitstreams

A bitstream has values of general_profile_idc, general_level_idc corresponding to a set of specified constraints on a bitstream for which a decoder conforming to a specified profile and level is required in Annex A of ITU-T H.265 | ISO/IEC 23008-2 to properly perform the decoding process.

### Contents of the bitstream file

The conformance bitstreams are included in this Recommendation | International Standard as an electronic attachment. The following information is included in a single zipped file for each such bitstream.

–	bitstream;

–	decoded pictures or hashes of decoded pictures (may not be present);

–	short description of the bitstream;

–	trace file (the bitstream in ASCII format).

In cases where the decoded pictures or hashes of decoded pictures are not available, the reference software decoder shall be used to generate the necessary reference decoded pictures from the bitstream.

### Requirements on output of the decoding process and timing

Two classes of decoder conformance are specified:

–	output order conformance; and

–	output timing conformance.

The output of the decoding process is specified in clauses 8 and Annex C of ITU-T H.265 | ISO/IEC 23008-2.

For output order conformance, it is a requirement that all of the decoded pictures specified for output in Annex C of ITU-T H.265 | ISO/IEC 23008-2 shall be output by a conforming decoder in the specified order and that the values of the decoded samples in all of the pictures that are output shall be (exactly equal to) the values specified in clause 8 of ITU-T H.265 | ISO/IEC 23008-2.

For output timing conformance, it is a requirement that a conforming decoder shall also output the decoded samples at the rates and times specified in Annex C of ITU-T H.265 | ISO/IEC 23008-2.

The display process, which ordinarily follows the output of the decoding process, is outside the scope of this Recommendation | International Standard.

### Recommendations (informative)

This clause does not form an integral part of this Recommendation | International Standard.

In addition to the requirements, it is desirable that conforming decoders implement various informative recommendations specified in ITU-T H.265 | ISO/IEC 23008-2 that are not an integral part of that Recommendation | International Standard. This clause lists some of these recommendations.

It is recommended that a conforming decoder be able to resume the decoding process as soon as possible after the loss or corruption of part of a bitstream. In most cases it is possible to resume decoding at the next start code or slice header. It is recommended that a conforming decoder be able to perform concealment for the macroblocks or video packets for which all the coded data has not been received.

### Static tests for output order conformance

Static tests of a video decoder require testing of the decoded samples. This clause will explain how this test can be accomplished when the decoded samples at the output of the decoding process are available. It may not be possible to perform this type of test with a production decoder (due to the lack of an appropriate accessible interface in the design at which to perform the test). In that case this test should be performed by the manufacturer during the design and development phase. Static tests are used for testing the decoding process. The test will check that the values of the samples decoded by the decoder under test shall be identical to the values of the samples decoded by the reference decoder. When a hash of the values of the samples of the decoded pictures is attached to the bitstream file, a corresponding hash operation performed on the values of the samples of the decoded pictures produced by the decoder under test shall produce the same results.

### Dynamic tests for output timing conformance

Dynamic tests are applied to check that all the decoded samples are output and that the timing of the output of the decoder's decoded samples conforms to the specification of clauses 8; and Annex C of ITU-T H.265 | ISO/IEC 23008-2, and to verify that the HRD models (as specified by the CPB and DPB specification in Annex C of ITU-T H.265 | ISO/IEC 23008-2) are not violated when the bits are delivered at the proper rate.

The dynamic test is often easier to perform on a complete decoder system, which may include a systems decoder, a video decoder and a display process. It may be possible to record the output of the display process and to check that display order and timing of fields or frames are correct at the output of the display process. However, since the display process is not within the normative scope of ITU-T H.265 | ISO/IEC 23008-2, there may be cases where the output of the display process differs in timing or value even though the video decoder is conforming. In this case, the output of the video decoder itself (before the display process) would need to be captured in order to perform the dynamic tests on the video decoder. In particular the field or frame order and timing shall be correct.

If buffering period SEI and picture timing SEI are included in the test bitstream, HRD conformance shall be verified using the values of initial_cpb_removal_delay, initial_cpb_removal_delay_offset, cpb_removal_delay and dpb_removal_delay that are included in the bitstream.

If buffering period SEI and picture timing SEI are not included in the bitstream, the following inferences shall be made to generate the missing parameters:

–	fixed_pic_rate_flag shall be inferred to be 1.

–	low_delay_hrd_flag shall be inferred to be 0.

–	cbr_flag shall be inferred to be 0.

–	The frame rate of the bitstream shall be inferred to be the frame rate value specified in the corresponding table of clause 6.7, where the bitstream is listed. If this is missing, then a frame rate of either 25 or 30000 ÷ 1001 can be inferred.

–	time_scale shall be set to 90,000 and the value of num_units_in_tick shall be computed based on field rate (twice the frame rate).

–	The bit rate of the bitstream shall be inferred to be the maximum value for the level specified in Table A-1 in ITU-T H.265 | ISO/IEC 23008-2.

–	CPB and DPB sizes shall be inferred to be the maximum value for the level specified in Table A-1 in ITU-T H.265 | ISO/IEC 23008-2.

With the above inferences, the HRD shall be operated as follows.

–	The CPB is filled starting at time t = 0, until it is full, before removal of the first access unit. This means that the initial_cpb_removal_delay shall be inferred to be equal to the total CPB buffer size divided by the bit rate divided by 90000 (rounded downwards) and initial_cpb_removal_delay_offset shall be inferred to be equal to zero.

–	The first access unit is removed at time t = initial_cpb_removal_delay ÷ 90000 and subsequent access units are removed at intervals based on the frame distance, i.e., 2 * (90000 ÷ num_units_in_tick) or the field distance, i.e., (90000 / num_units_in_tick), depending on whether the access unit is coded as a frame picture or field picture.

–	Using these inferences, the CPB will not overflow or underflow and the DPB will not overflow.

### Decoder conformance test of a particular profile-and-level

In order for a decoder of a particular profile-and-level to claim output order conformance to ITU-T H.265 | ISO/IEC 23008-2 as specified by this Recommendation | International Standard, the decoder shall successfully pass the static test specified in clause 6.5.5 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level.

In order for a decoder of a particular profile and level to claim output timing conformance to ITU-T H.265 | ISO/IEC 23008-2 as specified by this Recommendation | International Standard, the decoder shall successfully pass both the static test specified in clause 6.5.5 and the dynamic test specified in clause 6.5.6 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level. Tables 1 specify the normative test suites for each profile, -and-level combination. The test suite for a particular profile-and level combination is the list of bitstreams that are marked with an 'X' in the column corresponding to that profile-and-level combination. In the column 'Main tier', 'X' indicate the bitstream is for Main tier. A decoder conformed to Main tier shall be capable of decoding the the specified bitstreams, among the testing profile-level combination, indicated by 'X' at 'Main tier' column in Tables 1. A decoder conformed to High tier shall be capable of decoding the the all specified bitstreams, among the testing profile-level combination, in Tables 1.

'X' indicates that the bitstream is designed to test both the dynamic and static conformance of the decoder.

The bitstream column specifies the bitstream used for each test.

A decoder that conforms to the Main and Main10 profile at a specific level shall be capable of decoding the specified bitstreams in Tables 1.

## Specification of the test bitstreams

Some characteristics of each bitstream listed in Tables 1 are specified in this clause. In Tables 1, the value "29.97" shall be interpreted as an approximation of an exact value of 30000 ÷ 1001 and the value "59.94" shall be interpreted as an approximation of an exact value of 60000 ÷ 1001.

### Test bitstreams – Block structure

#### Test bitstreams #STRUCT_A

Specification: All slices are coded as I, P or B slices. Each picture contains one slice. Various CTU and maximum CU size are used.

- Functional stage: Test the reconstruction process of slices.

Purpose: Check that decoder can properly decode I, P and B slices with various CTU and maximum CU size.

#### Test bitstreams #STRUCT_B

Specification: All slices are coded as I, P or B slices. Each picture contains one slice. Various CTU and minimum CU size are used.

- Functional stage: Test the reconstruction process of slices.

Purpose: Check that decoder can properly decode I, P and B slices with various CTU and minimum CU size.

### Test bitstreams – Intra coding

#### Test bitstreams #IPRED_A, IPRED_B, IPRED_C

Specification: All slices are coded as I slices. Each picture contains one slice. All intra prediction modes (35 modes for each of luma 32x32, luma 16x16, luma 8x8, luma 4x4, chroma 16x16, chroma 8x8 and chroma 4x4 for a total 245 modes) are used.

- Functional stage: Test the reconstruction process of I slice.

Purpose: Check that decoder can properly decode I slices with all intra prediction modes.

#### Test bitstreams #CIP_A

Specification: The bitstream contains one I slice and one B slice, using one slice per picture. Both SAO and DBF are disabled.

- Functional stage: Test the reference sample substitution process for intra sample prediction.

Purpose: Check that decoder can correctly decode slices of coded pictures containing intra PUs with unavailable samples for intra prediction.

#### Test bitstreams #CIP_B

Specification: The bitstream contains an I-picture and 4 P-pictures. Each picture contains only one slice. constrained_intra_pred_flag is equal to 1.

- Functional stage: Test the reference sample substitution process for intra sample prediction.

Purpose: Check that decoder can correctly decode slices of coded pictures containing intra PUs with unavailable samples for intra prediction.

#### Test bitstreams #CIP_C

Specification: The bitstream contains one I slice and one B slice, using more than one slice per picture. Both SAO and DBF are disabled.

- Functional stage: Test the reference sample substitution process for intra sample prediction.

Purpose: Check that decoder can correctly decode slices of coded pictures containing intra PUs with unavailable samples for intra prediction.

### Test bitstreams – Inter frame coding

#### Test bitstreams #MERGE_A

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 4.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_B

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 3.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_C

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 2.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_D

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 1.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_E

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 0.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_F

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. sps_temporal_mvp_enable_flag is equal to 0 and five_minus_max_num_merge_cand is equal to 0.

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check whether Merge operations perform correctly when temporal merging candidate is not included in merge candidate set.

#### Test bitstreams #MERGE_G

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 0

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check if decoder properly handles merge index ranging from 0 to 4.

#### Test bitstreams #PMERGE_A

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 0.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_B

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 1.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_C

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 2.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_D

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 3.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_E

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 4.

Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #AMVP_A

Specification: All slices are coded as I or P slices. Each picture contains only one slice.

num_ref_idx_l0_default_active_minus1 is equal to 0, num_ref_idx_l1_default_active_minus1 is equal to 0 and num_ref_idx_active_override_flag is equal to 0.

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check whether motion vector prediction candidate generation and signalling perform correctly when MV scaling is not needed for spatial MVP candidate generation (All inter-coded PUs within the same slice have the same inter_pred_idc and ref_idx_l0).

#### Test bitstreams #AMVP_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice. Multiple reference pictures are used. For GPB slices, num_ref_idx_l0_default_active_minus1 is equal to 3 and num_ref_idx_active_override_flag is equal to 0. For non-GPB B slices, num_ref_idx_l0_default_active_minus1 is equal to 1, num_ref_idx_l1_default_active_minus1 is equal to 1 and num_ref_idx_active_override_flag is equal to 0

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check whether motion vector prediction candidate generation and signalling perform correctly when MV scaling is not needed for spatial MVP candidate generation.

#### Test bitstreams #AMVP_C

Specification: All slices are coded as I or P slices. Each picture contains only one slice. The purpose of the stream is to exercise AMVP for low delay condition.

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check whether motion vector prediction candidate generation and signalling perform correctly when MV scaling is not needed for spatial MVP candidate generation.

#### Test bitstreams #TMVP_A

Specification: Each picture contains only one slice. slice_temporal_mvp_enable_flag is set to 0 for picture 0-8 and 1 for picture 9-16.

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check if slice_temporal_mvp_enable_flag is parsed and temporal motion vector predictor is generated correctly.

#### Test bitstreams #MVDL1ZERO_A

Specification: The bitstream contains multiple B slices per picture. Mvd_l0_zero_flag is set equal to 1.

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: The purpose of the stream is to exercise the parsing of list 1 motion vecotor difference for bi-prediction according to mvd_l1_zero_flag in slice header (randamized on/off switching of mvd_l1_zero_flag for multiple B slices).

#### Test bitstreams #MVCLIP_A

Specification: Each picture contains only one slice. MVP and Merge candidate motion vectors are clipped to 16-bit values. Clipped MVP and merge candidates are selected as MVP and Merge motion vectors.

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check if decoder properly handles clipping of MVP and Merge candidate motion vectors to 16-bit values

#### Test bitstreams #MVEDGE_A

Specification: Each picture contains only one slice. The bitstream includes motion vectors pointing to the padded edge regions in a picture.

- Functional stage: Test the reconstruction process of motion vector prediction.

Purpose: Check if decoder properly handles motion vectors pointing to the padded edge regions in a picture.

#### Test bitstreams #WP_A

Specification: All slices are coded as I or P slices. Each picture contains only one slice. weighted_pred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage: Weighted sample prediction process for P slices with plural reference indices.

Purpose: Check that the decoder handles weighted sample prediction for P slices with plural reference indices.

#### Test bitstreams #WP_B

Specification: All slices are coded as I, P or B slices. Each picture contains only one slice. weighted_pred_flag is equal to 1 and weighted_bipred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage: Weighted sample prediction process for P and B slices with plural reference indices.

Purpose: Check that the decoder handles weighted sample prediction for P and B slices with plural reference indices.

### Test bitstreams – Transform and quantization

#### Test bitstreams #RQT_A

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 0.

- Functional stage: Test the reconstruction process of slices with RQT.

Purpose: Check if decoder properly decodes slices with RQT with intra and inter depth 0.

#### Test bitstreams #RQT_B

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 1.

- Functional stage: Test the reconstruction process of slices with RQT.

Purpose: Check if decoder properly decodes slices with RQT with intra and inter depth 1.

#### Test bitstreams #RQT_C

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 2.

- Functional stage: Test the reconstruction process of slices with RQT.

Purpose: Check if decoder properly decodes slices with RQT with intra and inter depth 2.

#### Test bitstreams #RQT_D

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 3.

- Functional stage: Test the reconstruction process of slices with RQT.

Purpose: Check if decoder properly decodes slices with RQT with intra and inter depth 3.

#### Test bitstreams #RQT_E

Specification: All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 4.

- Functional stage: Test the reconstruction process of slices with RQT.

Purpose: Check if decoder properly decodes slices with RQT with intra and inter depth 4.

#### Test bitstreams #RQT_F

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter is set equal to 2 and max_transform_hierarchy_depth_intra is set equal to 0.

- Functional stage: Test the reconstruction process of slices with RQT.

Purpose:  Check if decoder properly decodes slices with RQT with different intra and inter depths.

#### Test bitstreams #RQT_G

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter is set equal to 0 and max_transform_hierarchy_depth_intra is set equal to 2.

- Functional stage: Test the reconstruction process of slices with RQT.

Purpose:  Check if decoder properly decodes slices with RQT with different intra and inter depths.

#### Test bitstreams #TUSIZE_A

Specification: All slices are coded as I, or P slices. Each picture contains only one slice. log2_min_transform_block_size_minus2 is set equal to 2. For current bit stream Maximum CU size is 64x64, miminun CU size is 32x32, miminun transform size for Luma is 16x16 (for Chroma 8x8).

Functional stage: Test the reconstruction process of slices with limited minimum transfrom size.

Purpose: Check if decoder properly decodes slices with RQT with minimum transfrom size different from 4x4 (default).

#### Test bitstreams #DELTAQP_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. Max CU size is equal to 64 and Min CU size is equal to 8. Diff_cu_qp_delta_depth is set randomly from 0 to 3. cu_qp_delta is set randomly from -26 to 25.

- Functional stage: Test the reconstruction process of slices with delta QP.

Purpose: Check if decoder properly decodes slices with delta QP.

#### Test bitstreams #DELTAQP_B

Specification: All slices are coded as I, P or B slices. Each picture contains more than one slice. Max CU size is equal to 64 and Min CU size is equal to 8. cu_qp_delta is set randomly from -26 to 25. slice_cb_qp_offset and slice_cr_qp_offset are set randomly from -4 to 4.

- Functional stage: Test the reconstruction process of slices with delta QP.

Purpose: Check if decoder properly handles various combination of chroma QP offset.

#### Test bitstreams #DELTAQP_C

Specification: All slices are coded as I or B slices. Each picture contains only one slice. Max CU size is equal to 64 and Min CU size is equal to 8. Diff_cu_qp_delta_depth is set randomly from 0 to 3. cu_qp_delta is set randomly from -26 to 25. In some TU, Cbf is set equal to 0.

- Functional stage: Test the reconstruction process of slices with delta QP.

Purpose: Check if decoder properly decodes slices with delta QP.

#### Test bitstreams #INITQP_A

Specification: All slices are coded as I or B slices. The value of init_qp_minus26 is set from -26 to 25.

- Functional stage: Test the initial QP.

Purpose: Check if decoder properly handles init_qp_minus26.

      - Test bitstreams #SLIST_A

Specification: All slices are coded as I or B slices. Each picture contains one slice. One sequence parameter set and more than one picture parameter sets are included. The sequence parameter set includes scaling list data. One of the picture parameter set doesn’t include scaling list data. In other picture parameter sets, different scaling lists data is included. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 1.

- Functional stage: Test the reconstruction process of scaling list. Tests switching of scaling list in sequence parameter set and picture parameter set.

Purpose: Check that decoder can properly decode slices of coded frames with scaling list. Check that decoder can properly reconstruct scaling list with all coding modes of scaling list. Check that scaling list in sequence parameter set is applied when no scaling list is included in picture parameter set. Check that appropriate scaling list in picture parameter set is applied when scaling list data is included in picture parameter set.

#### Test bitstreams #SLIST_B

Specification: All slices are coded as I or B slices. Each picture contains one slice. More than one sequence parameter set and more than one picture parameter sets are included. One of the sequence parameter set doesn’t includes scaling list data. One of the picture parameter set doesn’t include scaling list data. In other sequence parameter sets and picture parameter sets, different scaling lists data is included. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 0 or 1.

- Functional stage: Test the reconstruction process of scaling list. Tests switching of scaling list off, default scaling list and scaling list in parameter sets.

Purpose: Check that decoder can properly decode slices of coded frames with scaling list. Check that decoder can properly reconstruct scaling list with all coding modes of scaling list. Check that appropriate scaling list in sequence parameter set and picture parameter set is applied with multiple sequence parameter sets and picture parameter sets.

#### Test bitstreams #SLIST_C

Specification: All slices are coded as I or B slices. Each picture contains one slice. One sequence parameter set and more than one picture parameter sets are included. The sequence parameter set doesn’t include scaling list data. One of the picture parameter set doesn’t include scaling list data. In other picture parameter sets, different scaling lists data is included. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 1.

- Functional stage: Test the reconstruction process of scaling list. Tests switching of default scaling list and scaling list in picture parameter set.

Purpose: Check that decoder can properly decode slices of coded frames with scaling list. Check that decoder can properly reconstruct scaling list with all coding modes of scaling list. Check that default scaling list is applied when no scaling list data is present. Check that decoder properly handle switching of default scaling list and scaling list data in picture parameter set.

#### Test bitstreams #SLIST_D

Specification: All slices are coded as I or B slices. Each picture contains more than one slice. More than one sequence parameter set and more than one picture parameter sets are included. One of the sequence parameter set doesn’t includes scaling list data. One of the picture parameter set doesn’t include scaling list data. In other sequence parameter sets and picture parameter sets, different scaling lists data is included. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 0 or 1.

- Functional stage: Test the reconstruction process of scaling list. Tests switching of scaling list off, default scaling list and scaling list in parameter sets.

Purpose: Check that decoder can properly decode slices of coded frames with scaling list. Check that decoder can properly reconstruct scaling list with all coding modes of scaling list. Check that appropriate scaling list in sequence parameter set and picture parameter set is applied with multiple sequence parameter sets and picture parameter sets.

### Test bitstreams – Deblocking filter

#### Test bitstreams #DBLK_A

Specification: All slices are coded as I, P or B slices. Each picture contains more than one slice. More than one picture parameter sets are included. QP is set randomly from 22 to 51. pps_beta_offset_div2 is randomly set in each picture from -6 to 6. slice_beta_offset_div2 and slice_tc_offset_div2 are randomly set in each slice from -6 to 6.

- Functional stage: Test the deblocking filter process.

Purpose: Check that decoder can properly decode slices with various combinations of deblocking filter control parameters.

#### Test bitstreams #DBLK_B

Specification: All slices are coded as I, P or B slices. Each picture contains more than one slice. More than one picture parameter sets are included. pps_cb_qp_offset and pps_cr_qp_offset are randomly set from -12 to 12. slice_cb_qp_offset and slice_cr_qp_offset are randomly set from -4 to 4.

- Functional stage: Test the deblocking filter process.

Purpose: Check that decoder can properly handle deblocking filter with various combinations of QP.

#### Test bitstreams #DBLK_C

Specification: All slices are coded as I, P or B slices. Each picture contains more than one slice. pps_disable_deblocking_filter_flag is set equal to 0. slice_disable_deblocking_filter_flag is randomly set equal to 0 or 1.

- Functional stage: Test the deblocking filter process.

Purpose: Check that decoder can properly handle deblocking filter with random deblocking filter on/off in each slice.

#### Test bitstreams #DBLK_D

Specification: All slices are coded as I, or B slices. Each picture contains more than one slice and tile. loop_filter_across_slices_enabled_flag is set equal to 0 and loop_filter_across_tiles_enabled_flag is set equal to 1.

- Functional stage: Test the deblocking filter process.

Purpose: Check that decoder can properly handle deblocking filter at slice and tile boundary.

#### Test bitstreams #DBLK_E

Specification: All slices are coded as I, or B slices. Each picture contains more than one slice and tile. loop_filter_across_slices_enabled_flag is set equal to 1 and loop_filter_across_tiles_enabled_flag is set equal to 0.

- Functional stage: Test the deblocking filter process.

Purpose: Check that decoder can properly handle deblocking filter at slice and tile boundary.

#### Test bitstreams #DBLK_F

Specification: All slices are coded as I, or B slices. Each picture contains more than one slice and tile. loop_filter_across_slices_enabled_flag is set equal to 0 and loop_filter_across_tiles_enabled_flag is set equal to 1.

- Functional stage: Test the deblocking filter process.

Purpose: Check that decoder can properly handle deblocking filter at slice and tile boundary.

#### Test bitstreams #DBLK_G

Specification: All slices are coded as I, or B slices. Each picture contains more than one slice and tile. loop_filter_across_slices_enabled_flag is set equal to 1 and loop_filter_across_tiles_enabled_flag is set equal to 0.

- Functional stage: Test the deblocking filter process.

Purpose: Check that decoder can properly handles deblocking filter at slice and tile boundary.

### Test bitstreams – Sample adaptive offset

#### Test bitstreams #SAO_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. sao_merge_left_flag and sao_merge_up_flag are randomly set equal to 0 or 1.

Functional stage: Test the reconstruction process of sample adaptive offset.

Purpose: Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with random SAO merge left/up flag values.

#### Test bitstreams #SAO_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice and contains more than one tile. slice_sao_luma_flag and slice_sao_chroma_flag are randomly set equal to 0 or 1.

Functional stage: Test the reconstruction process of sample adaptive offset.

Purpose: Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with tiles, and randomely enabled SAO for luma and/or SAO for chroma per slice.

#### Test bitstreams #SAO_C

Specification: All slices are coded as I or P slices. Each picture contains only one slice. All SAO offset values in this bitstream have maximum allowed magnitude 7 and random sign.

Functional stage: Test the reconstruction process of sample adaptive offset.

Purpose: Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with extreme SAO offset values.

#### Test bitstreams #SAO_D

Specification: All slices are coded as I or P slices. Each picture contains only one slice. SAO offset values in this bitstream have random values in range -7..7.

Functional stage: Test the reconstruction process of sample adaptive offset.

Purpose: Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with random SAO offset values.

#### Test bitstreams #SAO_E

Specification: All slices are coded as I or B slices. Each picture contains only one slice. A set of SAO parameters is associated to each CTB for all frames. This means that no SAO merge flag (up or left) is used. Only the Band Offset (BO) SAO type is used and the 4 BO offsets are forced to be -7 or 7 in a random way. The Coded Tree Block size is set to 16x16.

Functional stage: Tests loading of maximum SAO information at CTB level and frame.

Purpose: Check that decoder can properly decode slices of coded frames with full SAO information.

#### Test bitstreams #SAO_F

Specification: All slices are coded as I or B slices. Each picture contains only one slice. A set of SAO parameters is associated to each CTB for all frames. This means that no SAO merge flag (up or left) is used. Only the Band Offset (BO) SAO type is used and the 4 BO offsets are forced to be -7 or 7 in a random way. The Coded Tree Block size is set to 32x32.

Functional stage: Tests loading of maximum SAO information at CTB level and frame.

Purpose: Check that decoder can properly decode slices of coded frames with full SAO information.

#### Test bitstreams #SAO_G

Specification: All slices are coded as I or B slices. Each picture contains only one slice. A set of SAO parameters is associated to each CTB for all frames. This means that no SAO merge flag (up or left) is used. Only the Band Offset (BO) SAO type is used and the 4 BO offsets are forced to be -7 or 7 in a random way. The Coded Tree Block size is set to 64x64.

Functional stage: Tests loading of maximum SAO information at CTB level and frame.

Purpose: Check that decoder can properly decode slices of coded frames with full SAO information.

### Test bitstreams – Entropy coding

#### Test bitstreams #MAXBINS_A

Specification: All slices are coded as I slices. Each picture contains only one slice. The number of bins per LCU is reached within 95% of the maximum number which is 4096 bits per 16x16 LCU. pcm_enabled_flag is set equal to 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check that decoder can properly decode slices with the maximum number of bins per LCU.

#### Test bitstreams #MAXBINS_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The number of bins per LCU is reached within 95% of the maximum number which is 4096 bits per 16x16 LCU. pcm_enabled_flag is set equal to 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check that decoder can properly decode slices with the maximum number of bins per LCU.

#### Test bitstreams #MAXBINS_C

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The number of bins per LCU is reached within 95% of the maximum number which is 4096 bits per 16x16 LCU. pcm_enabled_flag is set equal to 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check that decoder can properly decode slices with the maximum number of bins per LCU.

#### Test bitstreams #CAINIT_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is equal to 0 in picture parameter set.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization process when cabac_init_flag is not signaled in slice header of B or P slices.

#### Test bitstreams #CAINIT_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is equal to 1 in picture parameter set. cabac_init_flag is signaled for B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization process when cabac_init_flag is switched in B slice.

#### Test bitstreams #CAINIT_C

Specification: All slices are coded as I or P slices. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is equal to 1 in picture parameter set. cabac_init_flag is signaled for P slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization ptocess when cabac_init_flag is switched in P slces.

#### Test bitstreams #CAINIT_D

Specification: All slices are coded as I or B slices which are uni-directionally predicted. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is equal to 1 in picture parameter set. cabac_init_flag is signaled for B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization process when cabac_init_flag is switched in P slices.

#### Test bitstreams #CAINIT_E

Specification: All slices are coded as I or P slices. Each picture contains only one slice. Each slice contains 4 tiles (2 columns of tiles and 2 rows of tiles with uniform spacing). There is one picture parameter set. cabac_init_present_flag is equal to 1 in picture parameter set. cabac_init_flag is signaled for P slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization process when cabac_init_flag is switched in P slices in presence of tiles.

#### Test bitstreams #CAINIT_F

Specification: All slices are coded as I or uni-directionally predicted B slices. Each picture contains only one slice. Each slice contains 4 tiles (2 columns of tiles and 2 rows of tiles with uniform spacing). There is one picture parameter set. cabac_init_present_flag is equal to 1 in picture parameter set. cabac_init_flag is signaled for uni-directionally predicted B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization process when cabac_init_flag is switched in B slices in presence of tiles.

#### Test bitstreams #CAINIT_G

Specification: All slices are coded as I or P slices. Each picture contains only one slice. Each slice contains multple dependent slices. Each dependent slice contains 3 CTBs or less. There is one picture parameter set. cabac_init_present_flag is equal to 1 in picture parameter set. cabac_init_flag is signaled for P slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization process when cabac_init_flag is switched in P slices in presence of dependent slices.

#### Test bitstreams #CAINIT_H

Specification: All slices are coded as I or uni-directionally predicted B slices. Each picture contains only one slice. Each slice contains multple dependent slices. Each dependent slice contains 3 CTBs or less. There is one picture parameter set. cabac_init_present_flag is equal to 1 in picture parameter set. cabac_init_flag is signaled for uni-directionally predicted B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Check if decoder properly handles CABAC initialization process when cabac_init_flag is switched in B slices in presence of dependent slices.

#### Test bitstreams #SDH_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. sign_data_hiding_enabled_flag is set equal to 1. The bitstream includes various configurations of sign data hiding.

Functional stage: Test the reconstruction process of entropy coding.

Purpose: Test SDH (Sign Data Hiding) sign inference, and test SDH in conjunction with cu_ transquant_bypass_flag.

### Test bitstreams – Temporal scalability

#### Test bitstreams #TSCL_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The bitstream includes 4 temporal layers.

Functional stage: Test temporal scalability.

Purpose: Check if decoder properly handles temporal layers.

#### Test bitstreams #TSCL_B

Specification: All slices are coded as I or P slices. Each picture contains only one slice. The bitstream includes 4 temporal layers.

Functional stage: Test temporal scalability.

Purpose: Check if decoder properly handles temporal layers.

### Test bitstreams – Parallel processing tools

#### Test bitstreams #TILES_A

Specification: All slices are coded as I or P slices. Each picture contains only one slice. num_tiles_columns_minus1 and num_tiles_rows_minus1 are set equal to 4, which is the maximum value for level 4.1. uniform_spacing_flag is set equal to 0. The values of column_width_minus1[i] and row_height_minus1[i] are set randomly for each picture. loop_filter_across_tiles_enabled_flag is set randomly for each picture.

Functional stage: Test dependency breaks at tile boundaries.

Purpose: Test random non-uniform tile spacing with maximum number of tiles and disabling/enabling of deblocking filter across tile boundaries.

#### Test bitstreams #TILES_B

Specification: All slices are coded as I or P slices. Each picture contains random number of slices. All slice boundaries aligned with tile boundaries. num_tiles_columns_minus1 and num_tiles_rows_minus1 are set equal to 4, which is the maximum value for level 4.1. uniform_spacing_flag is set equal to 0. The values of column_width_minus1[i] and row_height_minus1[i] are set randomly for each picture. loop_filter_across_tiles_enabled_flag is set randomly for each picture. pps_loop_filter_across_slices_enabled_flag is set randomly for each frame. slice_loop_filter_across_slices_enabled_flag is set randomly for each slice.

Functional stage: Test dependency breaks at tile boundaries and enabling/disabling the deblocking filter at tile/slice boundaries.

Purpose: Test random non-uniform tile spacing with maximum number of tiles and randomly disabling/enabling the deblocking filter across tile/slice boundaries.

#### Test bitstreams #WPP_A

Specification: entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,  QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_B

Specification: entropy_coding_sync_enabled_flag is set to 1. A CTU size of 32x32 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,  QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_C

Specification: entropy_coding_sync_enabled_flag is set to 1. A CTU size of 16x16 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,  QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_D

Specification: entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 1 CTU wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised when a picture is 1 CTU wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 1 CTU wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_E

Specification: entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 2 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised when a picture is 2 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 2 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_F

Specification: entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 3 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised when a picture is 3 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 3 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #ENTP_A

Specification: All slices are coded as I slices. Each picture contains only one slice. Four tiles are included in a picture. num_tiles_columns_minus1 and num_tiles_rows_minus1 are set equal to 1. uniform_spacing_idc is set equal to 1. There are some tiles in POC 4 that contains emulation prevention bytes.

- Functional stage: Test entry point signaling.

Purpose: Check that decoder can correctly handles entry point signalling in slice header when tile is used and when emulation prevention bytes occur in substream(s)

      - Test bitstreams #ENTP_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice. Six tiles are included in a picture. num_tiles_columns_minus1 is set equal to 1 and num_tiles_rows_minus1 are set equal to 2. uniform_spacing_idc is set equal to 1. There are some pictures (e.g., POC 4, 6, 10, 12, 18, and 20) contains a tile in which emulation prevention bytes occur.

- Functional stage: Test entry point signaling.

Purpose: Check that decoder can correctly handles entry point signalling in slice header when tile is used and when emulation prevention bytes occur in substream(s)

      - Test bitstreams #ENTP_C

Specification: All slices are coded as I or B slices. Each picture contains only one slice. WaveFrontSynchro is set equal to 1.

- Functional stage: Test entry point signaling.

Purpose: Test entry point signalling in slice header when wavefront is used.

    - Test bitstreams – Other coding tools

#### Test bitstreams #IPCM_A

Specification: All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 7. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 2 and 0, respectively.

- Functional stage: Test parsing of pcm_flags in Coding unit syntax.

Purpose: Check that decoder can correctly decode slices of coded pictures containing pcm_flags.

#### Test bitstreams #IPCM_B

Specification: All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 5. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 1 and 0, respectively.

- Functional stage: Test parsing of pcm_flags in Coding unit syntax. Test parsing of pcm_sample_luma and pcm_sample_chroma data in PCM sample syntax.

Purpose: Check that decoder can correctly decode slices of coded pictures containing pcm_flags, and pcm_sample_luma and pcm_sample_chroma data.

#### Test bitstreams #IPCM_C

Specification: All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 7. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 1 and 1, respectively.

- Functional stage: Test parsing of pcm_flags in Coding unit syntax. Test parsing of pcm_sample_luma and pcm_sample_chroma data in PCM sample syntax. Test skipping of loop filtering on samples associated with pcm_flag equal to 1.

Purpose: Check that decoder can correctly decode slices of coded pictures containing pcm_flags, and pcm_sample_luma and pcm_sample_chroma data. Check that decoder can skip loop filtering on samples associated with pcm_flag equal to 1.

#### Test bitstreams #IPCM_D

Specification: All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 7. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 1 and 0, respectively. transquant_bypass_enable_flag is equal to 1.

- Functional stage: Test parsing of pcm_flags in Coding unit syntax. Test parsing of pcm_sample_luma and pcm_sample_chroma data in PCM sample syntax. Test skipping of loop filtering on samples associated with both cu_transquant_bypass_flag and pcm_flag equal to 1.

Purpose: Check that decoder can correctly decode slices of coded pictures containing pcm_flags, and pcm_sample_luma and pcm_sample_chroma data. Check that decoder can skip loop filtering on samples associated with both cu_transquant_bypass_flag and pcm_flag equal to 1.

#### Test bitstreams #IPCM_E

Specification: Contain single coded picture. The coded picture contains only one intra slice. pcm_enabled_flag is equal to 1. pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 5 and 7, respectively. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 1, 0 and 0, respectively.

- Functional stage: Test parsing of pcm_flags in coding unit syntax. Test parsing of pcm_sample_luma and pcm_sample_chroma data in PCM sample syntax with different bit depth precisions.

Purpose: Check that decoder can correctly decode the slice of the coded frame containing pcm_flags, and pcm_sample_luma and pcm_sample_chroma data with different pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 values.

#### Test bitstreams #TS_A

Specification: Each picture contains only one slice. transform_skip_enabled_flag is set to 1 for picture 0-8 and 0 for picture 9-16.

- Functional stage: Test reconstruction process of slices with transform_skip_enabled_flag is equal to 1.

Purpose: Check if transform_skip_enabled_flag is parsed and transform skipping functions correctly.

#### Test bitstreams #AMP_A, #AMP_D and #AMP_E

Specification: All slices are coded as I, P or B slices. Each picture contains only one slice. All asymmetric motion partition modes (2NxnU, 2NxND, nLx2N, nRx2N) are included.

- Functional stage: Test reconstruction process of slices with amp_enabled_flag equal to 1.

Purpose: Check if decoder properly decodes slices with all asymmetric motion partition modes.

#### Test bitstreams #AMP_B

Specification: All slices are coded as I, P or B slices. Each picture contains only one slice. Asymmetric motion partition is only utilized for PUs of which size is larger than minimum CU.

- Functional stage: Test reconstruction process of slices with amp_enabled_flag equal to 1.

Purpose: Check if decoder properly decodes slices with a constraint of asymmetric motion partition modes.

#### Test bitstreams #LS_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. All the CUs are lossless. SAO and deblocking filter are enabled in the SPS and PPS.

- Functional stage: Test reconstruction process of lossless coding.

Purpose: Check if decoder properly handles lossless coding.

#### Test bitstreams #LS_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice. Bitstream where at least 50% of the CUs do not use transform/quantization/filtering bypass, and where there at are least 100 CUs in each of the following categories

- The CU is 64x64, it has cu_transquant_bypass_flag on, at least one of the neighboring CU uses SAO, at least one of the neighboring CU uses deblocking filter

- The CU is 32x32, it has cu_transquant_bypass_flag on, at least one of the neighboring CU uses SAO, at least one of the neighboring CU uses deblocking filter

- The CU is 16x16, it has cu_transquant_bypass_flag on, at least one of the neighboring CU uses SAO, at least one of the neighboring CU uses deblocking filter

- The CU is 8x8, it has cu_transquant_bypass_flag on, at least one of the neighboring CU uses SAO, at least one of the neighboring CU uses deblocking filter

- Functional stage: Test reconstruction process of lossless coding.

Purpose: Check if decoder properly handles lossless coding.

### Test bitstreams – High level syntax

#### Test bitstreams #NUT_A

Specification: All slices are coded as I or P slices. Each picture contains only one slice. Three temporal layers are used. The bitstream exercises various VLC NAL unit types.

- Functional stage: Test decoding of various video coding layer (VCL) NAL unit types.

Purpose: Check decoding of VCL NAL unit types TRAIL_N, TRAIL_R, TSA_N, TSA_R, STSA_N, STSA_R, RADL_R, RASL_R, RADL_N, RASL_N, BLA_W_LP, BLA_W_DLP, BLA_N_LP, IDR_W_DLP, IDR_N_LP, and CRA.

#### Test bitstreams #FILLER_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. This bitstream contains NAL unit with nal_unit_type equal to 38 (filler data) at the end of every AU

- Functional stage: Test decoding of filler data NAL unit.

Purpose: Check if decoder properly handles NAL unit type FD_NUT.

#### Test bitstreams #VPSID_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. This bitstream contains two vps's. The correct one has the value "4". The bitstream has 3 temporal layers and the correct VPS has the temporal_nesting_flag turned off.

- Functional stage: Test decoding of vps_video_parameter_set_id.

Purpose: Check if decoder properly handles vps_video_parameter_set_id.

#### Test bitstreams #PS_A

Specification: All slices are coded as I or P slices. Each picture contains only one slice. vps_extention_flag, sps_extension_flag and pps_extension_flag are set equal to 1. Random data is included after each extension_flag.

- Functional stage: Test decoding of VPS, SPS and PPS.

Purpose: Check if decoder properly handles extension_flag in VPS, SPS and PPS.

#### Test bitstreams #PS_B

Specification: All slices are coded as I or P slices. Each picture contains only one slice. sps_extension_flag is set equal to 1. Random data is included after sps_extension_flag.

- Functional stage: Test decoding of VPS, SPS and PPS.

Purpose: Check if decoder properly handles extension_flag in SPS.

#### Test bitstreams #PPS_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. Bitstream includes multiple PPS signalling with random PPS parameters (constrained intra, transform skip, tile configurations, WP, loop filter, etc.) that get randomly selected by coded pictures.

- Functional stage: Test decoding of PPS.

Purpose: Check if decoder properly handles Multiple PPS signalling.

#### Test bitstreams #SLPPLP_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The bitstream contains 3 temporal layers (two sublayers).

- Functional stage: Test decoding of sub_layer_profile_present_flag and sub_layer_level_present_flag.

Purpose: Check if decoder properly handles sub_layer_profile_present_flag and sub_layer_level_present_flag.

      - Test bitstreams #OPFLAG_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The value of output_flag_present_flag is 1, indicating that the syntax element pic_output_flag is signaled in the slice header.

- Functional stage: Test parsing of output_flag_present_flag in PPS. Test parsing of pic_output_flag in slice header syntax.

Purpose: Check whether the decoder can decode slice headers containing pic_output_flag.

#### Test bitstreams #OPFLAG_B

Specification: All slices are coded as I slices. Each picture contains only one slice. The value of output_flag_present_flag is 1, indicating that the syntax element pic_output_flag is signaled in the slice header. Picture with POC 39 and 73 are set to be not for output.

- Functional stage: Test picture output

Purpose: Test picture output flag indication

#### Test bitstreams #OPFLAG_C

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The value of output_flag_present_flag is 1, indicating that the syntax element pic_output_flag is signaled in the slice header. Picture with POC 20, 31, 56, and 72 are set to be not for output.

Functional stage: Test picture output

Purpose: Test picture output flag indication

#### Test bitstreams #NoOutPrior_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The value of no rasl output flag and no output pic prior flag is equal to 1 when when CRA follows and end of sequence.

Functional stage: Test picture output

Purpose: Test no rasl output flag and no output pic prior flag when CRA follows and end of sequence.

#### Test bitstreams #NoOutPrior_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The value of no output pic prior flag is equal to 1 when when IDR follows.

Functional stage: Test picture output

Purpose: Test no output pic prior flag when IDR follows.

      - Test bitstreams #PICSIZE_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. Each picture includes 4 tile columns.

- Functional stage: Test verious picture size.

Purpose: Check if decoder properly decode pictures whose size is 1056x8440.

#### Test bitstreams #PICSIZE_B

Specification: All slices are coded as I or B slices. The bitstream is designed to test maximum height for level 5.1. Picture size is 8440x1056. Picture width is not a multiple of 16

- Functional stage: Test verious picture size.

Purpose: Check if decoder properly decode pictures whose size is 8440x1056 (Picture width is not amultiple of 16).

#### Test bitstreams #PICSIZE_C

Specification: All slices are coded as I or B slices. Each picture includes 2 tile columns.

- Functional stage: Test verious picture size.

Purpose: Check if decoder properly decode pictures whose size is 528x4216.

#### Test bitstreams #PICSIZE_D

Specification: All slices are coded as I or B slices. The bitstream is designed to test maximum height for level 4.1. Picture size is 4216x528

- Functional stage: Test verious picture size.

Purpose: Check if decoder properly decode pictures whose size is 4216x528.

#### Test bitstreams #POC_A

Specification: All slices are coded as I or B slices. The bitstream is designed to test some rules related to POC derivation

- Functional stage: Test POC derivation process.

Purpose: Check if decoder properly derive POC.

#### Test bitstreams #RAP_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The first picture in the bitstream is CRA and is followed by 7 RASL pictures that are not decodable. There are two subsequent CRA pictures with RASL pictures, following the first CRA picture in this bitstream. These subsequent RASL pictures should be decodable since the associated CRA is not the first CRA picture in the bitstream.

- Functional stage: Test reconstruction process starting with a CRA picture followed by RASL pictures that cannot be decoded of slices with the inter RPS prediction.

Purpose: Check if decoder properly decodes a conforming bitstream where the CRA is the first picture in the bitstream and is followed by RASL pictures that are not decodable.

#### Test bitstreams #RAP_B

Specification: All slices are coded as I or B slices. CRA picture with leading pictures following EOS NAL unit is included. VPS, SPS and PPS are present in the bitstream repeatedly. The conformance window for cropping is signaled in the bitstream.

- Functional stage: Test reconstruction process of CRA picture.

Purpose: Check if decoder properly decodes a conforming bitstream with CRA picture with leading pictures following EOS nal unit.

#### Test bitstreams #RPS_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The slice header includes the inter-RPS prediction method for sending the RPS for short-term pictures. The last 3 frames of this 44 frames sequence contain Slice header RPS using the inter-RPS in addition to the RPS sent in the PPS.

- Functional stage: Test reconstruction process of slices with inter-RPS prediction.

Purpose: Check if decoder properly decodes slices with inter-RPS prediction method for sending the RPS for short-term pictures in the slice header.

#### Test bitstreams #RPS_B

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The bitstream includes random RPS signaling in slice header along with random picture coding order within a GOP.

- Functional stage: Test reconstruction process of slices without inter-RPS prediction.

Purpose: Check if decoder properly decodes slices with the inter-RPS prediction method for sending the RPS for short-term pictures in the slice header.

#### Test bitstreams #RPS_C

Specification: All slices are coded as I or P slices. Each picture contains only one slice. Two temporal layers are used. 15 reference pictures are used. The bitstream exercises short-term reference pictures in the RPS.

- Functional stage: Test short-term RPS handling.

Purpose: Check short-term picture handling in the RPS.

#### Test bitstreams #RPS_D

Specification: All slices are coded as I or P slices. Each picture contains only one slice. Two temporal layers are used. The bitstream exercises short-term and long-term reference pictures in the RPS.

- Functional stage: Test long-term and short-term RPS handling.

Purpose: Check long-term and short-term picture handling in the RPS.

#### Test bitstreams #RPS_E

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The bitstream includes random RPS signaling with LTRPs in slice header along with random picture coding order within a GOP.

- Functional stage: Test reconstruction process of slices without inter-RPS prediction.

Purpose: Check if decoder properly decodes slices with long-term reference pictures, without inter-RPS prediction.

#### Test bitstreams #RPS_F

Specification: The inter-RPS prediction signals some RPS entries that are not used by the current picture. (used_by_curr_pic_flag[ j ] equal to 0 and use_delta_flag[ j ] equal to 1).

- Functional stage: Test reconstruction process of slices without inter-RPS prediction.

Purpose: Check if decoder properly decode slices with the inter-RPS prediction method for sending the RPS for short-term pictures in the slice header. The inter-RPS prediction signals some RPS entries that are not used by the current picture. (used_by_curr_pic_flag[ j ] equal to 0 and use_delta_flag[ j ] equal to 1).

#### Test bitstreams #LTRPSPS_A

Specification: The bitstream is coded under the random access conditions described in the CTC, with the following modifications. Eight long-term reference picture candidates (four difference POC LSB values and 2 values of used_by_curr, giving a total of 8) are signaled in the SPS. The slice headers refer to long-term reference pictures that are either referred to from the SPS or may be explicitly signaled in the slice header. Reference picture list modification is applied on some pictures.

Functional stage: Test parsing of long_term_ref_pics_present_flag, num_long_term_ref_pics_sps, lt_ref_pic_poc_lsb_sps, and used_by_curr_pic_lt_sps_flag in SPS. Test parsing of num_long_term_sps and lt_idx_sps in slice header syntax.

Purpose: Check whether the decoder can decode slice headers when the long-term reference pictures from the list of candidates in the SPS are referred to.

      - Test bitstreams #RPLM_A

Specification: All slices are coded as I or B slices. Each picture contains only one slice. The bitstream includes random reference picture list modification with varying list sizes.

- Functional stage: Test reconstruction process of slices with reference list modification.

Purpose: Check if decoder properly decodes slices with reference list modification.

#### Test bitstreams #RPLM_B

Specification: All slices are coded as I or B slices. Each picture contains more than one slice. The bitstream includes random reference picture list modification with varying list sizes.

- Functional stage: Test reconstruction process of slices with reference list modification.

Purpose: Check if decoder properly decodes slices with reference list modification.

#### Test bitstreams #SLICES_A

Specification: Each picture contains more than one slice with different slice type.

- Functional stage: Test reconstruction process of pictures comprising slices of different slice_type.

Purpose: Check if decoder properly decodes pictures comprising slices of different slice_type.

#### Test bitstreams #DSLICE_A

Specification: All slices are coded as I or B slices. Each picture contains more than one slice. dependent_slice_segments_enabled_flag is set equal to 1.

- Functional stage: Test reconstruction process of independent and dependent slice segments.

Purpose: Check if decoder properly decodes independent and dependent slice segments.

#### Test bitstreams #DSLICE_B

Specification: All slices are coded as I or B slices. Each picture contains more than one slice. dependent_slice_segments_enabled_flag is set equal to 1. entropy_coding_sync_enabled_flag is set to 1.

- Functional stage: Test reconstruction process of dependent slice segments.

Purpose: Check if decoder properly decodes dependent slice segments in combination with substreams (WPP).

#### Test bitstreams #DSLICE_C

Specification: All slices are coded as I or B slices. Each picture contains more than one slice. dependent_slice_segments_enabled_flag is set equal to 1. tiles_enabled_flag is set to 1.

- Functional stage: Test reconstruction process of dependent slice segments.

Purpose: Check if decoder properly decodes dependent slice segments in combination with tiles.

#### Test bitstreams #BUMPING_A

Specification: All slices are coded as I or B slices. Each picture contains more than one slice. All pictures with POC 0-65 are to be output except the pictures with the following POC values:

4, 5, 6, 7, 15, 21, 22, 23, 30, 31, 36, 37, 38, 39, 54, 55, 56. Those pictures are not output since they have not been output yet when IRAPs with no_output_of_prior_pics_flag equal to 1 are encountered in the bitstream.

- Functional stage: Test bumping process.

Purpose: Check if decoder properly handles tests output order conformance, in particular the bumping・process. Four temporal layers are used and IRAP pictures with no_output_of_prior_pics_flag equal to 1 are present in the bitstream.

#### Test bitstreams #CONFWIN_A

Specification: All slices are coded as I or B slices. Each picture contains more than one slice. The value of conf_win_bottom_offset, conf_win_top_offset, conf_win_left_offset and conf_win_right_offset is set equal to 1.

- Functional stage: Test conformance window.

Purpose: Check if decoder properly handles conf_win_bottom_offset, conf_win_top_offset, conf_win_left_offset, and conf_win_right_offset.

      - Test bitstreams #HRD_A

Specification: All slices are coded as I or B slices. Each access unit (AU) contains 4 decoding units (DU). sub_pic_cpb_params_present_flag is set equal to 1 and du_common_cpb_removal_delay_flag is set equal to 0.

- Functional stage: Test the DU-based HRD.

Purpose: Check that decoder can properly decode with DU-based CPB removal time signaling.

#### Test bitstreams #EXT_A

Specification: A three picture stream containing extension fields in VPS, SPS, PPS, and slice header. The stream will later be updated to also include VPS extension fields, currently HM 9.1 does not support extensions in VPS. Note that this bitstream is not a conforming bitstream but conforming decoders are required to be able to parse it.

- Functional stage: Test decoding of extension fields.

Purpose: Check decoding of extension fields.

### Test bitstreams – 10 bit

#### Test bitstreams #WP_A_MAIN10

Specification: All slices are coded as I or P slices. Each picture contains only one slice. bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. weighted_pred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage: Weighted sample prediction process for P slices with plural reference indices.

Purpose: Check that the decoder handles weighted sample prediction for P slices with plural reference indices.

#### Test bitstreams #WP_B_MAIN10

Specification: All slices are coded as I, P or B slices. Each picture contains only one slice. bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. weighted_pred_flag is equal to 1 and weighted_bipred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage: Weighted sample prediction process for P and B slices with plural reference indices.

Purpose: Check that the decoder handles weighted sample prediction for P and B slices with plural reference indices.

#### Test bitstreams #TSUNEQBD_A_MAIN10

Specification: Each picture contains only one slice. bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 1. In PPS, transform_skip_enabled_flag set to 1. In Transform Unit (residual_coding()), transform_skip_flag set to 1 for Y, Cb, Cr for both intra and inter prediction modes.

- Functional stage: Test parsing and reconstruction process of slices with transform skip mode for luma and chroma with different bitdepths.

Purpose: Check Transform Skip intra and inter with unequal bitdepth (luma: 10-bit, chroma: 9-bit).

#### Test bitstreams #DBLK_A_MAIN10

Specification: All slices are coded as I or B slices. Each picture contains only one slice. bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. Some QP is seto to negative value.

- Functional stage: Test deblocking filter process for 10 bit video.

Purpose: Check if decoder properly handles negative value of QP for deblocking filter process.

#### Test bitstreams #INITQP_B_MAIN10

Specification: All slices are coded as I or B slices. bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. The value of init_qp_minus26 is set from -38 to 25.

- Functional stage: Test the initial QP.

Purpose: Check if decoder properly handles init_qp_minus26.

      - Test bitstreams #WPP_A_MAIN10

Specification: bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,  QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_B_MAIN10

Specification: bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 32x32 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,  QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_C_MAIN10

Specification: bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 16x16 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,  QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_D_MAIN10

Specification: bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 1 CTU wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised when a picture is 1 CTU wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 1 CTU wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_E_MAIN10

Specification: bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 2 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised when a picture is 2 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 2 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_F_MAIN10

Specification: bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 3 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage: Tests that entropy coding is correctly synchronised when a picture is 3 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose: The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 3 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

## Normative test suites for ITU-T H.265 | ISO/IEC 23008-2

Legend:

- X – Bitstream is for static and dynamic test

| Table 1 – Bitstreams for Main and Main10 profiles |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Sub category | Bitstream | File name | Main | Main 10 | Main still picture |  | Main tier | Level | Frame rate (Frames/sec) |
| Block structure | Block structure and partitioning | STRUCT_A | STRUCT_A_Samsung_5 | X | X |  |  | X | 5.1 and higher | 50 |
|  |  | STRUCT_B | STRUCT_B_Samsung_4 | X | X |  |  | X | 5.1 and higher | 50 |
| Intra coding | Intra prediction | IPRED_A | IPRED_A_docomo_2 | X | X |  |  | X | 5.1 and higher | 30 |
|  |  | IPRED_B | IPRED_B_Nokia_3 |  |  | X |  | X | 4.0 and higher | N/A |
|  |  | IPRED_C | IPRED_C_Mitsubishi_3 | X | X |  |  | X | 3.0 and higher | 30 |
|  | Constraint intra prediction | CIP_A | CIP_A_Panasonic_3 | X | X |  |  |  | 4.0 and higher | 30 |
|  |  | CIP_B | CIP_B_NEC_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | CIP_C | CIP_C_Panasonic_2 | X | X |  |  |  | 4.0 and higher | 30 |
| Inter frame coding | Merge | MERGE_A | MERGE_A_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | MERGE_B | MERGE_B_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | MERGE_C | MERGE_C_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | MERGE_D | MERGE_D_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | MERGE_E | MERGE_E_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | MERGE_F | MERGE_F_MTK_4 | X | X |  |  |  | 4.0 and higher | 30 |
|  |  | MERGE_G | MERGE_G_HHI_4 | X | X |  |  |  | 3.1 and higher | 60 |
|  | Parallel merge | PMERGE_A | PMERGE_A_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | PMERGE_B | PMERGE_B_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | PMERGE_C | PMERGE_C_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | PMERGE_D | PMERGE_D_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  |  | PMERGE_E | PMERGE_E_TI_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  | AMVP | AMVP_A | AMVP_A_MTK_4 | X | X |  |  |  | 4.0 and higher | 50 |
|  |  | AMVP_B | AMVP_B_MTK_4 | X | X |  |  |  | 4.0 and higher | 50 |
|  |  | AMVP_C | AMVP_C_Samsung_4 | X | X |  |  |  | 5.1 and higher | 30 |
|  | Temporal MVP | TMVP_A | TMVP_A_MS_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  | mvd_l1_zero | MVDL1ZERO_A | MVDL1ZERO_A _Docomo_4 | X | X |  |  |  | 4.0 and higher | 50 |
|  | MVP clipping | MVCLIP_A | MVCLIP_A_qualcomm_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  | MV pointing to picture edge | MVEDGE_A | MVEDGE_A_qualcomm_3 | X | X |  |  |  | 2.0 and higher | 30 |
|  | Weighted prediction | WP_A | WP_A_Toshiba_3 | X | X |  |  |  | 2.0 and higher | 60 |
|  |  | WP_B | WP_B_Toshiba_3 | X | X |  |  |  | 2.0 and higher | 60 |
| Transform and quantization | RQT | RQT_A | RQT_A_HHI_4 | X | X |  |  |  | 3.1 and higher | 60 |
|  |  | RQT_B | RQT_B_HHI_4 | X | X |  |  |  | 3.1 and higher | 60 |
|  |  | RQT_C | RQT_C_HHI_4 | X | X |  |  |  | 3.1 and higher | 60 |
|  |  | RQT_D | RQT_D_HHI_4 | X | X |  |  |  | 3.1 and higher | 60 |
|  |  | RQT_E | RQT_E_HHI_4 | X | X |  |  |  | 3.1 and higher | 60 |
|  |  | RQT_F | RQT_F_HHI_4 | X | X |  |  | X | 3.1 and higher | 60 |
|  |  | RQT_G | RQT_G_HHI_4 | X | X |  |  | X | 3.1 and higher | 60 |
|  |  | TUSIZE_A | TUSIZE_A_Samsung_1 | X | X |  |  | X | 5.0 and higher | 30 |
|  | Quantization | DELTAQP_A | DELTAQP_A_BRCM_4 | X | X |  |  | X | 5.0 and higher | 24 |
|  |  | DELTAQP_B | DELTAQP_B_Sony_3 | X | X |  |  |  | 4.0 and higher | 30 |
|  |  | DELTAQP_C | DELTAQP_C_Sony_3 | X | X |  |  |  | 4.0 and higher | 30 |
|  |  | INITQP_A | INITQP_A_Sony_1 | X | X |  |  |  | 4.0 and higher | 30 |
|  | Scaling list | SLIST_A | SLIST_A_Sony_4 | X | X |  |  | X | 4.0 and higher | 60 |
|  |  | SLIST_B | SLIST_B_Sony_8 | X | X |  |  | X | 4.0 and higher | 60 |
|  |  | SLIST_C | SLIST_C_Sony_3 | X | X |  |  | X | 4.0 and higher | 60 |
|  |  | SLIST_D | SLIST_D_Sony_9 | X | X |  |  | X | 4.0 and higher | 60 |
| In-loop filter | Deblocking filter | DBLK_A | DBLK_A_Sony_3 | X | X |  |  |  | 4.0 and higher | 30 |
|  |  | DBLK_B | DBLK_B_Sony_3 | X | X |  |  |  | 4.0 and higher | 30 |
|  |  | DBLK_C | DBLK_C_Sony_3 | X | X |  |  |  | 4.0 and higher | 30 |
|  |  | DBLK_D | DBLK_D_VIXS_2 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | DBLK_E | DBLK_E_VIXS_2 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | DBLK_F | DBLK_F_VIXS_2 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | DBLK_G | DBLK_G_VIXS_2 | X | X |  |  | X | 4.1 and higher | 60 |
|  | Sample adaptive offset | SAO_A | SAO_A_MediaTek_4 | X | X |  |  | X | 4.0 and higher | 60 |
|  |  | SAO_B | SAO_B_MediaTek_5 | X | X |  |  | X | 4.0 and higher | 60 |
|  |  | SAO_C | SAO_C_Samsung_4 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | SAO_D | SAO_D_Samsung_4 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | SAO_E | SAO_E_Canon_4 | X | X |  |  | X | 4.0 and higher | 50 |
|  |  | SAO_F | SAO_F_Canon_3 | X | X |  |  | X | 4.0 and higher | 50 |
|  |  | SAO_G | SAO_G_Canon_3 | X | X |  |  | X | 6.2 | 50 |
| Entropy coding | Maximum bins | MAXBINS_A | MAXBINS_A_TI_4 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | MAXBINS_B | MAXBINS_B_TI_4 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | MAXBINS_C | MAXBINS_C_TI_4 | X | X |  |  | X | 2.0 and higher | 30 |
|  | CABAC initialization | CAINIT_A | CAINIT_A_Sharp_4 | X | Ｘ |  |  | X | 3.0 and higher | 50 |
|  |  | CAINIT_B | CAINIT_B_Sharp_4 | X | Ｘ |  |  | X | 3.0 and higher | 50 |
|  |  | CAINIT_C | CAINIT_C_Sharp_3 | X | Ｘ |  |  | X | 3.0 and higher | 50 |
|  |  | CAINIT_D | CAINIT_D_Sharp_3 | X | Ｘ |  |  | X | 3.0 and higher | 50 |
|  |  | CAINIT_E | CAINIT_E_Sharp_3 | X | Ｘ |  |  | X | 3.0 and higher | 50 |
|  |  | CAINIT_F | CAINIT_F_Sharp_3 | X | Ｘ |  |  | X | 3.0 and higher | 50 |
|  |  | CAINIT_G | CAINIT_G_Sharp_3 | X | Ｘ |  |  | X | 3.1 and higher | 50 |
|  |  | CAINIT_H | CAINIT_H_Sharp_3 | X | Ｘ |  |  | X | 3.1 and higher | 50 |
|  | Sign data hiding | SDH_A | SDH_A_Orange_3 | X | X |  |  |  | 4.1 and higher | 50 |
| Temporal scalability | Temporal scalability | TSCL_A | TSCL_A_VIDYO_5 | X | X |  |  | X | 2.1 and higher | 50 |
|  |  | TSCL_B | TSCL_B_VIDYO_4 | X | X |  |  | X | 2.1 and higher | 50 |
| Parallel processing tools | Tiles | TILES_A | TILES_A_Cisco_2 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | TILES_B | TILES_B_Cisco_1 | X | X |  |  | X | 4.1 and higher | 60 |
|  | Wavefront parallel processing | WPP_A | WPP_A_Ericsson_ MAIN_2 | X | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_B | WPP_B_Ericsson_ MAIN_2 | X | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_C | WPP_C_Ericsson_ MAIN_2 | X | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_D | WPP_D_Ericsson_ MAIN_2 | X | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_E | WPP_E_Ericsson_ MAIN_2 | X | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_F | WPP_F_Ericsson_ MAIN_2 | X | X |  |  | X | 2.0 and higher | 50 |
|  | Entry point | ENTP_A | ENTP_A_Qualcomm_1 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | ENTP_B | ENTP_B_Qualcomm_1 | X | X |  |  | X | 4.1 and higher | 60 |
|  |  | ENTP_C | ENTP_C_Qualcomm_1 | X | X |  |  | X | 4.1 and higher | 60 |
| Other coding tools | IPCM | IPCM_A | IPCM_A_NEC_3 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | IPCM_B | IPCM_B_NEC_3 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | IPCM_C | IPCM_C_NEC_3 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | IPCM_D | IPCM_D_NEC_3 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | IPCM_E | IPCM_E_NEC_2 | X | X |  |  | X | 2.0 and higher | 30 |
|  | Transform skip | TS_A | TSKIP_A_MS_3 | X | X |  |  | X | 3.1 and higher | 30 |
|  | AMP | AMP_A | AMP_A_Samsung_4 | X | X |  |  | X | 5.1 and higher | 30 |
|  |  | AMP_B | AMP_B_Samsung_4 | X | X |  |  | X | 5.1 and higher | 30 |
|  |  | AMP_D | AMP_D_HiSilicon_3 | X | X |  |  | X | 6.2 and higher | 24 |
|  |  | AMP_E | AMP_E_HiSilicon_3 | X | X |  |  | X | 6.2 and higher | 50 |
|  |  | AMP_F | AMP_F_HiSilicon_3 | X | X |  |  | X | 6.2 and higher | 60 |
|  | Lossless | LS_A | LS_A_Orange_2 | X | X |  |  | X | 5.0 and higher | 30 |
|  |  | LS_B | LS_B_Orange_4 | X | X |  |  | X | 5.0 and higher | 30 |
| High level syntax | UAL unit types | NUT_A | NUT_A_Ericsson_5 | X | X |  |  | X | 3.0 and higher | 30 |
|  |  | FILLER_A | FILLER_A_Sony_1 | X | X |  |  |  | 4.0 and higher | 30 |
|  | VPS | VPSID_A | VPSID_A_VIDYO_1 | X | X |  |  | X | 3.1 and higher | 50 |
|  | Parameter set | PS_A | PS_A_VIDYO_2 | X | X |  |  | X | 2.1 and higher | 50 |
|  |  | PS_B | PS_B_VIDYO_2 | X | X |  |  | X | 2.1 and higher | 50 |
|  | Picture parameter set | PPS_A | PPS_A_qualcomm_7 | X | X |  |  | X | 6.2 and higher | 30 |
|  | Sub layer | SLPPLP_A | SLPPLP_A_VIDYO_1 | X | X |  |  | X | 3.1 and higher | 50 |
|  | Output flag | OPFLAG_A | QPFLAG_A_qualcomm_1 | X | X |  |  | X | 2.1 and higher | 50 |
|  |  | OPFLAG_B | OPFLAG_B_Qualcomm_1 | X | X |  |  | X | 3.1 and higher | 60 |
|  |  | OPFLAG_C | OPFLAG_C_Qualcomm_1 | X | X |  |  | X | 3.1 and higher | 60 |
|  |  | NoOutPrior_A | NoOutPrior_A_Qualcomm_1 | X | X |  |  | X | 3.1 and higher | 60 |
|  |  | NoOutPrior_B | NoOutPrior_B_Qualcomm_1 | X | X |  |  | X | 3.1 and higher | 60 |
|  | Picture size | PICSIZE_A | PICSIZE_A_Bossen_1 | X | X |  |  | X | 5.1 and higher | 50 |
|  |  | PICSIZE_B | PICSIZE_B_Bossen_1 | X | X |  |  | X | 5.1 and higher | 50 |
|  |  | PICSIZE_C | PICSIZE_C_Bossen_1 | X | X |  |  | X | 4.1 and higher | 50 |
|  |  | PICSIZE_D | PICSIZE_D_Bossen_1 | X | X |  |  | X | 4.1 and higher | 50 |
|  | Picture order count | POC_A | POC_A_Bossen_1 | X | X |  |  | X | 4.0 and higher | 50 |
|  | Random access | RAP_A | RAP_A_Docomo_5 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | RAP_B | RAP_B_Bossen_1 | X | X |  |  | X | 6.2 | 50 |
|  | RPS | RPS_A | RPS_A_Docomo_5 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | RPS_B | RPS_B_qualcomm_5 | X | X |  |  | X | 3.0 and higher | 30 |
|  |  | RPS_C | RPS_C_Ericsson_5 | X | X |  |  | X | 3.0 and higher | 30 |
|  |  | RPS_D | RPS_D_Ericsson_6 | X | X |  |  | X | 3.0 and higher | 30 |
|  |  | RPS_E | RPS_E_Qualcomm_5 | X | X |  |  | X | 3.0 and higher | 30 |
|  |  | RPS_F | RPS_F_docomo_2 | X | X |  |  | X | 6.2 | 30 |
|  | Long term reference | LTRSPS | LTRPSPS_A_Qualcomm_1 | X | X |  |  | X | 2.1 and higher | 50 |
|  | Reference picture list modification | RPLM_A | RPLM_A_qualcomm_4 | X | X |  |  | X | 2.0 and higher | 30 |
|  |  | RPLM_B | RPLM_B_qualcomm_4 | X | X |  |  | X | 2.0 and higher | 30 |
|  | Slice type | SLICES_A | SLICES_A_Rovi_3 | X | X |  |  | X | 6.2 | 30 |
|  | Dependent slice | DSLICE_A | DSLICE_A_HHI_4 | X | X |  |  | X | 3.1 and higher | 24 |
|  |  | DSLICE_B | DSLICE_B_HHI_4 | X | X |  |  | X | 3.1 and higher | 24 |
|  |  | DSLICE_C | DSLICE_C_HHI_4 | X | X |  |  | X | 3.1 and higher | 24 |
|  | DPB | BUMPING_A | BUMPING_A_ericsson_1 | X | X |  |  | X | 3.0 and higher | 30 |
|  | Conformance window | CONFWIN_A | CONFWIN_A_Sony_1 | X | X |  |  |  | 4.0 and higher | 30 |
|  | HRD | HRD_A | HRD_A_Fujitsu_2 | X | X |  |  | X | 6.2 | 50 |
|  | Extensions | EXT_A | EXT_A_Ericsson_4 | X | X |  |  | X | 3.0 and higher | 30 |
| 10 bit | Weighted prediction | WP_A_ MAIN10 | WP_A_MAIN10_ Toshiba_3 |  | X |  |  | X | 2.0 and higher | 60 |
|  |  | WP_B_ MAIN10 | WP_B_MAIN10_ Toshiba_3 |  | X |  |  | X | 2.0 and higher | 60 |
|  | Transform Skip | TSUNEQBD_A | TSUNEQBD_A_MAIN10_ Technicolor_1 |  | X |  |  | X | 5.1 and higher | 30 |
|  | Deblocking filter | DBLK_A_ MAIN10 | DBLK_A_MAIN10_ VIXS_3 |  | X |  |  | X | 4.0 and higher | 30 |
|  | Quantization | INITQP_B_ Main10 | INITQP_B_Main10_Sony_1 |  | X |  |  |  | 4.0 and higher | 30 |
|  | Wavefront parallel processing | WPP_A_ MAIN10 | WPP_A_MAIN10_ Ericsson_2 |  | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_B_ MAIN10 | WPP_B_MAIN10_ Ericsson_2 |  | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_C_ MAIN10 | WPP_C_MAIN10_ Ericsson_2 |  | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_D_ MAIN10 | WPP_D_MAIN10_ Ericsson_2 |  | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_E_ MAIN10 | WPP_E_MAIN10_ Ericsson_2 |  | X |  |  | X | 2.0 and higher | 50 |
|  |  | WPP_F_ MAIN10 | WPP_F_MAIN10_ Ericsson_2 |  | X |  |  | X | 2.0 and higher | 50 |

This Recommendation | International Standard includes an electronic attachment containing the conformance bitstreams identified within the text. The bitstreams can also be downloaded from the ITU-T Test Signal Database.