| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q1009 |
| --- | --- |

| Title: | Common SHM test conditions and software reference configurations |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin Yong He | Email: | vseregin@qti.qualcomm.com Yong.He@InterDigital.com |
| Source: | AHG12 chairs |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations to be used in the development of HEVC scalable extension (SHVC) conducted between the 17th and the 18th JCT-VC meetings. These common test conditions are also recommended for use in technical contributions to the 18th JCT-VC meeting, if applicable.

# Introduction

Common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

This document defines 8 mandatory and 3 optional test conditions for 2x, 1.5x, and SNR scalability ratios for both HEVC and AVC base layers. AVC base layer test configurations are mandatory only for the technical contributions specifically developed for the base layer coded with AVC.

| Mandatory test conditions | Intra, spatial 2x Intra, spatial 1.5x Random access, spatial 2x Random access, spatial 1.5x Random access, SNR Low delay, B slices, spatial 2x Low delay, B slices, spatial 1.5x Low delay, B slices, SNR |
| --- | --- |
| Optional test conditions | Low delay, P slices only, spatial 2x Low delay, P slices only, spatial 1.5x Low delay, P slices only, SNR |

A subset of these conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

SHM-6.0 software (based on HM-14.0) is expected to be used for experiments, software package can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/SHM-6.0/. HM14.0 shall be used to generate the anchor of simulcast and single layer high quality.

The following sections define test sequences, quantization parameter values, encoder configuration files, and compile-time options to be used.

People bringing input contributions should provide a set of results as complete as possible that apply to the proposal. Results should be reported using the attached Excel sheets.

# Test sequences

Table 1. Test sequences defines the set of test sequences, base (BL) and enhancement (EL) layers resolution and scalability ratios to be used for intra, random-access, and low-delay conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences and test cases described below.

Test sequences are available at ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/ and ftp://hevc@ftp.tnt.uni-hannover.de/scalable/sequences/. AVC base layer bitstreams are available at the ftp://hevc@ftp.tnt.uni-hannover.de/scalable/bitstreams/AVC_CTC (please contact the JCT-VC chairs for login information).

Table 1. Test sequences

| Class | Sequence name | Frame count | Frame rate, Hz | BL resolution | EL resolution | Intra | Random access | Low-delay P or B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30 | 1280x800 2560x1600 | 2560x1600 2560x1600 | Spatial 2x | Spatial 2x SNR | Spatial 2x SNR |
| A | PeopleOnStreet | 150 | 30 | 1280x800 2560x1600 | 2560x1600 2560x1600 | Spatial 2x | Spatial 2x SNR | Spatial 2x SNR |
| B | Kimono | 240 | 24 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | ParkScene | 240 | 24 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | Cactus | 500 | 50 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | BasketballDrive | 500 | 50 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | BQTerrace | 600 | 60 | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |

# Quantization parameter values

For each video sequence 8 quantization parameter sets are to be used depending upon the test configuration for both AVC and HEVC base layers.  These values define the QP values used for the I-frames of the base (BL) and enhancement (EL) layers in a sequence (configuration files further define QP values used for other frames):

| Scalability ratio | BL QP | EL delta QP |
| --- | --- | --- |
| Spatial 2x and 1.5x | 22, 26, 30, 34 | 0, 2 |
| SNR | 26, 30, 34, 38 | -6, -4 |

# Configuration files

This section defines encoder configuration files to be used for each test case. Only two layers are considered for the common testing conditions. Parameters to be changed for each test point are the following (where N=0 for the base layer and N=1 for the enhancement layer):

- InputFileN to reflect the location of the source video sequence for layer N on the test system.

- FrameRateN to reflect the frame rate of a given sequence for layer N as per Table 1. Test sequences.

- SourceWidthN to reflect the width of the source video sequence for layer N.

- SourceHeightN to reflect the height of the source video sequence for layer N.

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1. Test sequences.

- IntraPeriodN to reflect the intra refresh period for layer N in the random access test cases. The intra refresh period is dependent on the frame rate of the source: a value 16 shall be used for sequences with a frame rate equal to 20fps, 24 for 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

- QPN to reflect the quantization parameter values for layer N defined in section 3.

These configuration files are provided in the cfg/ folder of the SHM software package (available at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/).

Test sequences are configured differently for HEVC and AVC base layers and sequence specific configuration files are located in cfg/per-sequence-svc/ and cfg/per-sequence-svc-avcbase/ folders correspondingly. There is one common configuration file with layer specific parameters which is cfg/layers.cfg for HEVC base layer tests and cfg/layers_avcbase.cfg for AVC base layer tests.

There are 4 configurations provided as follows:

- “All Intra – Main” (AI-Main): encoder_intra_main.cfg

- “Random access – Main” (RA-Main): encoder_randomaccess_main.cfg

- “Low-delay B – Main” (LB-Main): encoder_lowdelay_main.cfg

- “Low-delay P – Main” (LP-Main, optional): encoder_lowdelay_P_main.cfg

Finally, the SEIDecodedPictureHash parameter must also be configured to use MD5 hashes, i.e. set to 1.

# Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/TLibCommon folder of the software package. The default settings provided in the source code should be used.

# Results

Test results shall be reported using the Excel template attached to this document. Both HEVC and AV C base layers templates are provided.

In the template, the “Test” tab has to be populated with the test data to be compared against the anchor, containing reference data in the “Anchor” tab.

The format of the data is explained in the Figure 1. Data format description. and mandatory data to be filled correspond to the red columns while green column is optional.

Figure 1. Data format description.

# Colour gamut scalability

This document also includes anchor data with the accompanied Excel templates used in the SCE1 and colour gamut scalability related tests. The detailed description of the test sequences and test configuration can be found in the SCE1 description document [1].

# Reference

[1] A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, and X. Li “HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability”, JCTVC-Q1101