| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q1101 |
| --- | --- |

| Title: | Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Alberto Duenas, Pierre Andrivon, Yan Ye, Elena Alshina Xiang Li, Kemal Ugur, Cheung Auyeung, Seung-Hwan Kim | Email: | alberto@ngcodec.com, pierre.andrivon@technicolor.com, yan.ye@interdigital.com, elena_a.alshina@samsung.com, lxiang@qti.qualcomm.com, kemal.ugur@nokia.com, cheung.auyeung@am.sony.com, kimse@sharplabs.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 1 (SCE1) on Improved Weighted Prediction for Color Gamut Scalability and 8x2x2 non-uniform regions matrix based color difference prediction.

# Introduction

Color Gamut Scalability has been mentioned in the requirements of Scalable Coding Extension of HEVC. It allows addressing the cases the original Enhancement Layer uses a different color gamut than the Base Layer. This can be useful for instance in case of deployment of UHD services compatible with legacy HD devices: HD is using the Rec.709, while UHD is likely to use some of the parameters defined in the Rec.2020. Another realistic use case described in Error: Reference source not found considers UHDTV forward compatibility where color gamut is different between base and enhancement layers.

This SCE aims to test utilizing Weighted Prediction (WP) design for more efficient coding of color gamut scalability and 8x2x2 non-uniform regions matrix based color difference prediction.

# Participants

P = Participants (Contribution),

C = Crosscheckers.

| Company | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| Technicolor | P.Bordes P.Andrivon E.François | philippe.bordes@technicolor.com pierre.andrivon@technicolor.com edouard.francois@technicolor.com |  | X |
| InterDigital | Y. He Y. Ye | Yuwen.he@interdigital.com Yan.ye@interdigital.com |  | X |
| Samsung | E. Alshina A. Alshin A. Saxena | elena_a.alshina@samsung.com alexander_b.alshin@samsung,com asaxena@sta.samsung.com | X | X |
| Qualcomm | X. Li J. Chen | lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com |  | X |
| Sharp Labs | S.-H. Kim K. Misra | kimse@sharplabs.com misrak@sharplabs.com | X |  |
| Nokia | K.Ugur A.Alireza | kemal.ugur@nokia.com ext-alireza.aminlou@nokia.com | X |  |
| Sony | C.Auyeung | cheung.auyeung@am.sony.com | X | X |

# Test sequences

Same test sequences as used in the SCE1 established in the 16th JCT-VC meeting will be used.

The sequences with the associated license conditions are available at the Hannover FTP site: ftp.tnt.uni-hannover.de/scalable/sequences/CGS with scalability use credentials.

An MD5 file per file is provided to verify that the locally down scaled version of the sequences are identical to the sequences on the FTP site.

Table 1: List of test sequences.

| Test Sequences | size | fps | Frames | Bit-depth |
| --- | --- | --- | --- | --- |
| BT709_Birthday_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart1_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart2_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Parakeets_1920x1080_50_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 8 |
| BT709_TableCar_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Birthday_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart1_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart2_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_Parakeets_1920x1080_50_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 10 |
| BT709_TableCar_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_Birthday_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart1_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart2_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_Parakeets_1920x1080_50_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 10 |
| BT2020_TableCar_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT2020_Birthday_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart1_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_BirthdayFlashPart2_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |
| BT2020_Parakeets_3840x2160_50_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 50 | 250 | 10 |
| BT2020_TableCar_3840x2160_60_10bit_zerophase_0.9pi.yuv | 3840 x 2160 | 60 | 300 | 10 |

# Reference software (Anchor)

The reference software will be SHM6.0 including the color-gamut scalability feature adopted from JCTVC-Q0048 at the 17th meeting with the following modifications in TEncGOP.cpp to avoid potential GCC compiler optimization issues with the default makefile:

Add the followings to line 4326 in TEncGOP.cpp

#ifdef __GNUC__

__attribute__((optimize("no-tree-vectorize")))

#endif

to just before :

Void TEncGOP::filterImg(

Pel           *src,

Int           iSrcStride,

# Description of Experiments

2x and 1x scalability shall be tested with RA and AI configurations with the following contents:

Table 2: Test conditions

|  | Base layer | Enhancement layer |
| --- | --- | --- |
| Test cases | Base layer | Enhancement layer |
| 2x: RA and AI | 1080p 8-bit BT.709 QP_BL={22, 26 ,30, 34} | 2160p 10-bit BT.2020 QP_EL1={22, 26, 30, 34} QP_EL2={24, 28, 32, 36} |
| 1x: RA and AI | 1080p 10-bit BT.709 QP_BL={22, 26 ,30, 34} | 1080p 10-bit BT.2020 QP_EL1={22, 26, 30, 34} QP_EL2={18, 22, 26, 30} |

The common SHVC test conditions (QPs) will be used for AI and RA configurations, 2x.

For the 1x case we will use the same QPs for the base layer as on the SHVC test conditions and for the enhancement layer delta QPs are 0 and -4.

At the encoder side, the computation of the CGS information shall not utilize information other than current frame. Encoder may signal CGS information for each frame if necessary.

## JCTVC-Q0056 (Nokia, Samsung)

Encoder only algorithmError: Reference source not found that improves the coding efficiency of weighted prediction enabled in SHVC for inter-layer reference was proposed in JCTVC-Q0056.

The proposed encoder algorithm works as follows:

- Each pixel in (upsampled) inter-layer reference picture is classified into two classes based on their U&V values as follows:

C = (u[i] + v[i]) < threshold ,					(1)

threshold = DCCb+DCCr, and DCCb+DCCr here are mean value of Cb and Cr  on inter-layer reference  frame.

- Two sets of weight & offset parameters are calculated using the pixel values belonging to each class C and corresponding pixels in the enhancement layer frame.

- Inter-layer reference picture is placed in both lists and the weight and offset parameters found in step-2 are used for each list.

- Encoder selects the best mode and reference picture for each block based on rate-distortion cost.

In this test following encoder only algorithms for WP will be tested as described in JCTVC-Q0056:

- More accurate estimation of WP parameters: All the pixels in inter-layer reference picture are used to determine the weight and offset parameters. Weight parameters will be refined by only using pixels referenced in enhancement layer. In addition the separation criteria will be improved to take into account the triplet of Luma and Chroma values.

- Using more ILR pictures: More inter-layer reference pictures can be added to reference picture lists and separate weight and offset parameters will be signaled for each one of those pictures. N ILR pictures is equivalent to usage of 2*N color space regions in 3D LUT approach. The study of the trade-off between performance an number of extra ILR pictures is the goal of this test.

## JCTVC-Q0129 (Sony) and JCTVC-Q0141 (Sharp)

In JCTVC-Q0129, the colour space is divided into 8x4x4 regions. In JCTVC-Q0141, the coding of matrix coefficient is improved. In both contributions, the colour prediction is performed after re-sampling process.

In order to make complexity and coding performance trade off, this experiment tests and evaluates the coding performance and complexity of joint JCTVC-Q0129 and JCTVC-Q0141 with the following conditions.

- The number of regions in the colour space from JCTVC-Q0129 is changed to 8x2x2 non-uniform regions. The partition of Y is fixed. The partitions of Cb and Cr are adaptive and signalled in the bitstream.

- The matrix coefficients are encoded with the method in JCTVC-Q0141.

- The colour prediction is performed before the re-sampling process.

Figure 1: The number of color space regions is 8x2x2.

# Cross-checks

| Test | Proponent | Description | Cross-Checker |
| --- | --- | --- | --- |
| 1.1 | Nokia and Samsung | 1 ILP in both refernce lists, 2 classes of samples in WP parameters selection | InterDigital |
| 1.2 | Nokia and Samsung | 2 ILPs in both refernce lists, 4 classes of samples of samples in WP parameters selection | Sony |
| 1.3 | Nokia and Samsung | 4 ILPs in both refernce lists, 8 classes of samples of samples in WP parameters selection | Qualcomm |
| 1.4 | Nokia and Samsung | 8 ILPs in both refernce lists, 16 classes of samples of samples in WP parameters selection | Technicolor |
| 2.1 | Sony and Sharp | As described in 5.2 | Samsung Qualcomm |

# Time-line and Responsibilities

T0:					Reference software SHM6.0 available.

April 25th 2014:	Finalization of the CE description.

T0 + 1 week:		Create anchor results.

T0 + 4 weeks:		Test 1.x cross-verification begins: proponents provide software, draft contribution text and results to cross-verifiers(s) and CE coordinators.

T0 + 5 weeks:		Test 2.1 cross-verification begins: proponents provide software, draft contribution text and results to cross-verifiers(s) and CE coordinators.

June 2th 2014	Cross-verifiers report results to CE participants.