| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q1122 |
| --- | --- |

| Title: | HEVC Screen Content Coding Core Experiment 2 (SCCE2): Line-based Intra Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Xiaozhong Xu Li Zhang | Email: | cheerchen.cs98g@g2.nctu.edu.tw xiaozhong.xu@mediatek.com lizhang@qti.qualcomm.com |
| Source: | SCCE coordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Screen Content Coding Core Experiment 2 on line-based intra copy.

# Introduction

This document defines the HEVC Screen Content Coding Core Experiment 2 (SCCE2) on line-based intra copy to be performed for the upcoming July 2014 JCT-VC meeting. There are four experiments to be tested in this SCCE: line-based intra copy with 1-D BV, line-based intra copy with 2-D BV, constrained line splitting, and BV merging for line-based intra copy.

# List of Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| R. Cohen | cohen@merl.com | MERL |  |  |
| C.-C. Chen W.-H. Peng C.-C. Lin | cheerchen.cs98g@g2.nctu.edu.tw wpeng@cs.nctu.edu.tw JackLin.itri@gmail.com | NCTU/ITRI | X | X |
| X. Xiu | Xiaoyu.Xiu@interdigital.com | InterDigital |  |  |
| Y.-W. Huang P. Lai S. Liu X. Xu P. Chuang J. Ye | yuwen.huang@mediatek.com polin.lai@mediatek.com shan.liu@mediatek.com xiaozhong.xu@mediatek.com peter.chuang@mediatek.com jing.ye@mediatek.com | MediaTek | X | X |
| T. Lin | lintao@tongji.edu.cn | Tongji University | X |  |
| A. Saxena | a.saxena1@samsung.com | Samsung |  |  |
| Z. Ma W. Wang M. Xu H. Yu | Zhan.Ma@huawei.com Wei.Wang.sc@huawei.com m.xu@huawei.com haoping.yu@huawei.com | Huawei |  | X |
| O. Patrice G. Christophe L. Guillaume C. Rosewarne | Patrice.Onno@crf.canon.fr Christophe.Gisquet@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr Chris.Rosewarne@cisra.canon.com.au | Canon |  |  |
| J. Sole L. Zhang K. Rapaka | joels@qti.qualcomm.com lizhang@qti.qualcomm.com krapaka@qti.qualcomm.com | Qualcomm |  | X |
| A. Minezawa K. Miyazawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp | Mitsubishi |  |  |
| J.-Z. Xu | jzxu@microsoft.com | Microsoft |  |  |

# Tools to be tested

## Test A: Line-based Intra Copy with 1-D BV and CU-level Splitting (JCTVC-Q0033, MediaTek)

JCTVC-Q0033 introduces a line-based intra copy which divides a CU equally into multiple 2N×1 lines or 1×2N lines. Pixels within a line are predicted in the same way as intra block copy except that the basic unit for intra copy is not a block but a line. In this test, 1-D horizontal BV search is performed only for 1×2N lines while 1-D vertical BV search is performed only for 2N×1 lines (Figure 1). A flag is present at CU level to specify whether the 2N×1 splitting or the 1×2N splitting is in use.

| (a) | (b) |
| --- | --- |

- (a) Horizontal search for 1×2N lines and (b) vertical search for 2N×1 lines

## Test B: Line-based Intra Copy with 2-D BV and PU-level Splitting (JCTVC-Q0032, NCTU/ITRI)

JCTVC-Q0032 extends the notion of the line-based intra copy as described in Test A to performing intra copy for lines at PU level with 2-D BV for each line (Figure 2). In this test, a flag is present at PU level to indicate whether the row-wise splitting or the column-wise splitting is in use.

- An example of 2-D BV search for lines within PUs

## Test C: Test B + Constrained PU-level Splitting (JCTVC-Q0032, NCTU/ITRI, Tongji University)

This test imposes an additional constraint on Test B, which requires that all the PUs within a CU use the same form of line splitting, row-wise or column-wise. Only one flag needs to be signaled at the CU level to indicate which mode is in use.

## Test D: Test C + Block Vector Merging (JCTVC-Q0032, NCTU/ITRI)

To reduce BV overhead for Test C, this test reuses one previously decoded BV for representing the BV of the current line, and no block vector difference needs to be signaled. A flag is present for each line to switch adaptively between BV merging and BV difference coding.

# List of Testers and Cross-checks

| Test | Method | Tester | Cross-checker |
| --- | --- | --- | --- |
| A | Line-based Intra Copy with 1-D BV and CU-level Splitting | Mediatek | NCTU/ITRI |
| B | Line-based Intra Copy with 2-D BV and PU-level Splitting | NCTU/ITRI | Qualcomm |
| C | Test B + Constrained PU-level Splitting | NCTU/ITRI Tongji University | Huawei |
| D | Test C + Block Vector Merging | NCTU/ITRI | MediaTek |

# Test Conditions

The SCM-1.0 software will be used for all experiments. The common test conditions for screen content coding specified in JCTVC-Q1015 (tentatively to be released on May 2, 2014) will be used to evaluate the coding results for both the lossy and lossless operating points.

# Evaluation of SCCE results

Results of the SCCE will be evaluated on the basis of BD-rate results and complexity. The complexity assessment will be performed on the basis of encoding/decoding complexity.

## Complexity

To measure software run times, screen content coding software with the provided configurations and the proposals implemented on screen content coding software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposals. Relative computational time calculated against the anchor must be presented.

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

# Timeline

- T1 (April 18, 2014): Finalization and release of SCCCE description.

- T2 (April 25, 2014, tentative): Release of SCM-1.0 software.

- T3 (T2 + 3 weeks): Release of software, draft specification text and simulation results to SCCE participants and cross-checkers.

- T4 (T3 + 2 weeks): Release of cross-checking results.

- T5 (June 20, 2014): Submission deadline for the 18th JCT-VC meeting.

# References

- P. Lai, T.-D. Chuang, Y.-C. Sun, X. Xu, J. Ye, S.-T. Hsiang, Y.-W. Chen, K. Zhang, X. Zhang, S. Liu, Y.-W. Huang, S. Lei, “Description of screen content coding technology proposal by MediaTek,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0033, Mar. 2014.

- C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, Y.-J. Chang, C.-H. Hung, C.-C. Lin, J.-S. Tu, E.-C. Ke, J.-Y. Kao, C.-L. Lin, F.-D. Jou, F.-C. Chen, “Description of screen content coding technology proposal by NCTU and ITRI International,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0032, Mar. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1015, Mar. 2014.