| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q1123 |
| --- | --- |

| Title: | Description of Screen Content Core Experiment 3 (SCCE3): Palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang (Primary) Patrice Onno Rajan Joshi Robert Cohen Xiaoyu Xiu Zhan Ma | yuwen.huang@mediatek.com patrice.onno@crf.canon.fr rajanj@qti.qualcomm.com cohen@merl.com xiaoyu.xiu@interdigital.com Zhan.Ma@huawei.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Screen Content Core Experiment 3 (SCCE3) on palette mode.

# Introduction

The palette mode has been used in all the responses to the Screen Content Call for Proposals (CfP).  Most of the proposed palette techniques correspond to variations of previous contributions on palette mode submitted at the 16th JCT-VC meeting: JCTVC-P0108 and JCTVC-P0303. Through the descriptions of the responses to the CfP and the related contributions submitted at the 17th JCT-VC meeting, several variants of the palette modes have been proposed. These variants can be roughly categorized as follows: palette table coding, color index coding, and single color modes.

The goal of this Core Experiment (CE) is to further investigate each of these different technical parts of the palette mode and evaluate their individual performance according to the screen content test conditions. Evaluation of combinations of some individual parts of the palette mode is also included in this CE.

# Participant list

| Company/Organization | Participant Name | Email Address |
| --- | --- | --- |
| Broadcom | Minhua Zhou | minhua@broadcom.com |
| Canon | Chris Rosewarne Christophe Gisquet Guillaume Laroche Patrice Onno | Chris.Rosewarne@cisra.canon.com.au christophe.gisquet@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Huawei | Haoping Yu Meng Xu Wei Wang Zhan Ma | haoping.yu@huawei.com m.xu@huawei.com Wei.Wang.sc@huawei.com Zhan.Ma@huawei.com Zhan.ma@outlook.com |
| InterDigital | Chia-Ming Tsai Xiaoyu Xiu Yan Ye Yuwen He | chia-ming.tsai@interdigital.com xiaoyu.xiu@interdigital.com yan.ye@interdigital.com yuwen.he@interdigital.com |
| ITRI | Chao-Hsiung Hung Ching-Chieh Lin Jih-Sheng Tu Jung-Yang Kao Yao-Jen Chang | chhung@itri.org.tw JackLin.itri@gmail.com sunriseJSTu@itri.org.tw Yang_Kao@itri.org.tw britpablo@itri.org.tw |
| KDDI | Kei Kawamura | ki-kawamura@kddi.com |
| MediaTek | Jing Ye PoLin Lai Shan Liu Shih-Ta Hsiang Tzu-Der Chuang Yi-Wen Chen Yu-Chen Sun Yu-Wen Huang | jing.ye@mediatek.com polin.lai@mediatek.com shan.liu@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yiwen.chen@mediatek.com yc.sun@mediatek.com yuwen.huang@mediatek.com |
| MERL | Robert Cohen Xingyu Zhang | cohen@merl.com xzhang@merl.com |
| Microsoft | Jizheng Xu Xun Guo | jzxu@microsoft.com xunguo@microsoft.com |
| NCTU | Chun-Chi Chen Tsui-Shan Chang Wen-Hsiao Peng | cheerchen.cs98g@g2.nctu.edu.tw abaibai1122.cs01g@nctu.edu.tw wpeng@cs.nctu.edu.tw |
| Nokia | Jani Lainema Kemal Ugur Döne Bugdayci | jani.lainema@nokia.com kemal.ugur@nokia.com ext-done.bugdayci@nokia.com |
| Qualcomm | Feng Zou Joel Sole Krishna Rapaka Marta Karczewicz Rajan Joshi Vadim Seregin Wei Pu | fzou@qti.qualcomm.com joels@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com rajanj@qti.qualcomm.com vseregin@qti.qualcomm.com wpu@qti.qualcomm.com |
| Samsung | Ankur Saxena | a.saxena1@samsung.com saxena.ankur0@gmail.com |
| Sharp | Kiran Misra Seung-Hwan Kim | misrak@sharplabs.com kimse@sharplabs.com |
| Sony | Jun Xu | Jun.Xu@am.sony.com |

# Software

Test C.1 and Test C.3 are to be built on top of SCM-1.0.

The rest tests are to be built on top of SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047.

# Test conditions

All to-be-tested methods shall be tested under the following two test conditions. Each test condition has two mandatory anchors.

## Test condition 1 – IntraBC with full frame search

Anchor 1-1: SCM-1.0 common test condition with intraBC using full frame search + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

Anchor 1-2: SCM-1.0 common test condition with intraBC using full frame search

## Test condition 2 – IntraBC with 2-CTU search

Anchor 2-1: SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU) + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

Anchor 2-2: SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU)

# To-be-tested methods

## Category A – Palette table coding related

### Test A.1 – Triplet palette representation and coding

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Nokia

This test evaluates performance of the triplet palette representation technique proposed in JCTVC-Q0083 by MediaTek. The information of palette composes component-wise palette and triplet palette. The palettes are predictive encoded by a palette copy mode or a palette modification mode in JCTVC-Q0083. In the palette copy mode, the palette will be the same as the predictor. In the palette modification mode, the palette will be derived from the predictor according to the signaled modification information. The palette predictor will be updated according to modification and fully propagated to the next CU.

More details can be found in section 2.1 of JCTVC-Q0083.

### Test A.2 – Palette coding and palette predictor updates

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Canon

This test evaluates performance of the palette predictive coding technique proposed in JCTVC-Q0083 by MediaTek. The palette is predictive encoded by a palette copy mode and a palette modification mode in JCTVC-Q0083. In the palette copy mode, the palette will be the same as the predictor. In the palette modification mode, the palette will be derived from the predictor according to the signaled modification information. The palette predictor will be updated according to modification and fully propagated to the next palette coded CU.

More details can be found in section 2.1.2.3 of JCTVC-Q0083.

### Test A.3 – Palette stuffing

- Proponent: Canon, JCTVC-Q0063

- Crosschecker: InterDigital

JCTVC-Q0063 proposes to combine the last coded palette and the last predictor palette by stuffing unused elements of the last predictor at the end of the last coded palette, using the resulting palette as predictor for the next palette-coded CU, and signalling when there are no predicted elements. This predictor and flags can then be used to determine the current CU palette.

### Test A.4 – Color table processing

- Proponent: Huawei, JCTVC-Q0034/JCTVC-Q0176

- Crosschecker: MediaTek

JCTVC-Q0034/JCTVC-Q0176 proposed to derive color table using packed (interleaved) components together. It includes color table merge from neighboring blocks as implicit signaling method. It further contains the inter table color sharing and intra table color DPCM as explicit signaling mechanism.

More details could be found in section 2.4.1.of JCTVC-Q0034/JCTVC-Q0176.

### Test A.5 – Palette table prediction

- Proponent: InterDigital, JCTVC-Q0037

- Crosschecker: Canon

A representative color dictionary is used to store the representative colors from previously coded palette tables in previous CUs. When coding the current CU, the representative color dictionary is used to predict the representative colors in the current palette table. After the current CU is coded, the representative color dictionary is updated by: 1) adding the representative colors in the current CU into the dictionary, and 2) removing old representative colors from the dictionary adaptively. Additionally, a palette table skip mode is applied to directly re-use the representative color dictionary for the current CU. The signaling of the position of the last predictable palette table entry will be tested.

### Test A.6 – Palette table generation

- Proponent: InterDigital, JCTVC-Q0037

- Crosschecker: Huawei

At the encoder side, an improved palette table generation method is applied to generate the representative colors in the palette table. The proposed palette table generation consists of three main steps: 1) representative color initialization, 2) representative color refinement, and 3) RDO based palette table entry selection. The histogram based method is used to calculate an initial set of M representative colors in the current CU. In the second step, for each quantization zone, the refined representative color is calculated as the weighted average of all the colors within that quantization zone.

### Test A.7 – Encoder-improved palette generation

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047

- Crosschecker: InterDigital

In this test, improvement of palette generation is investigated. The main steps are the use of histogram of the pixel values in a CU to derive the palette, use of rate-distortion checking to prune the palette (decide whether pixels should be coded as escape pixel or as index), use of centroids in deriving the final palette. Results for subsets of these steps will also be provided.

More details can be found in sections 2.2.1 and 2.2.2 of JCTVC-Q0047.

### Test A.8 – Improvements on palette prediction vector signaling

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0225

- Crosschecker: InterDigital

In this test, methods of coding the binary predictor vector are investigated. A hierarchical binary tree data structure is used to code the binary palette predictor vector. Use of contexts for encoding the higher levels of the tree, starting and stopping the coding of the binary tree at various levels in the hierarchy will be evaluated. Sending the number of ones in the binary predictor vector or the total number of palette entries before sending the bits will also be tested. Additionally, these methods will be tested on top of palette stuffing.

More details can be found in section 1.1 of JCTVC-Q0225.

### Test A.9 – Long-term palette prediction

- Proponent: Nokia, JCTVC-Q0071

- Crosschecker: Canon

This test evaluates performance of the long-term palette prediction technique proposed in JCTVC-Q0071 by Nokia. Proposed method signals a long-term palette at the slice header to be used in the coding of the palette of a CU that is coded with palette mode within that slice. Long term palette information signaled at the slice header consists of the size and entries of the long term palette. At CU level, while coding the palette of a CU, for each palette entry that is not predicted from a previous slice, one bit is signaled to indicate whether this entry is copied from the long term palette or explicitly signaled. If the bit indicates the entry is copied from the long term palette, then the index of the entry within the long term palette is signaled.

## Category B – Color index map coding

### Test B.1 – Line-based color index map coding

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Huawei

The color indices of all samples in the CU are coded line by line. For each sample line, an “index line mode” is signaled to indicate the coding mode. The index line mode includes vertical mode, horizontal mode, and normal mode:

- If the vertical mode is selected, the color indices of the current line are copied from the above line.

- Else if the horizontal mode is selected, one color index is signaled and the color indices of all rest samples in the current line are set to the signaled color index.

- Else (the normal mode is selected), the color indices of the line are signaled sample by sample. For each sample, a color prediction index is signaled to specify how to predict the color index, and a color index of the sample is transmitted directly when the color prediction index indicates no prediction.

The index coding in normal mode includes the methods described in Test B.2, Test B3, and Test B.4 in line-based color index map coding, as shown in JCTVC-Q0083, while Test B.2, Test B3, and Test B4 are in run-based color index map coding.

More details can be found in section 2.2 of JCTVC-Q0083.

### Test B.2 – Transition copy mode

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Huawei

Transition copy (TC) candidates are considered as one of the prediction modes in the color index map coding. In each CU, a TC table inherits the TC table of the previous CU. The TC table is maintained as the samples in the CU are coded. The input of the TC table is called a pilot color index and is set to the color index of the left sample, and the outputs of the TC table are TC color indices, where the indices are likely to occur right after an occurrence of the pilot color index. The color indices in TC table are updated according to coded samples.

More details can be found in section 2.2.2.2 of JCTVC-Q0083.

### Test B.3 – Candidate-based color index map coding

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Qualcomm

A candidate list which includes copy-above, copy-left, copy-above-left, copy-above-right, copy-above-above (indices from neighboring samples), and transition-copy, is first generated. The redundant candidates in the list are removed. A candidate index is signaled to specify which candidate is used. If a new color index is signaled, the redundant index values from predictors are removed.

More details can be found in section 2.2.2 of JCTVC-Q0083.

### Test B.4 – Adaptive color index map scan

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Microsoft

This test evaluates performance of the adaptive color index map scan technique proposed in JCTVC-Q0083 by MediaTek. For a palette coded CU, if the proposed tool is signaled to be enabled, the sample position axes for the color index map will be swapped before being encoded.

More details can be found in section 2.2.3 of JCTVC-Q0083.

### Test B.5 – Palette index coding

- Proponent: Canon, JCTVC-Q0064

- Crosschecker: Qualcomm

In JCTVC-Q0064 it is proposed to use two code lengths depending on the index, as well as removing impossible values from the set instead of using FLC as in AHG10 software.

### Test B.6 – Transition copy mode

- Proponent: Canon, JCTVC-Q0065

- Crosschecker: ITRI

In JCTVC-Q0065 it is proposed a new index prediction method using the position of so-called transitions to determine a position from which the prediction copies indexes. This contribution presents a small modification to the update mechanism of the initial proposition JCTVC-P0115, and also describes a simpler prediction method that encodes a run of values being the expected transition. This test will evaluate both prediction methods.

### Test B.7 – Run coding

- Proponent: Canon, JCTVC-Q0066

- Crosschecker: InterDigital

In JCTVC-Q0066 it is proposed to limit the number of runs coded after a palette index for the run mode. A syntax element is transmitted for each palette CU to determine when not to decode the run.

### Test B. 8 – Color index map processing

- Proponent: Huawei, JCTVC-Q0034/JCTVC-Q0176

- Crosschecker: Qualcomm

This test evaluates the color index map coding proposed in JCTVC Q0034/Q0176 using 1D and 2D string match based index compression.

More details could be found in section 2.4.2 of JCTVC Q0034/JCTVC-Q00176.

### Test B.9 – BWT-based index grouping

- Proponent: InterDigital, JCTVC-Q0037

- Crosschecker: Canon

The Burrows–Wheeler transform (BWT) is used to increase the correlation between adjacent positions in the palette index map. The BWT is applied before encoding the palette index map. After scanning each position as a 1-D index sequence, this sequence is fed into the BWT process and the end position is also coded as BWT side-information (overhead) for decoder side reconstruction. Due to the side information needed for BWT reconstruction, a 1-bit flag for each CU is used to indicate if the BWT is performed or not. The BWT with adaptive scanning methods will be tested to get best performance.

### Test B.10 – Palette index value mapping

- Proponent: InterDigital, JCTVC-Q0037

- Crosschecker: Qualcomm

To reduce the magnitude of the coded palette index value, the palette index value is mapped before being coded. The proposed palette index value mapping consists of two main steps: 1) reduce palette index value by comparing to the last compared palette index (LCPI), and 2) mapping of escape color index. In the first step, the palette index value is reduced by 1 if the palette index value is larger than its LCPI. In the second step, the index value of escape color pixels is set to a smaller index value, and the other entries in the palette table are shifted backward by 1 position. In addition, different escape color representation method will be tested.

### Test B.11 – Escape color prediction

- Proponent: InterDigital, JCTVC-Q0037

- Crosschecker: Canon

Escape color prediction method is used to code the escape colors efficiently. A flag is signaled to indicate if every color component of the current escape color pixel is the same as the previous escape color pixel or not. And if not (flag is equal to 0), the current escape color value is predicted from an entry in the palette table. The index value of the table entry used for prediction, along with the prediction difference (including absolute difference and the sign bit), are signaled to the decoder.

### Test B.12 – Binarization of escape sample and palette index

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047

- Crosschecker: Canon

Alternatives to fixed length binarization of escape pixel and index coding are tested in this experiment. Variable length binarization (e.g. truncated binary code) will be applied for coding of escape pixel and palette index coding. Index adjustment as described in JCTVC-Q0047 will also be tested.

More details can be found in section 2.3 of JCTVC-Q0047.

### Test B.13 – Flag to indicate escape and index coding

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047

- Crosschecker: Microsoft

Instead of assigning an index to indicate escape pixel, a flag is used to indicate whether the current pixel is an escape pixel or index coded. Modifications to the signaling scheme to accommodate this flag will also be tested.

More details can be found in section 2.3 of JCTVC-Q0047.

### Test B.14 – Restricted run coding

- Proponent: Nokia, JCTVC-Q0071

- Crosschecker: KDDI

This test evaluates performance of the run mode restriction technique proposed in JCTVC-Q0071 by Nokia. Proposed method signals which palette indexes have “palette_run” associated with them in “run mode” and which don’t. For the palette indexes for which the palette_run is not signaled those are assumed to be 0. This avoids signaling unnecessary run-length information for “stand-alone” samples that are not repeating consequently within the block. A single CU level syntax element is included in the bitstream to indicate the maximum palette index for which the “palette_run” is indicated.

### Test B.15 – Scanning indicator for the palette coded CUs

- Proponent: Nokia, JCTVC-Q0071

- Crosschecker: KDDI

This test evaluates performance of the palette CU scanning technique proposed in JCTVC-Q0071 by Nokia. Proposed method signals whether the palette coded CU is scanned in horizontal or vertical scan order. Proposed method does not change the coding/decoding of the remaining syntax elements of the palette coded CU.

## Category C – Combination of palette coding and color index map coding

### Test C.1 – Combination of Test A.1 and Test B.1

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Canon

This test combines Test A.1 and Test B.1, which cover the triplet palette representation and coding and the line-based color index map coding.

More details can be found in the entire JCTVC-Q0083.

### Test C.2 – Combination of palette coding tools

- Proponent: Canon, JCTVC-Q0069

- Crosschecker: MediaTek

This test includes JCTVC-Q0063, JCTVC-Q0064, JCTVC-Q0065, JCTVC-Q0066, and encoding choices. It is related to the combinations of the aforementioned tools. Encoding choices will incorporate further improvements related to said combinations, as well as palette derivation and palettization process improvements.

### Test C.3 – Combination of Test A.4 and Test B.8

- Proponent: Huawei, JCTVC-Q0034/JCTVC-Q0176

- Crosschecker: MediaTek

This test combines aforementioned tools in Test A4 and Test B8, which covers color table processing using packed components and string match based index map coding.

More details could be found in JCTVC-Q0034/Q0176.

### Test C.4 – Combination of Tests A.3, Test A.7, Test A.8, Test B.12, and Test B.13

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047/JCTVC-Q0225

- Crosschecker: MediaTek

A combination of palette stuffing (A.3), encoder palette improvements (Test A.7), binary prediction vector signaling (Test A.8), truncated binary coding of indices (Test B.12), and a flag to indicate escape pixel or index value (Test B.13) will be tested. Combinations of subsets of these methods will also be tested.

## Category D – Single color modes

### Test D.1 – Single color mode

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0093

- Crosschecker: MERL

In the proposed scheme in JCTVC-Q0093, one flag is signaled in the slice header to enable the single color mode for the entire slice. When the slice-level flag is enabled, one CU-level flag is further signaled to indicate the enabling of the single color mode for coding each CU.

To reconstruct a CU coded as single color mode, a sample color candidate list is first constructed by using the spatial neighboring samples of current CU in a predefined order into the candidate list. A redundancy pruning process is applied to the construction of sample candidate list. After the pruning process, if empty entry exists, a default candidate (e.g. value equal to 1<<(g_bitDepth-1)) will be used to fill in those empty entries. After the sample candidate list is constructed, a sample candidate index is signaled in the bitstream to indicate which sample candidate is used to fill the current CU.

More details can be found in the entire JCTVC-Q0093.

### Test D.2 – Independent uniform prediction

- Proponent: MERL, JCTVC-Q0036

- Crosschecker: MediaTek

In Independent Uniform Prediction (IUP), a small set of color triplets is available to each CU for generating a uniform prediction block for each color component, where all pixels in the predictor block have the same value as the corresponding element in the color triplet selected from the set. This predictor block is then treated as in other prediction methods, where the predictor is added to the reconstructed residual block to form the reconstructed block in the decoder.

More details can be found in the Independent Uniform Prediction (IUP) portion of JCTVC-Q0036.

# Timeline

T1: Release of SCM-1.0

T2: Release of SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047, T2 = T1 + 1 week

T3: Release of to-be-tested software, T3 = T2 + 3 weeks

T4: Release of all simulation results including crosscheck, T4 = T3 + 2 weeks

T5: Document upload deadline, Friday, June 20, 2014

Note: The 18th JCT-VC meeting will be in Sapporo, Japan, starting at 3:45pm, Monday, June 30, 2014 and ending by lunch time on Wednesday, July 9, 2014.

# References

- X. Guo, Y. Lu, S. Li, “RCE4: Test1. Major-color-based screen content coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P0108, 16th Meeting: San José, US, January 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, J. Xu, “Suggested software for the AHG on investigation of palette mode coding tools,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P0303, 16th Meeting: San José, US, January 2014.

- J. Chen, Y. Chen, T. Hsieh, R. Joshi, M. Karczewicz, W.-S. Kim, X. Li, C. Pang, W. Pu, K. Rapaka, J. Sole, L. Zhang, F. Zou, “Description of screen content coding technology proposal by Qualcomm,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0031, 17th Meeting: Valencia, ES, March-April 2014.

- JCTVC-Q0034, 17th Meeting: Valencia, ES, March-April 2014.

- P. Lai, T.-D. Chuang, Y.-C. Sun, X. Xu, J. Ye, S.-T. Hsiang, Y.-W. Chen, K. Zhang, X. Zhang, S. Liu, Y.-W. Huang, S. Lei, “Description of screen content coding technology proposal by MediaTek,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0033, 17th Meeting: Valencia, ES, March-April 2014.

- JCTVC-Q0034, 17th Meeting: Valencia, ES, March-April 2014.

- R. Cohen, A. Minezawa, X. Zhang, K. Miyazawa, A. Vetro, S. Sekiguchi, K. Sugimoto, T. Murakami, “Description of screen content coding technology proposal by Mitsubishi Electric Corporation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0036, 17th Meeting: Valencia, ES, March-April 2014.

- X. Xiu, C.-M. Tsai, Y. He, Y. Ye, “Description of screen content coding technology proposal by InterDigital,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0037, 17th Meeting: Valencia, ES, March-April 2014.

- W. Pu, F. Zou, R. Joshi, M. Karczewicz, J. Sole, “AHG10: Simplification of palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0047, 17th Meeting: Valencia, ES, March-April 2014.

- C. Gisquet, G. Laroche, P. Onno, “AhG10: Palette predictor stuffing,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0063, 17th Meeting: Valencia, ES, March-April 2014.

- C. Gisquet, G. Laroche, P. Onno, “AhG10: Palette index coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0064, 17th Meeting: Valencia, ES, March-April 2014.

- C. Gisquet, G. Laroche, P. Onno, “AhG10: Transition copy mode for palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0065, 17th Meeting: Valencia, ES, March-April 2014.

- G. Laroche, T. Poirier, C. Gisquet, P. Onno, “AhG10: Run coding for palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0066, 17th Meeting: Valencia, ES, March-April 2014.

- C. Gisquet, G. Laroche, P. Onno, “AhG10: Combination of Palette coding tools,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0069, 17th Meeting: Valencia, ES, March-April 2014.

- D. Bugdayci, J. Lainema, K. Ugur, M.M. Hannuksela, “AHG10: Improvements on palette coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0071, 17th Meeting: Valencia, ES, March-April 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, J. Xu, “AHG10: Suggested software for palette coding based on RExt6.0,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0094, 17th Meeting: Valencia, ES, March-April 2014.

- Y.-C. Sun, T.-D. Chuang, P. Lai, S.-T. Hsiang, Y.-W. Chen, X. Zhang, S. Liu, Y.-W. Huang, S. Lei, “AHG10: A triplet palette mode combining JCTVC-P0108 and JCTVC-P0198,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0083, 17th Meeting: Valencia, ES, March-April 2014.

- Y.-W. Chen, Y.-C. Sun, Y.-W. Huang, S. Lei, “Single color intra mode for screen content coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0093, 17th Meeting: Valencia, ES, March-April 2014.

- W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu, “AHG8: String match in coding of screen content,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0176, 17th Meeting: Valencia, ES, March-April 2014.

- W. Pu, F. Zou, R. Joshi, M. Karczewicz, J. Sole, “Improvements to palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0225, 17th Meeting: Valencia, ES, March-April 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1015, 17th Meeting: Valencia, ES, March-April 2014.