| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0004 |
| --- | --- |

| Title: | HEVC conformance test development (AHG4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Jill Boyce Kimihiko Kazui Adarsh K. Ramasubramonian Yan Ye | Email: | teruhikos@jp.sony.com jill@vidyo.com kazui.kimihiko@jp.fujitsu.com aramasub@qti.qualcomm.com Yan.Ye@InterDigital.com |
| Source: | AHG on HEVC conformance development |  |  |

_____________________________

# Abstract

# Introduction

The mandates of this AHG are:

- Study the requirements of HEVC conformance testing to ensure interoperability.

- Discuss the work plan needed to develop HEVC v.1, RExt and SHVC conformance testing.

- Study potential testing methodology to fulfil the requirements of HEVC conformance testing.

- Establish and coordinate bitstream exchange activities for HEVC.

- Identify needs for HEVC conformance bitstreams with particular characteristics.

- Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set.

# Activities

## Reflector

The e-mail reflector is [jct-vc@lists.rwth-aachen.de].  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG4] in the subject line for easy grouping.

## ftp site

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

## Bitstream generation guideline

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

## HEVC v.1 conformance

After the Valencia meeting, the following problems were reported. The status of all bitstreams is summarized in the attached Excel sheet and please check column ED. (JCTVC-R0004_HEVCv1).

### Annex C problem

Annex C problem is reported in the following bitstream. This was fixed by HRD_A_Fujitsu_3.

HRD_A_Fujitsu_2.:  Error :[SPS](A.4.1)NAL HRD: At least one CpbSize[i] shall be less than or equal to CpbBrNalFactor*MaxCPB.

### Annex B problem

Annex B problem was reported in the following bitstream

RAP_A_docomo_5  (This was fixed by RAP_A_docomo_6)
VPSID_A_VIDYO_1  (not fixed yet)

The zero_byte is not presenting in the necessary position in the bitstream. Annex B specifies as follows.

zero_byte is a single byte equal to 0x00.

When one or more of the following conditions are true, the zero_byte syntax element shall be present:

–	The nal_unit_type within the nal_unit( ) syntax structure is equal to VPS_NUT, SPS_NUT or PPS_NUT.

–	The byte stream NAL unit syntax structure contains the first NAL unit of an access unit in decoding order, as specified in subclause 7.4.2.4.4.

### Max bit per LCU

The maximum number of bits per LCU is constraint by Annex A.3.2 as follows.

"The number of times read_bits( 1 ) is called in subclauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3."

The following bitstream exceeds this limit.

SDH_A_Orange_3

This bitstream must be revised.

### vps/sps_extension_flag

In the following bitstream, the value of vps_extension_flag is equal to 1.

PS_A_VIDYO_3.bit

In the following bitstream, the value of sps_extension_flag is equal to 1.

PS_B_VIDYO_3

EXT_A_ericsson_4

The intension is to check if decoder can ignore the extension flag. But it is better to confirm if this is allowed in conformance test set.

### general_profile_compatibility_flag problem

In the following bitstreams, values of general_profile_compatibility_flag are not correct. The values in the current bitstreams are summarized in the attached Excel sheet. (JCTVC-R004_compatibility_flag)

AMP_B_Samsung_4.bit

AMVP_C_Samsung_4.bit

SAO_C_Samsung_4.bin

SAO_D_Samsung_4.bin

STRUCT_A_Samsung_5.bin

STRUCT_B_Samsung_5.bin

SLPPLP_A_VIDYO_1.bin

### slice_segment_header_extension_present_flag

The spec specifies as follows:

"slice_segment_header_extension_present_flag shall be equal to 0"

However this value is intentionally set equal to 1 to test slice segment header extension. The intention is to check if a decoder can ignore the extension flag. But it is better to confirm if this is allowed in conformance test set.

WPP_A_Ericsson_MAIN_2

WPP_B_Ericsson_MAIN_2

WPP_C_Ericsson_MAIN_2

WPP_D_Ericsson_MAIN_2

WPP_E_Ericsson_MAIN_2

WPP_F_Ericsson_MAIN_2

WPP_A_Ericsson_MAIN10_2

WPP_B_Ericsson_MAIN10_2

WPP_C_Ericsson_MAIN10_2

WPP_D_Ericsson_MAIN10_2

WPP_E_Ericsson_MAIN10_2

WPP_F_Ericsson_MAIN10_2

### md5

md5 in RAP_B_Bossen_1 is not correct. md5 must be revised. This was fixed by RAP_B_Bossen_2.

### Originally planned bitstream features

The following bitstreams were originally planned to generate and the yellow highlighted bitstreams are not generated yet.

Table: Candidates of HEVC conformance test features

|  | Categories | Sub categories | Feature Code | Bitstream features | Volunteers |
| --- | --- | --- | --- | --- | --- |
|  | Intra coding | Intra prediction | IPRED | Functional stage: Decoding of intra PU. Purpose: Check that the decoder can properly decode all values of intraPredMode for different Luma and Chroma transform block sizes. | C.S. Lim, F. Bossen, J. Lainema, K. Sugimoto |
|  |  | Constraint intra prediction | CIP | Functional stage: Decoding of intra PU in non-intra slice when constrained_intra_pred_flag is set to one Purpose: Check that the decoder can properly perform reference sample substitution process. | C.S. Lim, K. Chono |
|  | Inter frame coding | AMVP | AMVP | Check whether motion vector prediction candidate generation and signalling perform correctly. Check the process when neighboring PUs have various partition mode, prediction mode, reference index, inter_pred_idc, etc.. Related syntax element: mvp_l0_flag, mvp_l1_flag | C. Kim, M. Zhou, Y. -W. Huang |
|  |  | MVs | MV | maximum number of motion vectors permitted per CTU |  |
|  |  |  | MV | vectors reference padded picture edges. | M. Coban |
|  |  |  | MV | clipping of motion vector range in MVP/Merge candidates | M. Coban |
|  |  |  | MV | motion vectors are random, maximum page DRAM misses / minimize cache hits (according to memory bandwidth model... e.g. JCTVC-H007) |  |
|  |  | Merge | MERGE | Merge candidate list index signaling: Check that merge candidate list can deal with every merge index (0..4) | B. Bross |
|  |  |  | MERGE | Parallel merge: test all the possible combinations of log2_parallel_merge_level_minus2 and CTB sizes. | M. Zhou, Y. -W. Huang |
|  |  |  | MERGE | number of maximum merge candidates signalling: test all the values of five_minus_max_merge_cand, i.e. 0, 1, 2, 3, 4 for CTB size of 64x64 | M. Zhou, |
|  |  | WP | WP | weighted_pred_flag and/or weighted_bipred_flag are equal to 1. | T. Chujoh |
|  |  |  | WP | various weighted prediction parameter combinations | T. Chujoh |
|  |  | mvd_l1_zero_flag | MVD | Test mvd_l1_zero_flag | F. Bossen |
|  |  | temporal_mvp | TMVP | Test slice_temporal_mvp_enable_flag | J. Xu |
|  |  | Collocated ref idx | RIDX | Test collocated_ref_idx (single slice or multiple slice/picture with different reference lists.) |  |
|  | HL syntax | slices, PSs | PS | bitstreams with stuffed extensions (vps_extension_data_flag=1, sps_extension_data_flag=1, pps_extension_flag=1, ..) | W. Jang, M. Coban |
|  |  | dependent slice | DSLICE | Check: - dependent slices - WPP with dependent slices - tiles with dependent slices | B. Bross |
|  |  | POC | POC | POC increments > 1 | M. Coban |
|  |  | RPS | RPS |  | M. Coban |
|  |  |  | RPS |  | F. Bossen |
|  |  |  | RPS | Slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Slice header RPS with short-term and long-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures and inter-RPS prediction | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-and long-term pictures | R. Sjöberg |
|  |  | RPL | RPL |  | M. Coban |
|  |  | RAP | RAP | all 6 (currently identified) RAP types | F. Bossen |
|  |  | HRD | HRD | maximum number of pictures in DPB | TBD |
|  |  |  | HRD | bitstream with largest possible CPB delay (maximum number of coded bits per picture but within MinCR) | TBD |
|  |  |  | HRD | bumping DPB | R. Sjöberg |
|  |  |  | HRD | Low delay, Sub picture | K. Kazui |
|  | Transform | RQT | RQT | Check all depths of inter & intra transform tree: QuadtreeTUMaxDepthInter : 0..4 QuadtreeTUMaxDepthIntra : 0..4 | B. Bross |
|  | Quantization | dQP | DQP | all depths of CU and diff_cu_qp_delta_depth | W. Wan, T. Suzuki |
|  |  | Scaling list | SLIST | non-default scaling lists | T. Suzuki, M. Zhou |
|  | Other coding tools | AMP | AMP | Check all asymetric partition mode enable | X. Zheng, C. Kim |
|  |  | Transform skip | TSKIP |  | J. Xu |
|  |  | I_PCM | IPCM | Functional stage: Parsing of I_PCM size information at SPS and I_PCM flag at CU. Purpose: Check that the decoder can properly decode pcm_flag for different intra CU sizes. Related Syntax: log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_flag. | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Parsing and decoding of I_PCM CUs. Purpose: Check that the decoder can properly decode I_PCM CUs with different PCM bit depth. Related syntax: pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, pcm_flag | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Decoding of I_PCM CUs with loop filter skipping. Purpose: Check that the decoder can properly decode I_PCM CUs by skipping loop filter. Related syntax: pcm_loop_filter_disable_flag, pcm_flag. Note: Combination with cu_transquant_bypass_flag will be tested. | W. Wan, C.S. Lim, K. Chono |
|  |  | Lossless | LOSSLESS | transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles lossless coding. | F. Henry |
|  | Parallel processing tools | Tile | TILE | Maximum number of tiles in a random, non-uniform arrangement that changes from picture to picture, with random chosen value for loop_filter_across_tiles_enabled_flag, deblocking_filter_override_flag, slice_disable_deblocking_filter_flag, slice_loop_filter_across_slices_enabled_flag | A. Fuldseth, M. Zhou |
|  |  | WPP | WPP | entropy_coding_sync_enabled_flag on. Purpose: Check that the decoder handles wavefront parallel processing | F. Henry |
|  |  | WPP and slices | WPP | entropy_coding_sync_enabled_flag on, number of slice varies randomly between 2 and maximum number of slices permitted, each non-first slice is either regular or dependent in a random way. Purpose: Check that the decoder handles wavefront parallel processing and slices | F. Henry |
|  |  | Tile & WPP | WPP | entry point of WPP | Hendry |
|  | Temporal scalability |  | TSCL |  | W. Jang |
|  |  |  | TSCL | range of nested temporal_id's |  |
|  | Deblocking filter |  | DBLK | various cb, cr qp_offsets | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | various beta_offset_div2, tc_offset_div2 | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | trigger all deblocking decisions | T.Suzuki, R. Sjöberg |
|  |  |  | DBLK | deblocking + tile, etc |  |
|  | SAO |  | SAO | Check all CTU sizes with max. SAO information rate. | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random and extreme SAO offsets values | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random SAO merge left/up flag values | E. Francois, Y. -W. Huang |
|  |  | Slice- and component-level control | SAO | Pictures divided into x slices / randomly enabling SAO Y and/or SAO UV per slice | E. Francois |
|  |  | Tile- and component-level control | SAO | Pictures divided into x tiles / randomly enabling SAO Y and/or SAO UV per tile | E. Francois |
|  | Entropy coding |  |  |  |  |
|  |  | Sign data hiding | SDH | sign_data_hiding_flag on, transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles sign data hiding with lossless coding. | H. Henry |
|  |  | High through put binalizaion | HTB |  | M. Coban |
|  |  | cabac_init_idc | CAINIT |  | A. Segall |
|  |  | Max number of bins | MAXBIN |  | V. Sze |
| Corner case bitstream |  |  |  | Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics | C. Fogg |
|  |  |  |  | CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012) | C. Fogg |
|  |  |  |  | A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset | C. Fogg |
|  |  |  |  | inferred splits along non-CTU multiple picture borders |  |
|  |  |  |  | combinations of transform_skip_enabled_flag, transquant_bypass_enable_flag, cu_transquant_bypass_flag |  |
|  |  |  |  | something to test the vast universe that is: POC signalling |  |
|  | Video resolution |  |  | pic_width_in_luma_samples = pic_height_in_luma_samples = (1 << Log2MinCbSize), with various CTU sizes. |  |

## RExt conformance

### Initial plan

- Bit depth & chroma format to cover in RExt conformance are as follows.

- 4:0:0, 4:2:0, 4:2:2, 4:4:4

- 8, 10, 12, 16b

Profiles in RExt are as follows.

Monochrome 12, Monochrome 16, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 10 Intra, Main 4:4:4 12 Intra, and Main 4:4:4 16 Intra profiles

- The plan is to cover all profiles. It is not necessary to re-test HEVC v1 features, but new features must be checked.

The initial idea of bitstream features is summarized in the following Table.

Table: Candidate features of RExt conformance

| Chroma format | Bit depth | Category | Sub category | Bitstream | Volunteers | Candidates |
| --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 | 12 |  |  |  |  |  |
| 4:2:2 | 10 | CU/TU/PU |  |  |  |  |
| 4:2:2, 4:4:4 |  |  | CBF |  |  | Qualcomm/Canon/Various? |
| 4:2:2, 4:4:4 |  | Intra coding | Intra smoothing disable |  | Fujitsu | Fujitsu ? |
| 4:2:2 |  |  | Adjustment to intra chroma prediction angle | ADJUST_IPRED_ANGLE_A_RExt_Mitsubishi_1 | A.Minezawa | Mitsubishi |
| 4:2:2, 4:4:4 |  | Inter frame coding | WP (high_precision_offsets_enabled_flag) |  |  | Toshiba ? |
| 4:4:4 |  |  | Cross component prediction |  | Qualcomm | HHI/Qualcomm |
| 4:4:4 | L/C 10/8 8/10 |  | Different bit depth for luma and chroma | Bitdepth_A_RExt_Sony_1 Bitdepth_B_RExt_Sony_1 | Sony |  |
|  |  | Transform |  |  | Sony |  |
|  |  | Quantization | Scaling list |  | Sony | Sony ? |
|  |  |  | Chroma_qp_adjustment |  | Black Berry | Black Berry ? Apple ? |
|  |  | Loop filter | Deblocking |  |  | ? |
|  |  |  | SAO | SAO_A_RExt_MediaTek_1 | Media Tek | Samsung ? Media Tek ? |
|  |  | Entropy coding | Rice parameter initialization |  | Qualcomm | Qualcomm, Sony ? |
|  |  |  | Persistent Rice parameter tool | PERSIST_RPARAM_A_RExt_Sony_1 | Sony |  |
|  |  |  | CABAC bypass alignment |  | Sony | Qaulcomm ? Sony ? |
| 4:4:4 | 16 | Precision | Extended precision |  | Sony |  |
|  |  | HL syntax |  |  |  |  |
| 4:2:2, 4:4:4 | 10, 12, 16 | Others | PCM | IPCM_A_RExt_NEC IPCM_B_RExt_NEC | NEC | NEC |
| 4:4:4 | 8, 10, 12, 16 |  | Transform skip rotation |  |  | Microsoft, Qualcomm (N0288) |
| 4:4:4 | 8, 10, 12, 16 |  | Transform skip context |  | Sharp | Qualcomm (N0044) Sharp |
| 4:4:4 | 8, 10, 12, 16 |  | RDPCM (implicit & explicit) |  | Samsung BBC(explicit) | Media Tek (P0154) |
| 4:4:4 | 8, 10, 12, 16 |  | Max transform skip block size |  | BBC | Many. Perhaps BBC? |
| 4:2:2 | 10 |  | Various combination of tools/parameters | Main_422_10_A_RExt_Sony_1 Main_422_10_B_RExt_Sony_1 | Sony |  |
| 4:4:4 | 8/16 |  | Various combination of tools/parameters |  | Sony |  |
| 4:0:0 | 8/16 |  |  |  |  |  |

Basic features can be combines into bitstreams to reduce the number of conformance bitstreams. Volunteers identified so far are as follows.

- O. Nakagami (Sony)

- D. Flynn (RIM)

- K. Sharman (Sony)

- K. Kazui (Fujitsu)

- D. Nicholson (VITEC)

- A. Minezawa (Mitsubishi)

- K. Chono (NEC)

- Yuwen (yuwen.huang@mediatek.com) (MediaTek)

- M. Naccari (BBC)

- J. Sole (Qualcomm)

- P. Onno (Canon)

- E. Alshina (Samsung)

- K. Tsukuba (Sharp)

### Plan

#### Procedure

- Same as HEVC v.1 conformance

- See bitstream generation guideline in JCTVC-O1010

#### Time line

- When should we start ?

  - Start the first exchange starting now and collect initial bitstreams by July

- Probably issue PDAM of conformance in July and then revise the bitstreams

#### Consideration

Consider to remove the tool from the text if no conformance stream, same as HEVC v.1

## SHVC conformance

A preliminary list of SHVC conformance bitstreams to be prepared has been identified. It is included in an attached spreadsheet.

So far, 7 companies have volunteered to participate in generation of SHVC conformance bitstreams: ETRI, Fraunhofer HHI, Nokia, Qualcomm, Sony, Technicolor, and Vidyo. Additional companies are requested to volunteer.

It is recommended that at this meeting the preliminary list of conformance bitstreams be reviewed and volunteers identified for each bitstream.

### Conformance point

In SHVC conformance, it is not clear the conformance point which is tested by conformance testing. For example, the output of SHVC bitstreams could be pictures from multiple layers (i.e. an output layer set could have multiple output layers). The question was raised if the volunteers provide md5, yuv (optional), trc (optional) files separately for each layer.

Guideline for SHVC bitstream generation should be clarified and the guideline of the bitstream generation, JCTVC-O1010, should be revised if necessary.

# Recommendations

The AHG recommends

- To finalize HEVC v.1 conformance at this meeting

- to discuss the further plan of HEVC extensions conformance test

- to update the guideline of the bitstream generation for RExt and SHVC conformance