| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0006 |
| --- | --- |

| Title: | JCT-VC AHG report: Range extensions draft text (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole (Qualcomm) David Flynn (Blackberry) Matteo Naccari (BBC) Chris Rosewarne (Canon) Karl Sharman (Sony) Gary Sullivan (Microsoft) Teruhiko Suzuki (Sony) | Email: | joels@qti.qualcomm.com dflynn@blackberry.com matteo.naccari@bbc.co.uk chris.rosewarne@cisra.canon.com.au karl.sharman@eu.sony.com garysull@microsoft.com teruhikos@jp.sony.com |
| Source: | AhG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on Range extensions draft text (AHG6) between the 17th JCT-VC meeting in Valencia, ES (March-April 2014) and the 18th JCT-VC meeting in Sapporo, JP (June-July 2014).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-O1005 HEVC RExt draft text. - Gather and address comments for refinement of the text. - Coordinate with AHG7 on range extensions software development to address any identified issues regarding text and software relationship. | J. Sole (primary) D. Flynn (co-chairs) M. Naccari C. Rosewarne K. Sharman G. Sullivan T. Suzuki | No |

# Background

The High Efficiency Video Coding (HEVC) Range Extensions test model was developed and finalized following the decisions taken at the 17h JCT-VC meeting in Valencia, ES (March-April 2014).

# Status

Nine versions of JCTVC-Q1005 were published by the Editing AhG following the 17th JCT-VC meeting in Valencia, which were based upon JCTVC-L1003_v34 and JCTVC-Q1003_v1. First 7 drafts were based upon JCTVC-L1003_v34. The text was rebased upon Q1003_v1 in the last two versions. The text of JCTVC-Q1005 (revision 9) was submitted to ISO/IEC JTC1/SC29 for Final Draft International Standard.

Changes in JCTVC-Q1005 Draft 7 relative to the previous version and based upon JCTVC-L1003_v34 are follows:

- Replaced table A–3 with reference to table A–2

- Updated version information

- Updated unit definitions

- Renamed fast_rice_adaptation_enabled_flag to persistent_rice_adaptation_enabled_flag

- Editorial update to rice adaptation, chroma qp offset lists, chroma deblocking process, limited EGk process, VCL/NAL bit rate scale factors, time code SEI, mastering display colour volume SEI, VUI

- Initial integration of segmented rectangular FPA SEI message [JCTVC-Q0256]

- Added missing chroma cbf contexts [#1256]

- Updated context tables to reflect software values [mtgNotes]

- Removed no-longer used intra slice contexts for explicit rdpcm

- Added Digital Cinema EOTF to VUI (JCTVC-Q0084)

- Change to tabular form for bitstream constraints for profiles and refined related aspects.

- Changed method of specifying new constraint flags in profile_tier_level( ) syntax (only editorially) to clarify the conditions under which they have specified values.

- Removed collective names for subsets of range extensions profiles, since they didn't prove very useful after being added in the previous version. Introduced "format" into collective name for all range extensions profiles for improved clarity of the description of these profiles, since all of these profiles differ in some aspect of basic format from the previously specified profiles.

- Renamed syntax elements for CU-level chroma QP adjustment and refined semantics.

- Editorial refinement of mastering display colour volume SEI message.

- Added general_one_picture_only_constraint_flag to identify range extensions still picture profiles

- Corrected a few details of specifications for Main 4:4:4 and Main 4:4:4 Intra profiles.

- Changed to tabular form for specification of decoder conformance to range extensions profiles.

- Changed to tabular form for specification of bitstream indications for conformance to range extensions profiles.

- Removed the definition of Extra High tier (and its associated TierValue and general_high_tier_flag).

- Introduced collective names for range extensions profiles and renamed SPS and PPS syntax flags accordingly

- Initial integration of limited EGk binarization process [JCTVC-Q0073]

- Initial fixes for chroma cbf for 4:2:2 and 4:4:4, needs checking [#1225, #1252, #1256]

- Integrated update to mastering display colour volume SEI message [JCTVC-Q0086]

- Typographical fixes

- Removed redundent condition in default weighted sample prediction process [#1272]

- Fixed usage of sps_extension_{flag[ 0 ], 7bits} and pps_extension_{flag[ 0 ], 7bits} [mtgNotes]

- Removed profile constraints on SPS and PPS extension flags [mtgNotes]

- Fixed misapplication of strong intra smoothing for 32x32 chroma blocks [JCTVC-Q0128]

- Renumbered payloadType for the time code and mastering display colour volume SEI messages

- Removed motion the constrained tile sets, knee function, and chroma resampling filter SEI messages [mtgNotes]

- Removed intraBC [mtgNotes]

Draft 7 (JCTVC-Q1005) of High efficiency video coding range extensions, based upon JCTVC-Q1003_v1

- Improvement of uniformity of vertical spacing in syntax tables

- Clarification and cross-reference fixes for binarization process (9.3.3 and its subclauses)

- Update unavailable picture generation for monochrome case §8.3.3.2 [#1138]

- Fixed incorrect derivation of scanIdx for 4:4:4 in §7.4.9.11 [#1294]

- Fixed incorrect luma qualification of prediction block size in §8.5.3.3.1, §8.5.3.3.4

- Removed unnecessary cbf_cb/cbf_cr inference rules

- Removal of application nickname and changed 3-digit approximation 0.333 to 1 ÷ 3 in colour_primaries table

- Clarification of clock time stamp SEI message semantics

- Fixed use incorrect reference to sps_extension_flag and pps_extension_flag instead of sps_range_extensions_flag and pps_range_extensions_flag

- Fixed use of initial_alt_cpb_removal_delay[ i ] and initial_alt_cpb_removal_offset[ i ]

- Minor clarification of conformance check in Annex A

- Fixed missing derivation of IntraPredModeC in §8.4.1 for 4:4:4 with IntraSplitFlag=1

- Updated Figure 9–4

- Editorial fixes for spelling and typesetting

- Fixed ordering of initValues for sig_coeff_flag

- Delete spurious intraBC contexts from part_mode table

- Updated field numbering (may need update again because of next item) and fixed some cross-references

- Restructured SPS extension and PPS extension syntax and semantics subclauses to avoid non-RBSP headings at top level of RBSP definition subclause

- Renamed syntax element string "sub_set" as "subset" (already being changed for other reasons)

- Added figure for segmented rectangular SEI message

- Fine-tuned table specifications for new EOTFs

- Checked and fixed bold []

- Updated and fixed field numbering

- Rebased Q1005_v5 text against Q1003_v1

# Recommendations

The recommendations of the HEVC Range extensions draft text AHG are to:

- Approve the edited JCTVC-Q1005 document as JCT-VC output

- Encourage the use of the issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc/) to facilitate the reporting of issues with the text

- Resolve all text issues logged in trac

- Compare the HEVC Range Extensions document with the HEVC Range Extensions software and resolve any discrepancies that may exist, in collaboration with the Range extensions software development (AHG7)