| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0009 |
| --- | --- |

| Title: | JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | J. Boyce and M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs) | Email: | jill@vidyo.com miska.hannuksela@nokia.com |
| Source: | AHG9 |  |  |

_____________________________

# Abstract

This report summarizes the activities of the ad-hoc group of high-level syntax for HEVC extensions (AHG9) between the 17th JCT-VC meeting and the 18th JCT-VC meeting.

# Mandates

- Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC.

- Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs.

- Study SEI messages and VUI syntax designs needed for HEVC extensions.

- Study enhancement-layer video parameter set design on the basis of the agreements noted in the high-level syntax break-out group report JCTVC-Q0223

- Study the operation of the Hypothetical Reference Decoder for multi-layer operation.

- Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs.

- Coordinate efforts with JCT-3V AHG6 on high-level syntax issues in relation to 3D extensions.

# Activities

Related to mandate 4, the following contribution was submitted on June 11th, and its availability was announced in the JCT-VC and JCT-3V reflectors:

JCTVC-R0049 MV-HEVC/SHVC HLS: VPSs with nuh_layer_id greater than 0 [M. M. Hannuksela (Nokia)]

Abstract: This contribution proposes allowing a VPS with a non-zero nuh_layer_id to be delivered to the decoder in addition to a "base" VPS with nuh_layer_id equal to 0, both with the same vps_video_parameter_set_id value. The VPS with a non-zero nuh_layer_id is required to be a semantic superset of the base VPS and may for example include information about additional layers. The VPS with the highest nuh_layer_id value among the VPSs with the same vps_video_parameter_set_id value is activated.

No other coordinated AHG activity took place, and no other emails were exchanged over the JCT-VC reflector.

There are about 40 input contributions on the topic areas of AHG9.