| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0010 (draft 10) |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 2–9 July 2014 | Document: JCT3V-I0010 (draft 10) |
| --- | --- |

| Title: | Ad hoc group report: Layered coding constraint specifications and capability indications (AHG10) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC and JCT-3V ad hoc group on layered coding constraint specifications and capability indications, including a report of the teleconferences held and the progress made during the interim period since the preceding JCT-VC and JCT-3V meetings. (The reader of this report is assumed to be familiar with various jargon and abbreviations used in JCT-VC and JCT-3V; recent meeting reports and the draft specification text for HEVC and its SHVC and MV-HEVC extensions can be consulted for definitions of terms.)

The AHG held 9 teleconferences of 2.5–3.5 hours duration each, with substantial discussions of the issues. Approximately 10–20 people participated in each call – each of which was announced with at least 7 days of advance notice. The calls were held at different times of day to ensure reasonable opportunities for participation from different time zones (with the last few calls held during the mornings in the U.S. West Coast time zone by unanimous agreement, as this was a convenient time for all participants).

The AHG reached agreement on a number of conclusions and recommendations, as recorded in section 3 of the AHG report..

- Mandates and communication methods

The JCT-VC and JCT-3V ad hoc group on layered coding constraint specifications and capability indications had the following mandates:

- Study how constraints on bitstream syntax affect specifications of bitstream conformance and decoder conformance for layered coding.

- Study indications of bitstream properties and decoding process requirements for layered coding.

- Identify potential side-effects of constraint specifications and capability indications and recommend alternative approaches.

- Identify potential needs for modifications and additional indications of constraints and decoding process requirements for layered coding.

- Coordinate with JCT-VC AHG 9 / JCT-3V AHG 6 on high-level syntax for constraint specifications and capability indications for layered coding.

The reflector used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

The reflector used for discussions by the JCT-3V and all of its AHGs is the JCT-3V reflector:
jct-3v@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-3v.

Email discussions for this AHG were sent to both of the above-mentioned reflectors.

# Work of the AHG

## Kick-off and teleconference planning

A kick-off message was sent by the chair on 10 April 2014. It included an announcement of two planned teleconferences, the first of which was to be held at 1700 Seattle local time on Thursday 17 April 2014 (maximum duration 3 hours) and the second of which was to be held at 0800 Seattle time on Thursday, 24 April (maximum duration 3 hours). The chair suggested discussing the following topics in the first call:

- How to handle the capability necessary to decode auxiliary pictures. This includes consideration of contribution JCTVC-Q0078 / JCT3V-H0026 and the case where the output of the rewriting process would not contain a VPS for the case of single-layer rewritten bitstreams.

- How to handle the potential difference in bit rate, decoding process requirements, etc., when the BL is provided by external means.

- Considering side-effects of aspects such as the level interaction with tile size specifications.

Additional suggestions of particular topics to discuss in this AHG were indicated to be welcome.

The following topics were suggested by Y.-K. Wang (Qualcomm) in subsequent email on the same day:

- How to define level constraints that would be scalable to the number of layers in a bitstream (i.e. the constraints still make sense for bitstreams that contain a large number of layers). This includes consideration of the possibility of specifying a layer-specific level definition and how to also specify the required decoding capability of an entire bitstream (that corresponds to an output layer set)

- How to define level constraints and indications of decoder capabilities that are suitable for the type of decoders that take advantage of the "high-level syntax only" design of both SHVC and MV-HEVC, by reusing existing HEVCv1 decoders without touching the existing hardware cores. It was suggested by Y.-K. Wang that this type of decoders may be the most easily constructed and most important type of SHVC and MV-HEVC decoders.

## First teleconference

### General

The first teleconference was held at 1700 Seattle time on Thursday 17 April 2014 with a duration of 2.5 hours. The teleconference was hosted through Microsoft Lync, which allows either dial-in (through a variety of local phone numbers world-wide) or online access (with shared document viewing). Approximately 15 people attended the teleconference (with a peak of approximately 12 people on the call at once). The screen content coding capability of the Lync software was used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

A preliminary report of the first teleconference and planned topics for discussion for the second teleconference were sent by email shortly after the call on Thursday 17 April (Seattle time).

Plans for further discussion were established. In addition to the second planned teleconference that had previously been planned to be held at 0800 Seattle time on Thursday, 24 April (maximum duration 3 hours), it was suggested to also plan a 3rd teleconference to be held at 1400 Seattle local time on Thursday 1 May (maximum duration 3 hours).

Topics for discussion and further action in the subsequent work were identified as follows:

- Further discussion of how decoding capabilities for auxiliary pictures (and non-target output layers) can be identified (incl. JCTVC-Q0078 / JCT3V-H0026).

- Further consideration of the implications of the case where a base layer is provided by external means.

- Study of the implications of using single-layer decoders to construct a multi-layer decoder.

- DPB specification (incl. JCTVC-Q0103 / JCT3V-H0034).

    - Discussion of capability of decoding auxiliary pictures and non-target output layers

How to handle the capability necessary to decode auxiliary pictures was discussed. The solution agreed at the Valencia meetings related to contribution JCTVC-Q0078 / JCT3V-H0026 and included the possibility where the output of the rewriting process would not contain a VPS for the case of single-layer rewritten bitstreams. It was expressed that this seems generally adequate in principle.

The editors had not yet tried to include this in the output draft text.

NOTE: See 2nd teleconference notes below.

The case where the target output layers of an output layer set do not include auxiliary pictures seems adequately resolved by the decision made in Valencia (to count the bits in the CPB buffer flow of data but not include the auxiliary pictures in the decoding process requirements).

It was asked whether it is possible to have other (non-auxiliary) pictures that are included in an output layer set but are not target output layers of the output layer set and are not in layers that are needed for decoding the target output layers (e.g., an independently coded layer). A participant indicated that the design intent did not seem to allow this.

It was discussed whether this use case might make sense or not, and what the effect would be for the semantics of the default output layer mechanism (e.g., because of the expressed semantics of default_target_output_layer_idc).

NOTE: See 2nd teleconference notes below.

It was suggested that if auxiliary pictures are to be included as target output layers of an output layer set, the profile for the output layer set would need to include decoding process requirements for decoding those pictures. This may be an adequate solution. In any case, we don't seem to have an alternative available.

NOTE: See 2nd teleconference notes below.

It was asked whether it could be possible to have only auxiliary pictures in the target output layers of an output layer set. This would be possible if there is only one target output layer, but if there is more than one target output layer in the output layer set, it may require a profile definition that includes the decoding process requirements.

It was asked whether it is possible to have scalable / layered coding of auxiliary pictures. It was said that this is not possible as currently specified.

NOTE: See 2nd teleconference notes below.

It was noted that the drafted Scalable Main profile does not support monochrome decoding.

It was remarked that the current draft does not prohibit having target output layers of an output layer set that are auxiliary pictures. This would need to be disallowed unless some way is found to specify the necessary decoding process capabilities (e.g., by a profile definition that includes such capabilities).

### Discussion of when the BL is provided by external means

The case when the BL is provided by external means was discussed.

Contribution JCTVC-Q0145 / JCT3V-H0043 considered this topic.

It still seemed possible that we may not need a separate profile for the external base layer case, if the specification is drafted appropriately, as the question of whether the base layer is provided within the bitstream or by external means could be a system configuration question rather than a profiling issue. However, this question seemed to remain in need of further consideration.

### Discussion of using single-layer decoders to construct a multi-layer decoder

The current draft Scalable Main profile requires a 2x pixel count capability for level specifications and only allows at most one upsampling process to be performed for each picture, and has up to 8-deep layering dependency.

On a picture-by-picture basis, the layer that is to be upsampled can change.

The Stereo Main (multiview) profile, of course, only allows 2 layers.

It was remarked that decoder conformance requirements (versus bitstream conformance requirements) are not specified in the current draft and should be added by the editors. Also, the profile_idc values have not been assigned. Contribution JCTVC-Q0145 / JCT3V-H0043 considered this topic.

Our understanding of the intent is that Scalable Main and Stereo Main decoders should be capable of decoding Main profile bitstreams (at least when the base layer is not provided by external means).

The 8-deep layering dependency requirement was discussed.

It was suggested that each layer need to conform to a specific set of constraints rather than, or in addition to, having a set of layers conform to a specific profile/level, so that the capability for each layer is distinctly indicated.

Another suggestion was that if only two layers is envisioned to be supported for the application (e.g., if the high-level structuring and multi-DPB handling are considered burdensome for the decoder), a different profile, e.g., a two-layer scalable profile, might be needed.

Further study was needed on this issue of the capability signalling needed for enabling single-layer decoders to be repurposed to decode multi-layer bitstreams. A key question is whether (and to what degree) we can assume that one single-layer decoder can be repurposed to decode multiple layers.

### General discussion

It was remarked that JCTVC-Q0103 / JCT3V-H0034 relates to constraint indications and should be discussed in this AHG.

## Second teleconference

### General

The second teleconference was held at 0800 Seattle time on Thursday, 24 April 2014 with a duration of 3 hours. The teleconference was hosted through Microsoft Lync, as with the previous call. Approximately 20 people attended the teleconference (with a peak of approximately 16 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

Topics for discussion and further action in the subsequent work had been identified as follows:

- Further discussion of how decoding capabilities for auxiliary pictures (and non-target output layers) can be identified (incl. JCTVC-Q0078 / JCT3V-H0026).

- Further consideration of the implications of the case where a base layer is provided by external means.

- Study of the implications of using single-layer decoders to construct a multi-layer decoder.

- DPB specification (incl. JCTVC-Q0103 / JCT3V-H0034).

Prior to the call, email was sent to the reflector by M. M. Hannuksela and J. Boyce, each containing attachments with relevant comments that were suggested to be considered by the AHG.

Included in the email from Hannuksela was a suggestion that JCTVC-Q0115/JCT3V-H0038 (on decoding process indication) is another contribution that should be discussed in the AHG.

It was suggested that further review of JCTVC-Q0145 / JCT3V-H0043 (on level specifications) should also be conducted, and noted that there is a substantial number of details discussed in that contribution.

During the first teleconference, a 3rd teleconference had also been planned to be held at 1400 Seattle local time on Thursday 1 May.

A preliminary report of the second teleconference was sent by email shortly after the second teleconference on 24 April 2014 (Seattle time).

Regarding plans for further teleconferences, the following plans were established:

- The third teleconference was planned to be held at 1400 Seattle local time on Thursday 1 May.

- A fourth call was planned for 8 May 2014 at 1900 Seattle local time.

### Discussion of M. M. Hannuksela emailed remarks

Emailed remarks from M. M. Hannuksela were discussed.

Regarding the solution agreed at the Valencia meetings related to contribution JCTVC-Q0078 / JCT3V-H0026 and the possibility where the output of the rewriting process would not contain a VPS for the case of single-layer rewritten bitstreams, it was noted that text had actually been provided in JCTVC-Q0078v4, which was uploaded April 4th (during the Valencia meeting). At the time of the 2nd teleconference, the editors were working on its inclusion.

It was further discussed whether it is possible to have other (non-auxiliary) pictures that are included in an output layer set but are not target output layers of the output layer set and are not in layers that are needed for decoding the target output layers (e.g., an independently coded layer).

Hannuksela indicated that he believed this is allowed, and that layer sets can include any layers, and output_layer_flag[ i ][ j ] can be set without constraints (except for the requirement that each output layer set contain at least one target output layer).

It seemed that further study of those semantics and that question would be needed.

It was asked whether the extraction process can include such "extra" non-target output layers in an extracted bitstream (after recent modifications).

It was suggested that the profile/tier/level information for such an output layer set would include all the layers in the output layer set in the CPB operation but would not include the "unnecessary" layers in its decoding process requirements.

It was remarked that the current specification assumes that the full decoding process is applied to the "unnecessary" layers as well as to the "necessary" layers.

Future extensibility was suggested to be a use case that would motivate this behaviour.

It was remarked that we may not have adequate text language currently regarding identification of which layers the profile/tier/level information applies to. It was remarked that it should be possible to draft such language, although it is not currently there.

In principle, it was suggested to be desirable to support this possibility (esp. for the future extensibility purpose), and suggested that we should conduct further study to ensure that the specification is consistent with this design intent. However, there was no clear agreement that this needed to be supported, pending further study.

It was asked whether the "necessary" layer subset should be explicitly identified rather than determined from dependency tree information, to ease the identification of the relevant bitstream subset.

In regard to auxiliary pictures as (possibly the only) target output layers in an output layer set, Hannuksela indicated that two different cases have to be considered separately:

- When the output layer set also contains at least the base layer and may also contain other primary picture layers. In this case, it is possible to have (one or more) auxiliary picture layers as output layers, if the profile for the output layer set so allows.

(The current specification assumes that all layers in the output layer set are decoded.)

- When output layer set does not contain the base layer. In this case, the rewriting process is (conceptually) applied and those layers that are allowed as output layers in the rewritten bitstream can also be output layers in the output layer set (of the original VPS/bitstream).

Regarding whether it is possible to have scalable / layered coding of auxiliary pictures, Hannuksela indicated that he thought it is possible, and asked which part of the specification text would disallow it – suggesting that it is intended to use view scalability with auxiliary depth pictures. The layer ID of the "base layer" of the auxiliary pictures would not be zero, but the (virtual) rewriting process would rewrite one of those to become zero.

Regarding having target output layers of an output layer set that are auxiliary pictures. In the previous call, it was noted that this would need to be disallowed unless some way is found to specify the necessary decoding process capabilities (e.g., by a profile definition that includes such capabilities).

Hannuksela questioned why this would need to be disallowed – suggesting that we need to say that the current profiles do not require decoding or output of the auxiliary picture layers regardless of their value of output_layer_flag. He asserted that including auxiliary picture layers as output layers in an output layer sets enables encoders to include and decoders to use proper HRD parameters (both CPB and DPB) when auxiliary picture layers are decoded and output.

Discussed examples included coding multiple resolutions for alpha, or view scalable coding of depth, or view scalability for overlay coding.

To support decoders that won't decode auxiliary picture layers, Q0078 originally proposed that whenever one or more auxiliary picture layers are included in an output layer set, it is required to also have a respective principal output layer set that contains no auxiliary picture layer but is otherwise identical (the same primary picture layer, the same output layers among the primary picture layers, the same setting for alternative output layers).

This suggests allowing auxiliary pictures to be target output layers even when the decoding process requirements do not include a requirement to decode those layers – in order for the DPB parameters to include those layers.

The concept of target output layers includes two aspects:

- Profile/tier/level decoding process requirements

- DPB requirements

An alternative suggestion was to specify a profile that includes the decoding process requirements.

It was agreed that it is not really necessary to allow auxiliary pictures to be target output layers even when the decoding process requirements do not include a requirement to decode those layers, since the DPB is not shared and the DPB requirements for the auxiliary pictures can be determined from the VPS rewriting SEI message.

### Discussion of JCTVC-R0041 / JCT3V-I0023 "Use of general_profile_space for auxiliary picture profile indication" [J. Boyce (Vidyo)]

A document by J. Boyce that was planned to be registered for the next meeting was discussed. The document was later registered as JCTVC-R0041 / JCT3V-I0023, "Use of general_profile_space for auxiliary picture profile indication".

It was proposed to use an unused value of the general_profile_space syntax element to indicate that a layer with a non-zero value of nuh_layer_id in a multi-layer bitstream is otherwise conforming to a profile.  It is asserted that this proposal could enable signalling of conformance points for auxiliary pictures without the need for the changes adopted in JCTVC-Q0078, which include a normative rewriting process, VPS rewriting SEI message and output layer set nesting SEI message. Conceptually, an additional "Extension" profile would be defined for an existing profile, such that, for example, an Extension Main profile would be defined which allows general_profile_space to be equal to 1, without modifying the existing Main profile. The "Extension" profiles could also be used to indicate conformance for simulcast enhancement layers.

This would not only define "Extension" profiles for v.1 and RExt profiles, but also for the layered coding profiles.

For the single layer case, this is essentially just an alternative to the Q0078 approach.

For other cases, this would enable the base layer to conform to a non-single-layer profile.

Some aspects of JCTVC-Q0078 (e.g., additional layer sets) would still apply, but the rewriting process would not be needed.

Some concern was expressed regarding whether it is really needed to define the additional usage as a distinct profile, or whether some alternative editorial language could be applied in regard to how the syntax combination is described in the text.

Concern was also expressed that the general_profile_space is only 2 bits, which are precious since they provide the ability to have more than 32 profiles.

It was noted that another way to do the same thing would be to defined other values of profile_idc. The value of using general_profile_space rather than profile_idc was suggested to be the clarity of identification of the profile association.

A question was asked about whether HRD conformance is an issue.

The concept would be envisioned to be used both for simulcast layers and auxiliary picture layers, and to be used for specification of a conformance point for layered coding of auxiliary pictures.

It was agreed that further study of this proposal was desired.

    - DPB specification (incl. JCTVC-Q0103 / JCT3V-H0034)

Proposal 2 in Q0103 was for DPB sharing, and it was agreed in Valencia not to use DPB sharing, so this was not further discussed.

Proposal 1: Per-Layer DPB Capacity Limit

Maximum DPB size level limit constraints are proposed for per-layer sub-DPB parameters max_vps_layer_dec_pic_buff_minus1[ ][ ][ ].

Regarding level limit constraints related to maximum DPB size:

- SHVC Scalable Main and Scalable Main 10 profiles define that each layer in the TargetDecLayerIdList shall obey profile-specific level limit in A.4.1 d.

The constraint in A.4.1 d applies to sps_max_dec_pic_buffering_minus1[ HighestTid ].

- MV-HEVC Stereo Main profile defines a similar constraint on sps_max_dec_pic_buffering_minus1[ HighestTid ].

It was asked whether the proposal is essentially a clarification to add something clearly missing in the current specification, and it seemed that this was the case – i.e., that the VPS level DPB needs to have constraints as well as that the SPS level having such constraints. The representation format in the VPS is used in the contribution to impose these constraints.

It was agreed in spirit to recommend that such constraints need to be specified, but that the exact wording should be studied – as an editorial matter.

### Third teleconference

### General

The third teleconference was held at 1400 Seattle time on Thursday, 1 May 2014 with a duration of 3 hours. The teleconference was hosted through Microsoft Lync, as with the previous calls. Approximately 15 people attended the teleconference (with a peak of approximately 13 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

Prior to the teleconference, the following new documents had been registered for discussion:

- JCTVC-R0041 / JCT3V-I0023, "Use of general_profile_space for auxiliary picture profile indication" [J. Boyce (Vidyo)] (already discussed in the 2nd teleconference)

- JCTVC-R0042 / JCT3V-I0021 "AHG10: Profile indication for additional layer sets" [M. M. Hannuksela (Nokia)]

- JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

A preliminary report of the third teleconference was sent by email shortly after the teleconference, on 1 May 2014 (Seattle time).

Additional teleconference plans were made as follows:

- 8 May 2014 at 1900 (Seattle time)

- 15 May 2014 at 1200 (Seattle time)

    - Discussion of JCTVC-R0042 / JCT3V-I0021 [M. M. Hannuksela (Nokia)]

The contribution proposes changes to enable indication of profile_tier_level( ) for additional layer sets (i.e. layer sets without the base layer) with the following design principles:

- Avoiding the requirement of including VPS rewriting SEI messages in the bitstream for additional layer sets containing more than one layer, as adopted with JCTVC-Q0078.

- Avoiding the duplication of the number of profiles and the use of a scarce general_profile_space value proposed in JCTVC-R0041.

It was asserted that this contribution and JCTVC-Q0078 provide the following benefits over JCTVC-R0041 for decoders when it comes to decoding independent non-base layers:

- The decoder does not need to decode the VPS (both the base part and the vps_extension part), as required in JCTVC-R0041, to figure out the decoding capability requirements for independent non-base layers. The applicable profile_tier_level( ) structure is proposed to be available in the active SPSs of the independent non-base layers.

- The decoder does not need have the capability of decoding SPSs and PPSs as specified in MV-HEVC/SHVC, as required by JCTVC-R0041.

It is asserted that this contribution, JCTVC-Q0078 and JCTVC-R0041 enable indicating the profile conformance multiple auxiliary picture layers which use inter-layer prediction.

The proposal can be summarized as follows:

- The profile specifications of the Stereo Main profile, the Scalable Main profile and the Scalable Main 10 profile were proposed to be modified as follows:

  - The output layer set and the corresponding extracted sub-bitstream is not required to contain a layer with nuh_layer_id equal to 0.

  - AuxId greater than 0 is allowed.

  - Monochrome sampling is allowed.

  - baseBitstream is the output of the conventional sub-bitstream extraction process, if the output layer set contains nuh_layer_id equal to 0; and of the independent non-base layer rewriting process otherwise.

For an output layer set conforming to the Stereo Main profile, baseBitstream is required to conform to the Main profile or the Monochrome 8 profile.

For an output layer set conforming to the Scalable Main profile, baseBitstream is required to conform to the Main profile or the Monochrome 8 profile.

For an output layer set conforming to the Scalable Main 10 profile, baseBitstream is required to conform to the Main 10 profile or the Monochrome 8 profile.

- The non-base layer subtree extraction process was modified to apply to an independent non-base layer only and renamed to independent non-base layer rewriting process. In other words, the rewriting process applies to a single layer only rather than a whole additional layer set as in JCTVC-Q0078. Consequently, the rewriting process was asserted to become straightforward:

- NAL units with nal_unit_type not equal to SPS_NUT, PPS_NUT, EOS_NUT and EOB_NUT and with nuh_layer_id not equal to the assignedBaseLayerId are removed from outBitstream.

- NAL units with nal_unit_type equal to SPS_NUT, PPS_NUT, EOS_NUT or EOB_NUT with nuh_layer_id not equal to 0 are removed from outBitstream.

- For each NAL unit, the following applies:

  - When nuh_layer_id is equal to assignedBaseLayerId, nuh_layer_id is set equal to 0.

The proposed rewriting process is used in the defining the conformance of independent non-base layers (but not in defining the conformance of sub-bitstreams with multiple layers). The profile_tier_level( ) indicated for an additional layer set consisting of one layer applies to the output of the independent non-base layer rewriting process. The output of the rewriting process is otherwise conforming to the indicated profile but does not contain VPS NAL units.

The proposed rewriting process enables to use the specification of the existing single-layer profiles (Main, Main 10, Main Still Picture, and all the 19 RExt profiles) for independent non-base layers.

- A sub-bitstream extraction process for additional layer sets was added.

This process is similar to the conventional sub-bitstream extraction process, but keeps SPS_NUT, PPS_NUT, EOS_NUT and EOB_NUT with nuh_layer_id equal to 0 included in the output bitstream, as those SPSs and PPSs may be active for the layers of the additional layer set and EOS_NUT and EOB_NUT affect the conformance requirements.

It is asserted that a similar extraction process additional layers sets would be needed also in JCTVC-R0041 to keep EOS_NUT and EOB_NUT NAL units in the bitstream.

The extracted sub-bitstream is otherwise conforming but does not contain VCL NAL units with nuh_layer_id equal to 0.

The sub-bitstream extraction process for additional layer sets is used in clause 8 when creating a sub-bitstream to be decoded from the selected output layer set and in the profile specifications the Stereo Main, Scalable Main and Scalable Main 10 profiles.

- The output layer set nesting SEI message was removed.

- The VPS rewriting SEI message message was removed.

The detailed proposal is provided in the accompanying change-marked text relative to an editors' version of the MV-HEVC specification that includes the integration of JCTVC-Q0078.

It was remarked that the SPS and PPS of an independent non-base layer for this proposal would be required to have layer_id equal to 0 (although not necessarily sharing the SPS and PPS of the base layer itself), and suggested that this would not allow an application to require the PSs to have the same layer ID as the pictures that use it, which would enable simpler layer extraction based on layer ID alone.

The proponent said that a version of JCTVC-Q0078 considered an alternative form in which syntax within the parameter sets would indicate whether they corresponds to v1 or v2 syntax (perhaps SPS max sublayers minus1).

In the discussion, it was noted that the proposal would allow an independent non-base layer to be identified as conforming to the Main profile. A participant commented that this might be confusing, as such a layer cannot be decoded by a Main profile decoder.

Relative to the JCTVC-R0041 / JCT3V-I0023 proposal, the proposal is based on using the layer ID value rather than profile space to indicate an independent non-base layer.

In this proposal, the only thing that is changed by the rewriting process for an independent non-base layer is the layer ID, which is changed from a non-zero value to a zero value.

It was noted that monochrome is not a capability within Main profile, which creates an issue with regard to whether a decoder that can decode such pictures could be constructed from existing Main profile decoders. Using the pseudo-monochrome syntax found in version 1 might be a way to avoid this.

It was suggested that one way to handle auxiliary pictures would be to identify them simply as layers to be decoded and conforming to Scalable Main profile. This might create a requirement that, in order to decoded auxiliary pictures, a decoder would need to conform to the Scalable Main profile (or Stereo Main, to avoid supporting upsampling).

A participant remarked that negotiation and capabilities are often indicated by the profile/tier/level syntax structure, so using something within that syntax structure may be helpful – i.e., not just layer ID. Others remarked that this information is interpreted in a layer-specific way in the SHVC/MV-HEVC specification, and the base layer would need to be compatible with prior profiles and thus could not use "v1-hostile" syntax there (such as alternative values of profile space).

Some remarks of participants included the following comments:

- Avoid creating new profiles just for layer syntax support.

- Avoid using profile space unless really necessary.

- This implies following something along the lines of this suggestion or the suggestion to define a profile that includes the requirements of the decoding process of both layers (which may be awkward with base layers that use RExt profiles).

One suggestion was to describe layer syntax support as an optional feature of existing profiles rather than defining that capability as a separate profile as proposed in JCTVC-R0041 / JCT3V-I0023.

For some applications, a non-base layer is supplemental information, and all that is needed is a way to indicate the capabilities needed to decode it, without necessarily calling that a profile.

It was remarked that this proposed approach seems better than JCTVC-Q0078, so the VPS rewriting SEI message and output layer set nesting SEI message don't seem desirable to retain anymore.

It was noted that the JCTVC-Q0078 approach enables rewriting of arbitrary layer sets, whereas this one does not. However, it allows the Scalable Main and Stereo Main profile to exclude base layers from expressed output layer sets. It was asked whether we can clearly define the capability necessary to decode such a layer set without having a conforming bitstream to refer to.

Monochrome remains a separate question.

### Discussion of JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

This proposal is a follow-up of JCTVC-Q0145/JCT3V-H0043.

JCTVC-Q0145/JCT3V-H0043 reported and discussed some asserted issues associated with the current level definitions in SHVC, and proposed a different approach to define levels and decoder capabilities that were asserted to also apply to MV-HEVC.

The level definitions and decoder capability requirements proposed in JCTVC-Q0145/JCT3V-H0043 are summarized as follows. As a part of the proposed level definitions, a decoder capability is specified in a way where the level is associated with a scale, which is needed together with the level to express the decoder capability. Two categories of conforming decoders may exist. Category I decoders can be implemented without reusing the hardware cores of existing single-layer HEVC decoders as they are. Category II decoders are those that are implemented using more than one existing single-layer HEVC decoder by reusing the hardware cores as they are.

During the discussion at the Valencia meeting and at conference calls after the meeting for the AHG on layered coding constraint specifications and capability indications, it seems that it was generally agreeable that Category II decoders are essential to consider, and possibly more important than Category I decoders. For Category II decoders, layer-specific profile, tier, and level specification was also mentioned as a promising direction, though even with that, it would still need to be considered whether bitstream-specific profile, tier, and level specification is also needed. Another topic discussed was whether it is easy to repurpose a single-layer HEVC decoder to decode multiple layers of an SHVC/MV-HEVC bitstream, which is a key decision factor on the need for the definition of a two-layer profile.

This document firstly discusses reusing of a single-layer HEVC decoder for decoding of multiple layers of an SHVC/MV-HEVC bitstream without touching the hardware, but with only firmware changes and the addition of resampling. After that, a specification of partition-specific profile, tier, and level is proposed that is suitable for Category II decoders, while leaving it open on whether and how to specify profile, tier, and level for Category I decoders.

The proposed partition-specific profile, tier, and level specification is summarized as follows.

- For each output layer set, one or more partitioning schemes of layers into partitions are signalled. Each partition can contain one or more layers, as currently specified (the current term is bitstream partition).

- A profile, tier, and level are proposed to be signalled for each partition.

- All level limits and restrictions, except for the three on picture width, picture height, and sub-DPB size that are naturally layer specific, are specified to be partition specific (e.g., CPB size and bit rate).

- The bitstream conformance tests are proposed to be specified to test the conformance of each partition, in a manner that decoded pictures and other necessary information for all direct or indirect reference layer of the layers in the partition are assumed to be available similarly as the current specification for the base layer when it is externally provided.

- A decoder's decoding capability is proposed to be expressed as conforming to a list of triplets of profile, tier and level, wherein the number of entries in the list indicates the number of single-layer decoders used to build the multi-layer decoder, and each triplet of profile, tier and level indicates the profile, tier and level of one of the single-layer decoders.

- A decoder that conforms to a list of triplets of profile, tier and level is proposed to be required to be able to decode any conforming output layer set for which there is at least one partitioning scheme that satisfies the following condition: for each partition of the partitioning scheme one of the single-layer decoders of the multi-layer decoder can be exclusively assigned for decoding the partition.

Note that when each layer is in its own partition, it is effectively layer-specific profile, tier, and level.

The attached slide set contains the discussion on reusing of a single-layer HEVC decoder for decoding of multiple layers of an SHVC/MV-HEVC bitstream.

The text changes that implement the proposed partition-specific profile, tier, and level specification are in another attachment of this document. The text changes are based on a recent draft version of MV-HEVC, however the approach and most of the text changes are applicable to SHVC as well.

The proposal leaves open the question of whether to also define "Category I" profiles.

The contribution asserts that it is viable and easy to re-purpose one legacy HEVC decoder to decode multiple layers of a scalable bitstream, reporting that firmware-level changes are sufficient to accomplish this.

A participant suggested that the upsampler requirement may interfere with this ability – saying that while the concept is generally feasible otherwise, having flexible data paths for upsampling may be difficult.

## Fourth teleconference

### General

The fourth teleconference was held at 1700 Seattle time on Thursday, 8 May 2014 with a duration of 3 hours. The teleconference was hosted through Microsoft Lync, as with the previous calls. Approximately 15 people attended the teleconference (with a peak of approximately 11 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

Prior to the meeting, a revised version of the following document had been registered for discussion:

- JCTVC-R0042 / JCT3V-I0021 "AHG10: Profile indication for additional layer sets" [M. M. Hannuksela (Nokia)]

A preliminary report of the fourth teleconference was sent by email shortly after the teleconference, on 8 May 2014 (Seattle time).

Additional teleconference plans were made as follows:

- The previous plan to hold another teleconference on 15 May 2014 at 1200 (Seattle time) was confirmed.

- Another call on Friday May 23rd 0800 (Seattle time) was also planned.

    - Further discussion of JCTVC-R0043 / JCT3V-I0022 SHVC/MV-HEVC level definitions and related decoder capability requirements [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

It was asked what is the difference, in concept, between the proposed partitioning scheme and the concept of an externally defined base layer.

The proponent indicated that, basically, there is no difference between the enhancement partition of such a partitioned bitstream and an enhancement partition for which the base layer pictures are provided by external means.

If an HRD buffer flow description is needed that covers the entire bitstream, the bitstream would contain a description of a partition that includes the entire bitstream.

Multiple partitioning schemes would be supported to be expressed in the bitstream.

Some details of exactly how this would work had not yet been sorted out by the contributor.

The existing concept of signalling HRD parameters for layer sets could be used for partitions that contain entire layer sets. However, in addition, HRD parameters and decoding process constraint information could be provided for enhancement-only partitions.

Partitions would be established within output layer sets.

It was asked how to deal with the need for resamplers between layers.

Since, in principle, a single-layer decoder could be assigned to decode multiple layers of a multi-layer bitstream, the DPB size of a partition to which such a decoder is assigned would need to fit within the DPB capacity of a single-layer decoder.

It was asked whether the DPB capacity is measured in terms of the number of pictures at the maximum picture size or based on the memory size of the pictures, such that a picture of a given size would be treated as equivalent to four pictures of 1/4 size when there are layers with different picture size in the same partition. The contributor suggested the latter.

Bit depth mixtures would be treated as if they all used the higher bit depth.

Auxiliary pictures can have different chroma format than primary pictures. In such a case, it was suggested that the pictures would be treated as if they all used the higher chroma format.

It was commented that the basic idea seemed good, and further study was encouraged. Resampling was suggested to be a critical issue needing further thought.

M. Zhou emphasized the implications of bit rate, pixel rate, the impact on line buffers (dependent on picture size), memory capacity, and memory bandwidth. The 64x64 maximum CTU size was said to be too large to hold on chip for upsampling, so off-chip storage prior to upsampling was suggested to be necessary. He commented that it may be desirable to have a profile in which each partition can contain only one layer. This would enable only one upsampler to be needed, which sits between such partitions.

It was suggested that this question is, at least primarily, outside the scope of the AHG.

### Discussion of version 2 of JCTVC-R0042 / JCT3V-I0021 [M. M. Hannuksela (Nokia)]

Revision 1 of the contribution responds to the open issues noted in the third AHG10 teleconference, categorized into the following numbered topics.

Topic #1: Differentiating single-layer (Annex A) profile decoding capability applied to the base layer and applied to independent non-base layer.

Proposal: In addition to characterizing a decoder with profile-tier-level triplet(s) it can support (as already specified in Annex A), it is proposed to add the following recommendation in Annex A related to indicating decoder capabilities: When expressing the capabilities of a decoder for a profile specified in Annex A, the capability of decoding an independent non-base layer should be expressed.

This seemed to essentially be the same concept previously discussed as being an optional capability for profiles specified in Annex A. For example, this could be called a "Main+" decoder capability, without defining it as a separate profile. However, as previously another participant suggested that having syntax within the profile/tier/level syntax of an enhancement layer seemed like a clearer approach.

It was suggested for that question to not be decided within the AHG, but to wait for that to be decided by JCT-VC and its parent bodies. (This topic was further considered in subsequent AHG teleconferences.)

Topic #2: Requirement for supporting monochrome sampling in the layered profiles.

Suggested alternatives:

- Require monochrome support in scalable profiles.

- Require alpha and depth pictures to use non-monochrome syntax in some profiles (e.g., for stereo depth with inter-layer prediction of the depth).

It was suggested for that question to not be decided within the AHG, but to wait for that to be decided by JCT-VC and its parent bodies.

Topic #3: SPS and PPS with nuh_layer_id equal to 0 are required to be used for independent non-base layers

The following alternative proposals are made:

- Require SPS and PPS of an independent non-base layer to have layer_id equal to 0 (although not necessarily use the same the SPS and PPS as that used for the base layer itself). (This alternative was included in the original submission of the contribution.)

  - Advantages:

    - No need to change the SPS and PPS design from version 1 and the current MV-HEVC/SHVC draft.

    - The base layer and the independent non-base layers can share the same SPSs and PPSs with nuh_layer_id equal to 0, which could provide bit count savings.

  - Disadvantages:

    - It cannot be concluded from the SPS and PPS itself which layer will activate it. However, it is remarked that SPSs and PPSs that are never activated are allowed to be present in the bitstream, and hence an SPS or PPS in the bitstream will not necessarily indicate the bitstream properties (prior to their activation is known).

    - The rewritten bitstream continue to include SPSs and PPSs of the former base layer that is no longer present in the rewritten bitstream. This causes a minor bit rate increase for the rewritten bitstream, but is probably not an issue.

    - An extracted sub-bitstream of the original bitstream that excludes an independent non-base layer continues to include the SPSs and PPSs for the independent non-base layer. This causes a minor bit rate increase for the extracted bitstream, but is probably not an issue.

- Specify the syntax of SPS and PPS with nuh_layer_id greater than 0 so that version 1 compatibility can be maintained, as proposed in JCTVC-Q0078 v1. For independent non-base layers, require the activated SPS or PPS to have nuh_layer_id equal to that of the layer that is activating the SPS or PPS, respectively.

Version 1 compatibility of SPS is proposed to be achieved as follows: Rather than using the nuh_layer_id value to include syntax elements conditionally in the SPS, a v1CompatibleSPSFlag is derived from the sps_max_sub_layers_minus1 (which is the second syntax element in the SPS). If sps_max_sub_layers_minus1 is equal to 7, v1CompatibleSPSFlag is set equal to 0. Otherwise, v1CompatibleSPSFlag is set equal to 1. Consequently, an SPS with nuh_layer_id greater than 0 can use the SPS syntax of HEVC version 1. In the rewriting process, such as an SPS can be converted to an SPS with nuh_layer_id equal to 0 simply by changing its nuh_layer_id value.

The VPS information could/should be required to be consistent with that provided in the SPS when the data is in the SPS.

Version 1 compatibility of PPS is proposed to be achieved as follows: pps_scaling_list_ref_layer_id is moved to PPS extension, and hence the "base part" of the PPS is identical to the PPS syntax in HEVC version 1 regardless of the nuh_layer_id value of the PPS.

For further details on version 1 compatible SPS and PPS, please reer to JCTVC-Q0078 v1.

  - Advantages:

    - It can be concluded from the SPS and PPS itself which independent layer, if any, will activate it. However, PPSs may still be shared by many layers (one of which may be an independent layer and others may be predicted layers).

  - Disadvantages:

    - As SPSs and PPSs cannot be shared by multiple independent layers, separate SPSs and PPSs are needed for each independent layer. This causes a minor the bitrate increase of the original bitstream. Additionally, a more substantial impact might be that a greater number of values of seq_parameter_set_id are consumed out of the value range of 0 to 15, inclusive.

- Specify the syntax of SPS and PPS with nuh_layer_id greater than 0 so that version 1 compatibility can be maintained, as proposed in JCTVC-Q0078 v1 and above in alternative 2. For independent non-base layers, require the activated PPS to have nuh_layer_id equal to 0 or to that of the layer that is activating the PPS. For independent non-base layers, require the activated SPS to have nuh_layer_id equal to that of the layer that is activating the SPS.

  - Advantages:

    - It can be concluded from the SPS itself which independent layer, if any, will activate it.

    - PPSs can be shared among multiple independent layers.

  - Disadvantages:

    - An extracted sub-bitstream of the original bitstream that excludes an independent non-base layer continues to include the PPSs that are activated only for the independent non-base layer. This causes a minor bitrate increase for the extracted bitstream, but is probably not an issue.

- Specify the syntax of SPS and PPS with nuh_layer_id greater than 0 so that version 1 compatibility can be maintained, as proposed in JCTVC-Q0078 v1 and above in alternative 2. For independent non-base layers, require the activated SPS or PPS to have nuh_layer_id equal to 0 or equal to that of the layer that is activating the SPS or PPS, respectively.

  - Advantages:

    - PPSs and SPSs that are not activated by the base layer need not have nuh_layer_id equal to 0. Consequently, an extracted sub-bitstream of the original bitstream that excludes an independent non-base layer typically excludes the SPSs and PPSs only used by the independent non-base layer.

  - Disadvantages:

    - It cannot be concluded from the SPS and PPS itself which layer will activate it. However, it is remarked that SPSs and PPSs that are never activated are allowed to be present in the bitstream, and hence an SPS or PPS in the bitstream will not necessarily indicate the bitstream properties (prior to their activation is known).

The proponent's preference order among the alternatives is: 1 (most preferred), 4, 3, 2 (least preferred).

In the discussion, some other participants expressed a preference for method 2 or method 4 as a cleaner design – somewhat of a bug fix for the lack of ability to use v1 syntax in a layer other than layer 0, and an improvement of flexibility in the design.

Option 4 was suggested as preferable to option 2. There was no strong objection to option 4.

It was agreed to try to decide in next call between option 4 and option 1.

Topic #4: Specifying the capability necessary to decode a layer set without a base layer, when there is no specification of a conforming bitstream without a base layer.

Due to lack of time, topic #4 was not discussed in the fourth teleconference.

## Fifth teleconference

### General

The fifth teleconference was held at 1200 Seattle time on Thursday, 15 May 2014 with a duration of 3 hours. The teleconference was hosted through Microsoft Lync, as with the previous calls. Approximately 15 people attended the teleconference (with a peak of approximately 12 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

Prior to the meeting or at the beginning of the meeting, the following documents had been registered for discussion:

- JCTVC-R0044 / JCT3V-I0024 "AHG10: On decoder capability indication" [M. M. Hannuksela (Nokia)]

- Version 2 of JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

A preliminary report of the fifth teleconference was sent by email shortly after the teleconference, on 15 May 2014 (Seattle time).

Additional teleconference plans were made as follows:

- The previous plan to hold a call on Friday 23rd at 0800 (Seattle time) was confirmed.

- Another call on Friday 6th June at 0900 (Seattle time) was also planned.

### Further discussion of version 2 of JCTVC-R0042 / JCT3V-I0021 [M. M. Hannuksela (Nokia)]

In the previous call, we had said we would try to decide in the fifth call between option 4 and option 1.

In the interest of stability, a couple of participants expressed a relatively weak preference for option 1. One participant expressed a relatively strong preference for option 4, motivated by a desire to allow a system to "cleanly separate" different layers.

Option 2 was also discussed. However, it was pointed out that the syntax space of the SPS ID is shared across all layers, and prohibiting sharing of SPSs for independent layers, as would be the case in option 2, would limit the total number of supported independent layers to 16 and when many independent layers are present, there would be limited room remaining for use by dependent layers.

Option 4 was agreed to be recommended for adoption by the AHG.

Topic #4 (specifying the capability necessary to decode a layer set without a base layer, when there is no specification of a conforming bitstream without a base layer) was discussed.

A related excerpt from the AHG10 meeting notes was as follows:

"It was noted that the JCTVC-Q0078 approach enables rewriting of arbitrary layer sets, whereas this one does not. However, it allows the Scalable Main and Stereo Main profile to exclude base layers from expressed output layer sets. It was asked whether we can clearly define the capability necessary to decode such a layer set without having a conforming bitstream to refer to."

Background (from the original submission of this contribution):

It was remarked that the decoding process handles an output layer set without a base layer correctly. This is ensured by created BitstreamToDecode with the newly specified sub-bitstream extraction process for additional layer sets. The related specification text in clause 8 from the original submission of this contribution was quoted.

The profile constraints apply to the subBitstream that is either derived with the (conventional) sub-bitstream extraction process or the sub-bitstream extraction process for additional layer sets.

An underlying thought in the sub-bitstream extraction process for additional layer sets is that the process is never used to generate a self-standing bitstream but merely create a sub-bitstream used within the decoding process or profile specification only.

The following alternative proposals were made:

- No changes to the original submission of this contribution, i.e. do not use the sub-bitstream extraction process for additional layer sets to create a self-standing conforming bitstream.

- Replace vps_base_layer_internal_flag and vps_reserved_one_bit with a two-bit indicator vps_base_layer_presence_idc. The semantics for the values of vps_base_layer_presence_idc are specified as follows:

3: the base layer is present and conforms to HEVC.

2: the base layer is not present but decoded base layer pictures are provided by external means (i.e., the same semantics as vps_base_layer_internal_flag equal to 1).

1: the base layer is not present, decoded base layer pictures are not provided by external means, and the properties of the base layer are documented in the VPS. vps_base_layer_presence_idc equal to 1 is used when additional layer sets (without the base layer) are extracted.

0: the base layer is not present, decoded base layer pictures are not provide by external means, and the properties of the base layer are not documented in the VPS. This value may be used in bitstreams where the base layer has never been present.

As values 0 to 2, inclusive, are disallowed in HEVC v1, decoders conforming to v1 only will not consider bitstreams with such values to be conforming.

In addition, a change in the sub-bitstream extraction process for additional layer sets is proposed such that in its output bitstream vps_base_layer_presence_idc is set equal to 1.

Bitstreams of Stereo Main, Scalable Main and Scalable Main 10 profiles are allowed to contain vps_base_layer_presence_idc equal to 0, 1 and 3, and decoders of these profiles are required to decode bitstreams with vps_base_layer_presence_idc equal to 0, 1, and 3. It is subject to another discussion whether the support for a non-HEVC base layer (i.e vps_base_layer_presence_idc equal to 2) is included in these profiles.

If it is considered desirable to keep vps_reserved_one_bit unchanged (e.g. to enable future non-backward-compatible extensions indicated early in the VPS), it is possible use split the proposed indication into two parts, the first re-defining vps_base_layer_internal_flag to indicate that no base layer is present in the bitstream, and a second part appearing in the VPS extension subject to vps_base_layer_internal_flag being equal to 0 to indicate values 0 to 2 as proposed above.

The proponent expressed a lack of preference between the alternatives.

It was remarked that option 2 particularly makes sense for multiview, where the use of independent layers and self-contained extractions seems especially plausible in practice.

Option 2 was agreed to be recommended by the AHG, at least in regard to the idea of allowing a bitstream to be considered conforming without containing a base layer (i.e., without containing pictures of layer 0).

It was agreed to be useful to distinguish between the case where the base layer is provided by external means versus being actually non-existing or being absent and unnecessary.

It was asked what is the actual value of the proposed semantics for vps_base_layer_presence_idc equal to 0.

It was remarked that it would be possible to simply allow bitstreams that do not contain base layers to be considered conforming without providing an explicit indicator for this case in the VPS. The contributor suggested that the indicator may be helpful to indicate to version 1 decoders that the bitstream is not an ordinary decodable bitstream (although, strictly speaking, version 1 decoders are not required to pay attention to the content of the VPS).

It was agreed that these aspects should be further studied. A later discussion resolved this issue, where it was agreed by the AHG to indicate such a bitstream using the vps_reserved_one_bit (with the syntax element name and semantics changed).

### Discussion of JCTVC-R0044 / JCT3V-I0024 "AHG10: On decoder capability indication" [M. M. Hannuksela (Nokia)]

Topic #1: Number of layers as a decoder capability

It is proposed that the decoder capability indications include the number of layers or the number of inter-layer processing operations supported by the decoder. It was suggested that the number of resampling units in a HW decoder implementation might be limited – and hence, that it would be important to include it as decoder capability.

It was asserting that adding the number of layers as a decoder capability would enable for example indicating that a decoder is capable of decoding a Scalable Main bitstream with two layers.

For example, it was suggested that the following specification text could be used:

"Capabilities of video decoders conforming to this Specification are specified in terms of the ability to decode video streams conforming to the constraints of profiles, tiers and levels. When expressing the capabilities of a decoder for a specified profile, the tier and level supported for that profile should also be expressed. When expressing the capabilities of a decoder to decode output layer sets conforming to a profile specified in Annex G or H, the capabilities of a decoder also include the number of layers that the decoder has the ability to decode. When a decoder is indicated to be capable of decoding a certain profile, a certain tier, the level levelIdc and the number of layers numLayers, the decoder shall be capable of decoding any output layer set of that profile and tier and with a level less than or equal to levelIdc and with the number of layers in the output layer set less than or equal to numLayers."

It was noted that this proposes to add another dimension to the profile/tier/level concept, in terms of decoder capabilities. It was remarked that such a distinction could alternatively be established as a profile/tier/level specification rather than by defining another dimension – e.g., there could be two profiles that have similar decoder requirements other than the number of layers they will decode. It was questioned whether introducing a new dimension was preferable to, hypothetically, introducing additional profiles or levels.

It was remarked that our current decoding requirement specification does not limit the number of layers in the bitstream or the number of layers that the decoder can be asked to decode (although it limits the dependency depth for each picture).

It was noted that the output layer set concept was used as part of the definition, such that an output layer set is the way that a bitstream is identified in terms that include a particular number of layers to be decoded, but that we currently have no way other than profile/tier/level to tie these to decoder capabilities.

One participant said that the profile should not limit the number of layers in the bitstream.

Another participant said that if we allow a bitstream to have a very substantial number of layers, we should have some concept of decoding capability that doesn't assume that all decoders must be capable of decoding all layers.

It was suggested for that question to not be decided within the AHG, but to wait for that to be decided by JCT-VC and its parent bodies, although the relevant questions and the available options should be outlined by the AHG for that purpose. Thus it was agreed to raise, as a parent-level question, whether the number of layers supported by a decoder should be part of the profile/tier/level specification or could be something chosen by individual decoder designers versus decoders just being required to decode an unlimited number of layers. This issue was further discussed in subsequent teleconferences, as recorded below.

Topic #2 Inferred profile decoding capability

It is proposed that it is explicitly specified which profiles shall be supported by a decoder, when support for decoder capability for a "layered" profile is indicated. While the exact lists of profiles that shall be supported are probably outside the scope of the mandate of AHG10, examples (in terms of specification text) are given here to make the intent of this proposal clearer:

- When the ability to decode the Scalable Main 10 profile is indicated for a decoder, the decoder shall also have the ability to decode the Scalable Main profile, the Main profile, the Main Still Picture profile and the Main 10 profile.

- When the ability to decode the Scalable Main profile or the Stereo Main profile in indicated for a decoder, the decoder shall also have the ability to decode the Main profile and the Main Still Picture profile.

It is asserted that such specification has the following benefits:

- It may reduce the signalling needed to indicate decoder capability, as all supported profiles need not be separately listed.

- Profile downgrading in mode negotiation, such as in SDP Offer/Answer procedure, becomes clear. For example, if an offerer offers a Scalable Main 10 profile but the answerer can only support Scalable Main profile, the answerer can downgrade the profile in the answer to the Scalable Main profile. Without the clearly specified downgrading of profiles, the offerer would have to list each profile separately in its offer.

It was remarked that this seemed to essentially just be an editorial remark, as it is generally understood that if the capabilities of one profile are "nested" within the capabilities of another profile, decoders should be required to recognize that and decode the bitstream. This was agreed.

It was noted also that our understanding is that the Scalable Main 10 profile includes support for base layers that conform to Main 10 profile and requires Scalable Main 10 decoders to decode Main 10 profile bitstreams (and Main profile and Main Still Picture profile bitstreams).

Topic #3 Assignment on bitstream partitions to processing units (comment on JCTVC-R0043/JCT3V-I0022)

JCTVC-R0043/JCT3V-I0022 proposes to indicate decoder capabilities as a set of profile, tier and level triplets denoted by (Pi, Ti, Li) for i in the range of 0 to q − 1, inclusive, where q is a positive integer essentially indicating the number of processing units or cores in the decoder.

It is remarked that the specification text of JCTVC-R0043/JCT3V-I0022 assumes that bitstream partitions can be assigned to any processing units as long as the processing core is indicated to be capable of decoding the required profile-tier-level combination. For a computing architecture of a decoder, this arbitrary assignment might require a certain type of a data access approach between the processing units, e.g. a bus.

It was asserted that conscious decision should be made whether such an assignment scheme of bitstream partitions is acceptable. An alternative might be to require certain order of assigning bitstream partition to processing units.

It is noted that in order to confirm whether an output layer set is decodable with certain decoding capabilities, potentially a large number of combinations of how to assign bitstream partitions to processing units has to be tested in brute-force manner. That is because no ordering of assigning a processing unit to a bitstream partition has been specified in the algorithm in G.11.3, but rather any available processing unit fulfilling the profile-tier-level requirement for the bitstream partition can be taken. The phrasing "if( there is a non-empty UnassignedProfileTierLevelTriples[ x ] such that" has been used in the algorithm in subclause G.11.3.

It was suggested to consciously decide whether such an approach is acceptable or whether bitstream partition assignment to processing units should be performed in an unambiguously specified order.

It was agreed that the ordered assignment of partitions in the decoding process seemed desirable, and text was requested to be prepared accordingly.

It was asked how the upsampling is handled for an externally-provided base layer, and said that the upsampling process is part of the enhancement decoding process in this case (and that this is already specified), so the association of the upsampler to the decoding process would be the same in this case as for when the base layer is provided internally.

### Discussion of version 2 of JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

In revision 1 of this document, the following changes were made based on the discussions in the fourth conference call of the AHG held on 8 May, 2014:

- Removal of redundant signalling of partitioning schemes and partitions in VPS VUI bitstream partition HRD parameters syntax structure.

- Conformance testing

- Definition of partition unit

- Definition of conformance of partitions, and the conformance of the bitstream based on partitions.

In addition, the proponent further considered whether it is not necessary to include the resampling capability in the definition of level capabilities, and if yes, how to specify, and asserted that it is not necessary, because of two reasons: 1) typically the complexity of resampler is much smaller compared to complexity of the decoder, e.g. less than 5%; and 2) for any given bitstream, the layer structure, including which layers of which spatial resolutions need resampling, is known and the resampling capability needed is also known.

It was remarked that, from the VPS, it cannot be concluded whether a layer is a spatial enhancement layer or quality enhancement layer, as this is determined by the scaled reference layer offsets, which are in the SPS.

The question remained open whether there is a need to specify resampling capability distinctly as part of profile/tier/level or VPS-level syntax.

## Sixth teleconference

### General

The sixth teleconference was held at 0800 Seattle time on Friday, 23 May 2014 with a duration of 3 hours. The teleconference was hosted through Microsoft Lync, as with the previous calls. Approximately 15 people attended the teleconference (with a peak of approximately 13 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

A preliminary report of the sixth teleconference was sent by email shortly after the teleconference, on 23 May 2014 (Seattle time).

Prior to the teleconference, the following documents had been registered and uploaded for discussion:

- Version 3 of JCTVC-R0042 / JCT3V-I0021 "AHG10: Profile indication for additional layer sets" [M. M. Hannuksela (Nokia)]

- Version 3 of JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

- JCTVC-R0046 / JCT3V-I0026 "AHG10: Comments on JCTVC-R0043/JCT3V-I0022" [M. M. Hannuksela (Nokia)]

Additional teleconference plans were made as follows:

- The previous plan to hold a call on Friday 6th June at 0900 (Seattle time) was confirmed.

- Another call was planned for Thursday June 12th at 0900 (Seattle time).

### Discussion of Version 3 of JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

In version 3 of this document (revision 2), with respect to the decoder capability description in version v2 of this document, the assignment of partitions to cores is restricted as follows: if two partitions with indices i and j are assigned to cores with indices m and n, respectively, and if i is less than j, m is less than n. Partitions with increasing index within the partitioning scheme are assigned to cores with increasing index; some cores, however, may not be assigned any partitions and hence there may be gaps in the assignment of the cores. This change was said to be in response to an agreed suggestion from the fifth conference call of the AhG held on 15 May, 2014.

Conceptually, each "core" (each value of i in the proposed G.11.3) is proposed to have one "HEVC version 1" quantity of decoding capability, which can be assigned to one partition of the bitstream, and each partition of the bitstream may contain multiple layers, where the sum of all CTUs in the layers of an AU within the partition is equivalent to one picture of the v1 capability. And each partition is associated with a profile/tier/level triplet.

An output layer set can have one or more partitionings defined in associated syntax. Each partition can have an associated HRD model.

A participant commented that he liked the idea of defining conformance based on partitioning the layers of the bitstream, but thought that when the number of layers increases, the sample processing speed capability should decrease – to account for layer-specific processing overhead – but that may not be clear by how much or how to compensate for that overhead. For example, an X% reduction relative to the total processing capability of the single-layer decoder could be subtracted for each additional layer:

- One layer: 100% capability

- Two layers: 100 − X% capability (e.g., 90%)

- Three layers: 100 − 2*X% capability (e.g., 80%)

- etc., for total capability 100 − (N − 1) * X % for N layers (e.g., 60% reduced for 7 layers).

It was asked whether such a scheme could be expressed using an SEI message rather than being the basis for conformance requirements of a decoder. This would be possible, but we do need to have some basis for establishing bitstream and decoder conformance.

It was commented, and agreed, that the concept, and the possible X% reduction scheme, is only needed if the number of layers is large (e.g., more than 3 layers and X more than 3%), and that most anticipated use cases would not need more than 3 layers. Another participant suggested that using more layers, e.g., 8–12 layers, may be used for dynamic streaming applications (although, today, this scenario typically uses simulcast), and another participant noted that the CTC for 3D-HEVC uses 6 layers.

This raises again the question of whether a profile is needed that restricts the number of layers to only 2 or 3.

### Discussion of JCTVC-R0046 / JCT3V-I0026 "AHG10: Comments on JCTVC-R0043/JCT3V-I0022" [M. M. Hannuksela (Nokia)]

This contribution contains technical remarks on JCTVC-R0043/JCT3V-I0022, proposes some changes relative to it, and identifies some topics that were suggested to be studied further.

On partition-wise conformance testing

JCTVC-R0043/JCT3V-I0022 proposes bitstream conformance to be tested partition-wise, summarized in that contribution as follows:

"The bitstream conformance tests are specified to test the conformance of each partition, in a manner that decoded pictures and other necessary information for all direct or indirect reference layer of the layers in the partition are assumed to be available similarly as the current specification for the base layer when it is externally provided."

This design choice was asserted to have several challenges, including:

- The properties of the reference-layer pictures affect the decoding process, e.g. in VPS activation and layer-wise start-up. The exhaustiveness of the present specification text in this regard was not felt to be sufficient.

- The joint HRD behavior of the bitstream partitions is not controlled. This is asserted to lead to similar issues as reported in JCTVC-Q0188 for the joint HRD conformance checking of the external base layer and HEVC enhancement layers. In particular, it was suggested that the buffering for bitstream partitions should be such that

  - The pictures in the referenced bitstream partition have to be decoded before they are needed as inter-layer references in a "predicted" bitstream partition.

  - The pictures in the referenced bitstream partition must still remain in the DPB when they are needed as inter-layer references in a "predicted" bitstream partition.

- When conformance of each partition is tested separately, it is asserted that the above-mentioned cross-partition constraints are not checked and cannot be verified.

- It is allowed to create output layer sets and bitstream partitions such that pictures of more than bitstream partition of an output layer set are output. It was asserted that the output of pictures of the same access unit should be synchronized, and bitstream and decoder conformance specifications should guarantee such synchronized output. However, it was asserted that when partition-wise bitstream conformance testing is performed, bitstream and decoder conformance specifications do not guarantee such synchronized output.

To overcome the above-described challenges, it was proposed to specify bitstream conformance to be tested for the entire sub-bitstream of the output layer set using the bitstream partition buffering (BPB) operation.

Precise text was not provided for the proposed modification.

A proponent of JCTVC-R0043/JCT3V-I0022 agreed that it would be desirable to establish bitstream conformance in that manner.

On level specifications

It was remarked that in JCTVC-R0043, level limits are essentially specified as if all pictures of a partition unit would be handled as one big picture. For example, partitionSizeinSamplesY is derived as a sum of PicSizeInSamplesY and many of the constraints of Annex A apply without changes other than referring to partitionSizeInSamplesY rather than PicSizeInSamplesY. For instance, partitionSizeinSamplesY is limited by MaxLumaPs of the level.

The contribution said that the way levels are specified in JCTVC-R0043 has some nice consequences, such as the ability to downgrade a profile in session negotiation while keeping the level unchanged.

As a general remark, the contribution asserted that JCTVC-R0043 lacks the intuitive association of levels to specific spatial resolutions of individual pictures or layers. Options that could be considered instead were suggested to include:

- Include a scale factor in the level limits, as was done e.g. in MVC (mvcScaleFactor is equal to 2) and MVC+D (mvcdScaleFactor is equal to 2.5). It was asserted that the intuitive association of levels to specific spatial resolutions would then be maintained with the typical/target number of layers and the typical/target spatial resolution scaling factor.

- Include a scale factor in the level limits that is a function or a multiple of the number of layers in the output layer sets. It was asserted that the intuitive association of levels to specific spatial resolutions would then be maintained with the typical/target spatial resolution scaling factor.

These comments were noted.

In addition, it was suggested that the following details of the level specifications proposed in JCTVC-R0043 require further studying, particularly in regard to the following aspects:

- Picture aspect ratio constraints.

The following picture aspect ratio constraints are proposed in JCTVC-R0043:

- For each layer in the partition, the value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- For each layer in the partition, the value of pic_height_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

The above specifications allow more extreme picture aspect ratios compared to those allowed by Annex A profiles, in cases where the layer is smaller than the total capacity. It was suggested to be considered whether picture aspect ratio limitations should be kept similar to or the same as in Annex A.

This was noted, but it was not clear whether some action was needed about it.

- CtbSizeY constraint.

The following constraint on CtbSizeY is proposed in JCTVC-R0043:

For level 5 and higher levels, the value of CtbSizeY for each layer in the partition shall be equal to 32 or 64.

It was reported that the above constraint does not overcome "problem 1" described in JCTVC-Q0145 as follows:

"If more than 4 SNR layers of 720p resolution (or equivalent number of luma pixels e.g. more layers with combined spatial and SNR layers) are needed, then Level 5 or above would have to be used. Consequently, the value of CtbSizeY shall be equal to 32 or 64 (per item e of the general limits). For resolution like 720p or lower, this restriction disallows the use of smaller coding tree block sizes such as 16x16 and can therefore result in sub-optimal coding efficiency."

It was suggested that the importance of the problem should be agreed by AHG10, and, if considered important, a change in the CtbSizeY constraint should be designed to solve the problem.

In discussion, it seemed like it might not be a big enough problem to worry about.

- Constraints on the rate or number of tiles

In JCTVC-R0043 the number of tiles (or, more precisely, the rate of tiles) is proposed to be limited for partition units rather than for pictures. Such a limitation would allow, for example, a partition unit where an EL picture is coded as one tile and a BL picture with large number of tiles (or vice versa). It was suggested that it should be considered whether constraints to disallow radically uneven distribution of tiles across layers could cause problems for decoder's capability of handling all bitstream partitions conforming to a certain level. If so, it is suggested to design suitable picture-wise limits for the number or rate of tiles.

In discussion, it was noted that some degree of "unfriendly" tile structuring is allowed in the single-layer case. However, it seemed somewhat difficult to fully understand the implications of the tile structuring issue, and it was agreed that further study would be needed on that.

### Discussion of partition-based constraints

Partition-based constraints versus layer-based constraints were discussed in regard to the concept of repurposing one single-layer decoder to decode multiple layers.

One suggestion was to define the partition-based scheme, but decide later whether to allow one partition to contain more than one layer (e.g., as a profile-specific constraint). Another view that was expressed was that we should avoid defining a generalized scheme until we need it.

The view was expressed that the value of the partition-based scheme, as a conformance model (versus, e.g., an SEI message), is partly a question of how many layers we want to consider important.

A contributor remarked that if we don't need the concept of multiple layer conformance, we don't need the concept of partitioned HRD operation with multiple layers assigned to a single HRD.

A contributor suggested that it is important to ensure that a bitstream subset can be decoded by a particular core – to guarantee its properties to be adequate for core sharing.

A suggestion was to require the bitstream to contain characteristics associated with each layer being one partition, and to optionally allow the encoder to also provide characteristics for multi-layer partitions. Another participant suggested that this brings back the idea that the multi-layer partition information becomes non-essential and could therefore hypothetically be put into an SEI message.

Further consideration was needed regarding these issues.

### Discussion of version 3 of JCTVC-R0042 / JCT3V-I0021 [M. M. Hannuksela (Nokia)]

There was insufficient time to get to discussion of this contribution in the sixth teleconference.

## Seventh teleconference

### General

The seventh teleconference was held at 0900 Seattle time on Friday, 6 June 2014 with a duration of 3.5 hours. (The extended length of the call was approved without objection during the call.) The teleconference was hosted through Microsoft Lync, as with the previous calls. Approximately 15 people attended the teleconference (with a peak of approximately 14 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

A preliminary report of the seventh teleconference was sent by email shortly after the teleconference, on 6 June 2014 (Seattle time).

It was noted that in the previous teleconference, there had been insufficient time to discuss the following contribution:

- Version 3 of JCTVC-R0042 / JCT3V-I0021 "AHG10: Profile indication for additional layer sets" [M. M. Hannuksela (Nokia)]

Prior to the teleconference, the following documents had been registered and uploaded for discussion:

- Version 2 of JCTVC-R0041 / JCT3V-I0023 "Use of general_profile_space for auxiliary picture profile indication" [J. Boyce (Vidyo)]

- Versions 4 and 5 of JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

Additional teleconference plans were made as follows:

- The previous plan to hold a call on Thursday June 12th at 0900 (Seattle time) was confirmed.

- Another call was planned for Tuesday June 17th at 0900 (Seattle time).

It was remarked and agreed that we should try to merge the agreed changes into a single document as an output of the AHG activity (e.g., relative to the MV-HEVC text). Ye-Kui Wang and Miska Hannuksela were asked to begin preparing the output text.

### Discussion of Version 5 of JCTVC-R0043 / JCT3V-I0022 "SHVC/MV-HEVC level definitions and related decoder capability requirements" [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

Version 5 of this document contained the following:

- Responses to comments received during the sixth conference call of the AHG held on 22 May, 2014.

- Updated signalling of bitstream partitions and profile, tier and level in a way asserted to be more in tune with the proposed partition-specific profile, tier and level concept.

- Discussion and changes to signalling of HRD parameters in VPS and SEI messages, including:

  - Signalling of partitioning scheme index and output layer set index in the bitstream partition nesting SEI message

  - Signalling a flag in the vps_vui_bsp_hrd_parameters( ) to allow not signalling the HRD parameters for some partitioning schemes of output layer sets

  - Signalling of combination of index of HRD parameter and index of initial cpb removal information to be used in each delivery schedule of partition of an output layer set

This version did not change the associated proposed specification text.

Asserted drawbacks of existing level specifications:

It was asked at the sixth conference call what was wrong with the current level definitions. Here is a list of problems of the current specification of level definitions:

- The current specification of level definitions was asserted to not be friendly to SHVC/MV-HEVC decoders built based on existing single-layer decoders reusing the hardware part as is, while such decoder implementations are most likely the most important and most likely SHVC/MV-HEVC decoder implementations that make use of the high-level syntax only design of SHVC/MV-HEVC.

  - For example, if an SHVC decoder is implemented in a way that consists of 4 cores of HEVC Level 3.1 decoders to be able to decode 4 SNR layers of 720p, per the current level definition, it would have to be said to be conforming to Level 4 or above. Consequently, the decoder needs to be able to decode any Level 4 bitstreams. However, with only high-level syntax changes (i.e. no changes to the hardware cores), such a decoder would not be able to decode an SHVC Level 4 bitstream with 2 SNR layers of 1080p resolution.

  - For another example, if an SHVC decoder is implemented in a way that consists of 4 cores of HEVC 3.1 decoders to be able to decode 4 SNR layers of 720p, per the current level definition, it would be said to be conforming to Level 4 or above. Consequently, each enhancement-layer picture can have more than 3 tile rows and more than 3 tile columns, e.g. 5 tile rows and 5 tile columns, each tile with width of 256 luma samples and height of 144 luma samples. However, this would go beyond the Level 3.1 limits and consequently the decoder cores for decoding the enhancement layers would have problem.

- All items in subclause A.4.1 of HEVC text are currently specified to be applied to each layer. However, some of these items are not directly applicable to each layer. However, for item d on DPB size, the SPS syntax element is not applicable for enhancement layers.

- Bistream- or partition-specific restrictions on CPB size (by items h and i in subclause A.4.1 of HEVC text) are needed. However, the items h and i in subclause A.4.1 of HEVC text cannot be directly applied on bitstream or partition level, because if directly applied, the same CPB size limit for single-layer bitstreams would also be the limit for multi-layer bitstreams or partitions. This is not scalable to the number of layers in a bitstream or partition and would only allow for low picture quality when there are many layers in a bitstream or partition. Similarly, the restrictions by items b, c, d, g, h, i, and j in subclause A.4.2 of HEVC text are currently specified to be layer-specific. However, bitstream-specific or partition-specific restriction by these items should be specified.

Bitstream partitions with more than one layer:

One or more partitioning schemes may be defined for an output layer set (OLS), and each partitioning scheme contains one or more bitstream partitions. A bitstream partition is said to be conforming to a particular profile, tier and level when all the constraints specified for that particular profile, tier, and level, are satisfied during the bitstream conformance test. The bitstream conformance test is performed based on the HRD parameters of all partitions and the partitions themselves. A bitstream (of an OLS) is said to be conforming to a list of profile-tier-level triplets, if there exists at least one partitioning scheme specified such that the i-th bitstream partition conforms to the i-th profile-tier-level triplet, i.e. (P, T, L)-triplets, in the list, and the number of bitstream partitions in the partitioning scheme is equal to the number of triplets in the list. A figure was provided in the contribution to show an example of multiple partitioning schemes of an OLS of a bitstream, and the (P, T, L)-triplets associated with each bitstream partition.

For ease of description, the contribution highlighted two partitioning schemes of each OLS: the all-layer partitioning scheme where there is only one bitstream partition that includes all the layers of the OLS, and the per-layer partitioning scheme where there are as many bitstream partitions as there are layers in the OLS, and each layer belongs to a separate bitstream partition.

Decoder capability is defined to indicate what bitstreams can be decoded by a decoder that advertises a certain capability. If the conformance parameters of only the per-layer partitioning scheme are defined, that bitstream would only be conforming to a very specific list of (P, T, L)-triplets. A decoder that has at least as many cores as the number of layers in the bitstream such that each bitstream partition (containing one layer each) can be decoded by a different core, is said to be able to decode the bitstream. A second figure was provided in the contribution to show how SHVC decoder 1 (made out of 3 HEVC decoder cores) is able to determine that the bitstream OLS2 is decodable by comparing the capabilities of the decoder cores and the (P, T, L)-triplets of the per-layer partitioning scheme.

However, defining the conformance parameters of per-layer partitioning scheme is not sufficient to determine all the conformance parameters for bitstream partitions that contain more than one layer; parameters such a resolution, picture unit size, etc. could be derived from the per-layer partitioning scheme parameters, but parameters related to bit rate and timing such as CPB removal time cannot be derived. Without knowing the combined requirement of all the layers in a bitstream partition, assuming the bitstream partition has more than one layer, it would not be possible to easily determine whether a decoder core would be able to decode the bitstream partition. If only partitioning scheme 0 is specified, as was shown in the provided figure, SHVC decoder 0 would not be able to determine whether the bitstream OLS2 is decodable. If the partitioning schemes 1 or 2 are specified, SHVC decoder 0 could conclude that it would be able to decode the bitstream.

It was asserted that, therefore, if only the conformance parameters for the per-layer partitioning scheme are specified, only those decoders that decode one layer in each decoder core would be able to advertise that the decoder is conforming to a particular list of (P, T, L)-triplets. Decoders that are capable of decoding more than one layer have no way of using their capabilities to decode more than one layer in a core.

Signalling partitioning schemes in SEI messages

A bitstream could be conforming to multiple lists of (P, T, L)-triplets. It was commented during the previous conference call that it would be sufficient to specify the per-layer partitioning scheme in the VPS, and the remaining partitioning schemes could be specified in SEI messages. However, firstly all the signalled partitioning schemes are parts of conformance definitions for both bitstreams and decoders, and secondly the partitioning scheme information is essential for session negotiation, as this information is needed to determine what bitstreams and what operating points are decodable by a decoder, and based on this information a certain bitstream may be requested. Therefore, signalling of only some partitioning schemes in the parameter sets, while signalling other information in the SEI messages would thus not be a good design choice.

Testing of bitstream conformance

Based on comments received during the sixth conference call of AHG, the bitstream conformance tests are modified so that each test applies to the sub-bitstream of an output layer set, instead of a bitstream partition.

HRD parameters signalled in the base VPS

In the base VPS, a list of HRD parameter structures are signalled and each HRD parameter structure is associated with a layer set that is specified by the syntax element hrd_layer_set_idx[ ]. If the syntax structure hrd_parameters( ) only contained HRD parameters that are related to the CPB, then it would be sufficient to relate each hrd_parameters( ) syntax structure with a layer set. However, the hrd_parameters( ) structure also includes syntax elements fixed_pic_rate_general_flag[ ], fixed_pic_rate_cvs_flag[ ], and elemental_duration_in_tc_minus1[ ] and these syntax elements are dependent on the OLS to which they apply to, rather than just the layer set. Therefore, assigning hrd_parameters( ) to a layer set may have the unintended consequence that only those values of syntax elements fixed_pic_rate_general_flag[ ], fixed_pic_rate_cvs_flag[ ], and elemental_duration_in_tc_minus1[ ] could be signalled that apply to all output layer sets corresponding to that layer set. This is the reason why in the attached spec text the proponent made the assignment of the hrd_parameters( ) syntax structure to be OLS-specific. For this reason and also because some of the syntax elements in the buffering period, picture timing, and decoding unit information SEI messages are directly related to DPB behaviors, the proponent added the OLS index to the bitstream partition nesting SEI message.

Removal of bitstream partition HRD parameters SEI message

Bitstream partition HRD parameters SEI message provides means to provide HRD parameters for bitstream partitions of OLS and HRD parameters for the described bitstream partitions. However, partitions of OLSs are signalled in VPS extension and HRD parameters for the described partitions may be present in VPS VUI, the functionality that are enabled by bitstream partition HRD parameters SEI message has been covered, i.e. the SEI message is not really useful. Yet inclusion of this SEI message resulted in a lot of texts, e.g. in the specification of bitstream conformance tests, which is already complicated even without those significant amount of text removed. The proponent thus proposed to remove the SEI message.

It was commented that this SEI message related to the desire to have layers added after-the-fact, but the proponent responded that this use case was not really properly supported anyway. and that this SEI message would still really not be needed for it. Another participant indicated that some further work was ongoing regarding that topic.

On the bitstream partition nesting SEI message

Bitstream partition nesting SEI message is contained in a scalable nesting SEI message and applies to a bitstream partition of an operation point (i.e., layer set and hightest temporal ID) that is specified in the scalable nesting SEI message. However, since a layer set can be associated with more than one output layer set, it is unclear as to which output layer set the bitstream partition nesting SEI message applies to. Furthermore, it is also not clear as to which partitioning scheme of the output layer set the bitstream partition nesting SEI message applies to. Therefore, it is proposed that the index of the OLS and the index of the partitioning scheme of the OLS to which bitstream partition belongs to are added in the syntax of bitstream partition nesting SEI message.

It was commented that the prior scheme may have been motivated to have different delivery schedules on a partition basis. The proponent indicated that another way to approach this would be to send different HRD parameters, although it was commented that the number of HRD parameters sent might become large for this. The proponent indicated that there might not really be an increase of the number of HRD parameters caused by that. This issue seemed worth further study.

On signalling a combination of delivery schedule index

When HRD parameters are signalled for partitition of an OLS, one or more delivery schedule may be signalled for the partition in which for each delivery schedule, an index of HRD parameter to be used and and index of which initial CPB removal information to be used for the delivery schedule. It is unclear why combination of both indices are needed for each delivery schedule of an partition.

Overall discussion

It was suggested that the use cases may be a bit exotic and may make the text too complicated to justify the partitioning scheme for level definition with multiple layers in a partition – and that perhaps just having layer-specific capability definition may be adequate and that fully clear specification of whether a decoder can actually decode a particular bitstream may not be essential. An alternative view was expressed that it is important to have a clear specification of the conformance capabilities needed without apparent gross overprovisioning.

Another participant commented that the best alternative to this approach is to have a very limited number of layers supported in a profile, and that this would be too constraining. The participant indicated that he had studied the proposal and was comfortable that it was a good approach.

Another participant commented that some encoders might not want to use the full supported flexibility.

The group was inclined to adopt the proposed approach, unless further study uncovers some significant problem in it.

A suggestion was to have a limit – e.g., to have a maximum number of layers established for a partition – suggesting a maximum of 4 layers per partition. This was agreed to be a reasonable restriction.

There were additional details discussed.

- Regarding the proposed deletion of the bitstream partition HRD parameters SEI message: This was agreed to be recommended by the AHG (pending further study indicating otherwise).

- Regarding section 2.1 of the contribution (HRD parameters signalled in the base VPS), a participant asked to review the text relating to this aspect, and this was shown and seemed OK.

- It was noted that in the proposed scheme there is no longer a (P, T, L) indicated for the entire OLS. Such a (P, T, L) could potentially be specified, although it might not obey the limit of 4 layers per partition. This was agreed to be desirable to enable – the ability for encoders to "opt out" of the partition-specific limit concept. (Further consideration of this topic was conducted in subsequent teleconferences.)

- It was noted that some rewording may be needed, either now or in the future, in the profile definitions, relating to the addition of additional profiles in the future, and it was agreed to go ahead and take care of this to eliminate the need for future changes and to ensure that it will work properly.

- Inference rules in the section relating to num_signalled_partitioning_schemes, num_partitions_in_scheme_minus1, and layer_included_in_partition_flag were discussed to understand why they were proposed. The proponent indicated that this was just there for the sake of completeness in relation to the base layer (defined with consideration of the case with an externally provided base layer), because a loop index starts from 1 rather than 0 in the syntax.

- The bitstream partition nesting SEI message was discussed, and it was agreed to encourage further study of whether it would be desirable to enable more than one OLS index and more than one partitioning scheme index and perhaps more than one BSP index, which could reduce the number of such SEI messages needed in the bitstream.

- The modification of the bitstream partition initial arrival time SEI message was discussed, and agreed that there was an unnecessary removal on a mention of scalable nesting in the proposed text. This change should not be made.

- Why "of the same layer" was deleted in the profile-specific level limits - that text change did not seem adequate – the intent was to consider output times of pictures in different AUs. Further study was needed on this.

- Regarding G.11.2.1 bullet d, max_vps_dec_pic_buffering and PicSizeInSamplesY where discussed in regard to establishing an appropriate upper bound – it was suggested to use the value from the VPS rather than SPS for PicSizeInSamplesY. This was agreed. Notes relating to this topic were also in JCTVC-Q0103.

### Discussion of Version 3 of JCTVC-R0042 / JCT3V-I0021 "AHG10: Profile indication for additional layer sets" [M. M. Hannuksela (Nokia)]

Version 3 of this document provides the specification text responding to the agreements in the fifth AHG10 teleconference. The proposed specification text changes are summarized in Section 3 of revision 2 of the contribution.

Regarding the ability to have a bitstream that does not contain a layer 0, a proposed VPS signalling was provided along with modifications relating to monochrome.

A vps_layer_not_extracted_flag is proposed to replace a vps_reserved_one_bit in the VPS syntax. Additional changes were included to reflect the implications of this on OLSs. This was agreed to be desirable.

It was asked whether we still need the rewriting process to create a base layer. It was remarked that it would be desirable to remove it if it is not needed. Further study of that was agreed to be needed.

In 10.2, SPS NUT, PPS NUT and EOB NUT should also be kept for nuh_layer_id that belongs to the list of layers in the list target. This was agreed (if the proposal in question is to be adopted).

### Discussion of Version 2 of JCTVC-R0041 / JCT3V-I0023 "Use of general_profile_space for auxiliary picture profile indication" [J. Boyce (Vidyo)]

Version 2 of the contribution proposes alternative proposed specification changes, based upon AHG10 discussion, in which profile names do not change when general_profile_space equal to 1.

In the v2 version of this proposal, general_profile_space is still modified to be equal to 1, but the profile names are not changed when general_profile_space is equal to 1. With this alternative, to indicate that a bitstream may contain NAL units with nuh_layer_id greater than 0, the bitstream can be indicated to conform to a single layer profile with general_profile_space equal to 1, e.g. Main profile with general_profile_space equal to 1.

The suggested advantage here is for the ordinary v1 profile decoder to have the ability to identify that it cannot decode the bitstream. (This was later concluded not to be the case, in discussions for the eighth teleconference.)

The difference between the proposal and the prior version of the proposal is essentially editorial – just a matter of whether to re-use profile names or define new profile names.

It was remarked that a v1 decoder would just ignore the NAL units of the relevant bitstream and never activate any parameter sets if there is no such indication and that this would not necessarily be a problem.

The basic question is just a matter of whether there should be a positive indication that the bitstream should be rejected versus having a bitstream that it would continue to scan while ignoring the contents of all of its NAL units. (This was later concluded not to be the case, in discussions for the eighth teleconference.)

It was agreed to defer the question of the desirability of setting general_profile_space equal to 1 to be decided at the main meeting of JCT-VC and its parent bodies.

Again, the question of whether we need the rewriting process for independent non-base layers also remains for further study, as noted above.

## Eighth teleconference

### General

The eighth teleconference was held at 0900 Seattle time on Thursday, 12 June 2014 with a duration of 2.5 hours. The teleconference was hosted through Microsoft Lync, as with the previous calls. Approximately 10 people attended the teleconference (with a peak of approximately 10 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

A preliminary report of the eighth teleconference was sent by email and uploaded to the JCT-VC and JCT-3V web sites on the day after the teleconference, on 13 June 2014 (Seattle time).

Additional teleconference plans were made as follows:

- The previous plan to hold a call on Tuesday June 17th at 0900 (Seattle time) was confirmed.

It was remarked and agreed that we should try to merge the agreed changes into a single document as an output of the AHG activity (e.g., relative to the MV-HEVC text). Ye-Kui Wang and Miska Hannuksela were asked to begin preparing the output text.

During the teleconference, J. Boyce was also asked to prepare draft text for how we could define bitstream characteristics of an independent non-base layer that otherwise conforms to existing single-layer profiles while avoiding the need to specify a rewriting process to convert it into a base layer. (See more detailed notes on this topic below).

Following the teleconference, the following draft texts were provided for consideration:

- Two variants of edited changes prepared as described above by Ye-Kui Wang and Miska Hannuksela. These were submitted as attachments to a draft AHG report uploaded on 13 June 2014 (Seattle time).

- Text edited toward potential removal of the need to specify a rewriting process, which was submitted by J. Boyce in a version 3 of JCTVC-R0041 / JCT3V-I0023.

### Further discussion of Version 2 of JCTVC-R0041 / JCT3V-I0023 "Use of general_profile_space for auxiliary picture profile indication" [J. Boyce (Vidyo)]

It was asked whether, in the current specification, if the bitstream contains a base layer, the first AU of the bitstream is required to contain a base layer picture. The participants said that the answer is Yes – the first AU is required to be an IRAP AU, which is an AU that contains a base layer picture that is an IRAP.

It was noted that the general_profile_space in an SPS with layer ID > 0 would not be detected by a v1 decoder.

It was asked what a v1 decoder does for general_profile_space > 0 in an SPS. It was suggested that, per the spec, the SPS would be ignored by the decoder unless it is activated, and this SPS – as proposed in JCTVC-R0041 for SHVC – would not ever be activated in a bitstream as observed by a v1 decoder.

The proponent indicated that the potential value of the general_profile_space > 0 concept was not so much a matter of decoding process as specified, but more a matter of how systems might handle such bitstreams – and that it would help in the clarity of the specification of capabilities by providing an indication in the (P, T, L) syntax rather than depending on the layer ID of the activating NAL unit.

### Discussion of the need for a specified rewriting process

A participant suggested that if there are auxiliary pictures of other than monochrome 8-bit format, and we do think there are important use cases for such auxiliary pictures, then we should either

- A: keep the rewriting process, with the simplification as proposed in R0042, or

- B: Remove the rewriting process but add the “plus” decoder capabiilty expression.

Otherwise (if there are no auxiliary pictures of other than monochrome 8-bit OR we do think there are important use cases for such auxiliary pictures), then the participant suggested it to be more preferable to specify:

- C: remove the rewriting process and slightly changing the Monochrome 8-bit profile to require the “plus” capability.

For both options B and C, the participant said that it is a of course a good question on what exactly list of capabilities are required for the “plus” capability.

It was agreed that we do think there could be important use cases for auxiliary pictures other than monochrome 8-bit.

Between options A and B, it was suggested that without the rewriting process, it may be difficult to specify the constraints that make the bitstream decodable as a "plus" bitstream. However, another participant thought this should be editorially feasible.

J. Boyce was asked to draft such text for consideration by the end of the day on 13 June.

Aspects to consider were discussed and identified as follows:

- Parameter set parsing

- POC decoding per Annex F

- RPS decoding

It was noted that part of the rewriting capability can already be indicated by a VUI flag (base_layer_parameter_set_compatibility_flag).

In the discussion, there appeared to be no real need to have extra capabilities required, as there may be no significant benefit to using non-v1 header-level functionality for an independent base layer.

### Discussion of partitioning specifications

Regarding whether to mandate the presence of a partition that includes all layers of an OLS, the disadvantage of doing this would be that the encoder would need to ensure that the bitstream conforms to the constraints of this (lack of) partitioning – although the encoder could hypothetically just set the level specification to a high level to get around that. It was suggested that if we do not have such a partition, the idea of associating an OLS with a (P, T, L) becomes decoupled, and that this would be a significant change of concept.

Having simple specification of layer-specific partitioning also seems desirable.

As an example, we discussed  an OLS that is identified as level 4.1 but has a partitioning within it that consists of two level 4 partitions. The envisioned scheme is such that a decoder could be said to have "two level 4" decoding core capabilities, and would be able to decoded (and may be required to decode) all bitstreams that contain at least one partitioning scheme that is compatible with that capability.

### Discussion of drafting efforts

Ye-Kui Wang and Miska Hannuksela had been working on drafting of text to reflect what had been agreed thus far. Two variants of the text had been developed in regard to the aspects discussed in the preceding section. It was agreed that text for both variants should be uploaded rapidly (by EOD 13 June) for study along with a draft AHG report.

Subclause F.7.4.4 (P, T, L) semantics was suggested to be checked to ensure proper drafting, as there had been some problem in preparing the text in that section.

Further study of these texts was planned, in preparation for the next teleconference.

## Ninth teleconference

### General

The ninth teleconference was held at 0900 Seattle time on Tuesday, 17 June 2014 with a duration of 3.5 hours. (The extended length of the call was approved without objection during the call.) The teleconference was hosted through Microsoft Lync, as with the previous calls. Approximately 10 people attended the teleconference (with a peak of approximately 10 people on the call at once). The screen content coding capability of the Lync software was again used during the teleconference to show the presented contribution documents and the meeting notes as they were being recorded.

A preliminary report of the ninth teleconference was sent by email and uploaded to the JCT-VC and JCT-3V web sites shortly after the teleconference, on 17 June 2014 (Seattle time).

Prior to the teleconference, the following documents were submitted for consideration:

- JCTVC-R0041 / JCT3V-I0023 version 4, "Use of general_profile_space for auxiliary picture profile indication" [J. Boyce (Vidyo)], with text edited toward potential removal of the need to specify a rewriting process.

- JCTVC-R0052 / JCT3V-I0028, "Picture size related constraints and level limits" [S. Deshpande (Sharp), Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm), M. M. Hannuksela (Nokia)]

- JCTVC-R0053 / JCT3V-I0029 "AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture" [M. M. Hannuksela (Nokia)]

- JCTVC-R0054 / JCT3V-I0030 "AHG10: Comments on JCTVC-R0041/JCT3V-I0023" [M. M. Hannuksela (Nokia)]

During the teleconference, it was noted that JCTVC-R0053 / JCT3V-I0029 is on a topic somewhat different than what fits into the main activities conducted in this AHG, and there seemed to be insufficient time to discuss it, so its consideration was deferred.

Miska Hannuksela, Ye-Kui Wang, and Jill Boyce were asked to prepare the AHG output text for inclusion with the final AHG report.

### JCTVC-R0041 / JCT3V-I0023 version 4, "Use of general_profile_space for auxiliary picture profile indication" [J. Boyce (Vidyo)], with text edited toward potential removal of the need to specify a rewriting process.

This contribution proposes text to define the "plus" capability as an alternative to specifying a rewriting process for independent non-base layer decoding (INBLD) capability.

(Note that it was remarked that the parameter sets of an independent non-base layer are required to either be in layer 0 or in the layer that contains its VCL NAL units.)

The concept in this proposed text could be applied with or without using general_profile_space equal to 1 for the INBLD.

The text could, hypothetically, be put either in Annex A or Annex F.

### JCTVC-R0054 / JCT3V-I0030 "AHG10: Comments on JCTVC-R0041/JCT3V-I0023" [M. M. Hannuksela (Nokia)]

The contribution provides some comments on JCTVC-R0041/JCT3V-I0023. Primarily, the proposal in JCTVC-R0041/JCT3V-I0023 version 4 is commented on, while some comments are also made on JCTVC-R0041/JCT3V-I0023 version 3.

The following general remarks are made, summarizing some of the specific comments:

- It is asserted that a new sub-bitstream extraction process is needed such that keeps the essential NAL units from the base layer (VPS, EOB, EOS) and otherwise keeps the layers of the OLS with the independent non-base layer or additional layer set. Such extraction is similar to rewriting proposed in JCTVC-R0042/JCT3V-I0021, which raises the question of why the rewriting process should be avoided.

- The semantic constraints could potentially make it easier for an encoder to write an independent non-base layer with "INBLD" capability. However, it is asserted that they could disallow the encoder to use some features that are desirable, such as sending poc_resetting_period_id for error resilience or discardable_flag for scalability/thinning indications. Furthermore, it was asserted to remain unclear whether the decoder can use version 1 decoding processes for decoding the layers with "INBLD" property. It is suggested that the rewriting process, such as that proposed in JCTVC-R0042/JCT3V-I0021, makes it very clear that the version 1 decoding process is applicable as such, after the nuh_layer_id values of the NAL unit have been changed according to the rewriting process.

The contribution noted that VPS, end of sequence and end of bitstream are currently required to have layer ID equal to 0.

It was noted that JCTVC-R0042/JCT3V-I0021 has a proposal to remove the SEI messages without removing the specification of the rewriting process.

It was suggested that we could allow VPS to be repeated in a higher layer.

It was suggested that we could actually define the "plus" capability concept without removing the specification of the rewriting process.

The contribution asserted that the constraints suggested in JCTVC-R0042/JCT3V-I0021 are too restrictive, or could be addressed without affecting the ability to use the approach.

There is a statement in the bitstream rewriting process that may "hide" some bitstream conformance requirements, by just requiring its output to conform to the Annex A profile. The text does not currently provide the details behind the requirement (e.g., perhaps in regard to POC handling).

Essentially, it seems to be only an editorial matter whether we describe the technical content as a rewriting process or not, but if we try to express it in a different way, it may not be clear exactly what is the minimum set of constraints that would fulfil the requirements.

It was agreed that, at least in a NOTE, we should try to provide information to encoders about how they can satisfy the requirements.

Further study is needed to determine whether it would be possible to remove the rewriting process specification without excessive undesirable side effects.

It was agreed to adopt the simplifications of the bitstream rewriting process of JCTVC-R0042.

It was agreed to define the INBLD capability (in Annex F) for profiles specified in Annex A (with the use of general_profile_space remaining TBD). The question of whether rewriting can be removed also remaining TBD – this will be drafted with wording that assumes rewriting is kept.

### Discussion of whether to mandate a PTL for each complete OLS

Two variants of the text had been provided, one with and one without this requirement.

It was noted that a decoder is expected to decode an entire OLS, not a subset of it.

Some participants suggested that having such a requirement would be a burden to encoders.

However, it was remarked that if a decoder was implemented without regard to the partitioning concept, such a decoder might not be able to decode a bitstream that was only providing partitioned decoding requirements for decoding the bitstream. If we require the whole OLS to have unpartitioned PTL information, a sufficiently capable decoder would be able to decode the OLS without paying attention to the partitioning concept.

It was suggested to let encoders and applications determine whether unpartitioned OLS expressions are needed for their environment.

So, between the two variants, it was agreed that there should not be a PTL required for each complete OLS (i.e., preferring "variant 2").

It was agreed that decoders should not be required to decode partitions that contain more than 4 layers.

Further study is needed regarding whether there are some "simple" cases (such as Stereo Main or a case where each layer would be a separate partition) in which decoder capabilities can be expressed as a single PTL rather than a "list of PTLs".

### JCTVC-R0052 / JCT3V-I0028, "Picture size related constraints and level limits" [S. Deshpande (Sharp), Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm), M. M. Hannuksela (Nokia)]

In AHG 10 conference calls, it was agreed that the value of the width and height signalled for the layer in the VPS are used to derive constraints for level limits of DPB parameters. Along with this document, specification text is provided considering that and other picture size-related level limits.

Following is the summary of the proposed text changes:

- For profile definition, a constraint is defined on the values of pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ i ] ]-th rep_format( ) syntax structure and those values for its direct reference layers. This is not a fully necessary addition, and it was agreed that it would be simpler to define the profile without including this constraint.

- The partitionSizeInSamplesY is derived based on VPS signalled parameters. This is used to define constraint on max_vps_dec_pic_buffering_minus1 parameters.

- The value of pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples of each layer in a bitstream partition are constrained to be less than or equal to Sqrt( MaxLumaPs * 8 ).

- Case-by-case checking of constraints in profile specific limits is performed to utilize partitionSizeInSamplesY value when appropriate.

It was agreed to integrate the agreed result (aspects 2-4 above) in to the AHG10 output text.

Offline discussion was planned to double-check that excessive frame rate requirements for a particular layer within a partition should not result from the way the text is drafted (e.g., if another layer in the same partition has a low frame rate). This was confirmed offline shortly after the call, as the frame rate constraints were planned to be drafted based on partition sizes, not picture sizes.

# AHG conclusions and recommendations

[Ed. (MH): It was also agreed to raise this issue in the parent body level, as per item #2 of R0042 in the fourth call: "It was suggested for that question to not be decided within the AHG, but to wait for that to be decided by JCT-VC and its parent bodies." This should be listed below in the list of parent body issues.]The AHG concluded the following, according to its understanding:

- That decoder conformance requirements (versus bitstream conformance requirements) had not been specified in the current draft and should be added by the editors. Also, the profile_idc values had not been assigned and should be added. Our understanding of the intent is that Scalable Main and Stereo Main decoders should be capable of decoding Main profile bitstreams (at least when the base layer is not provided by external means).

- Regarding the handling of auxiliary pictures and target output layers of output layer sets:

  - When the target output layers of an output layer set do not include the auxiliary pictures, the bits of the auxiliary pictures in the extracted bitstream subset would be counted in the CPB buffer flow, but the decoding of the auxiliary pictures would not be part of the associated decoding process requirements.

  - When the target output layers of an output layer set do include the auxiliary pictures, the profile specification for (partitions of) the output layer set would need to include decoding process requirements for decoding those pictures. Unless some way is found to specify the necessary decoding process capabilities (e.g., by a profile definition that includes such capabilities), the specification would need to disallow having target output layers of an output layer set that are auxiliary pictures.

  - It appears that layer sets can include any layers, and output_layer_flag[ i ][ j ] can be set without constraints (except for the requirement that each output layer set contain at least one target output layer). This appears to make it possible to have other (auxiliary or non-auxiliary) pictures that are included in an output layer set but are not target output layers of the output layer set and are not in layers that are needed for decoding the target output layers (e.g., an independently coded layer). It was suggested that the profile/tier/level information for such an output layer set should include all the layers in the output layer set in the CPB operation but not include the "unnecessary" layers in its decoding process requirements. In principle, it was suggested to be desirable to support this possibility (esp. for the future extensibility purpose), and suggested that we should conduct further study to ensure that the specification is consistent with this design intent. However, there was no clear agreement that this needed to be supported, pending further study.

- Regarding the case where a base layer is provided by external means, it still seems possible that we may not need a separate profile for the external base layer case, if the specification is drafted appropriately, as the question of whether the base layer is provided within the bitstream or by external means could be a system configuration question rather than a profiling issue. However, this question seemed to remain in need of further consideration.

- [Ed. (MH): Yes, Q0078v4 has been fully integrated in MV-HEVC/SHVC specification texts. This bullet can be removed from AHG conclusions IMO.]Regarding DPB specification (incl. JCTVC-Q0103 / JCT3V-H0034) level limit constraints related to maximum DPB size, the VPS-level DPB needs to have constraints as well as the SPS level having such constraints (each layer in the TargetDecLayerIdList shall obey profile-specific level limit in A.4.1 d, which applies to sps_max_dec_pic_buffering_minus1[ HighestTid ]).

The AHG reached agreement on the following recommendations, for which either detailed text changes or editor's notes are included in the attached AHG output text:

- The text should clearly specify the following:

  - Values of general_profile_idc for indicating SHVC and MV-HEVC profiles.

  - Decoding requirements specification needs to require decoding (an editorial oversight)

  - If the capabilities of one profile are "nested" within the capabilities of another profile, decoders should be required to recognize that and decode the bitstream.

- Disallow an auxiliary picture layer to be indicated as a target output layer (unless there is a profile defined that contains the required decoding capability for the auxiliary picture layer) for Scalable Main and Stereo Main profile OLSs (depending on the rewriting process to handle that, as applicable).

- Make the PPS syntax HEVCv1 compatible, and enable the syntax of SPS with nuh_layer_id > 0 to be either HEVCv1 compatible or not.

- Simplify the bitstream rewriting process for independent non-base layers (part of JCTVC-R0042).

- Specify independent non-base layer decoding for profiles specified in Annex A. For example, the following phrasing could be used "When expressing the capabilities of a decoder for a profile specified in Annex A, the capability of decoding an independent non-base layer should be expressed."

- Allow a bitstream to be considered conforming without containing a base layer (i.e., without containing pictures of layer 0), and indicate such a bitstream using the vps_reserved_one_bit (with the syntax element name and semantics changed).

- Remove the bitstream partition HRD parameters SEI message.

- Specify signalling for indication of decoder capability as follows:

  - Each output layer set is associated with one or more partitioning schemes, and for each partitioning scheme, one or more bitstream partitions are signalled, and each bitstream partition contains one or more layers.

  - Each bitstream partition is associated with a set of profile, tier, and level. Level limits are basically bitstream partition specific.

  - A decoder specification conceptually based on a number of single-layer decoder cores without changing the existing hardware cores (but with addition of resampling and firmware changes) expresses its decoding capability by expressing the list of PTLs (profile, tier, and level syntax structures) of the single-layer decoder cores, each of which is suitable for decoding a bitstream partition.

  - In the semantics for partition signalling, when the number of partitions in the partitioning scheme of an OLS is greater than 1, the number of layers in each partition is required to be less than or equal to 4 (as a profile or semantics constraint). Decoders should not be required to decode partitions that contain more than 4 layers, and an OLS should not be required to have a PTL (profile, tier, and level syntax structure) that applies to the entire OLS.

  - Picture size related constraints and level limits should be established, based on the partitioning.

The following needs for further study or work were identified:

- Whether the bitstream rewriting process is needed, and how to specify/indicate the "plus" capability of HEVCv1 and RExt profiles (where the "plus" capability refers to the decoding capability required for decoding of an independent non-base layer that is not rewritten to be a base layer).

- Whether there is a need to specify resampling capability distinctly as part of profile/tier/level or VPS-level syntax.

- Currently, for bitstream partitions, instead of using the different delivery schedules signalled in an hrd_parameters( ) syntax structure as for layer sets in HEVCv1, a combination of delivery schedules from different hrd_parameters( ) syntax structures are used. Should the same way as for layer sets be used for bitstream partitions?

- For the bitstream partition nesting SEI message syntax, would it be desirable to enable more than one OLS index, more than one partitioning scheme index, and perhaps more than one BSP index, which could reduce the number of such SEI messages needed in the bitstream?

- Wording improvement is needed for item b of the profile specific level limits, to correctly implement the intent to consider output times of pictures in different AUs.

- Should the level limits on the number of tiles (or, more precisely, the rate of tiles) be layer specific (i.e. picture based) instead of bitstream partition specific? (see JCTVC-R0046)

- Contribution JCTVC-R0053 / JCT3V-I0029 (On invocation of a correct decoding process for the current picture) should be considered.

The following observations and questions were agreed by the AHG to be raised for discussion at JCT-VC / JCT-3V and the parent bodies:

- It is noted that the drafted Scalable Main profile does not support monochrome decoding

- It is also noted that 8-deep layering dependency is present in the current SHVC profile specifications.

- It is noted that the proposed modified signalling scheme for indication of decoder capability (as described above) differs in a substantial way from the content of the current draft specification. However, no other method had been identified to express decoding capabilities in a way that would support a substantial number of layers together with the intent to enable the construction of decoders from existing single-layer implementations (without a need to restructure the associated low-level computing resources in a fundamentally different way or to grossly-overprovision the resource requirements). The need for this design selection should be checked and confirmed by the parent bodies.

- Should the number of layers supported by a decoder be part of the profile/tier/level specification or can this be something chosen by individual decoder designers versus decoders just being required to decode an unlimited number of layers? For example, should there be a profile that restricts the number of layers to only 2 or 3?

- Whether there are some "simple" cases (such as Stereo Main or a case where each layer is a separate partition) in which decoder capabilities can be expressed as a single PTL rather than a "list of PTLs".

- Is it desirable to use general_profile_space equal to 1 for independent non-base layers for profiles specified in Annex A (HEVCv1 and RExt profiles), or is it adequate for this to be handled outside of the profile-tier-level syntax structure (e.g., based only on layer ID and a VPS indication)?

- Should some additional adjustment be made of the syntax of independent non-base layers in regard, e.g., to VPS repetition and handling of end of sequence and end of bitstream NAL units?

The following contributions registered for the Sapporo meeting were considered by the AHG.

- JCTVC-R0041 / JCT3V-I0023

- JCTVC-R0042 / JCT3V-I0021

- JCTVC-R0043 / JCT3V-I0022

- JCTVC-R0044 / JCT3V-I0024

- JCTVC-R0046 / JCT3V-I0022

- JCTVC-R0052 / JCT3V-I0028

- JCTVC-R0054 / JCT3V-I0030

The following additional contributions are noted to be relevant to the work of the AHG:

- JCTVC-R0053 / JCT3V-I0029 "AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture" [M. M. Hannuksela (Nokia)]