| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0013 |
| --- | --- |

| Title: | JCT-VC AHG report: SHVC inter-layer re-sampling process (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | E. Alshina, J. Chen, P. Topiwala, T. Yamamoto, Y. Ye | Tel: Email: | elena_a.alshina@samsung.com, cjianle@qti.qualcomm.com pankaj@fastvdo.com yamamoto.tomoyuki@sharp.co.jp yan.ye@interdigital.com |
| Source: | AHG 13 |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC AHG on SHVC inter-layer re-sampling process (AHG13) between the 17th JCT-VC meeting in Valencia, Spain, (27 March – 4 April 2014) and 18th JCT-VC meeting in Sapporo, Japan, (30 June – 9 July 2014), and lists the related input documents.

# Mandates and related activity summary

SHVC inter-layer re-sampling process (AHG13)

(jct-vc@lists.rwth-aachen.de)

- Develop methodology for evaluating the accuracy of re-sampling processing;

- Study generic solutions for phase adjustment in re-sampling process;

- Study re-sampling process modification related to extracting regions of interest;

- Discuss and identify additional issues related to re-sampling filtering.

Four teleconferences (about 2 hours each) were conducted between the 17th and the 18th JCT-VC meetings. Participants are Arris, Ericsson, InterDigital, Microsoft, Qualcomm, Samsung, Sharp, Vidyo.

Brief summary of discussion during teleconferences:

- List of practically important use-cases affected by re-sampling process was created. It includes picture to picture, field to frame and aspect ratio scalability with most commonly used picture sizes and scaling factors.

- JCTVC-Q0203 template for re-sampling process accuracy estimation was verified and modified allowing easy accuracy evaluation for different use-cases from the list. No critical problems with re-sampling process accuracy were identified for all studied use-cases. However several SHVC specification changes listed below were agreed to be reasonable.

- Current draft text explicitly uses reference layer conformance window parameters during re-sampling process. But actual reference picture size might be different from conformance window (for example, if region of interest extraction is considered) or conformance window information can un- available (if non-HEVC reference layer is used). It is recommended to re-place conformance window parameters in re-sampling process by reference layer offsets as was proposed several times before [JCTVC-P0049, JCTVC-Q0159].

- Current draft text derives phases and delta parameters for re-sampling process from system of 3 flags [JCTVC-P0312, JCTVC-Q0120] allowing more freedom in vertical re-sampling process variants rather than horizontal. In order to make phase adjustment more generic it is recommended to combine phase and delta parameters (since they never used separately) and signal them in a bit-stream for Luma and Chroma both for vertical and horizontal re-sampling process [JCTVC-Q0203]. Suggested change replaces “system of 3 flags” in the specification.

- In current draft text parameters used in re-sampling process are signalled in VPS, SPS and slice-header. Signaling of all related parameters in PPS (as was suggested in [JCTVC-Q107, JCTVC-Q-0168, JCTVC-Q0254]) was agreed to be cleaner. At the same time PPS signalling allows enough freedom for re-sampling parameters adaptation.

- It was mentioned that non-4:2:0 color format support is desirable feature. Even SHVC is supposed to support only 4:2:0 color formats specification text should not contain restriction preventing easy extension of SHVC spec for non 4:2:0 color format support during future standards development.

Recommended SHVC specification draft text can be found in the document attached to this report.

# Open issues

Several issues still remain un-clear. It is easier to decide about them during live discussion in the meeting place.

- In "Picture parameter set RBSP semantics" the ranges for scaled reference layer offsets, reference layer offsets and resampling phases need to be specified.

- In "Derivation process for inter-layer reference pictures" the derivation for unequalPictureSizeFlag may need to be adjusted in order to skip re-sampling process when not needed.

- The value of resampling offset may need to be restricted to be zero for non-spatial scalability, i.e. when spatial ratio is equal to 1 so that the results of direct copy and using interpoation process are identical.

- Clipping in "Resampling process of luma sample values" and "Luma interpolation process" put the restriction that pixels on a reference layer picture accessed in those processes are restricted within "the reference layer offset window extended with filter-tap length". Need to discuss and determine if this is reasonable restriction.

- Current syntax signal 1) scaled reference layer offsets, 2) reference layer offsets, and 3) resample phases in independent loops. In two layers case, it could result in signaling the same 6 bit layer id syntax three times. It is suggested to discuss whether we should keep this design or signaling the parameters associated with the same reference layer in one loop.

# List of related contributions

| Doc # | Title | Authors |
| --- | --- | --- |
| JCTVC-R0209 | AHG13: Re-sampling process with generic phase | E. Alshina (Samsung) |
| JCTVC-R0220 | AHG13: On reference position calculation | T. Yamamoto,T. Tsukuba,T. Ikai (Sharp) |
| JCTVC-R0312 | AHG13 Crosscheck for Re-sampling process with generic phase (JCTVC-R0209) | K. Minoo (Arris Group) |

# Recommendations

- Discuss specification changes recommended by AhG13

- Review related contributions.