| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0031 |
| --- | --- |

| Title: | Summary report of HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Joel Sole Shan Liu | Email: | joels@qti.qualcomm.com shan.liu@mediatek.com |
| Source: | SCCE coordinators |  |  |

_____________________________

# Abstract

A summary of SCCE1 on Intra block copy extensions is reported. The core experiment consisted of four subtests that investigated 1) the impact of the search area size, 2) intra block copying modes, 3) intra block vector coding (prediction and binarization) and 4) flipping the predictor block. Performance of the proposed methods as well as combinations was evaluated for lossy and lossless configurations based on the SCC common test conditions.

# List of CE contributions

## Test 1: IBC Search Area

- JCTVC-R0184, SCCE1: Test 1.1 – Intra block copy with different search areas, C. Pang, J. Sole, M. Karczewicz (Qualcomm)

  - JCTVC-R0244, Cross check of ‘SCCE1: Test 1.1 – Intra block copy with different search areas’ (JCTVC-R0184) by Qualcomm, C. Rosewarne, M. Maeda (Canon)

- JCTVC-R0141, SCCE1: Result of Test 1.2 for IBC with constrained buffers from left CTUs, W. Wang, Z. Ma, M. Xu, H. Yu (Huawei Technologies)

  - JCTVC-R0211, SCCE1: Cross-verification of test 1.2 results, M. Zhou (Broadcom)

## Test 2: IBC modes

- JCTVC-R0190, SCCE1 Test 2.1: IntraBC coded as Inter PU, X. Xu, S. Liu, S. Lei (MediaTek)

  - JCTVC-R0129, Crosscheck of SCCE1 Test 2.1 on IBC as Inter PU, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)

- JCTVC-R0097, SCCE1: Result of Test 2.2, 2.3 and 4.2, B. Li, J. Xu (Microsoft)

  - JCTVC-R0195, Cross check of SCCE1 Test 2.2: Intra block copy skip mode, C. Pang (Qualcomm)

  - JCTVC-R0171, SCCE1: Crosscheck report of SCCE1 test 2.3 Merge mode, Y. He, Y. Ye (InterDigital)

- JCTVC-R0241, SCCE1 subtest 2.4: AMP for Intra BC prediction, L. Zhu, J. Xu, G. J. Sullivan, Y. Wu, S. Sankuratri, B. A. Kumar (Microsoft)

  - JCTVC-R0245, Cross check of ‘SCCE1 subtest 2.4: AMP for Intra BC prediction’ (JCTVC-R0241) by Microsoft, C. Rosewarne, M. Maeda (Canon)

## Test 3: IBC Block Vector Coding

### Test 3A: IBC Block Vector Prediction

- JCTVC-R0185, SCCE1: Test 3.1 – Block vector prediction method for Intra block copy, C. Pang, Y. Chen, J. Sole, M. Karczewicz (Qualcomm)

  - JCTVC-R0192, Cross check of SCCE1 Test 3.1 Block vector prediction method for Intra block copy, X. Xu, S. Liu (MediaTek)

- JCTVC-R0240, SCCE1 subtest 3.2: Ping-Pong block vector predictor test, L. Zhu, J. Xu, G. J. Sullivan, Y. Wu, S. Sankuratri, B. A. Kumar (Microsoft)

  - JCTVC-R0090, SCCE1: Cross-check of Test 3.2, P. Onno

- JCTVC-R0081, SCCE1: Test 3.3 - BV prediction using the three last predictors , G. Laroche, T. Poirier, C. Gisquet, P. Onno

  - JCTVC-R0107, SCCE1: Cross-check of test 3.3 (JCTVC-R0081), B. Li, J. Xu (Microsoft)

- JCTVC-R0061, SCCE1 Test 3.4: IntraBC BV prediction, X. Xu, S. Liu (MediaTek)

  - JCTVC-R0248, SCCE1: Cross-check results of Test 3.4, M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)

### Test 3B: IBC Block Vector Binarization

- JCTVC-R0133, SCCE1: Test 3.5 - Block vector difference coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

  - JCTVC-R0196, Cross check of SCCE1 Test 3.5: BVD coding for Intra block copy, C. Pang (Qualcomm)

- JCTVC-R0186, SCCE1: Test 3.6 – Block vector coding for Intra block copy, C. Pang, K. Rapaka, J. Sole, M. Karczewicz (Qualcomm)

  - JCTVC-R0138, SCCE1: Crosscheck of SCCE1 Test 3.6 - BVD binarization and coding, S.-T. Hsiang, T.-D. Chuang (MediaTek)

## Test 4: Block copying modifications

- JCTVC-R0116, SCCE1 Test 4.1: Symmetric intra block copy, K. Zhang, J. An, X. Zhang, H. Huang, S. Lei (MediaTek)

  - JCTVC-R0108, SCCE1: Cross-check of test 4.1 (JCTVC-R0116), B. Li, J. Xu (Microsoft)

- JCTVC-R0097, SCCE1: Result of Test 2.2, 2.3 and 4.2, B. Li, J. Xu (Microsoft)

  - JCTVC-R0273, Crosscheck for SCCE1 Test 4.2 in JCTVC-R0097, K. Zhang (MediaTek)

## Test 5: Combinations

- JCTVC-R0191, SCCE1 Test 5.1: Combination tests of 2.1, 2.3 and 2.4, X. Xu, S. Liu, S. Lei (MediaTek)

  - JCTVC-R0246, Cross check of ‘SCCE1 Test 5.1: Combination tests of 2.1, 2.3 and 2.4’ (JCTVC-R0191) by Mediatek, C. Rosewarne, M. Maeda (Canon)

- JCTVC-R0187, SCCE1: Test 5.2 – Combination of Test 3.1 and Test 3.6, C. Pang, Y. Chen, K. Rapaka, J. Sole, M. Karczewicz (Qualcomm)

  - JCTVC-R0193, Cross check of SCCE1 Test 5.2: combination of 3.1 and 3.6, X. Xu, S.-T. Hsiang (MediaTek)

## Summary of SCCE1 contributions

| Test | Description | Proponent | JCTVC # | Status | Crosscheck | JCTVC # |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1 | IBC search area | Qualcomm | R0184 | Provided | Canon | R0244 |
| 1.2 | IBC search area | Huawei | R0141 | Provided | Broadcom | R0211 |
| 2.1 | IBC as inter PU | MediaTek | R0190 | Provided | NCTU | R0129 |
| 2.2 | Skip mode | Microsoft | R0097 | Provided | Qualcomm | R0195 |
| 2.3 | Merge mode | Microsoft | R0097 | Provided | Interdigital | R0171 |
| 2.4 | AMP | Microsoft | R0241 | Late | Canon | R0245 |
| 3.1 | BV prediction | Qualcomm | R0185 | Provided | MediaTek | R0192 |
| 3.2 | PingPong BV pred | Microsoft | R0240 | Provided | Canon | R0090 |
| 3.3 | BV prediction | Canon | R0081 | Provided | Microsoft | R0107 |
| 3.4 | BV prediction | Mediatek | R0061 | Provided | Huawei | R0248 |
| 3.5 | BVD coding | Mediatek | R0133 | Provided | Qualcomm | R0196 |
| 3.6 | BVD coding | Qualcomm | R0186 | Provided | MediaTek | R0138 |
| 4.1 | Symmetric IBC | MediaTek | R0116 | Provided | Microsoft | R0108 |
| 4.2 | Block flipping | Microsoft | R0097 | Provided | MediaTek | R0273 |
| 5.1a | 2.1 + 2.3 | MediaTek Microsoft | R0191 | Provided | Canon | R0246 |
| 5.1b | 2.1 + 2.3 + 2.4 | MediaTek Microsoft | R0191 | Provided | Canon | R0246 |
| 5.2 | 3.1 + 3.6 | Qualcomm | R0187 | Provided | MediaTek | R0193 |

# Related non-SCCE1 contributions

There are 20 contributions related to SCCE1 topics:

- JCTVC-R0050, Non-SCCE1: Intra block copy masking, J. Lainema, K. Ugur, M. M. Hannuksela (Nokia)

- JCTVC-R0074, Non-SCCE1: Adaptive switching between differential and direct coding for intra block copy vectors, K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)

- JCTVC-R0088, Non-SCCE1: Vector residual coding for Inter and IBC modes, G. Laroche, T. Poirier, C. Gisquet, P. Onno

- JCTVC-R0089, Non-SCCE1: IBC BV throughput issue, C. Gisquet, G. Laroche, T. Poirier, P. Onno

- JCTVC-R0096, Non-SCCE1: Intra block copy with modified motion vector predictor, D. Jiang, X. Zhang, Y. Wu (Lenovo)

- JCTVC-R0100, Non-SCCE1: Unification of intra BC and inter modes, B. Li, J. Xu (Microsoft)

- JCTVC-R0117, Non-SCCE1: Segmental prediction for intra block copy, K. Zhang, J. An, X. Zhang, H. Huang, S. Lei (MediaTek)

- JCTVC-R0123, Non-SCCE1: On block vector coding, C. Rosewarne, M. Maeda (Canon)

- JCTVC-R0134, Non-SCCE1: Unified BVD & MVD coding using a universal entropy coding scheme, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

- JCTVC-R0162, Non-SCCE1: Intra block copy hash search enhancement, Y. He, X. Xiu, Y. Ye (InterDigital)

- JCTVC-R0165, Non-SCCE1: Improved intra block copy coding with block vector derivation, Y. He, Y. Ye, X. Xiu (InterDigital)

- JCTVC-R0181, Non-SCCE1: Block vector coding for Intra block copy, M. Karczewicz, K. Rapaka, C. Pang, J. Sole (Qualcomm)

- JCTVC-R0182, Non-SCCE1: On Intra block vector difference redundancy removal, K. Rapaka, J. Sole, M. Karczewicz (Qualcomm)

- JCTVC-R0188, Non-SCCE1: Memory bandwidth reduction for Intra block copy, C. Pang, K. Rapaka, J. Sole, X. Li, T. Hsieh, M. Karczewicz (Qualcomm)

- JCTVC-R0189, Non-SCCE1: Intra block copy deblocking, C. Pang, Y. Chen, J. Sole, M. Karczewicz (Qualcomm)

- JCTVC-R0201, Block vector predictor selection for intra block copy, S. Yang, H. Shim, D. Lee, B. Jeon (SKKU)

- JCTVC-R0203, Non-SCCE1: Default vector selection for Intra block copy BV prediction, X. Xu, T.-D. Chuang, S. Liu, J. Ye, S. Lei (MediaTek)

- JCTVC-R0204, Non-SCCE1: PU Intra block copy with flipping mode, X. Xu, S. Liu, J. Ye, K. Zhang, S. Lei (MediaTek)

- JCTVC-R0208, Non-SCCE1: Additional test results on intra block copy (IBC), M. Zhou (Broadcom)

- JCTVC-R0269, Non-SCCE1: Analysis of Full Frame IBC Block Vector Distribution, F. Duanmu, Z. Ma, M. Xu, W. Wang, H. Yu (Huawei R&D USA)

# Summary of Results

The proposed methods were implemented on top of SCM1.0 and simulated under the SCC common test conditions.

## Test A: IBC Search Area

Tested several search areas (JCTVC-R0141, JCTVC-R0184):

- 2 CTUs (left and current)

- 3 CTUs (current CTU + 2 left CTU)

- 4 CTUs (current CTU + 3 left CTU)

- 4 CTUs (current CTU + 3 left CTU) + 4 rows above

- Surrounding the current CTU:  2×3 CTUs

- Surrounding the current CTU:  3×5 CTUs

- Full frame for intra frames and local search (2 CTUs) for inter frames

- Full frame (anchor)

### Results for IBC search areas

BD-rates with respect to full-frame search.

|  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 34.7% | 25.5% | 21.3% | 19.2% | 20.1% | 11.4% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 22.6% | 15.8% | 12.9% | 12.0% | 14.2% | 8.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 15.1% | 11.7% | 10.2% | 9.4% | 8.6% | 5.2% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 20.3% | 17.0% | 15.9% | 14.5% | 10.5% | 4.7% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 35.2% | 25.9% | 21.7% | 19.6% | 20.5% | 11.5% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 22.4% | 15.7% | 12.9% | 12.0% | 14.1% | 8.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 16.5% | 12.9% | 11.3% | 10.3% | 9.3% | 5.5% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 21.8% | 18.4% | 17.3% | 15.9% | 11.3% | 5.1% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 92% | 109% | 134% | 123% | 95% | 96% | 103% | 100% |
| Dec Time[%] | 105% | 106% | 112% | 101% | 104% | 103% | 103% | 100% |
|  | Random Access |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 17.5% | 12.9% | 10.7% | 9.8% | 10.5% | 6.0% | 7.7% | 0.0% |
| RGB, text & graphics with motion,720p | 15.2% | 10.8% | 8.7% | 8.1% | 9.8% | 5.6% | 1.3% | 0.0% |
| RGB, mixed content, 1440p | 8.3% | 6.6% | 5.9% | 5.4% | 4.3% | 2.7% | 2.1% | 0.0% |
| RGB, mixed content, 1080p | 12.0% | 10.1% | 9.5% | 8.8% | 6.7% | 3.0% | 2.4% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 16.9% | 12.5% | 10.4% | 9.6% | 10.0% | 5.6% | 6.9% | 0.0% |
| YUV, text & graphics with motion,720p | 15.1% | 10.6% | 8.6% | 8.1% | 9.9% | 5.7% | 1.2% | 0.0% |
| YUV, mixed content, 1440p | 10.0% | 8.0% | 7.3% | 6.7% | 5.3% | 3.2% | 2.1% | 0.0% |
| YUV, mixed content, 1080p | 14.4% | 12.3% | 11.6% | 10.9% | 7.8% | 3.4% | 2.5% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 104% | 100% | 103% | 101% | 95% | 93% | 100% |
| Dec Time[%] | 104% | 105% | 101% | 100% | 102% | 96% | 96% | 100% |
|  | Low Delay |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 11.3% | 8.2% | 6.9% | 6.3% | 6.5% | 3.4% | 9.4% | 0.0% |
| RGB, text & graphics with motion,720p | 5.8% | 4.2% | 3.6% | 3.4% | 4.1% | 2.6% | 3.9% | 0.0% |
| RGB, mixed content, 1440p | 4.4% | 3.8% | 3.6% | 3.2% | 2.5% | 1.2% | 2.0% | 0.0% |
| RGB, mixed content, 1080p | 4.4% | 3.5% | 3.3% | 3.2% | 2.6% | 1.3% | 3.1% | 0.0% |
| RGB, Animation, 720p | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.1% | 7.5% | 6.3% | 5.8% | 5.8% | 3.0% | 8.3% | 0.0% |
| YUV, text & graphics with motion,720p | 5.0% | 3.6% | 3.2% | 3.0% | 3.6% | 2.3% | 3.3% | 0.0% |
| YUV, mixed content, 1440p | 5.3% | 4.3% | 4.2% | 3.9% | 2.5% | 1.5% | 1.9% | 0.0% |
| YUV, mixed content, 1080p | 5.8% | 5.0% | 4.8% | 4.3% | 3.3% | 1.9% | 2.9% | 0.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 96% | 105% | 101% | 98% | 93% | 93% | 93% | 100% |
| Dec Time[%] | 102% | 105% | 100% | 95% | 101% | 95% | 96% | 100% |

### Memory Access

Assuming that the memory block size is 4×2 for both luma and chroma, then the average memory access is summarized as follows when the alignment size is 64 and burst size is 256.

|  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 62.1% | - | - | - | 101.7% | 100.9% | 100.0% | 100.0% |
| RGB, text & graphics with motion,720p | 56.5% | - | - | - | 101.2% | 100.6% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 55.3% | - | - | - | 99.6% | 99.3% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 57.5% | - | - | - | 100.9% | 100.1% | 100.0% | 100.0% |
| RGB, Animation, 720p | 52.4% | - | - | - | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 50.7% | - | - | - | 100.1% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 61.3% | - | - | - | 101.6% | 100.9% | 100.0% | 100.0% |
| YUV, text & graphics with motion,720p | 55.6% | - | - | - | 101.1% | 100.6% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 54.2% | - | - | - | 99.5% | 99.2% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 56.7% | - | - | - | 100.8% | 100.1% | 100.0% | 100.0% |
| YUV, Animation, 720p | 51.3% | - | - | - | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 50.2% | - | - | - | 100.0% | 100.0% | 100.0% | 100.0% |
|  | Random Access |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 72.1% | - | - | - | 100.4% | 100.2% | 72.7% | 100.0% |
| RGB, text & graphics with motion,720p | 71.9% | - | - | - | 100.0% | 100.1% | 72.9% | 100.0% |
| RGB, mixed content, 1440p | 71.7% | - | - | - | 100.0% | 99.8% | 72.5% | 100.0% |
| RGB, mixed content, 1080p | 72.1% | - | - | - | 99.9% | 99.9% | 72.5% | 100.0% |
| RGB, Animation, 720p | 77.0% | - | - | - | 100.0% | 100.0% | 78.0% | 100.0% |
| RGB, camera captured, 1080p | 77.8% | - | - | - | 100.0% | 100.0% | 78.6% | 100.0% |
| YUV, text & graphics with motion, 1080p | 72.2% | - | - | - | 100.3% | 100.2% | 72.7% | 100.0% |
| YUV, text & graphics with motion,720p | 72.0% | - | - | - | 100.1% | 100.0% | 73.0% | 100.0% |
| YUV, mixed content, 1440p | 72.2% | - | - | - | 100.0% | 100.0% | 72.7% | 100.0% |
| YUV, mixed content, 1080p | 72.2% | - | - | - | 100.0% | 100.0% | 72.7% | 100.0% |
| YUV, Animation, 720p | 76.6% | - | - | - | 100.0% | 100.0% | 77.5% | 100.0% |
| YUV, camera captured, 1080p | 77.6% | - | - | - | 100.0% | 100.0% | 78.4% | 100.0% |
|  | Low Delay |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 69.7% | - | - | - | 100.5% | 100.3% | 69.8% | 100.0% |
| RGB, text & graphics with motion,720p | 67.6% | - | - | - | 100.0% | 100.0% | 67.7% | 100.0% |
| RGB, mixed content, 1440p | 68.0% | - | - | - | 100.1% | 100.0% | 68.2% | 100.0% |
| RGB, mixed content, 1080p | 68.4% | - | - | - | 100.0% | 100.0% | 68.5% | 100.0% |
| RGB, Animation, 720p | 76.0% | - | - | - | 100.0% | 100.0% | 76.0% | 100.0% |
| RGB, camera captured, 1080p | 78.2% | - | - | - | 100.0% | 100.0% | 78.3% | 100.0% |
| YUV, text & graphics with motion, 1080p | 69.8% | - | - | - | 100.4% | 100.2% | 69.8% | 100.0% |
| YUV, text & graphics with motion,720p | 67.5% | - | - | - | 100.0% | 100.0% | 67.6% | 100.0% |
| YUV, mixed content, 1440p | 67.8% | - | - | - | 100.1% | 100.1% | 68.0% | 100.0% |
| YUV, mixed content, 1080p | 68.1% | - | - | - | 100.0% | 100.0% | 68.1% | 100.0% |
| YUV, Animation, 720p | 75.1% | - | - | - | 99.7% | 99.9% | 75.3% | 100.0% |
| YUV, camera captured, 1080p | 77.3% | - | - | - | 100.0% | 100.0% | 77.6% | 100.0% |

## Test 2: IBC modes

- Test 2.1. IBC as inter PU (JCTVC-R0190)

Several aspects of IBC are modified to align it with inter predicted CU and PU syntax including: signaling of the block vector (BV) predictor, deblocking filter, DCT, and IBC related syntax moved from coding_unit to prediction_unit and harmonized with inter PU syntax. IntraBC mode is treated as an inter mode with inter_pred_idc equals to 0 (L0). 	4x4 partition mode is turn on or off for IntraBC.

- Test 2.2. Skip mode (JCTVC-R0097)

IBC skip mode is a 2N×2N partition IBC mode without residue. A flag is signaled for each CU to indicate the usage of the IBC skip mode.

- Test 2.3. Merge mode (JCTVC-R0097)

For IBC merge mode, the BV used for the current CU uses the BV in the left  or the above block. No BV difference is signaled for IBC merge mode. Applied only to 2N×2N partitions. A flag per CU indicates the usage. Another flag is signaled to indicate the BV used for the current CU.

- Test 2.4. AMP mode (JCTVC-R0241)

AMP for IBC introduces the motion partitions of 2N×nU, 2N×nD, nL×2N, and nR×2N into the IBC design.

### Results for tests on IBC modes

Note: Test 2.3 should be considered along with the BV prediction methods

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| RGB, text & graphics with motion, 1080p | -3.9% | -1.3% | 0.0% | -7.2% | -0.8% |
| RGB, text & graphics with motion,720p | -2.0% | -0.7% | 0.1% | -4.0% | -0.5% |
| RGB, mixed content, 1440p | -1.6% | -0.8% | 0.0% | -2.9% | -0.5% |
| RGB, mixed content, 1080p | -1.9% | -0.7% | 0.0% | -2.9% | -0.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.6% | -2.1% | 0.0% | -7.4% | -1.1% |
| YUV, text & graphics with motion,720p | -2.4% | -1.3% | 0.0% | -4.3% | -0.7% |
| YUV, mixed content, 1440p | -1.8% | -1.0% | 0.0% | -3.1% | -0.5% |
| YUV, mixed content, 1080p | -2.1% | -1.0% | 0.0% | -3.2% | -0.6% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 91% | 98% | 110% | 139% |
| Dec Time[%] | 98% | 100% | 100% | 98% | 119% |
|  | Random Access |  |  |  |  |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| RGB, text & graphics with motion, 1080p | -2.0% | -0.9% | 0.0% | -4.3% | -0.6% |
| RGB, text & graphics with motion,720p | -1.5% | -0.5% | 0.0% | -2.7% | -0.6% |
| RGB, mixed content, 1440p | -1.2% | -0.6% | 0.1% | -1.8% | -0.5% |
| RGB, mixed content, 1080p | -1.3% | -0.4% | 0.0% | -1.9% | -0.4% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.8% | -0.7% | -0.1% | -4.4% | -0.8% |
| YUV, text & graphics with motion,720p | -1.5% | -0.6% | 0.0% | -2.8% | -0.5% |
| YUV, mixed content, 1440p | -0.9% | -0.3% | 0.1% | -1.9% | -0.3% |
| YUV, mixed content, 1080p | -1.3% | -0.7% | 0.0% | -2.3% | -0.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 98% | 97% | 98% | 99% | 112% |
| Dec Time[%] | 97% | 98% | 100% | 97% | 115% |

|  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.1% | 0.0% | -3.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.9% | -0.3% | -0.1% | -1.0% | -0.4% |
| RGB, mixed content, 1440p | -0.7% | -0.5% | 0.0% | -1.0% | -0.4% |
| RGB, mixed content, 1080p | -0.9% | -0.7% | -0.2% | -1.3% | -0.4% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -0.8% | -0.1% | -3.2% | -0.6% |
| YUV, text & graphics with motion,720p | -0.5% | -0.1% | 0.0% | -1.2% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | 0.0% | -1.2% | -0.4% |
| YUV, mixed content, 1080p | -0.5% | -0.1% | 0.3% | -0.8% | 0.2% |
| YUV, Animation, 720p | 0.2% | 0.1% | 0.0% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.2% | 0.2% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 97% | 96% | 98% | 99% | 108% |
| Dec Time[%] | 99% | 97% | 99% | 98% | 119% |

|  | BV Memory Requirements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| Line Buffer | No | No | Yes | Yes | No |
| #vectors to be stored | Additional 24 for AI and 0 for RA and LD | Additional 24 for AI and 0 for RA and LD | - | Additional 24 for AI and 0 for RA and LD | Additional 24 for AI and 0 for RA and LD |

## Test 3A: IBC Block Vector Prediction

- Test 3.1: BV prediction (JCTVC-R0185)

The block vector prediction method of IBC is modified to use either an spatial neighboring BV or default vector is used as BV predictor. Up to two BV from those in spatial neighboring blocks as used in HEVC are used to form a BV candidate list and one BV is selected.

- Test 3.2: Ping-Pong BV prediction (JCTVC-R0240)

Block vector prediction values are maintained as state information. For each BV to be sent, a flag is sent to select between them for the BV prediction value.

- Test 3.3: BV prediction (JCTVC-R0081)

The BV predictor is selected among a predictors list that includes the three last decoded BVs. The selected predictor is indicated through a syntax element. Additional fixed predictor values are added when the full list does not contain three elements.

- Test 3.4 AMVP prediction (JCTVC-R0061)

The block vector prediction method of IBC is modified to use either an spatial neighboring BV or default vector is used as BV predictor. A flag is used to signal which of the predictors is used. When there is only one unique candidate BV available, (-2*CU_width, 0) is used for the second candidate. If both candidates are not available, (-2*CU_width, 0) and (-CU_width, 0) are used.

### Results for tests on BV prediction

Note: Encoders have different levels of optimization, which makes difficult to compare the algorithms. Same encoder results in table below.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| RGB, text & graphics with motion, 1080p | -2.6% | -2.0% | -1.6% | -3.8% |
| RGB, text & graphics with motion,720p | -0.8% | -1.2% | -0.9% | -1.9% |
| RGB, mixed content, 1440p | -1.0% | -0.8% | -0.7% | -1.4% |
| RGB, mixed content, 1080p | -1.4% | -0.8% | -0.8% | -1.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.8% | -2.3% | -1.8% | -4.5% |
| YUV, text & graphics with motion,720p | -1.1% | -1.4% | -1.1% | -2.4% |
| YUV, mixed content, 1440p | -1.3% | -1.0% | -0.8% | -1.7% |
| YUV, mixed content, 1080p | -1.5% | -1.0% | -0.9% | -2.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% | 107% | 102% | 101% |
| Dec Time[%] | 104% | 101% | 100% | 99% |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.6% | -1.2% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.3% | -0.8% |
| RGB, mixed content, 1440p | -0.8% | -0.6% | -0.5% | -1.0% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.9% | -1.1% | -0.8% | -1.7% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.8% | -0.7% | -1.5% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% | -0.6% | -0.9% |
| YUV, mixed content, 1440p | -1.1% | -0.6% | -0.4% | -1.3% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | 0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | -0.6% | -0.6% | -0.5% | -1.0% |
| Enc Time[%] | 104% | 99% | 103% | 99% |
| Dec Time[%] | 107% | 98% | 99% | 102% |

|  | Low Delay |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.5% | -0.2% | -0.4% |
| RGB, text & graphics with motion,720p | -0.3% | 0.1% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | -0.3% | -0.8% | -0.5% | -0.7% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | -0.3% | -0.4% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.7% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | 0.1% | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1440p | 0.1% | -0.4% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | -0.2% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.2% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 100% | 104% | 98% |
| Dec Time[%] | 104% | 101% | 99% | 100% |

Results with following encoder settings for 3.1, 3.3 and 3.4:

- Software bug fix (could also be applied to SCM1.0)

- BV costs are estimated by considering all predictors

- Allow the use of predictors to be the BV of current IBC block

- BV predictor is RD estimated even if the predictor is outside the 2 CTBs

Test 3.1 provides further results with full RD check at BV predictor position 3.1*.

Test 3.3 provides further results with full RD check of BV predictor and testing neighboring left and above BVs: test 3.3*.

|  |  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3.1 | 3.3 |  | 3.4 | 3.1* | 3.3* |
| RGB, text & graphics with motion, 1080p |  | -7.3% | -6.5% |  | -7.2% | -9.7% | -8.9% |
| RGB, text & graphics with motion,720p |  | -4.2% | -3.8% |  | -4.2% | -5.3% | -5.3% |
| RGB, mixed content, 1440p |  | -2.9% | -2.6% |  | -2.9% | -3.7% | -3.7% |
| RGB, mixed content, 1080p |  | -3.5% | -3.1% |  | -3.5% | -4.2% | -4.2% |
| RGB, Animation, 720p |  | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p |  | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p |  | -7.7% | -6.7% |  | -7.7% | -10.5% | -9.2% |
| YUV, text & graphics with motion,720p |  | -4.7% | -4.4% |  | -4.7% | -5.9% | -6.0% |
| YUV, mixed content, 1440p |  | -3.3% | -3.1% |  | -3.3% | -4.1% | -4.2% |
| YUV, mixed content, 1080p |  | -3.7% | -3.5% |  | -3.8% | -4.8% | -4.6% |
| YUV, Animation, 720p |  | -0.1% | -0.2% |  | -0.2% | -0.1% | -0.3% |
| YUV, camera captured, 1080p |  | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  | 99% | 101% |  | 96% | 113% | 113% |
| Dec Time[%] |  | 98% | 98% |  | 98% | 96% | 96% |
|  |  |  |  | Random Access |  |  |  |
|  |  | 3.1 | 3.3 |  | 3.4 | 3.1* | 3.3* |
| RGB, text & graphics with motion, 1080p |  | -3.3% | -3.6% |  | -3.3% | -5.4% | -6.0% |
| RGB, text & graphics with motion,720p |  | -2.7% | -2.5% |  | -2.7% | -3.5% | -3.5% |
| RGB, mixed content, 1440p |  | -1.6% | -1.6% |  | -1.7% | -2.1% | -2.2% |
| RGB, mixed content, 1080p |  | -2.1% | -2.0% |  | -2.2% | -2.7% | -2.8% |
| RGB, Animation, 720p |  | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p |  | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p |  | -3.1% | -3.3% |  | -3.2% | -5.6% | -6.1% |
| YUV, text & graphics with motion,720p |  | -3.0% | -2.7% |  | -2.9% | -3.9% | -3.8% |
| YUV, mixed content, 1440p |  | -1.7% | -1.8% |  | -1.9% | -2.3% | -2.7% |
| YUV, mixed content, 1080p |  | -2.4% | -2.4% |  | -2.6% | -3.2% | -3.3% |
| YUV, Animation, 720p |  | 0.0% | -0.1% |  | -0.2% | -0.1% | -0.2% |
| YUV, camera captured, 1080p |  | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  | 101% | 102% |  | 96% | 106% | 106% |
| Dec Time[%] |  | 99% | 99% |  | 98% | 103% | 91% |
|  |  |  |  | Low Delay |  |  |  |
|  |  | 3.1 | 3.3 |  | 3.4 | 3.1* | 3.3* |
| RGB, text & graphics with motion, 1080p |  | -1.8% | -2.6% |  | -1.8% | -3.6% | -4.9% |
| RGB, text & graphics with motion,720p |  | -1.0% | -1.1% |  | -1.0% | -1.5% | -1.6% |
| RGB, mixed content, 1440p |  | -0.7% | -0.8% |  | -0.9% | -1.1% | -1.3% |
| RGB, mixed content, 1080p |  | -0.8% | -1.4% |  | -0.8% | -1.1% | -1.4% |
| RGB, Animation, 720p |  | 0.0% | 0.0% |  | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p |  | 0.0% | -0.1% |  | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p |  | -1.6% | -2.2% |  | -1.6% | -3.7% | -4.9% |
| YUV, text & graphics with motion,720p |  | -1.0% | -1.2% |  | -1.0% | -1.6% | -1.7% |
| YUV, mixed content, 1440p |  | -0.7% | -0.8% |  | -0.7% | -1.3% | -1.6% |
| YUV, mixed content, 1080p |  | -0.5% | -1.0% |  | -0.7% | -1.0% | -1.5% |
| YUV, Animation, 720p |  | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p |  | 0.1% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  | 99% | 102% |  | 95% | 103% | 105% |
| Dec Time[%] |  | 101% | 102% |  | 96% | 106% | 95% |

|  | BV Memory Requirements |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| Line Buffer | No | No | No | No |
| #vectors to be stored | Additional 23 for AI and 0 for RA and LD | 2 | 3 | Additional 24 for AI and 0 for RA and LD |

## Test 3B: IBC Block Vector Binarization

- Test 3.5: BV difference coding (JCTVC-R0133)

A modified context modeling method for coding IBC block vector difference (BVD) is tested. Additional contexts are assigned for coding the prefix bins used for representing the absolute value of the BVD component.

- Test 3.6: BV binarization (JCTVC-R0186)

BVD binarization is modified: first, a flag is signaled to indicate whether the difference is zero. When BV difference is not zero, Exp-Golomb code of order 3 is used.

### Results for tests on BV binarization

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | 3.5 | 3.5 ctx rd | 3.6 |
| RGB, text & graphics with motion, 1080p | -3.2% | -3.0% | -2.3% |
| RGB, text & graphics with motion,720p | -2.0% | -1.9% | -1.3% |
| RGB, mixed content, 1440p | -1.0% | -0.9% | -0.8% |
| RGB, mixed content, 1080p | -1.3% | -1.2% | -1.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -4.0% | -3.7% | -2.9% |
| YUV, text & graphics with motion,720p | -2.5% | -2.3% | -1.6% |
| YUV, mixed content, 1440p | -1.3% | -1.3% | -1.0% |
| YUV, mixed content, 1080p | -1.5% | -1.4% | -1.2% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% | 97% | 101% |
| Dec Time[%] | 102% | 102% | 103% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | 3.5 | 3.5 ctx rd | 3.6 |
| RGB, text & graphics with motion, 1080p | -1.8% | -1.7% | -1.4% |
| RGB, text & graphics with motion,720p | -1.3% | -1.3% | -1.0% |
| RGB, mixed content, 1440p | -0.7% | -0.6% | -0.5% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.6% |
| RGB, Animation, 720p | -0.3% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -2.0% | -1.8% | -1.5% |
| YUV, text & graphics with motion,720p | -1.6% | -1.6% | -1.2% |
| YUV, mixed content, 1440p | -0.8% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | -1.1% | -1.0% | -1.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 97% | 102% |
| Dec Time[%] | 106% | 98% | 103% |

|  | Low Delay |  |  |
| --- | --- | --- | --- |
|  | 3.5 | 3.5 ctx r.d. | 3.6 |
| RGB, text & graphics with motion, 1080p | -1.4% | -1.4% | -1.1% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.6% |
| RGB, mixed content, 1440p | -0.7% | -0.5% | -0.6% |
| RGB, mixed content, 1080p | -0.4% | -0.9% | -0.5% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.4% | -1.2% |
| YUV, text & graphics with motion,720p | -0.9% | -0.8% | -0.5% |
| YUV, mixed content, 1440p | -0.4% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.1% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 96% | 99% |
| Dec Time[%] | 104% | 98% | 102% |

|  | Complexity |  |  |  |
| --- | --- | --- | --- | --- |
|  | SCM | 3.5 | 3.5 ctx rd | 3.6 |
| # regular bins per BV | 4 | 15 | 14 | 2 |
| # additional contexts | 0 | 17 | 4 | 2 |
| Binarization as in HEVC | Yes | Yes, except the prefix bins are regular bins |  | No |

## Test 4: Block copying modifications

- Test 4.1: Symmetric intra block copy (JCTVC-R0116)

Symmetric intra block copy (SIBC) is tested. When SIBC is applied, the reference block is flipped horizontally or vertically before it is used to predict the current block. SIBC is only allowed for IBC with block size 2Nx2N. A flag is signaled to indicate whether the current block chooses SIBC or normal IBC. If used, another flag indicates horizontal or vertical SIBC. When horizontal SIBC is on, BVDy is not signaled and BVy is inferred to be 0. When vertical SIBC is on, BVDx is not signaled and BVx is inferred to be 0.

- Test 4.2: Block flipping (JCTVC-R0097)

Vertical flipping is enabled for the CU using Intra BC prediction. One flag is signaled to distinguish flipping or not for every PU using Intra BC prediction (except the CU in Intra BC merge mode).

### Results for tests on block copying modifications

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4.1 hor. | 4.1 ver. | 4.1 h.+v. | 4.2 |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.7% | -1.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.4% | -0.7% | -0.8% | -0.1% |
| RGB, mixed content, 1440p | -0.8% | -0.3% | -1.0% | -0.1% |
| RGB, mixed content, 1080p | -0.7% | -0.2% | -0.8% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -0.6% | -1.2% | -0.6% |
| YUV, text & graphics with motion,720p | -0.5% | -0.8% | -0.9% | -0.5% |
| YUV, mixed content, 1440p | -0.8% | -0.4% | -1.0% | -0.2% |
| YUV, mixed content, 1080p | -0.7% | -0.1% | -0.8% | 0.0% |
| YUV, Animation, 720p | -0.1% | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 102% | 102% | 105% |
| Dec Time[%] | 100% | 100% | 99% | 102% |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4.1 hor. | 4.1 ver. | 4.1 h.+v. | 4.2 |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.9% | -0.3% |
| RGB, text & graphics with motion,720p | -0.3% | -0.5% | -0.6% | -0.2% |
| RGB, mixed content, 1440p | -0.4% | -0.2% | -0.4% | 0.0% |
| RGB, mixed content, 1080p | -0.3% | 0.0% | -0.4% | 0.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.4% | -0.7% | -0.2% |
| YUV, text & graphics with motion,720p | -0.3% | -0.6% | -0.6% | -0.3% |
| YUV, mixed content, 1440p | -0.4% | 0.0% | -0.4% | 0.1% |
| YUV, mixed content, 1080p | -0.4% | -0.1% | -0.5% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 99% | 99% |
| Dec Time[%] | 100% | 99% | 99% | 101% |

|  | Low Delay |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4.1 hor. | 4.1 ver. | 4.1 h.+v. | 4.2 |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.5% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.4% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.3% | 0.0% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.5% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% | -0.2% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.5% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.3% | 0.3% |
| YUV, mixed content, 1440p | -0.2% | 0.1% | -0.3% | -0.1% |
| YUV, mixed content, 1080p | 0.3% | 0.0% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 99% | 98% | 99% |
| Dec Time[%] | 101% | 99% | 99% | 101% |

## Test 5: Combinations

- Test 5.1: Combination of tests 2.1 + 2.3 + 2.4 (JCTVC-R0191)

In this test, the methods that are used in tests 2.1, 2.3 and 2.4 are tested in combination. Test A is 2.1 + 2.3 and test B is 2.1 + 2.3 + 2.4.

- Test 5.2: Combination of tests 3.1 + 3.6 (JCTVC-R0187)

In this test, the BV prediction method in test 3.1 and the BV coding method in test 3.6 are combined.

### Results for combination of tests

Note: enc. refers to the integration of the common encoder described for tests 3.1 and 3.3. enc* indicates the encoder with full RD in 3.1.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 5.1A | 5.1B | 5.2 | 5.2 enc | 5.2 enc* |
| RGB, text & graphics with motion, 1080p | -9.3% | -9.9% | -4.4% | -8.6% | -10.9% |
| RGB, text & graphics with motion,720p | -5.2% | -5.7% | -2.1% | -5.0% | -6.2% |
| RGB, mixed content, 1440p | -4.2% | -4.6% | -1.7% | -3.4% | -4.2% |
| RGB, mixed content, 1080p | -4.5% | -4.8% | -2.1% | -4.1% | -4.8% |
| RGB, Animation, 720p | -0.3% | -0.4% | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -9.7% | -10.6% | -5.1% | -9.5% | -12.0% |
| YUV, text & graphics with motion,720p | -5.7% | -6.2% | -2.4% | -5.7% | -6.9% |
| YUV, mixed content, 1440p | -4.1% | -4.5% | -2.1% | -4.0% | -4.8% |
| YUV, mixed content, 1080p | -4.6% | -5.0% | -2.4% | -4.5% | -5.5% |
| YUV, Animation, 720p | -0.2% | -0.5% | 0.0% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% | 166% | 100% | 102% | 115% |
| Dec Time[%] | 94% | 95% | 104% | 103% | 97% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 5.1A | 5.1B | 5.2 | 5.2 enc | 5.2 enc* |
| RGB, text & graphics with motion, 1080p | -5.8% | -6.4% | -2.1% | -4.3% | -6.3% |
| RGB, text & graphics with motion,720p | -3.8% | -4.2% | -1.3% | -3.2% | -4.0% |
| RGB, mixed content, 1440p | -2.6% | -3.0% | -1.0% | -2.0% | -2.6% |
| RGB, mixed content, 1080p | -3.0% | -3.2% | -1.3% | -2.6% | -3.1% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -5.6% | -6.2% | -2.3% | -4.3% | -6.7% |
| YUV, text & graphics with motion,720p | -3.8% | -4.3% | -1.5% | -3.7% | -4.5% |
| YUV, mixed content, 1440p | -2.4% | -2.6% | -1.3% | -2.5% | -2.9% |
| YUV, mixed content, 1080p | -3.2% | -3.5% | -1.7% | -3.1% | -3.8% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 116% | 97% | 102% | 105% |
| Dec Time[%] | 99% | 101% | 100% | 100% | 98% |

|  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 5.1A | 5.1B | 5.2 | 5.2 enc | 5.2 enc* |
| RGB, text & graphics with motion, 1080p | -4.9% | -5.4% | -1.4% | -2.7% | -4.6% |
| RGB, text & graphics with motion,720p | -2.1% | -2.2% | -0.8% | -1.4% | -2.0% |
| RGB, mixed content, 1440p | -2.0% | -2.2% | -0.4% | -1.0% | -1.4% |
| RGB, mixed content, 1080p | -1.7% | -2.0% | -0.6% | -1.4% | -1.9% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.2% | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -4.4% | -4.9% | -1.3% | -2.5% | -4.7% |
| YUV, text & graphics with motion,720p | -1.4% | -1.9% | -0.7% | -1.4% | -1.9% |
| YUV, mixed content, 1440p | -1.4% | -1.5% | -0.8% | -1.2% | -1.9% |
| YUV, mixed content, 1080p | -1.6% | -1.8% | -0.5% | -0.9% | -1.6% |
| YUV, Animation, 720p | 0.3% | 0.1% | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.3% | 0.3% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% | 112% | 102% | 103% | 102% |
| Dec Time[%] | 101% | 101% | 97% | 102% | 105% |