| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0032 |
| --- | --- |

| Title: | SCCE2: Summary Report of HEVC Screen Content Coding Core Experiment 2 on Line-based Intra Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Xiaozhong Xu Li Zhang | Email: | cheerchen.cs98g@g2.nctu.edu.tw xiaozhong.xu@mediatek.com lizhang@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This is a summary report on HEVC Screen Content Coding Core Experiment 2 (SCCE2) on line-based intra copy. Four experiments were investigated in this SCCE: line-based intra copy with 1-D BV, line-based intra copy with 2-D BV, constrained/unconstrained line splitting, and BV merging for line-based intra copy.

# Document List

## SCCE2 Contributions

- JCTVC-R0131, SCCE2: Line-based Intra Copy with 2-D BV and PU-level Splitting (Test B), and its variants with Constrained PU-level Splitting (Test C) and BV Merging (Test D) [T.-S. Chang, C.-C. Chen, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU), T. Lin, L. Zhao (Tongji Univ.), C.-L. Lin, F.-D. Jou (ITRI)]

- JCTVC-R0183, Cross check of SCCE2 Test-B: Line-based Intra Copy with 2-D BV and PU-level Splitting (JCTVC-R0131) [K. Rapaka (Qualcomm)]

- JCTVC-R0194, Cross check of JCTVC-R0131: SCCE2 Test D: Line-based Intra Copy with 2-D BV and PU-level Splitting and BV Merging [J. Ye, X. Xu (MediaTek)]

- JCTVC-R0249, SCCE2: Cross-check of Line-based Intra Copy with 2-D BV and with Constrained PU-level Splitting (Test C) (JCTVC-R0131) [M. Xu, F. Duanmu, Z. Ma, W. Wang, H. Yu (Huawei R&D USA)]

## SCCE2-related Contributions

- JCTVC-R0132, Non-SCCE2: Line-based Intra Copy with Unconstrained 2-D BV Coding for Test C [C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Juo (NCTU/ITRI)]

- JCTVC-R0264, Cross-check of JCTVC-R0132: Non-SCCE2: Line-based Intra Copy with Unconstrained 2-D BV Coding for Test C [J. Ye, X. Xu (MediaTek)]

- JCTVC-R0281, Non-SCCE2: An improvement on Intra Line Copy of SCCE2 [L. Zhao, X. Chen, T. Lin (Tongji Univ.)]

- JCTVC-R0292, Crosscheck of JCTVC-R0281: An improvement on Intra Line Copy of SCCE2 [R.-L. Liao, C.-C. Chen, T.-S. Chang, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

# Test Conditions

All the proposed methods were implemented on top of THE HM-14.0+RExt-7.0+SCM-1.0 software and are conducted using the common test conditions for screen content coding specified in JCTVC-Q1015.

# Test Description and Results

## Test A: Line-based Intra Copy with 1-D BV and CU-level Splitting (Withdrawn)

## Test B: Line-based Intra Copy with 2-D BV and PU-level Splitting

- Proponent: NCTU/ITRI (JCTVC-R0131)

- X-checker: Qualcomm (JCTVC-R0183)

- Description: It is proposed to performing intra copy for lines at PU level with 2-D BV for each line. In addition, a flag is present at PU level to indicate whether the row-wise splitting or the column-wise splitting is in use.

- Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -7.1% | -7.1% | -7.1% | -4.1% | -4.1% | -4.2% | -2.6% | -2.6% | -2.5% |
| RGB, text & graphics w/ motion,720p | -3.2% | -3.1% | -3.1% | -2.4% | -2.2% | -2.4% | -1.5% | -1.4% | -1.4% |
| RGB, mixed content, 1440p | -2.9% | -2.9% | -2.9% | -2.2% | -2.0% | -2.1% | -1.3% | -1.4% | -1.2% |
| RGB, mixed content, 1080p | -3.6% | -3.8% | -3.9% | -2.6% | -2.8% | -2.7% | -1.2% | -1.3% | -1.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.7% | -6.6% | -6.6% | -3.7% | -4.0% | -3.9% | -2.2% | -2.3% | -2.4% |
| YUV, text & graphics w/ motion,720p | -2.6% | -2.9% | -3.4% | -2.0% | -2.3% | -2.5% | -0.7% | -1.3% | -1.5% |
| YUV, mixed content, 1440p | -2.3% | -2.7% | -2.9% | -1.4% | -2.1% | -2.3% | -0.6% | -1.6% | -1.9% |
| YUV, mixed content, 1080p | -3.2% | -3.9% | -3.7% | -2.4% | -3.4% | -3.1% | -0.7% | -2.4% | -0.8% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 115% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 102% |  |  |

- Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 6.6% | 4.3% | 10.4% | 4.1% | 1.7% | 7.6% | 3.0% | 1.3% | 5.0% |
| RGB, text & graphics w/ motion,720p | 1.8% | 0.5% | 3.3% | 1.0% | 0.3% | 2.8% | 0.7% | 0.2% | 1.9% |
| RGB, mixed content, 1440p | 1.0% | 0.6% | 1.4% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 6.6% | 4.3% | 10.3% | 4.2% | 2.0% | 7.7% | 3.1% | 1.5% | 5.3% |
| YUV, text & graphics w/ motion,720p | 2.1% | 0.4% | 4.8% | 1.3% | 0.3% | 4.1% | 0.9% | 0.2% | 2.7% |
| YUV, mixed content, 1440p | 1.0% | 0.5% | 1.4% | 0.2% | 0.1% | 0.3% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |

## Test C: Test B + Constrained PU-level Splitting

- Proponent: NCTU/ITRI and Tongji University (JCTVC-R0131)

- X-checker: Huawei (JCTVC-R0249)

- Description: This test imposes an additional constraint on Test B, which requires that all the PUs within a CU use the same form of line splitting, row-wise or column-wise. Only one flag needs to be signaled at the CU level to indicate which mode is in use.

- Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -7.0% | -7.0% | -6.9% | -4.1% | -4.0% | -4.0% | -2.5% | -2.4% | -2.3% |
| RGB, text & graphics w/ motion,720p | -3.2% | -3.2% | -3.2% | -2.3% | -2.2% | -2.4% | -1.5% | -1.4% | -1.5% |
| RGB, mixed content, 1440p | -2.8% | -2.7% | -2.8% | -2.1% | -1.9% | -1.9% | -1.1% | -1.3% | -1.1% |
| RGB, mixed content, 1080p | -3.7% | -3.8% | -3.8% | -2.6% | -2.7% | -2.7% | -1.3% | -1.3% | -1.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.6% | -6.5% | -6.5% | -3.6% | -3.9% | -3.9% | -2.2% | -2.4% | -2.5% |
| YUV, text & graphics w/ motion,720p | -2.5% | -3.0% | -3.3% | -2.1% | -2.6% | -2.6% | -0.9% | -1.1% | -1.8% |
| YUV, mixed content, 1440p | -2.2% | -2.7% | -2.9% | -1.3% | -2.4% | -2.1% | -0.7% | -1.7% | -1.8% |
| YUV, mixed content, 1080p | -3.2% | -3.9% | -3.7% | -2.4% | -3.1% | -3.0% | -0.5% | -2.0% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.5% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 113% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  | 103% |  |  |

- Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 6.5% | 4.3% | 10.2% | 4.0% | 1.7% | 7.4% | 3.0% | 1.3% | 4.9% |
| RGB, text & graphics w/ motion,720p | 1.7% | 0.5% | 3.2% | 1.0% | 0.3% | 2.7% | 0.7% | 0.2% | 1.8% |
| RGB, mixed content, 1440p | 1.0% | 0.6% | 1.4% | 0.2% | 0.1% | 0.4% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 6.5% | 4.3% | 9.9% | 4.1% | 2.0% | 7.5% | 3.1% | 1.6% | 5.1% |
| YUV, text & graphics w/ motion,720p | 2.1% | 0.4% | 4.6% | 1.3% | 0.3% | 3.9% | 0.9% | 0.2% | 2.6% |
| YUV, mixed content, 1440p | 1.0% | 0.5% | 1.4% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 121% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 104% |  |  | 102% |  |  |

## Test D: Test C + Block Vector Merging

- Proponent: NCTU/ITRI (JCTVC-R0131)

- X-checker: MediaTek (JCTVC-R0194)

- Description: To reduce BV overhead for Test C, this test reuses one previously decoded BV for representing the BV of the current line, and no block vector difference needs to be signaled. A flag is present for each line to switch adaptively between BV merging and BV difference coding.

- Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -6.9% | -6.9% | -6.9% | -3.9% | -3.9% | -3.9% | -2.3% | -2.3% | -2.1% |
| RGB, text & graphics w/ motion,720p | -3.2% | -3.2% | -3.3% | -2.4% | -2.3% | -2.3% | -1.6% | -1.4% | -1.5% |
| RGB, mixed content, 1440p | -2.7% | -2.6% | -2.7% | -1.9% | -1.8% | -1.9% | -1.1% | -1.1% | -1.0% |
| RGB, mixed content, 1080p | -3.6% | -3.6% | -3.7% | -2.6% | -2.6% | -2.7% | -1.2% | -1.3% | -1.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.6% | -6.6% | -6.5% | -3.6% | -3.8% | -3.8% | -2.1% | -2.3% | -2.3% |
| YUV, text & graphics w/ motion,720p | -2.7% | -3.0% | -3.3% | -2.0% | -2.4% | -2.5% | -0.8% | -1.2% | -1.8% |
| YUV, mixed content, 1440p | -2.1% | -2.6% | -2.8% | -1.3% | -2.1% | -2.2% | -0.6% | -1.4% | -1.5% |
| YUV, mixed content, 1080p | -3.0% | -3.7% | -3.6% | -2.3% | -3.1% | -3.1% | -0.6% | -2.2% | -0.7% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.2% | 0.0% | -0.2% | 0.1% | -0.1% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 115% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  | 101% |  |  |

- Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 7.3% | 4.9% | 11.2% | 4.6% | 2.0% | 8.3% | 3.4% | 1.4% | 5.6% |
| RGB, text & graphics w/ motion,720p | 2.0% | 0.5% | 3.8% | 1.2% | 0.3% | 3.2% | 0.8% | 0.3% | 2.1% |
| RGB, mixed content, 1440p | 1.1% | 0.6% | 1.6% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 7.4% | 5.0% | 11.1% | 4.7% | 2.3% | 8.5% | 3.6% | 1.8% | 5.9% |
| YUV, text & graphics w/ motion,720p | 2.4% | 0.5% | 5.5% | 1.5% | 0.3% | 4.7% | 1.0% | 0.2% | 2.9% |
| YUV, mixed content, 1440p | 1.1% | 0.5% | 1.6% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 117% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  | 102% |  |  |